3GPP TSG RAN WG1 Meeting #103-e		R1-200XXXX

e-Meeting, October 26th – November 13th, 2020

Title:	Session notes for 8.10

(Enhancements to Integrated Access and Backhaul)

Source:	Ad-hoc chair (Samsung)

## Enhancements to Integrated Access and Backhaul

Please refer to RP-201293 for detailed scope of the WI

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2008858	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2009269	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2009301	Resource multiplexing and DC in enhanced IAB	Ericsson

R1-2007594	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2007684	Enhancement to resource multiplexing between child and parent links	vivo

R1-2008029	Discussion on resource multiplexing in the simultaneous operation	CMCC

R1-2008184	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2008312	Enhancements for IAB resource multiplexing	AT&T

R1-2008406	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2008863	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2008995	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2009018	Enhancements to resource multiplexing between child and parent links of an IAB node	ETRI

Withdrawn

R1-2009108	Enhancements to resource multiplexing for IAB	Lenovo, Motorola Mobility

R1-2009190	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2009220	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2009221	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

[103-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

R1-2007595	Enhancements for simultaneous operation of MT and DU	Huawei, HiSilicon

R1-2008407	Discussions on other enhancements for simultaneous operation	LG Electronics

R1-2009109	Enhancements for simultaneous operation in IAB systems	Lenovo, Motorola Mobility

R1-2007685	Other enhancements for simultaneous operation of child and parent links	vivo

R1-2007786	Other enhancements for simultaneous operation of IAB-node’s child and parent links	Fujitsu

R1-2008030	Discussion on the enhancement for simultaneous operation	CMCC

R1-2008185	Enhancements to Timing, Power Control and CLI for NR IAB	Samsung

R1-2008313	Enhancements for IAB interference management	AT&T

R1-2008816	Discussion on simultaneous operation of IAB-node’s child and parent links	CEWiT

R1-2008859	Enhancements for simultaneous operation of child and parent links	ZTE, Sanechips

R1-2008864	Other enhancements for simultaneous operation of IAB Parent and Child	Nokia, Nokia Shanghai Bell

R1-2008996	Power Control Enhancements for Simultaneous Operations	Intel Corporation

R1-2009019	Discussion on simultaneous operation enhancements	ETRI

R1-2009137	Power Control: Subsequent Contribution	Sharp

R1-2009191	Other enhancements for simultaneous operation of IAB-node’s child and parent links	NTT DOCOMO, INC.

R1-2009270	On enhancements for simultaneous operation of IAB-node's child and parent links	Qualcomm Incorporated

R1-2009302	Timing, CLI and power control in enhanced IAB	Ericsson

[103-e-NR-eIAB-02] Email discussion non other enhancements for simultaneous operation of IAB-node’s child and parent links – Luca (Qualcomm)

- 1st check point: 11/5

- 2nd check point: 11/10

- 3rd check point: 11/12

9567

Agreement

Select one or both of the following modes of operation for Case 7 timing in RAN1#104-e:

- symbol level alignment without slot level alignment

- slot level alignment

FL Proposal 2.2.v2:

Case 1, Case 6 and Case 7 timing modes are not restricted to specific multiplexing modes supported in Rel-17.

FFS: Details on how the timing modes are specified for specific multiplexing modes

FL Proposal 2.3.v2

An IAB-node operating in Case 6 timing mode can rely on the OTA timing synchronization mechanism defined in Rel-16 (based on TA and T_delta) to set its DL Tx timing.

- FFS any required change to the range of T_delta

### Others

R1-2007686	Other enhancements to Rel-17 IAB nodes	vivo

R1-2008314	Performance of simultaneous operation of IAB-node’s child and parent links	AT&T

R1-2008327	Other enhancements for IAB	Huawei, HiSilicon

R1-2008408	Discussion on RACH procedure for IAB enhancement	LG Electronics

R1-2008860	Discussion on multiplexing capability for IAB	ZTE, Sanechips

R1-2009020	Other enhancements to IAB	ETRI

R1-2009303	Refined beam calibration in enhanced IAB	Ericsson