3GPP TSG RAN WG1 #104-e                                             R1-2100474

e-Meeting, January 25th – February 5th, 2021

Source:	vivo

Title:	Discussion on joint scheduling

Agenda Item:	8.13.2

Document for:	Discussion and Decision

- Introduction

Regarding the study of scheduling PDSCH on multiple cells using a single DCI in R17 DSS WID [1], the following agreements on evaluation assumption have been made [2]:

Agreements: Further study with below simulation assumptions: Simulation scenarios: - For two-cell scheduling via a single DCI, PDCCH transmitted on a first cell schedules one PDSCH on the first cell and another PDSCH on a second cell. - For single-cell scheduling (baseline), one PDCCH transmitted on a first cell schedules one PDSCH on the first cell via self-scheduling and another PDCCH transmitted on the first cell schedules another PDSCH on a second cell via cross-carrier scheduling. - Companies can optionally compare to the case of PDCCH transmitted on each of the two cells via self-scheduling. In this case, company should provide details on how to calculate the PDCCH blocking rate. Simulation assumptions on carrier frequency, SCS, antenna configuration, carrier bandwidth as well as CORESET configuration - Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs - Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs - [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs] - [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs] Payload size of two-cell scheduling DCI (excluding CRC): - 60 for single-cell scheduling DCI (baseline). - 72/84/96/108 for two-cell scheduling DCI. - Companies are encouraged to report how the values are obtained, e.g., via separate or shared fields in DCI format. Target BLER for two-cell scheduling DCI: 1% (baseline), 0.5%(optional) Regarding the CCE-to-REG mapping, based on the agreed interleaved CCE-to-REG mapping, whether to adopt non-interleaved CCE-to-REG mapping is up to the proponent. Agreements: - Further study with below simulation assumptions: Table 2: System level simulation assumptions Parameters; Values. Carrier frequency; For scheduling cell, follow agreed link level simulation assumptions For scheduled cell, consider 700MHz/2GHz with 10/20MHz BW (LTE overhead on DSS carrier can be optionally provided, up to proponent). SCS. Simulation bandwidth. BS antenna height; 25 m. UE height; 1.5m. TRP transmit power; 46 dBm for 10MHz. Scenario; Urban Macro. ISD; 500m. TRP antenna configuration; (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 700MHz (M,N,P,Mg,Ng;Mp,Np)= (2,8,2,1,1;1,1) for 2GHz (M,N,P,Mg,Ng;Mp,Np)= (8,4,2,1,1;1,1) for 4GHz. UE antenna configuration; (M,N,P,Mg,Ng;Mp,Np)= (1,1,2,1,1;1,1) for 700MHz/2GHz (M,N,P,Mg,Ng;Mp,Np)= (1,2,2,1,1;1,1) for 4GHz. Device deployment; 80% indoor, 20% outdoor. UE speeds of interest; Indoor users: 3km/h. Outdoor users (in-car): 30 km/h. BS noise figure; 5 dB. BS antenna element gain; 8 dBi. UE noise figure; 9 dB. Thermal noise level; -174 dBm/Hz. Traffic; Full Buffer(baseline), FTP model 1 or 3 up to company. Macro sites; 19. Number of UEs per cell; 10/15/20 UEs. Downtilt; 102°. Minimum BS to UE distance; 35m.

In the contribution, we provide our views and simulation results on scheduling PDSCH on two cells with a single DCI.

- Evaluation

In this section, the evaluation results are provided for the following scenarios:

- Baseline for comparison: self-scheduling+cross-carrier-scheduling

  - Scheduling the scheduling cell and another cell by separate single-cell-DCIs

  - 100% UE are scheduled by single-cell-DCIs

- Scenario of multi-cell scheduling for evaluation:

  - Scheduling the scheduling cell and another cell simultaneously by a joint-DCI

  - 100% UE are scheduled by joint-DCI

- Combinations of SCS/BW/CORESET config according to the previous agreement

  - Combination 1

    - Scheduling cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW, 2-symbol CORESET with 96RBs

    - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

  - Combination 2

    - Scheduling cell: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz carrier BW, 1-symbol CORESET with 270RBs

    - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

  - Combination 3

    - Scheduling cell: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW, 3-symbol CORESET with 48RBs

    - Scheduled cell: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz carrier BW

  - Combination 4

    - Scheduling cell: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz carrier BW, 2-symbol CORESET with 96RBs

    - Scheduled cell: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz carrier BW

Figure 1. scenarios for comparison between single scheduling and multi-cell scheduling

Several aspects including CCE saving, PDCCH blocking, throughput, and scheduling flexibility are analysed. In the LLS simulation, we assume that the payload size of a single-cell-DCI that schedules a single serving cell is 60bits (excluding 24bits CRC), and the payload size of joint-DCI is 60/72/84/96/108 bits (excluding 24bits CRC) respectively, corresponding to a 50/40/30/20/10% DCI information bits compression rate compared with using 2 single-cell-DCIs.

- CCE saving

The following tables provide the required SNR to achieve 1% BLER as well as the corresponding ALs. Based on the SNR distribution CDF provided in the appendix, the distribution of UEs for each AL in the evaluated deployment is also provided. On top of it, the average number of CCEs occupied by a single-cell-DCI or a joint-DCI can be derived.

It can be observed that when the size of a joint-DCI (excluding CRC) is no larger than 90% of the total size of the two single-cell-DCIs, it saves more than 33.09%/28.13%/32.59%/18.14% CCE in combination 1/2/3/4 respectively. When the compression ratio increases, the amount of CCE saved becomes more significant.

Table 1. Required SNR and the distribution of each AL @ combination1

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.64 | 23 | 12.64 | 23 | 14.35 | 20 | 17.96 | 15 | / | / | / | / |
| AL=2 | 4.96 | 27 | 4.96 | 27 | 6.23 | 24 | 7.22 | 25 | 8.109 | 37 | 8.856 | 34 |
| AL=4 | 0.4998 | 24 | 0.4998 | 24 | 1.032 | 27 | 1.515 | 28 | 2.068 | 28 | 2.547 | 29 |
| AL=8 | -2.873 | 19 | -2.873 | 19 | -2.425 | 20 | -1.889 | 21 | -1.526 | 22 | -1.191 | 22 |
| AL=16 | -5.816 | 6 | -5.816 | 6 | -5.384 | 7 | -5.029 | 9 | -4.694 | 10 | -4.335 | 12 |
| Average No of CCEs | 4.25*2 |  | 4.25 |  | 4.57 |  | 4.99 |  | 5.38 |  | 5.69 |  |
| Saving ratio | / |  | 50% |  | 46.25% |  | 41.33% |  | 36.73% |  | 33.09% |  |

Table 2. Required SNR and the distribution of each AL @ combination2

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 6.169 | 43 | 6.169 | 43 | 7.662 | 38 | 10.73 | 30 | / | / | / | / |
| AL=2 | 1.258 | 23 | 1.258 | 23 | 1.969 | 23 | 2.341 | 29 | 3.012 | 56 | 3.594 | 53 |
| AL=4 | -2.123 | 19 | -2.123 | 19 | -1.695 | 22 | -1.26 | 23 | -0.8911 | 24 | -0.4904 | 24 |
| AL=8 | -4.927 | 8 | -4.927 | 8 | -4.57 | 10 | -4.139 | 10 | -3.845 | 11 | -3.531 | 12 |
| AL=16 | -7.14 | 3 | -7.14 | 3 | -6.798 | 2 | -6.442 | 3 | -6.177 | 4 | -5.882 | 6 |
| Average No of CCEs | 2.89*2 |  | 2.89 |  | 2.99 |  | 3.24 |  | 3.79 |  | 4.15 |  |  |
| Saving ratio | / |  | 50% |  | 48.20% |  | 43.82% |  | 34.33% |  | 28.13% |  |  |

Table 3. Required SNR and the distribution of each AL @ combination3

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 12.26 | 20 | 12.26 | 20 | 14.02 | 17 | 17.05 | 13 | / | / | / | / |
| AL=2 | 5.064 | 21 | 5.064 | 21 | 6.337 | 20 | 7.097 | 22 | 7.822 | 32 | 8.929 | 28 |
| AL=4 | 0.1091 | 30 | 0.1091 | 30 | 0.6573 | 31 | 1.27 | 28 | 1.777 | 27 | 2.357 | 27 |
| AL=8 | -3.042 | 21 | -3.042 | 21 | -2.522 | 21 | -2.05 | 23 | -1.663 | 25 | -1.276 | 27 |
| AL=16 | -5.872 | 7 | -5.872 | 7 | -5.405 | 10 | -4.964 | 12 | -4.642 | 13 | -4.253 | 14 |
| Average No of CCEs | 4.67*2 |  | 4.67 |  | 5.14 |  | 5.56 |  | 5.98 |  | 6.29 |  |
| Saving ratio | / |  | 50% |  | 44.91% |  | 40.42% |  | 35.94% |  | 32.59% |  |

Table 4. Required SNR and the distribution of each AL @ combination4

|  | baseline |  | Joint-DCI |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 60 bits |  | 60 bits |  | 72 bits |  | 84 bits |  | 96 bits |  | 108 bits |  |  |
| AL | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % | SNR | % |
| AL=1 | 5.785 | 44 | 5.785 | 44 | 7.176 | 39 | 10.08 | 31 | / | 0 | / | 0 |
| AL=2 | 0.4598 | 25 | 0.4598 | 25 | 1.294 | 25 | 1.87 | 29 | 2.5 | 57 | 3.003 | 55 |
| AL=4 | -2.662 | 17 | -2.662 | 17 | -2.22 | 20 | -1.791 | 22 | -1.388 | 23 | 0.9966 | 11 |
| AL=8 | -5.62 | 8 | -5.62 | 8 | -5.258 | 9 | -4.819 | 10 | -4.528 | 12 | -4.217 | 25 |
| AL=16 | -7.973 | 2 | -7.973 | 2 | -7.607 | 3 | -7.262 | 3 | -6.961 | 3 | -6.686 | 4 |
| Average No of CCEs | 2.69*2 |  | 2.69 |  | 3.01 |  | 3.21 |  | 3.68 |  | 4.40 |  |  |
| Saving ratio | / |  | 50% |  | 43.99% |  | 40.27% |  | 31.46% |  | 18.14% |  |  |

Observation 1. Compared with using single-cell-DCI, a joint-DCI scheduling two PDSCHs on two cells brings more than 
-  33.09% CCE saving for combination 1, 
-  28.13% CCE saving for combination 2,
-  32.59% CCE saving for combination 3, 
-  18.14% CCE saving for combination 4, 
and the CCE saving gain becomes more significant if the compression rate of joint-DCI size increases.

- PDCCH blocking

When the capacity of PDCCH is limited, conflicts between PDCCHs of different UEs will inevitably occur, resulting in some UEs not being able to be scheduled, as well as some PDSCH resources not being fully utilized. We evaluated the PDCCH blocking rate of the following schemes under different assumptions of CORESET bandwidth and the above AL distribution.

  - Scheme0: two single-cell-DCI, size=60 bits

  - Scheme1: joint-DCI, size=60 bits

  - Scheme2: joint-DCI, size=72 bits

  - Scheme3: joint-DCI, size=84 bits

  - Scheme4: joint-DCI, size=96 bits

  - Scheme5: joint-DCI, size=108 bits

Figure 2, Figure 3, and Figure 4 illustrate the trends of PDCCH blocking rate for scheme0/1/2/3/4/5 with different CORESET bandwidths. It can be observed that: for a given CORESET with the bandwidth of 96/270/48/96RB, the PDCCH blocking rate is reduced significantly if joint-DCI is used. More specifically, compared with using two single-cell-DCI, even with a joint-DCI size of up to 108 bits,

- joint-DCI brings about 8.58%/5.2%/9.07%/2.37% reduction in PDCCH blocking rate for combination 1/2/3/4 respectively when the number of UE is 10

- joint-DCI brings about 8.84%/7.01%/7.9%/4.91% reduction in PDCCH blocking rate for combination 1/2/3/4 respectively when the number of UE is 15

- joint-DCI brings about 8.22%/8.05%/6.49%/5.66% reduction in PDCCH blocking rate for combination 1/2/3/4 respectively when the number of UE is 20

Observation 2. With the same CORESET bandwidth, a joint-DCI with size (excluding CRC) =108 bits brings around
-  8.22%~8.84% reduction in PDCCH blocking rate for combination1, 
-  5.2%~8.05% reduction in PDCCH blocking rate for combination2,
-  6.49%~9.07% reduction in PDCCH blocking rate for combination3,
-  2.37%~5.66% reduction in PDCCH blocking rate for combination4
compared with using single-cell-DCI, and the reduction in PDCCH blocking rate becomes more significant if the joint-DCI size decreases.

| Figure 2. PDCCH blocking rate when No of scheduled UEs=10 |  |
| --- | --- |
| Figure 3. PDCCH blocking rate when No of scheduled UEs=15 |  |
| Figure 4. PDCCH blocking rate when No of scheduled UEs=20 |  |

- Throughput

The throughput for single-cell-DCI and joint-DCI is evaluated based on the results of PDCCH blocking rate. The CORESET BW required to achieve the same level of blocking rate as scheme0 for scheme1/2/3/4/5 can be derived and concluded in Table 5, Table 6, Table 7, and Table 8. It can be seen that scheme1/2/3/4/5 requires less CORESET RB than scheme0 to achieve the same blocking rate, while the number of UE has a minor impact on the CORESET RB saving. In other words, scheme1/2/3/4/5 require fewer CORESET RBs while providing the same scheduling opportunities as scheme0. Then gNB can provide a CORESET with less PRBs for joint-DCI scheduling than single-cell-DCI scheduling. These saved RBs can be reused for PDSCH to achieve higher throughput.

Observation 3. Joint-DCI requires fewer CORESET RBs to achieve the same scheduling opportunities as single-cell-DCI, thus gNB can provide a CORESET with less PRBs for joint-DCI scheduling than single-cell-DCI scheduling. These saved RBs can be reused for PDSCH to improve throughput.

For example, scheme5 saves 24 RB per CORESET symbol in combination 1, which means it has 24*2=48 RBs for PDSCH than scheme0, thus can bring 48/(2*106*14-96*2)=1.73%. gain compared to single-cell DCI scheduling (note: we assume that PDSCH rate matches around the CORESET, since there are two carriers with BW=20MHz, thus the total resource for PDSCH in scheme0 are 2*106*14-96*2=2776 RBs). It should be understood as a theoretical throughput improvement ratio assuming the same MCS for each RB. We summarized the theoretical throughput gain achieved by scheme1/2/3/4/5 in the following tables.

Table 5. CORESET BW required for different schemes with the same blocking rate@ combination1

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 72 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.73% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 72 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.73% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 66 | 69 |
| Throughput gain | / | 3.24% | 3.03% | 2.59% | 2.16% | 1.95% |

Table 6. CORESET BW required for different schemes with the same blocking rate@ combination2

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 270 | 138 | 156 | 168 | 192 | 216 |
| Throughput gain | / | 3.09% | 2.67% | 2.39% | 1.83% | 1.27% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 270 | 132 | 156 | 168 | 198 | 228 |
| Throughput gain | / | 3.23% | 2.67% | 2.39% | 1.69% | 0.98% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 270 | 138 | 150 | 168 | 204 | 228 |
| Throughput gain | / | 3.09% | 2.81% | 2.39% | 1.55% | 0.98% |

Table 7. CORESET BW required for different schemes with the same blocking rate@ combination3

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 48 | 32 | 32 | 32 | 36 | 36 |
| Throughput gain | / | 3.66% | 3.66% | 3.66% | 2.74% | 2.74% |

Table 8. CORESET BW required for different schemes with the same blocking rate@ combination4

| N_UE=10 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| --- | --- | --- | --- | --- | --- | --- |
| CORESET BW(RB) per symbol | 96 | 48 | 57 | 60 | 72 | 90 |
| Throughput gain | / | 4.79% | 3.89% | 3.59% | 2.39% | 0.60% |
| N_UE=15 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 78 | 84 |
| Throughput gain | / | 4.49% | 4.19% | 3.59% | 1.79% | 1.20% |
| N_UE=20 | Scheme0 | Scheme1 | Scheme2 | Scheme3 | Scheme4 | Scheme5 |
| CORESET BW(RB) per symbol | 96 | 51 | 54 | 60 | 78 | 84 |
| Throughput gain | / | 4.49% | 4.19% | 3.59% | 1.79% | 1.20% |

It can be observed that, if joint-DCI size is 96bits~108 bits, it can bring

- <=2.16% theoretical throughput gain compared with single-cell-DCI for combiantion1

- <=1.83% theoretical throughput gain compared with single-cell-DCI for combiantion2

- <=2.74% theoretical throughput gain compared with single-cell-DCI for combiantion3

- <=2.39% theoretical throughput gain compared with single-cell-DCI for combiantion4

Observation 4. Compared with using single-cell-DCI, joint-DCI brings around
-  24~27 RB reduction in CORESET BW and <=3.24% theoretical throughput gain for combination1, 
-  42~54 RB reduction in CORESET BW and <=3.32% theoretical throughput gain for combination2,
-  12~16 RB reduction in CORESET BW and <=3.66% theoretical throughput gain for combination3,
-  6~12 RB reduction in CORESET BW and <=4.79% theoretical throughput gain for combination4.

If the joint DCI size is large enough (i.e., 108bit) to provide independent resource allocation fields for each carrier as well as high scheduling flexibility, the corresponding observed throughput gain approximates the theoretical throughput gain. However, further compression of resource allocation fields in joint-DCI may be unavoidable for the small DCI size, the scheduling flexibility would be inevitably restricted – for instance, the granularity of frequency-domain scheduling resources (i.e., RBG size) becomes coarser in joint-DCI compared to single-cell-DCI, which may affect the spectrum efficiency adversely.

For example, in combination 4, assuming that the RBG size for single-cell DCI scheduling is 8 RB, to compress the joint DCI size to 96 bits, the RBG size for joint-DCI scheduling needs to be increased to be greater than 8 RB (i.e., 9 RB or larger). While for the intra-band cases of having the same numerology and same bandwidth (i.e., combination 1/3), the RBG size can remain the same as the single-cell DCI scheduling. For combination 2, the RBG is set to 16 RB for single-cell-DCI scheduling since the bandwidth of the scheduling cell is as large as 273 PRB, so the RBG size in the joint-DCI is set to 16RB as well. If the joint DCI size is reduced further, for example to 84bits, then the RBG size may need to be increased for all combinations.

Table 9. RBG size assumption for evaluation

|  | Single-cell-DCI | Scheme4(96bits) | Scheme5(108bits) |
| --- | --- | --- | --- |
| Combiantion1 | 8RB | 8RB | 8RB |
| Combiantion2 | 16RB | 16RB | 16RB |
| Combiantion3 | 4RB | 4RB | 4RB |
| Combiantion4 | 8RB | 9RB | 8RB |

We performed SLS based on this scheduling restriction (i.e., the determined RBG size) and the overhead reduction gain (i.e., the required CORESET BW in the above Tables). The throughput results of the joint-DCI are illustrated in Figure 5, Figure 6, and Figure 7, respectively for different number of UEs.

The gain achieved with the combination 1/2/3 is close to or even higher than the theoretical throughput gain. This is because in the above theoretical throughput analysis we have assumed the same MCS for each PRB, while in the SLS the MCS used for each schedule is related to the channel conditions, thus introducing some fluctuations.

It can be seen that the practical gain obtained in combination4 is much lower than the theoretical throughput gain. Moreover, the joint-DCI even brings 0.2%/0.31% performance loss if the number of UEs increases to 15 or 20 for combiantion4. The reason for this loss is that throughput gains from the resource savings of joint-DCI scheduling cannot compensate for the performance loss due to the increased scheduling granularity.

Observation 5. Joint-DCI with size=96bits~108 bits can bring
-  <=2.44% throughput gain compared with single-cell-DCI for combiantion1 in practical scenarios
-  <=2.32% throughput gain compared with single-cell-DCI for combiantion2 in practical scenarios
-  <=3.12% throughput gain compared with single-cell-DCI for combiantion3 in practical scenarios
compared with single-cell-DCI.

Observation 6. When the number of UE is 10, joint-DCI with size=96bits~108 bits can bring <=1.42% throughput gain compared with single-cell-DCI for combiantion4 in practical scenarios. However, it can also bring 0.2%~0.31% throughput loss if the number of UE increases to 15/20.

Figure 5. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=10

Figure 6. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=15

Figure 7. Throughput gain achieved by scheme4/5(96 bits/108bits) when No of UE=20

In summary, multi-cell scheduling could bring throughput gain thanks to PDCCH overhead reduction and PDCCH blocking rate reduction. However, the throughput gain may not be significant in practical due to potential scheduling restrictions (such as the increase of RBG size as shown on the above evaluation and analysis). How to make the trade-off to achieve maximum throughput gain in a typical scenario should be studied.

Proposal 1. The design of joint-DCI should achieve a good trade-off between system capacity improvement due to CCE saving/PDCCH blocking rate reduction and the spectrum efficiency loss due to the coarser scheduling granularity and degraded scheduling flexibility.

- Potential issues

As shown in the above evaluation, DCI size compression is important to achieve resource-saving and thus system throughput improvement. However, how to compress the DCI payload size for multi-cell scheduling of different TBs is a challenging task. On one side, in R15 and R16, most of the information fields in a DCI depend on the configurations of the scheduled cell, so an information field cannot be directly shared between the cells, especially if there is a big difference between the configurations of the two cells. An example is in Figure 8, CC#1 and CC#2 have different SCS, TDD configuration, and bandwidth, it would be hard for the two cells to share all fields. On the other side, if each functionality has two separate information fields for the cells in the DCI of multi-cell scheduling, the DCI payload size, as well as the number of CCE consumed, may double, which does not reduce the PDCCH overhead overall and PDCCH detection reliability is also impacted.

Figure 8. Example of multi-cell scheduling

It is preferable not to share the fields that heavily depend on the configuration of the scheduled cell and the fields that directly affect the transmission mechanism such as MCS. The fields that do not depend on the scheduled cells, such as MIMO related information and PUCCH-related information that is mainly associated with PUCCH Pcell, can be considered to be shared between two cells.

In the following table, we listed some major information fields and provides a preliminary analysis of the field type for combination1. The legacy-DCI is 60 bits while the joint-DCI based on Table 10 is 96 bits. In this case, the compression ratio of joint-DCI is around 20% which means it achieves less than 2.44% throughput gain. Thereby, some CC-dependent fields other than MIMO and PUCCH related fields should be further jointly coded for the scheduled carries to reduce DCI size.

Table 10. Example of DCI information field type

| Information field | Separate field or shared field | Number of bits |
| --- | --- | --- |
| Identifier for DCI formats – 1 bit | Shared field | 1 |
| Carrier indicator – 0 or 3 bits | Shared field | 3 |
| Bandwidth part indicator – 0, 1 or 2 bits | Separate field | 2 |
| FDRA – 13 bits for 106 RB | Separate field | 26 |
| TDRA – 0, 1, 2, 3 or 4 bits | Separate field | 6 |
| VRB-to-PRB mapping – 1 bit | Shared field | 1 |
| PRB bundling size indicator – 0 or 1 bit | Separate field | 2 |
| Rate matching indicator – 0, 1, or 2 bits | Separate field | 2 |
| ZP CSI-RS trigger – 0, 1, or 2 bits | Separate field | 2 |
| Modulation and coding scheme – 5 bits | Separate field | 10 |
| New data indicator – 1 bit | Separate field | 2 |
| Redundancy version – 2 bits | Separate field | 4 |
| HARQ process number – 4 bits | Separate field | 8 |
| Downlink assignment index – 0,2,4 bits | Shared field | 4 |
| TPC command for scheduled PUCCH – 2 bits | Shared field | 2 |
| PUCCH resource indicator – 3 bits | Shared field | 3 |
| PDSCH-to-HARQ feedback timing indicator – 3 bits | Shared field | 3 |
| Antenna port(s) – 4, 5, or 6 bits | Separate field | 10 |
| Transmission configuration indication 0 or 3 bits | Shared field | 0 |
| SRS request – 2, 3 bits | Separate field | 4 |
| DMRS sequence initialization – 1 bit. | Shared field | 1 |
| Joint-DCI size |  | 96 bits (excluding CRC) |

Proposal 2. Field type (i.e., shared or cell-specific) of each information field in joint-DCI needs to be investigated.

In addition to the field design of joint-DCI mentioned above, the following issues need to be resolved as well.

- The framework of multi-cell scheduling

In cross-carrier scheduling, UE is indicated by RRC IE CrossCarrierSchedulingConfig that the data on this cell is scheduled either by PDCCH in its own cell (i.e. ‘own’ in schedulingCellInfo) or PDCCH on another cell (i.e. ‘other’ in schedulingCellInfo). It is not clear if a cell that can be jointly scheduled is allowed to perform self-scheduling, e.g., UL scheduling. If yes, the design of multi-cell scheduling to some extent is similar to type3 CSS. If not, it should be further clarified that whether the UL grant for the jointly scheduled cell can be located on a cell other than the one on which joint-DCI is received. We need to clarify whether the support of multi-cell scheduling may reuse the current cross-carrier scheduling framework with some changes.

- Whether multi-cell scheduling reuses the framework of type3 CSS or cross-carrier scheduling

- Whether a cell jointly scheduled by another cell can be configured with PDCCH resources and can perform self-scheduling, e.g., for UL transmission

- Whether a joint-DCI monitored for a cell and a legacy-DCI monitored for the same cell should be located on the same serving cell

- Configuration of cell pairs for multi-cell scheduling

It is necessary to further clarify that how to configure the cell pairs for multi-cell scheduling and whether there are any restrictions on choosing cell pairs for multi-cell scheduling. For example, the following aspects need to be clarified:

- Whether there can be more than one cell configured with a joint-DCI

- Whether a Scell can be configured with a joint-DCI

- Whether a cell can jointly schedule other two cells with a joint-DCI

- Whether Pcell can be scheduled by a Scell with a joint-DCI

- SS configuration and DCI format for multi-cell scheduling

Another aspect that needs to study is the DCI format for multi-cell scheduling. If an existing DCI format such as DCI format 1_1 is used for multi-cell scheduling purpose, the DCI for single scheduling case should always perform zero-padding to keep alignment with the DCI size of the case where multi-cell scheduling is enabled, which implies that a large number of redundant bits will be inserted into the DCI format even for single scheduling. In this case, it not only leads to a higher DCI code rate but also leads to worse coverage and worse performance for single scheduling. By contrast, introducing a new DCI format for multi-cell scheduling can ensure a high degree of scheduling flexibility while avoiding negative impacts on the performance of the existing DCI. However, it raises some other issues, such as how to maintain the DCI size budget.

- PDCCH BD budget maintenance for multi-cell scheduling

In R15, the PDCCH BD budget is maintained for each scheduled cell and there is only one scheduled cell corresponding to a legacy-DCI. However, two scheduled cells are corresponding to a joint-DCI in the multi-cell scheduling case, it should be discussed how to maintain the PDCCH BD budget across the scheduled cell pair. For example, whether the two scheduled cells share the same budget, or if they should be handled individually.

- HARQ-ACK reporting for multi-cell scheduling

For single TB on every PDSCH occasion involved in a HARQ-ACK codebook in R15, there is only 1 HARQ-ACK bit generated in response to the corresponding DCI with an associated PUCCH. However, if multi-cell scheduling is supported, there would be more than 1 TB scheduled by a DCI, which may have different outcomes of PDSCH decoding. Thus, some discussions such as whether HARQ-ACK bits corresponding to the jointly scheduled PDSCHs should be multiplexed in the same codebook and how to determine the HARQ-ACK in response to a joint-DCI, as well as some enhancements for the HARQ-ACK codebook construction, i.e., how to guarantee the reliability of HARQ codebook in case of missed joint-DCI, are needed.

Observation 7. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

- Conclusion

In this contribution, we discuss some issues on multi-cell scheduling and have the following observations and proposals:

Observation 1. Compared with using single-cell-DCI, a joint-DCI scheduling two PDSCHs on two cells brings more than 
-  33.09% CCE saving for combination 1, 
-  28.13% CCE saving for combination 2,
-  32.59% CCE saving for combination 3, 
-  18.14% CCE saving for combination 4, 
and the CCE saving gain becomes more significant if the compression rate of joint-DCI size increases.

Observation 2. With the same CORESET bandwidth, a joint-DCI with size (excluding CRC) =108 bits brings around
-  8.22%~8.84% reduction in PDCCH blocking rate for combination1, 
-  5.2%~8.05% reduction in PDCCH blocking rate for combination2,
-  6.49%~9.07% reduction in PDCCH blocking rate for combination3,
-  2.37%~5.66% reduction in PDCCH blocking rate for combination4
compared with using single-cell-DCI, and the reduction in PDCCH blocking rate becomes more significant if the joint-DCI size decreases.

Observation 3. Joint-DCI requires fewer CORESET RBs to achieve the same scheduling opportunities as single-cell-DCI, thus gNB can provide a CORESET with less PRBs for joint-DCI scheduling than single-cell-DCI scheduling. These saved RBs can be reused for PDSCH to improve throughput.

Observation 4. Compared with using single-cell-DCI, joint-DCI brings around
-  24~27 RB reduction in CORESET BW and <=3.24% theoretical throughput gain for combination1, 
-  42~54 RB reduction in CORESET BW and <=3.32% theoretical throughput gain for combination2,
-  12~16 RB reduction in CORESET BW and <=3.66% theoretical throughput gain for combination3,
-  6~12 RB reduction in CORESET BW and <=4.79% theoretical throughput gain for combination4.

Observation 5. Joint-DCI with size=96bits~108 bits can bring
-  <=2.44% throughput gain compared with single-cell-DCI for combiantion1 in practical scenarios
-  <=2.32% throughput gain compared with single-cell-DCI for combiantion2 in practical scenarios
-  <=3.12% throughput gain compared with single-cell-DCI for combiantion3 in practical scenarios
compared with single-cell-DCI.

Observation 6. When the number of UE is 10, joint-DCI with size=96bits~108 bits can bring <=1.42% throughput gain compared with single-cell-DCI for combiantion4 in practical scenarios. However, it can also bring 0.2%~0.31% throughput loss if the number of UE increases to 15/20.

Observation 7. To support multi-cell scheduling, the following issues need to be resolved
-  DCI field design
-  Any restrictions on the scheduled cells to be paired for multi-cell scheduling
-  Framework of multi-cell scheduling
-  Whether to introduce a new DCI format 
-  PDCCH BD budget maintenance if multi-cell scheduling is enabled
-  HARQ-ACK codebook determination if multi-cell scheduling is enabled

Proposal 1. The design of joint-DCI should achieve a good trade-off between system capacity improvement due to CCE saving/PDCCH blocking rate reduction and the spectrum efficiency loss due to the coarser scheduling granularity and degraded scheduling flexibility.

Proposal 2. Field type (i.e., shared or cell-specific) of each information field in joint-DCI needs to be investigated.

- Reference

- RP-193260, “New WID on NR Dynamic spectrum sharing (DSS)”, Ericsson, RAN #86, Dec 2019.

- RAN1#104, e-Meeting, Chairman note

- Appendix

Table 11. LLS Simulation assumptions for PDCCH

| Parameters | Values |
| --- | --- |
| Carrier frequency(scheduling cell) Subcarrier spacing Simulation bandwidth/No. of PRBs Antenna configuration at BS/UE CORESET configuration | • Combination 1: 2 GHz, 15 kHz SCS, 2 Tx, 2 Rx, 20 MHz(106RB) carrier BW, 2-symbol CORESET with 96RBs • Combination 2: 4 GHz, 30 kHz SCS, 4 Tx, 4 Rx, 100 MHz(273RB) carrier BW, 1-symbol CORESET with 270RBs • [Combination 3: 700MHz, 15 kHz SCS, 2 Tx, 2 Rx, 10 MHz(52RB) carrier BW, 3-symbol CORESET with 48RBs] • [Combination 4: 4GHz, 30 kHz SCS, 4 Tx, 4 Rx, 40 MHz(106RB) carrier BW, 2-symbol CORESET with 96RBs] |
| Link-level Channel model | TDL-C |
| TDL correlation level | Low |
| Delay Spread | 300ns |
| UE speed | 3 km/h |
| payload | 60/72/84/96/108 |
| Aggregation level | 1/2/4/8/16 |
| CCE-to-REG mapping | interleaved |
| REG bundle size | 6 |
| Interleaver size | 2 |
| Tx Diversity | One port precoder cycling |
| Target BLER | 1% |
| Modulation | QPSK |
| Channel coding | Polar code |

Table 12. SNR CDF table @ combination1

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -10.2546 | 20 | -0.36453 | 40 | 3.22835 | 60 | 7.45343 | 80 | 14.7788 |
| 1 | -5.53642 | 21 | -0.13111 | 41 | 3.40787 | 61 | 7.69348 | 81 | 15.6094 |
| 2 | -4.74039 | 22 | -0.00364 | 42 | 3.55026 | 62 | 7.89926 | 82 | 16.0546 |
| 3 | -4.16097 | 23 | 0.154634 | 43 | 3.71084 | 63 | 8.15225 | 83 | 16.5229 |
| 4 | -3.6152 | 24 | 0.316127 | 44 | 3.86172 | 64 | 8.39473 | 84 | 17.4413 |
| 5 | -3.33601 | 25 | 0.465781 | 45 | 4.10461 | 65 | 8.71262 | 85 | 18.0229 |
| 6 | -2.94179 | 26 | 0.687305 | 46 | 4.34906 | 66 | 8.93064 | 86 | 18.7217 |
| 7 | -2.68084 | 27 | 0.790504 | 47 | 4.46795 | 67 | 9.20377 | 87 | 19.0812 |
| 8 | -2.46978 | 28 | 1.02582 | 48 | 4.63613 | 68 | 9.46371 | 88 | 20.0456 |
| 9 | -2.23169 | 29 | 1.1432 | 49 | 4.8972 | 69 | 9.76022 | 89 | 21.0523 |
| 10 | -1.9535 | 30 | 1.33747 | 50 | 5.10365 | 70 | 10.0948 | 90 | 21.7823 |
| 11 | -1.77057 | 31 | 1.51264 | 51 | 5.35755 | 71 | 10.3657 | 91 | 22.9945 |
| 12 | -1.63586 | 32 | 1.72767 | 52 | 5.61563 | 72 | 10.7204 | 92 | 23.8569 |
| 13 | -1.5181 | 33 | 1.86436 | 53 | 5.76374 | 73 | 11.0177 | 93 | 25.4977 |
| 14 | -1.32022 | 34 | 2.04097 | 54 | 5.89568 | 74 | 11.3839 | 94 | 26.4652 |
| 15 | -1.1529 | 35 | 2.29597 | 55 | 6.14152 | 75 | 11.8047 | 95 | 28.0576 |
| 16 | -0.9492 | 36 | 2.47988 | 56 | 6.32072 | 76 | 12.2188 | 96 | 29.2536 |
| 17 | -0.81625 | 37 | 2.63678 | 57 | 6.62136 | 77 | 12.9898 | 97 | 30.9672 |
| 18 | -0.67884 | 38 | 2.84109 | 58 | 6.90661 | 78 | 13.577 | 98 | 34.7759 |
| 19 | -0.49287 | 39 | 3.05602 | 59 | 7.14573 | 79 | 14.1688 | 99 | 38.0076 |
|  |  |  |  |  |  |  |  | 100 | 49.618 |

Figure 9. SNR CDF@combination1

Table 13. SNR CDF table @ combination2

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -24.1586 | 20 | -0.86392 | 40 | 2.20429 | 60 | 7.05718 | 80 | 16.4319 |
| 1 | -14.0176 | 21 | -0.73406 | 41 | 2.34103 | 61 | 7.56236 | 81 | 16.9294 |
| 2 | -10.3672 | 22 | -0.56698 | 42 | 2.63156 | 62 | 7.76868 | 82 | 17.6157 |
| 3 | -8.10383 | 23 | -0.44329 | 43 | 2.8635 | 63 | 8.24649 | 83 | 18.0561 |
| 4 | -6.85355 | 24 | -0.28876 | 44 | 3.07717 | 64 | 8.75504 | 84 | 18.8066 |
| 5 | -5.72503 | 25 | -0.13847 | 45 | 3.27446 | 65 | 9.02377 | 85 | 19.5125 |
| 6 | -5.12569 | 26 | 0.002199 | 46 | 3.42531 | 66 | 9.39361 | 86 | 20.1874 |
| 7 | -4.43667 | 27 | 0.135032 | 47 | 3.66266 | 67 | 9.79873 | 87 | 21.2498 |
| 8 | -4.0066 | 28 | 0.298252 | 48 | 3.8878 | 68 | 10.1475 | 88 | 22.0417 |
| 9 | -3.75121 | 29 | 0.425629 | 49 | 4.2176 | 69 | 10.6638 | 89 | 23.1602 |
| 10 | -3.53802 | 30 | 0.580691 | 50 | 4.48816 | 70 | 11.2491 | 90 | 23.9371 |
| 11 | -3.23432 | 31 | 0.842471 | 51 | 4.72583 | 71 | 11.6533 | 91 | 24.9571 |
| 12 | -2.8392 | 32 | 0.995557 | 52 | 5.04191 | 72 | 12.208 | 92 | 25.9982 |
| 13 | -2.46033 | 33 | 1.17571 | 53 | 5.35037 | 73 | 12.7977 | 93 | 26.9958 |
| 14 | -2.15962 | 34 | 1.27262 | 54 | 5.54458 | 74 | 13.2294 | 94 | 29.1328 |
| 15 | -1.87732 | 35 | 1.45418 | 55 | 5.84028 | 75 | 13.7596 | 95 | 30.5847 |
| 16 | -1.74311 | 36 | 1.66068 | 56 | 6.0695 | 76 | 14.1825 | 96 | 32.435 |
| 17 | -1.53056 | 37 | 1.79714 | 57 | 6.22072 | 77 | 14.6769 | 97 | 34.1081 |
| 18 | -1.25115 | 38 | 1.91417 | 58 | 6.47333 | 78 | 15.3013 | 98 | 36.8946 |
| 19 | -1.07078 | 39 | 2.06022 | 59 | 6.78597 | 79 | 15.7433 | 99 | 41.3785 |
|  |  |  |  |  |  |  |  | 100 | 53.7262 |

Figure 10. SNR CDF@combination2

Table 14. SNR CDF table @ combination3

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -7.26419 | 20 | -0.96545 | 40 | 1.7748 | 60 | 5.56674 | 80 | 12.4162 |
| 1 | -5.21172 | 21 | -0.88688 | 41 | 1.88603 | 61 | 5.92351 | 81 | 12.9676 |
| 2 | -4.80452 | 22 | -0.75159 | 42 | 2.03053 | 62 | 6.15983 | 82 | 13.6152 |
| 3 | -4.38764 | 23 | -0.61685 | 43 | 2.17083 | 63 | 6.57576 | 83 | 14.146 |
| 4 | -3.95765 | 24 | -0.47036 | 44 | 2.32176 | 64 | 6.75301 | 84 | 15.442 |
| 5 | -3.61514 | 25 | -0.33375 | 45 | 2.50363 | 65 | 7.11825 | 85 | 15.9526 |
| 6 | -3.35388 | 26 | -0.17279 | 46 | 2.64933 | 66 | 7.3838 | 86 | 16.6827 |
| 7 | -3.18269 | 27 | -0.0451 | 47 | 2.81553 | 67 | 7.65885 | 87 | 17.2967 |
| 8 | -2.91377 | 28 | 0.093423 | 48 | 3.01089 | 68 | 7.98236 | 88 | 18.028 |
| 9 | -2.74461 | 29 | 0.248865 | 49 | 3.24202 | 69 | 8.24703 | 89 | 18.9256 |
| 10 | -2.60492 | 30 | 0.438283 | 50 | 3.40196 | 70 | 8.50787 | 90 | 19.8244 |
| 11 | -2.36578 | 31 | 0.576086 | 51 | 3.61096 | 71 | 8.80913 | 91 | 21.1362 |
| 12 | -2.28163 | 32 | 0.680843 | 52 | 3.76803 | 72 | 9.2037 | 92 | 21.8695 |
| 13 | -2.11585 | 33 | 0.839814 | 53 | 3.9496 | 73 | 9.48494 | 93 | 22.4685 |
| 14 | -1.89804 | 34 | 0.917518 | 54 | 4.14849 | 74 | 9.77827 | 94 | 23.6129 |
| 15 | -1.66357 | 35 | 1.06479 | 55 | 4.40149 | 75 | 10.4052 | 95 | 25.2505 |
| 16 | -1.53982 | 36 | 1.23596 | 56 | 4.63591 | 76 | 10.8369 | 96 | 26.9431 |
| 17 | -1.36078 | 37 | 1.36786 | 57 | 4.81894 | 77 | 11.2731 | 97 | 28.5205 |
| 18 | -1.24942 | 38 | 1.5008 | 58 | 4.97215 | 78 | 11.5699 | 98 | 30.1764 |
| 19 | -1.07945 | 39 | 1.66537 | 59 | 5.2814 | 79 | 12.0173 | 99 | 32.1415 |
|  |  |  |  |  |  |  |  | 100 | 36.8842 |

Figure 11. SNR CDF@combination3

Table 15. SNR CDF table @ combination4

| Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] | Outage [%] | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | -22.0262 | 20 | -1.28806 | 40 | 1.99085 | 60 | 6.88581 | 80 | 16.481 |
| 1 | -14.7467 | 21 | -1.13791 | 41 | 2.21929 | 61 | 7.34555 | 81 | 17.2132 |
| 2 | -12.4635 | 22 | -0.88186 | 42 | 2.43963 | 62 | 7.73864 | 82 | 18.1 |
| 3 | -10.6342 | 23 | -0.64076 | 43 | 2.7346 | 63 | 8.08924 | 83 | 18.8629 |
| 4 | -7.57208 | 24 | -0.47889 | 44 | 2.93743 | 64 | 8.42503 | 84 | 19.7797 |
| 5 | -6.36859 | 25 | -0.35376 | 45 | 3.16592 | 65 | 8.79445 | 85 | 20.6463 |
| 6 | -5.56474 | 26 | -0.25832 | 46 | 3.39976 | 66 | 9.25276 | 86 | 21.2617 |
| 7 | -4.98859 | 27 | -0.09675 | 47 | 3.57166 | 67 | 9.6758 | 87 | 22.2406 |
| 8 | -4.35556 | 28 | 0.021877 | 48 | 3.75616 | 68 | 10.0586 | 88 | 23.2867 |
| 9 | -3.87205 | 29 | 0.256459 | 49 | 3.92262 | 69 | 10.4647 | 89 | 23.97 |
| 10 | -3.52634 | 30 | 0.429175 | 50 | 4.18057 | 70 | 10.9435 | 90 | 24.9883 |
| 11 | -3.25525 | 31 | 0.522494 | 51 | 4.4416 | 71 | 11.609 | 91 | 25.7142 |
| 12 | -2.96183 | 32 | 0.708476 | 52 | 4.60271 | 72 | 12.3138 | 92 | 27.0815 |
| 13 | -2.67992 | 33 | 0.898988 | 53 | 5.14542 | 73 | 12.9802 | 93 | 28.4339 |
| 14 | -2.429 | 34 | 1.02653 | 54 | 5.38973 | 74 | 13.3398 | 94 | 29.6644 |
| 15 | -2.24673 | 35 | 1.22581 | 55 | 5.59945 | 75 | 13.8092 | 95 | 31.6005 |
| 16 | -2.08099 | 36 | 1.3598 | 56 | 5.90199 | 76 | 14.4038 | 96 | 33.7328 |
| 17 | -1.87038 | 37 | 1.48601 | 57 | 6.09438 | 77 | 15.0527 | 97 | 36.3763 |
| 18 | -1.66034 | 38 | 1.63309 | 58 | 6.44936 | 78 | 15.6939 | 98 | 39.0046 |
| 19 | -1.49349 | 39 | 1.81223 | 59 | 6.60945 | 79 | 16.0457 | 99 | 41.9799 |
|  |  |  |  |  |  |  |  | 100 | 53.4473 |

Figure 12. SNR CDF@combination4