3GPP TSG RAN WG1 Meeting #104-e                            	        R1-2100814

E-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.7.1.2

Source:	Spreadtrum Communications

Title:	Consideration on TRS/CSI-RS occasion(s) for idle/inactive UEs

Document for:	Discussion and decision

Introduction

In RAN 86 meeting, it was agreed to specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, and minimize system overhead impact [1].

- Specify enhancements for idle/inactive-mode UE power saving, considering system performance aspects [RAN2, RAN1] - Study and specify paging enhancement(s) to reduce unnecessary UE paging receptions, subject to no impact to legacy UEs [RAN2, RAN1] - NOTE: RAN1 to check and update, if needed, evaluation methodology in RAN1 #100 meeting - Specify means to provide potential TRS/CSI-RS occasion(s) available in connected mode to idle/inactive-mode UEs, minimizing system overhead impact [RAN1] - NOTE: Always-on TRS/CSI-RS transmission by gNodeB is not required

In this paper, we discuss our views on this aspect.

Reasons to introduce TRS/CSI-RS in idle/inactive mode

When UE operates in RRC-IDLE or RRC-INACTIVE state, UE might need to perform RRM measurement, cell re-selection if triggered and paging monitoring every paging cycle. In general, UE is expected to perform the following activities in RRC-IDLE or RRC-INACTIVE state:

- Loop convergence, e.g., AGC/TTL/FTL and etc.

- RRM measurement for cell reselection.

- Paging monitoring

In LTE, all above activities could be performed based on CRS in each PO. For NR UE, these activities are based on SSB, which may not be presented close to PO. Therefore, in normal case (as shown in Figure 1), UE may need to wake up two times in every paging cycle. One is to perform AGC/TTL/FTL and RRM for serving cell based on SSB (SMTC), the other one is to monitor paging occasion.

Figure 1: Case-1: UE power consumption in each paging cycle

In worse case, e.g., for the poor channel condition or lager paging cycle (as shown in Figure 2 and Figure 3), UE may need to wake-up to receive 2 SSB burst or  3 SSB burst for AGC, T/F tracking and serving cell measurement before PO monitoring.

Figure 2: Case-2: UE power consumption in each paging cycle

Figure 3: Case-3: UE power consumption in each paging cycle

From above analysis, UE may need to wake up multiple times.

Observation 1: UE may need to wake up multiple times for AGC, T/F tracking, serving cell measurement and PO monitoring in each paging cycle.

TRS/CSI-RS occasion(s) for idle/inactive UEs

Function of TRS/CSI-RS

In last meeting, the following agreement has been achieved [2].

Agreements: - Target sending an LS to RAN2 and RAN4 to ask whether it is feasible to allow a UE to use the potential TRS/CSI-RS occasion to enhance the SSB based IDLE/Inactive mode evaluations of the serving cell. (to also include agreements from last meeting) * Further discussion whether any additional information needs to be included in the LS or not, including potential re-wording of the leading sentence

Regarding current wording of the LS, if the potential TRS/CSI-RS occasion is only used to enhance the SSB based serving cell measurements, this means the SSB is still needed to be received for serving cell measurement before paging reception. In this case, UE still needs to wake up two times in every paging cycle in normal case (i.e., Case-1). One is to perform AGC/TTL/FTL and RRM for serving cell based on SSB (SMTC), the other one is to monitor paging occasion. For Case-1, in our evaluation, CSI-RS transmission with function of RRM measurement for serving cell for idle/inactive mode UE can bring 12%~17% power saving gains.

Therefore, UE can perform serving cell measurement based on CSI-RS in idle/inactive mode should be supported. Furthermore, in our view, on which RS (i.e., SSB or CSI-RS) UE performs measurement of serving cell is by implementation. For example, the UE can select the RS (SSB or CSI-RS) close to the PO for serving cell RRM measurement based on the implementation.

Proposal 1: UE can perform serving cell measurement based on CSI-RS in idle/inactive mode.

Availability indication of TRS/CSI-RS

In RAN1#103e, the following agreement has been achieved.

Agreements: - Discuss further based on the following alternatives and down-select at RAN1#104-e: - Alt 1: The availability of TRS/CSI-RS at the configured occasion(s) is NOT informed to the UE. - Alt 2: The availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - Alt 3. The conditional availability of TRS/CSI-RS at the configured occasion(s) is informed to the UE. - The condition can be, e.g., existence of paging. - Alt 4. Combination of the above alternatives. - FFS for details - FFS for UE behavior when the availability is not informed. - Other techniques are not precluded. - Companies encourage to provide sufficient information for the proposal, e.g., - how to achieve power saving gain - how to minimize impact on NW how to minimize extra UE implementation complexity - feasibility check on sharing the TRS/CSI-RS between connected UEs and idle/inactive UEs - Proposals should be consistent with the WID objective.

The purpose of introducing TRS/CSI-RS is to prevent UE from waking up too early to get the finer frequency and time synchronization for paging reception. UE should know whether the TRS/CSI-RS is available. In some companies’ view, UE needs to detect TRS/CSI-RS blindly. In our view, blind detection of TRS/CSI-RS will complicate UE implementation significantly and the specification effort is large, e.g. RAN4 testing cases. Therefore, gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE.

For Alt-3, from network perspective, TRS/CSI-RS dedicated for idle/inactive mode UE, which is coupled with paging message, should not be introduced. From UE implementation perspective, this is new UE behavior to process TRS/CSI-RS coupled with paging message. Therefore, we do not support Alt 3.

Proposal 2: gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE in advance.

Evaluation

In this section, we will simply evaluate the energy-saving gain brought by TRS/CSI-RS transmission in idle/inactive mode.

Evaluation methodology

Baseline assumption in our evaluation is assumed as the following:

- SCS=30KHz

- Paging cycle=1280/640ms

- SSB burst periodicity=20ms

- UE bandwidth=20MHz

In RAN1#102e, it agreed that the following power consumption model for FR1 is utilized for the evaluations of Rel-17 UE power saving enhancements in idle/inactive mode [3].

Table 1: Power consumption for different power state

| Power State | Relative Power (FR1 reference from TR 38.840) | Relative Power (Idle/inactive-mode operation with reception bandwidth 20 MHz) |
| --- | --- | --- |
| Deep Sleep (PDS) | 1 | 1 |
| Light Sleep (PLS) | 20 | 20 |
| Micro sleep (PMS) | 45 | 45 |
| PDCCH-only (PPDCCH) | 100 | 50Note |
| PDCCH + PDSCH (PPDCCH+PDSCH) | 300 | 120 |
| PDSCH-only (PPDSCH) | 280 | 112 |
| SSB/CSI-RS proc. (PSSB) | 100 (synchronization or serving cell measurement) | 50 |
| Intra-frequency RRM measurement (Pintra) | · 150 (synchronous case, N=8, measurement only; Pintra, meas-only) · 200 (combined search and measurement; Pintra, search+meas) | · [60] (synchronous case, N=8, measurement only; Pintra, meas-only) · [80] (combined search and measurement; Pintra, search+meas) |
| Inter-frequency RRM measurement (Pinter) | · 150 (measurement only per freq. layer; Pinter, meas-only) · 150 (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer | · [60] (measurement only per freq. layer; Pinter, meas-only) · [150] (neighbor cell search power per freq. layer; Pinter, search-only) · Micro sleep power assumed for switch in/out a freq. layer |
| Note: Power scaling to 20MHz reception bandwidth follows the rule in Section 8.1.3 of TR 38.840, i.e., max{reference power * 0.4, 50}. |  |  |

Paging traffic model in our evaluation is assumed as the following table:

Table 2: Paging traffic model for evaluation

| Paging rate | 0.01 |
| --- | --- |
| UEs per group | 100 |
| Group paging rate | 10% |

To simplify the evaluation, for Case-1, no neighbor cell measurement is assumed; For Case-2 and Case-3, one 5ms SMTC window is assumed for neighbor cell measurement after PO. In our evaluation, the TRS/CSI-RS transmission occasion is assumed before the PO.

Evaluation results

To evaluate the power gains of introducing TRS/CSI-RS transmission in idle/inactive mode, six different test cases are assumed as the following table:

Table 3: Six different test cases

|  | Evaluation assumptions |
| --- | --- |
| Test 1 | Paging cycle=1.28s |
|  | Case-1 |
| Test 2 | Paging cycle=1.28s |
|  | Case-2 |
| Test 3 | Paging cycle=1.28s |
|  | Case-3 |
| Test 4 | Paging cycle=640ms |
|  | Case-1 |
| Test 5 | Paging cycle=640ms |
|  | Case-2 |
| Test 6 | Paging cycle=640ms |
|  | Case-3 |

For each test case, we evaluate the power saving gain with TRS/CSI-RS transmission compared to the scenario without TRS/CSI-RS transmission in idle/inactive mode. The evaluation time is 100 paging cycles for each test case. The power saving gains for different test cases are shown in the following tables:

Table 4: The evaluation results of test 1

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 4500 | 4500 |
| PDCCH+PDSCH energy | 1200 | 1200 |
| SSB or TRS/CSI-RS processing | 12000 | 12000 |
| Deep sleep | 126000 | 128000 |
| Light sleep | 18000 | NA |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 10000 | NA |
| Neighbor cell measurement | NA | NA |
| Total Power consumption | 216700 | 190700 |
| Power saving gains | 12.0% |  |
| Note1: No neighbor cell measurement is assumed. Note2: UE can perform serving cell measurement with TRS/CSI-RS. |  |  |

Table 5: The evaluation results of test 2

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 22000 | 22000 |
| Deep sleep | 123500 | 125500 |
| Light sleep | 64000 | 28000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 30000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 337300 | 293300 |
| Power saving gains | 13.0% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 6: The evaluation results of test 3

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 32000 | 22000 |
| Deep sleep | 121500 | 123500 |
| Light sleep | 200000 | 80000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 40000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 391300 | 303300 |
| Power saving gains | 22.5% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 7: The evaluation results of test 4

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 4500 | 4500 |
| PDCCH+PDSCH energy | 1200 | 1200 |
| SSB or TRS/CSI-RS processing | 12000 | 12000 |
| Deep sleep | 62000 | 64000 |
| Light sleep | 18000 | NA |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 10000 | NA |
| Neighbor cell measurement | NA | NA |
| Total Power consumption | 152700 | 126700 |
| Power saving gains | 17.0% |  |
| Note1: No neighbor cell measurement is assumed. Note2: UE can perform serving cell measurement with TRS/CSI-RS. |  |  |

Table 8: The evaluation results of test 5

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 22000 | 22000 |
| Deep sleep | 59500 | 61500 |
| Light sleep | 64000 | 28000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 30000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 273300 | 229300 |
| Power saving gains | 16.1% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

Table 12: The evaluation results of test 6

|  | Without TRS/CSI-RS transmission | With TRS/CSI-RS transmission |
| --- | --- | --- |
| PDCCH-only energy | 18000 | 18000 |
| PDCCH+PDSCH energy | 4800 | 4800 |
| SSB or TRS/CSI-RS processing | 32000 | 22000 |
| Deep sleep | 57500 | 59500 |
| Light sleep | 100000 | 40000 |
| Deep sleep to non-sleep | 45000 | 45000 |
| Light sleep to non-sleep | 40000 | 20000 |
| Neighbor cell measurement | 30000 | 30000 |
| Total Power consumption | 327300 | 239300 |
| Power saving gains | 26.9% |  |
| Note: One 5ms SMTC window is assumed for neighbor cell measurement after PO. |  |  |

As seen from the above evaluation results, TRS/CSI-RS transmission before PO can bring obvious power saving gain, especially when paging cycle is small. The power saving gain mainly comes from the power reduction of light sleep and the corresponding state transition.

Observation 2: TRS/CSI-RS transmission for idle/inactive mode UE can bring obvious power saving gains.

Conclusion

As summary, we have the following proposals.

Proposal 1: UE can perform serving cell measurement based on CSI-RS in idle/inactive mode.

Proposal 2: gNB needs to inform the availability of TRS/CSI-RS to idle/inactive mode UE in advance.

Observation 1: UE may need to wake up multiple times for AGC, T/F tracking, serving cell measurement and PO monitoring in each paging cycle.

Observation 2: TRS/CSI-RS transmission for idle/inactive mode UE can bring obvious power saving gains.

Reference

- RP-193239, “New WID: UE Power Saving Enhancements”, MediaTek Inc.

- 3GPP RAN1#103e Chairman’s Notes, October 26 - November 13, 2020.

- 3GPP RAN1#102e Chairman’s Notes, August 17 - 28, 2020.