3GPP TSG RAN WG1 #104-e		R1-2101157

e-Meeting, January 25th – February 5th, 2021

Title:	Draft reply LS on New Standardized 5QIs for 5G-AIS

Release:	Rel-17

Work Item:

Source:	RAN1

To:	SA2

Cc:

Attachments:

# Contact Person:

# Name:

E-mail Address:

# 1. Overall Description:

RAN1 would like to thank SA2 for the LS on the new standardized 5QIs for 5G-AIS.

For the new standardized 5QIs Value#1 and 2 defined for visual content of cloud/edge/split rendering, RAN1 in general will support them. On the one hand, both the latency and reliability requirements need to be satisfied. On the other hand, the resource types of them are GBR, and the traffic data rate could have a wide range. Therefore, for low data rate case, RAN1 thinks they can be supported based on TR 38.824. Meanwhile, there might be great challenges for high data rate case, e.g.,100Mbps, since the data rate of such case is much higher than the use cases of URLLC defined in TR 38.824. It should also be noted that the performance evaluation for high data rate case is now under the ongoing XR SI.

For the new standardized 5QIs Value#3 and 4 defined for motion tracking and sensor data of cloud/edge/split rendering, they have similar data volume, latency and reliability requirements to those for Rel-15 enabled use case (e.g., AR/VR) and transport industry as shown in the table below quoted from TR 38.824. According to the evaluations for URLLC in TR 38.824, RAN1 thinks they can be well supported by NR-RAN.

Table A.2-1: Representative use cases for Rel-16 NR URLLC evaluation [TR38.824]

| Use case | Reliability (%) | Latency | Data packet size and traffic model | Description |
| --- | --- | --- | --- | --- |
| Rel-15 enabled use case (e.g. AR/VR) | 99.999 | 1 MS (air interface delay) for 32 bytes 1 mms and 4 mess (air interface delay) for 200 bytes | DL & UL: 32 and 200 bytes FTP model 3 or periodic with different arrival rates |  |
|  | 99.9 | 7 mms (air interface delay) | DL & UL: 4096 and 10 K bytes FTP model 3 or periodic with different arrival rates |  |
| Transport Industry | 99.999 | 5 mess (end to end latency) Note: 3 ms air interface latency | UL: 2.5 Mpbs; Packet size 5220 bytes DL: 1Mbps; Packet size 2083 bytes Note: Data arrival rate 60 packets per second for periodic traffic model | Remote driving (TS 22.186: 5.5) |
|  | 99.999 | 10 ms (end to end latency) Note: 7ms air interface latency | UL&DL: 1.1 Mbps; Packet size 1370 bytes Note: Data arrival rate 100 packets per second for periodic traffic model | Intelligent transport system (ITS) (TS 23.501, TS 22.261) |

# 2. Actions:

RAN1 respectfully asks SA2 to take into account the above information.

# 3. Date of Next RAN1 Meetings:

TSG-RAN WG1 Meeting #104bis-e			12th April – 20th April 2021		E-meeting.

TSG-RAN WG1 Meeting #105				19th May – 27th May 2021		E-meeting.