3GPP TSG RAN WG1 Meeting #104-e	R1-2101170

e-Meeting, January 25th – February 5th, 2021

Title:	Draft reply LS on simultaneous CSI-RS

Response to:	LS on the use of simultaneous CSI-RS resources and ports (R2-2011274)

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Youngrok Jang

E-mail Address:		yr.jang@samsung.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: 	-

1. Overall Description:

RAN1 would like to provide replies on the following questions from RAN2 in R2-2011274:

Question: RAN2 would like to ask whether the fields maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC and totalNumberPortsSimultaneousNZP-CSI-RS are supposed to indicate: the number of active CSI-RS resources and ports, respectively; or the number of configured CSI-RS resources and ports (including active CSI-RS resources and ports), respectively?

Answer to Question:

RAN1 understands that the answer is the number of active CSI-RS resources and ports, respectively. Since the UE capabilities maxConfigNumberNZP-CSI-RS-PerCC and maxNumberSimultaneousNZP-CSI-RS-PerCC are defined separately, RAN1 understands that the maximum number of configured CSI-RS resources is denoted by maxConfigNumberNZP-CSI-RS-PerCC, not by maxNumberSimultaneousNZP-CSI-RS-PerCC, which denotes the maximum number of active CSI-RS resources. Also, in addition to the limits signalled by maxNumberSimultaneousNZP-CSI-RS-PerCC, the network applies a limit signalled maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC. Hence, the UE capability maxNumberSimultaneousNZP-CSI-RS-ActBWP-AllCC also indicates the number of active CSI-RS resources. Similar understanding can be applied to the number of CSI-RS ports.

2. Actions:

To RAN2

ACTION: 	RAN1 respectfully asks RAN2 to take the above information into account in their further work

3. Date of Next Meetings:

TSG-RAN WG1 Meeting #104-Bis-e 	April 12 – 20, 2021				e-Meeting

TSG RAN WG1 Meeting #105-e	May 19 – 27, 2021 				e-Meeting