3GPP TSG RAN WG1 #104-e			R1-2101404

e-Meeting, January 25th – February 5th, 2021

Agenda Item:	8.15.4

Source:	InterDigital, Inc.

Title:	HARQ enhancement for IoT NTN

Document for:	Discussion and Decision

# Introduction

In RAN#86, a new study item on NB-IoT/eMTC support for non-terrestrial networks was approved with the following RAN1 objectives [1]:

The first objective of this Study is to identify scenarios applicable to NB-IoT/eMTC [RAN1, RAN2], including:

-	Bands of interest in sub 6 GHz

-	Device type with PC3 or PC5 (LEO and GEO)

-	Satellite constellation orbit LEO and GEO

-	Transparent payload.

-	Link budget

NOTE 1: This first objective will be based on the scenarios documented in TR 38.821.

NOTE 2: UE mobility assumptions follow terrestrial NB-IoT/eMTC assumptions.

The second objective is, for the above identified scenarios, to study and recommend necessary changes to support NB-IoT and eMTC over satellite, reusing as much as possible the conclusions of the studies performed for NR NTN in TR38.821. This objective will address the following items:

-	Aspects related to random access procedure/signals [RAN1, RAN2]

-	Mechanisms for time/frequency adjustment including Timing Advance, and UL frequency compensation indication [RAN1, RAN2]

-	Timing offset related to scheduling and HARQ-ACK feedback [RAN1, RAN2]

-  Aspects related to HARQ operation [RAN2, RAN1]

In this contribution, we will focus on necessary changes to extend NB-IoT/eMTC to a non-terrestrial environment in terms of HARQ operation in the second objective.

# Discussion

A new SID on NB-IoT/eMTC device support for non-terrestrial networks was approved in RAN#86 to provide IoT operation to remote areas with low or no cellular connectivity. It is expected that such enhancements will support a broad range of industries, including transportation, farming, mining etc.

As described in the NB-IoT/eMTC study item description, non-terrestrial deployment scenarios under consideration consist of both LEO and GEO orbits with transparent payload. GEO deployments are characterized by large propagation delay and cell size, however stationary and approximately fixed relative to a point on earth. Though propagation delay and cell size is less in LEO deployments, satellites move relative to a fixed point on earth with cells either moving along the ground, or fixed via e.g. beam steering techniques.

Key modifications to enable terrestrial deployments are therefore enhanced time compensation techniques to accommodate greatly increased propagation delay, and enhanced frequency compensation due to fast moving LEO satellites. Specific values relevant for this contribution are included in Table 1 for reference, however a full description of NR NTN reference scenario parameters is found in Table 4.2-2 in TR 38.821 [2].

Table 1 [2]: Relevant reference parameters for max RTD and Doppler shift.

| Max Round Trip Delay (propagation delay only) | 541.46 ms (service and feeder links) | 25.77 ms (600km) 41.77 ms (1200km) |
| --- | --- | --- |
| Max differential delay within a cell (Note 6) | 10.3 ms | 3.12 ms and 3.18 ms for respectively 600km and 1200km |
| Max Doppler shift (earth fixed user equipment) | 0.93 ppm | 24 ppm (600km) 21ppm(1200km) |
| Max Doppler shift variation (earth fixed user equipment) | 0.000045 ppm/s | 0.27ppm/s (600km) 0.13ppm/s(1200km) |

## HARQ enhancement

Two HARQ aspects are currently under discussion in RAN1: 1) the enabling/disabling of HARQ feedback for downlink transmission and; 2) the extension of maximal HARQ process number. Such enhancements are necessary to avoid HARQ stalling in NTN due to large propagation delays. The following agreements from RAN1#102e are summarized below [3]:

Agreement:

Enabling/disabling on HARQ feedback for downlink transmission should be at least configurable per HARQ process via UE specific RRC signaling

Agreement:

The extension of maximal HARQ process number can be considered with following assumptions:

- The maximal supported HARQ process number is up to 32.

- FFS: Support on the maximal HARQ process number is up to UE capability

- Minimizing the impacts on specification and scheduling

Although agreements on disabling HARQ process can be applied to NB-IoT/eMTC, given the increased delay requirements (i.e., 10s from application to core network), solutions addressing HARQ stalling may not be as critical in NB-IoT/eMTC. Additionally, considering the low-cost nature of NB-IoT/eMTC devices, increasing the soft buffer requirements to support additional HARQ PIDs does not seem well motivated.

Observation 1: 	Due to low-cost nature of NB-IoT/eMTC devices, increasing soft buffer requirements to support additional HARQ PIDs is not well motivated.

Proposal 1: Maximum HARQ process number is not increased for NTN NB-IoT/eMTC devices.

Proposal 2: 	Discussion on enhancements to reduce HARQ stalling is deprioritized in NB-IoT/eMTC.

Other types of HARQ enhancements including HARQ codebook enhancement is currently under discussion in RAN1 for Rel-17 NTN WI. However, it seems to be premature to discuss those issues for IoT NTN until enough progress has made in the NTN WI.

Proposal 3: No discussion on the HARQ enhancement issues for IoT NTN until a sufficient progress is made for that in NTN.

# Summary

In this contribution, we discussed on HARQ enhancement for IoT NTN. Based on the discussion, we make the following observations and proposals:

Observation 1: 	Due to low-cost nature of NB-IoT/eMTC devices, increasing soft buffer requirements to support additional HARQ PIDs is not well motivated.

Proposal 1: Maximum HARQ process number is not increased for NTN NB-IoT/eMTC devices.

Proposal 2: 	Discussion on enhancements to reduce HARQ stalling is deprioritized in NB-IoT/eMTC.

Proposal 3: No discussion on the HARQ enhancement issues for IoT NTN until a sufficient progress is made for that in NTN.

# References

RP-193235, Study on NB-IoT/eMTC support for Non-terrestrial Network, MediaTek Inc., RAN#86, Dec 2019.

3GPP TR 38.821 Solutions for NR to support non-terrestrial networks (NTN)