3GPP TSG RAN WG1 #104-e	R1-210xxxx

e-Meeting, January 25th – February 5th, 2021

Agenda item:	7.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	[104-e-NR-7.1CRs-13]

Document for:	Discussion and Decision

- Background

In R1-2101432, it is proposed to define timelines for SRS carrier switching. The contribution highlights that, for priority rules related to SRS carrier switching, there are no timelines specified. For example, in the case of collision between PUCCH with HARQ-ACK and SRS, the following timeline is missing:

To solve this issue, the following text proposal is provided:

<TP1, 38.214>

#### 6.2.1.3	UE sounding procedure between component carriers

For an SRS transmission starting in symbol Nc1 of a carrier c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, and with c2 carrier the UE is configured to switch from according to higher layer parameters srs-SwitchFromServCellIndex and srs-SwitchFromCarrier, for a conflicting transmission in c2 starting in symbol Nc2  the UE shall apply the rules in the remaining of this subclause based on:

- DCI(s) for which the time interval between the last symbol of PDCCH and Nc1 is at least N2 symbols and an additional time duration TSRSCS,  and the time interval between the last symbol of PDCCH and Nc2 is at least   N2 symbols; and

- semi-persistent CSI reports or SRS for which the HARQ-ACK information on PUCCH in response to the activation command would be transmitted in slot n, and Nslot being the first symbol of the first slot which is after slot n+3Nslotsubframe,μ, and the time interval between Nslot and Nc1 is at least N2 symbols and an additional time duration TSRSCS, and the time interval between Nslot and Nc2 is at least N2 symbols, where μ is the SCS configuration of the PUCCH.

where TSRSCS=max⁡{switchingTimeUL,switchingTimeDL}, and the time interval unit of OFDM symbol is counted based on the minimum subcarrier spacing given by min⁡{μPDCCH,c1, μSRS,c1,μPDCCH,c2, μUL,c2}, with μPDCCH,c1, the SCS configuration for the PDCCH carrying the triggering commands for the SRS in c1, μSRS,c1 the SCS configuration for SRS in c1,  μPDCCH,c2  the SCS configuration for the PDCCH that schedules PUSCH in c2, and μUL,c2 the SCS configuration for the PUSCH in c2.

</TP1>

In the following, we try to collect views from companies on the following two questions: whether the change is needed (and if not, what is the UE behavior), and any comments on the provided TP:

- Discussion

Q1: Do you agree that the timelines for SRS carrier switching dropping rules are not specified and, therefore, a correction is needed?

- If you do not agree, please provide a spec reference / justification on what is the UE behavior.

| Company name | Answer (Yes/no) | If the answer is no, please provide reference / justification |
| --- | --- | --- |
| Qualcomm | Yes |  |
| Ericsson | Yes, although it is not clear that the feature is broken without a fix. |  |
| Samsung | No | We think that this CR is not essential in Rel-15. This problematic issue can be avoided by gNB implementation. |
| Nokia | OK to have an update | As long as the specification update is such that all existing UEs are compliant with the CR |
| HW | Partially Yes | We think that this CR is NBC in Rel1-15 so that gNB can avoid that by longer gap. If it is needed, we are open to address this in Rel-16 TEI. With possible different SCS, proposed spec may not be straightforward. |
| FUTUREWEI | Yes | This can be largely solved by gNB implementation though may be with some drawback and have to be conservative. To address this issue with newly introduced UE behavior (clearly UE timeline) and UE capability (for example for N_2) will cause NCB issue. Therefore, we think this is an enhancement, a valid and useful one. |
| OPPO | Yes |  |
| ZTE | Yes | But we think it can be an error case if the timeline issue happens. So gNB can avoid this situation by implementation. |
| vivo | Yes/No | In the scenario described above it seems dropping rule is not defined, the main points is when does the UE make decision to switch carrier for SRS transmission, 1) it could be avoided be gNB scheduling(?), 2) when UE successfully decodes PDCCH (for PDSCH) it knows PUCCH resource, is their UE implementation constraint to make decision at this point? |

Q1: •	Do you have any comments on the provided TP in 2101432?

| Company name | Comments on TP |
| --- | --- |
| Ericsson | To be honest, I’m having trouble parsing the TP. Some initial questions for my understanding: - Are the conflicting transmissions in response to the DCI(s)? If so, where is this stated? - Does ‘at least N_2 symbols and an additional time duration T_SRS_CS’ mean ‘N_2 + T_SRS_CS’? - When does either one or the other of the DCIs related to Nc1 and Nc2 apply in the remainder of the subclause? |
| Nokia | Have some difficulties with the TP as well What type of carrier does this refer to, a TDD carrier only, or is this meant to rule out SUL and SDL? a carrier c1 with slot formats comprised of DL and UL symbols What is a conflicting transmission? I suppose the intention is to say something like “for an uplink transmission in carrier C2 starting in symbol Nc2 that is to overlap with the SRS transmission, the…”? What is done if the rules are not satisfied? |
| FUTUREWEI | We prefer to treat this as a TEI and investigate a good approach to amend the spec. |
| OPPO | Understanding the intention of this TP. However, the TP seems not touching the case where different CCs are with different numerologies. |
| ZTE | Prefer to get a conclusion, such as UE does not expect .... |

- Summary

To be completed after discussion phase