3GPP TSG-RAN WG1 Meeting #104-e											R1-210xxxx

e-Meeting, Jan 25th – Feb 5th, 2020

Agenda Item:	7.2.1

Source:			Moderator (ZTE)

Title:			FL summary on the maintenance of 2-step RACH

Document for:	Discussion

# Introduction

This document contains the summary of issues related to the maintenance of Rel-16 2-step RACH WI in RAN1#104-e meeting.

# Maintenance issues

The following 3 issues are identified based on the submitted contributions in RAN1#104-e.

| Issue # | Description | Related TDoc # |
| --- | --- | --- |
| 1 | Correction on DMRS configuration for MsgA in 38.214 | R1-2100243 |
| 2 | Clarify in the description of the transmission timing adjustment procedure in 38.213 | R1-2101526 |
| 3 | Not support MsgB and unicast PDSCH TDMed multiplexing in a slot per CC | R1-2101573 |

FL comments:

For the above 3 issues, the proponents have clarified clearly the intention for changes in their TDocs. It would be rather straightforward to include them in a single email thread.

# Summary

The following Email discussion is proposed (will be updated based on companies’ feedback, if any).

Proposed Email thread #1:

Corrections of 2-step RACH related issues

- CR in R1-2008785, CR in R1-2101526, and TP in R1-2101573

Any comments?

| Company | Comment |
| --- | --- |

# References

- R1-2100243	Correction on DMRS configuration for MsgA in 38.214	Huawei, HiSilicon

- R1-2101526	Draft CR to 38.213 on corrections for 2-step RACH	Ericsson

- R1-2101573	Discussion on remaining issues on 2-step RACH	Spreadtrum Communications

# Appendix

List of proposals in the submitted contributions.

| TDoc | Proposals |
| --- | --- |
| R1-2100243 Huawei | Reason for change: - It is clear that there can be at most 2 ports configured per DM-RS CDM group for single-symbol DM-RS case. Then for MsgA PUSCH transmission, if a UE is not configured with msgA-PUSCH-NrofPort, the UE shall assume that it is only for the case of double-symbol DM-RS that 4 ports are configured per DM-RS CDM groups. This is also aligned with the possible DMRS configuration defined in TS 38.211 and TS 38.212. - For MsgA PUSCH transmission, only PUSCH DM-RS configuration type 1 is supported, and there is no higher layer parameter “dmrs-Type”. The corresponding correct description is already provided in TS 38.211. ------------------------- ### 6.2.2 UE DM-RS transmission procedure *** Unchanged text is omitted *** When transmitted PUSCH is scheduled by DCI format 0_1 with CRC scrambled by C-RNTI, CS-RNTI, SP-CSI-RNTI or MCS-C-RNTI, or corresponding to a configured grant, or being a PUSCH for Type-2 random access procedure, - the UE may be configured with higher layer parameter dmrs-Type in DMRS-UplinkConfig, and the configured DM-RS configuration type is used for transmitting PUSCH in as defined in Clause 6.4.1.1 of [4, TS 38.211]. *** Unchanged text is omitted *** For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-DMRS-CDM-group, the UE shall assume that 2 DM-RS CDM groups are configured. Otherwise, msgA-PUSCH-DMRS-CDM-group indicates which DM-RS CDM group to use from the set of {0,1}. For MsgA PUSCH transmission, if the UE is not configured with msgA-PUSCH-NrofPort, the UE shall assume that 4 ports are configured per DM-RS CDM groups for double-symbol DM-RS. Otherwise, msgA-PUSCH-NrofPort with value of 0 indicates the first port per DM-RS CDM group, while a value of 1 indicates the first two ports per DM-RS CDM group. *** Unchanged text is omitted *** |
| R1-2101526 Ericsson | Reason for change: A 12-bit absolute TA can be in a DL-SCH with Absolute Timing Advance Command MAC CE which is already specified in 38.321 from 2-step RACH work item in NR R16. But in 38.213, the 12 bits absolute TA is only assumed to be in RAR, and only 6-bit TA is assumed for all other cases. ----------------------------------------- ## 4.2 Transmission timing adjustments <omit unchanged text> In case of random access response or Absolute Timing Advance Command MAC CE, a timing advance command [11, TS 38.321], , for a TAG indicates values by index values of = 0, 1, 2, ..., 3846, where an amount of the time alignment for the TAG with SCS of kHz is . is defined in [4, TS 38.211] and is relative to the SCS of the first uplink transmission from the UE after the reception of the random access response. In other cases, a timing advance command [11, TS 38.321], , for a TAG indicates adjustment of a current value, , to the new value, , by index values of = 0, 1, 2,..., 63, where for a SCS of kHz, . <omit unchanged text> |
| R1-2101573 Spreadtrum | Reason for change: The payload size of MsgB is much larger than Msg2 and/or Msg4. The processing capability requirement for MsgB could be equivalent to unicast PDSCH. For UEs not supporting two unicast PDSCHs TDMed in a slot per CC, MsgB could not be treated. The accessing latency would be increased and the benefit of 2-step RACH would loss. Summary of change: Not support MsgB and unicast PDSCH TDMed multiplexing in a slot. Consequences if not approved: The benefit of 2-step RACH would loss, and even 2-step RACH could not be supported for UEs not supporting 2 unicast PDSCHs TDMed in a slot per CC. Clauses affected: TS38.214 g40, section 5.1 ----------------------Start of Text proposal#1 for TS 38.214---------------------------- ## 5.1 UE procedure for receiving the physical downlink shared channel --------------------------------------------------Unchanged text omitted--------------------------------------------- The UE is not expected to decode a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI if another PDSCH in the same cell scheduled with RA-RNTI or MSGB-RNTI partially or fully overlap in time. The UE is not expected to be scheduled a PDSCH scheduled with C-RNTI, MCS-C-RNTI, or CS-RNTI, and another PDSCH in the same cell scheduled with MSGB-RNTI in a slot. -----------------------End of Text proposal#1 for TS 38.214---------------------------- |