3GPP TSG-RAN WG1 Meeting #104b-e	R1-2102369

e-Meeting, April 12 – 20, 2021

Agenda item:	7.2.4

Title:	Remaining open issues and corrections for mode 2 RA

Source:	OPPO

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide discussion and our views on the remaining open issues and corrections needed for mode 2 resource allocation. Specifically, we focus on the following item:

- Infinite loop due to excessive exclusion

- Mode 2: Infinite loop due to excessive exclusion

The main procedure for determining a subset of resources within a resource selection window in mode 2 resource allocation is described in TS 38.214 as below:

4) The set SA is initialized to the set of all the candidate single-slot resources. 5) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: - the UE has not monitored slot tmSL in Step 2. - for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. 6) The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions: a) the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213]; b) the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than ThprioRX,prioTX; c) the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A, is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=TscalPrsvp_RX if Prsvp_RX< Tscal and n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms. 7) If the number of candidate single-slot resources remaining in the set SA is smaller than X⋅Mtotal, then Th(pi,pj) is increased by 3 dB for each priority value Th(pi,pj) and the procedure continues with step 4.

From the described step 4) to 7), it is clear that UE will increase the RSRP threshold Th(pi,pj) continuously by 3dB until the amount of candidate resources in SA achieves X⋅Mtotal. And the (pre-)configured value for X can be up to 50% according to the agreements of RAN1#101-e. However, if the number of resources in SA is already less than X⋅Mtotal after performing step 5), the condition of achieving X⋅Mtotal number of resources will never be met although the RSRP threshold is continuously increasing again and again. This is due to increasing the RSRP threshold can only reduce the number of resources excluded in step 6) rather than in step 5). There is a possible infinite loop problem especially when X is set to a large value like 50%.

To address this issue, we provide four potential solutions in the following.

First, the above issue is due to excessive resource exclusion in step 5). In step 5), all of the periodicity values in sl-ResourceReservePeriodList configured by higher layers are used to exclude resources in slots where the UE did not monitor. This can lead to excessive resource exclusion if small periodicity values like 2ms, 3ms and etc are configured for the selected resource pool. Assuming 2ms reservation period is configured, then there is a possibility that at least every second slot in the resource selection window are excluded in step 5). And after performing step 6), more resources will most likely be excluded, then the remaining resources will never reach target/configured 50% of the candidate set for the respective L1 priority. Hence a subset of the (pre-)configured periodicities should be used for resource exclusion in step 5). For example, the subset can be determined by taking some small periodicity values out of the sl-ResourceReservePeriodList.

Proposal 1: A subset of the (pre-)configured periodicities for reservation should be used to exclude resources in slots not monitored during sensing.

Typically, the most flexible way is to configured a subset of periodicity values from the sl-ResourceReservePeriodList that should be used in step 5) for exclusion. However, it is also possible to select a subset based on first X values or the highest X values from the configured sl-ResourceReservePeriodList to avoid any RRC impact. The value for X can be pre-determined as 3, 5, or 10 in the spec, since there can be up to 16 reservation period values in the sl-ResourceReservePeriodList.

========================Text Proposal for the 1st solution (38.214)========================

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for the first X periodicity values allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met. If the size of is greater than 10, X is 10. Otherwise X is 5.

====================End of Text proposal for the 1st solution (38.214) =======================

A second possible solution, the set of possible values for Prsvp_RX is {0、1-99、100、200…1000}ms for NR V2X as agreed in RAN1#100-e, as such, the Q value in step 6) would be quite large if  Prsvp_RX is very small e.g. 2ms. If Q is too large, many resources will be excluded because  tm+q×Prsvp_RX'SL represents many slots. This is another case which will lead to excessive exclusion in step 5). Therefore, we propose to replace Q=TscalPrsvp_RX with Q=Tscalmax⁡(20, Prsvp_RX) to avoid using small reservation periods in resource exclusion.

Proposal 2: Replace Q=TscalPrsvp_RX with Q=Tscalmax⁡(20, Prsvp_RX) to avoid excessive exclusion.

========================Text Proposal for the 2nd solution (38.214)========================

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

c)	the SCI format received in slot tmSLor the same SCI format which, if and only if the "Resource reservation period" field is present in the received SCI format 1-A,  is assumed to be received in slot(s) tm+q×Prsvp_RX'SL determines according to clause 8.1.5 the set of resource blocks and slots which overlaps with Rx,y+j×Prsvp_TX' for q=1, 2, …, Q and j=0, 1, …, Cresel-1. Here, Prsvp_RX' is Prsvp_RX converted to units of logical slots according to clause 8.1.7, Q=Tscalmax⁡(20, Prsvp_RX) Q=TscalPrsvp_RX  if Prsvp_RX< Tscal and  n'-m≤Prsvp_RX', where tn'SL = n if slot n belongs to the set t0SL,t1SL,...,tTmaxSL, otherwise slot tn'SL is the first slot after slot n belonging to the set t0SL,t1SL,...,tTmaxSL; otherwise Q=1. Tscal is set to selection window size T2 converted to units of ms.

=====================End of Text proposal for the 2nd solution (38.214) ======================

A third solution, another straightforward method is that, UE shall report the set SA  to high layers immediately if it finds that the number of resources in SA is less than or equal to X⋅Mtotal after step 5), without performing step 6). In this way, UE is able to jump out of the infinite loop and won’t increase the RSRP threshold again and again.

Proposal 3: If the number of resources in SA is already less than or equal to X⋅Mtotal after step 5), UE will report the current  SA to high layers immediately and not perform other steps (i.e. step 6 and 7).

========================Text Proposal for the 3rd solution (38.214)========================

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

6)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions and the number of candidate single-slot resources remaining in the set SA is larger than X⋅Mtotal:

a)	the UE receives an SCI format 1-A in slot tmSL, and "Resource reservation period" field, if present, and "Priority" field in the received SCI format 1-A indicate the values Prsvp_RX and prioRX, respectively according to Clause 16.4 in [6, TS 38.213];

b)	the RSRP measurement performed, according to clause 8.4.2.1 for the received SCI format 1-A, is higher than  ThprioRX;

<Unchanged parts omitted>

=====================End of Text proposal for the 3rd solution (38.214) ======================

A fourth solution, an alternative to the third solution is that the UE do not perform step 5) if number of resources to be excluded from step 5) will result in less than X⋅Mtotal.

Proposal 4: If the number of resources remaining in SA will be less than X⋅Mtotal after performing step 5), UE will not perform / skips step 5).

========================Text Proposal for the 4th solution (38.214)========================

8.1.4	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2

<Unchanged parts omitted>

5)	The UE shall exclude any candidate single-slot resource Rx,y from the set SA if it meets all the following conditions:

-	the UE has not monitored slot tmSL in Step 2.

-	for any periodicity value allowed by the higher layer parameter sl-ResourceReservePeriodList-r16 and a hypothetical SCI format 1-A received in slot tmSL with "Resource reservation period" field set to that periodicity value and indicating all subchannels of the resource pool in this slot, condition c in step 6 would be met.

-	if the number of candidate single-slot resources remaining in the set SA is less than X⋅Mtotal, the set SA is reinitialized to the set of all the candidate single-slot resources.

<Unchanged parts omitted>

=====================End of Text proposal for the 4th solution (38.214) ======================

- Conclusion

The remaining open issues of Mode 2 are discussed. The following proposals are given and one of them should be adopted to solve the infinite loop due to excessive exclusion problem.

Proposal 1: A subset of the (pre-)configured periodicities for reservation should be used to exclude resources in slots not monitored during sensing.

Proposal 2: Replace Q=TscalPrsvp_RX with Q=Tscalmax⁡(20, Prsvp_RX) to avoid excessive exclusion.

Proposal 3: If the number of resources in SA is already less than or equal to X⋅Mtotal after step 5), UE will report the current  SA to high layers immediately and not perform other steps (i.e. step 6 and 7).

Proposal 4: If the number of resources remaining in SA will be less than X⋅Mtotal after performing step 5), UE will not perform / skips step 5).