3GPP TSG RAN WG1 #104-bis-e	R1-2102561

e-Meeting, April 12th – April 20th, 2021

Agenda item:		8.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Beam Management Aspects

Document for:		Discussion and Decision

# Introduction

The revised work item on supporting NR from 52.6 GHz to 71 GHz [1] was approved at RAN#90-e. Before that 3GPP carried out a study on required changes to NR using existing DL/UL waveform to support operation between 52.6 GHz and 71GHz [2]. This contribution deals with the following objectives of the WID:

- Specify timing associated with beam-based operation to new SCS (i.e., 480kHz and/or 960kHz), study, and specify if needed, potential enhancement for shared spectrum operation

  - Rel-15/16 and any Rel-17 beam management enhancements can be considered for 52.6-71 GHz. Whether particular features should be excluded for 52.6-71 GHz can be further discussed

  - Note: As per usual procedure, duplication of work between work items in Rel-17 should be avoided

TR [2] captures Physical layer impacts to beam management and CSI in the following way:

For the study item, it is recommended to study potential enhancements or alternatives to the scheduling request mechanism to reduce scheduling latency due to beam sweeping, if needed.

For the study item, it is recommended to consider at least the following aspects in system operations with beams:

-	study of BFR mechanism enhancements, if supported,

-	e.g., the use of aperiodic CSI-RS for BFR, increased number of RSs for monitoring/candidates and efficient utilization of the increased number of RSs, enhanced reliability to cope with narrower beamwidth,

-	study of UE capabilities on beam switch timing in beam management procedure,

-	study of enhancements for beam management and corresponding RS(s) in DL and UL are needed further considering at least the following aspects, if supported:

-	beam switching time, beam alignment delay (including initial access), LBT failure, and potential coverage loss (if large SCS is supported),

-	study of beam switching gap handling for signals/channels (e.g. CSI-RS, PDSCH, SRS, PUSCH) for higher subcarriers spacing, if supported.

It is recommended to further investigate potential enhancements, if needed, to beam management at least considering one or more of potentially narrower beamwidths, CP duration, multiple beam indications for multi-PUSCH/PDSCH scheduling, triggering of reference signals for beam management, enhancements to beam management for random access procedure, intra- and/or inter-cell mobility, and adaptation to LBT failures. Minimum requirement on beam switching delay in > 52.6 GHz spectrum should be further studied by RAN4 when specification is further developed.

It is recommended to investigate whether or not enhancements to CSI processing unit (CPU) availability check is needed when the UE is required to process CSI reports corresponding to multiple numerologies across active BWPs in different component carriers.

# Discussion

## Timing related

RAN1#104-e made the following agreements related to timing aspects:

Agreement: - For NR operation in 52.6-71GHz with new SCSs, new parameter values for at least the following timing parameters are needed: - timeDurationForQCL - beamSwitchTiming - beamReportTiming - Companies are encouraged to provide preferred values on timeDurationForQCL, beamSwitchTiming and beamReportTiming Agreement: - Further study new parameter values for at least the following parameters: - maxNumberRxTxBeamSwitchDL - Additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS - Study whether/how to introduce a beam switching gap between signals/channels - FFS: condition to apply including potential UE capability definition - Study should account for inputs from RAN4

For PDSCH scheduling UE provides the following capability [3GPP TS 38.306]:

timeDurationForQCL Defines minimum number of OFDM symbols required by the UE to perform PDCCH reception and applying spatial QCL information received in DCI for PDSCH processing as described in TS 38.214 [12] clause 5.1.5. UE shall indicate one value of the minimum number of OFDM symbols per each subcarrier spacing of 60kHz and 120kHz.

As can be seen from the description the UE indicates one value per each subcarrier spacing of 60 and 120 kHz. Correspondingly, as now new subcarrier spacings are introduced the UE shall provide one value for the timeDurationForQCL for the new SCSs 480 and 960 kHz. Possible values for the timeDurationForQCL with 120 kHz SCS are 14 and 28 symbols. The same values in absolute time domain should be the maximum considered values but it should also be discussed whether shorter values in time could be supported as the SCS increases. It’s to be noted that with the same absolute times for instance the second capability value with 960 kHz SCS would correspond to 16 slots. That would practically mean that all PDSCHs in multi-PDSCH transmission would be received by the UE assuming default PDSCH QCL assumption. Thus, our considered values are as follows:

| µ | timeDurationForQCL value #1 [symbols] | timeDurationForQCL value #2 [symbols] |
| --- | --- | --- |
| 3 | 14 | 28 |
| 5 | ≤56 | ≤112 |
| 6 | ≤112 | ≤224 |

Proposal 1: Define parameter values (UE capabilities) for the timeDurationForQCL for the PDSCH scheduling for 480 kHz and 960 kHz SCS and values should be:

- first value: ≤ 56 symbols (≤ 4 slots) and second value: ≤ 112 symbols (≤ 8 slots) with 480 kHz SCS

- first value: ≤ 112 symbols (≤ 8 slots) and second value: ≤ 224 symbols (≤ 16 slots) with 960 kHz SCS

For A-CSI-RS triggering the UE provides the following capability dependent on the subcarrier spacing [3GPP TS 38.331]:

beamSwitchTiming                    SEQUENCE {

scs-60kHz                           ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL,

scs-120kHz                          ENUMERATED {sym14, sym28, sym48, sym224, sym336}                           OPTIONAL

Similarly, as new subcarrier spacings are introduced the UE shall provide beamSwitchTiming for 480 or 960 kHz SCS. At least for the two last values (that were introduced to take into account UE’s possible panel switch on time when the UE is triggered A-CSI-RS with repetition “ON”) could be defined in terms of slots.

Proposal 2: Define parameter values (UE capabilities) for the beamSwitchTiming for the A-CSI-RS triggering for 480 kHz and 960 kHz SCS and values should be

- {≤ 56 symbols/4 slots, ≤112 symbols/8 slots, ≤192 symbols, ≤64 slots, ≤96 slots) with 480 kHz SCS

- {≤ 112 symbols/8 slots, ≤224 symbols/16 slots, ≤384 symbols, ≤128 slots, ≤192 slots) with 960 kHz SCS

For beamReportTiming the possible values are {14, 28, 56} symbols for 120 kHz SCS. Corresponding maximum values for 480 and 960 kHz SCSs are as follows:

- {≤ 56 symbols/4 slots, ≤112 symbols/8 slots, ≤224 symbols/16 slots} with 480 kHz SCS

- {≤ 112 symbols/8 slots, ≤224 symbols/16 slots, ≤448 symbols/32 slots} with 960 kHz SCS

Proposal 3: Define parameter values (UE capabilities) for the beamReportTiming for 480 kHz and 960 kHz SCS and values should be:

- {≤ 56 symbols/4 slots, ≤112 symbols/8 slots, ≤224 symbols/16 slots} with 480 kHz SCS

- {≤ 112 symbols/8 slots, ≤224 symbols/16 slots, ≤448 symbols/32 slots} with 960 kHz SCS

maxNumberRxTxBeamSwitchDL defines the number of Tx and Rx beam changes UE can perform on the band within a slot. UE shall report one value per each subcarrier spacing supported by the UE. Available values for 120 kHz and 240 kHz SCSs are as follows:

maxNumberRxTxBeamSwitchDL                   SEQUENCE {

…

scs-120kHz                                  ENUMERATED {n4, n7, n14}                     OPTIONAL,

scs-240kHz                                  ENUMERATED {n4, n7, n14}                     OPTIONAL

}

Considering the UE’s reception of PDCCH and PDSCH within a slot in case different TCI states are used for those the UE would need be to able to perform at least two RX beam changes within a slot (first to receive PDCCH and second to receive PDSCH). Thus, the supported values should be ≥2.

Proposal 4: Values for maxNumberRxTxBeamSwitchDL should be ≥2 for both 480 and 960 kHz SCS.

Regarding

  - Additional beam switching time delay d for triggering AP-CSI-RS when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS

it’s noted that [TS38.214] defines that additional time delay d is used with the following condition:

“where if the µPDCCH < µCSIRS, the beam switching timing delay d is defined in Table 5.2.1.5.1a-1, else d is zero”

Following the same principle for the case PDCCH with smaller subcarrier spacing than AP-CSI-RS the d non-zero value could be defined.

Proposal 5: Additional time delay d is defined when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS. Value(s) for new SCSs FFS.

Regarding “whether/how to introduce a beam switching gap between signals/channels” it’s to be noted that RAN4 has studied analog beam switching as part of the SI and conclusions from BS perspective were captured into TR 38.808, taking into account both the time for analog components to change state and timing inaccuracy of control interface. The outcome was that to avoid performance degradations switching should take place in or less than 59 ns time window. There are phase shifter technologies which can react in approximately 10 ns, leaving sufficient time for the possible delays in the control interface and no explicit switching gap is needed between successive SSB blocks. Based on this analysis, it was further concluded that both 480 kHz and 960 kHz SCS are feasible from beam switching point of view.

Based on RAN4 study it can be concluded that there is no need to introduce beam switching gap between signals and channels.

Proposal 6: No explicit beam switching gap is introduced between DL signals and channels.

## On the beam indication in multi-PDSCH transmission

RAN1#104-e made the following agreements related to QCL assumption/TCI state indication:

Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when some of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL while some have scheduling offset equal to or greater than timeDurationForQCL. - For multi-PDSCH scheduling with a single DCI, study the QCL assumption(s) the UE should apply for each PDSCH for the case when all of the scheduled PDSCHs have scheduling offset less than timeDurationForQCL - Note: If the current Rel-16 behavior would be extended to multiple-PDSCH scheduling, it could result in a different QCL assumption for each PDSCH due to the fact the that the CORESET with the lowest ID can be different for different slots, resulting in a potentially different TCI state for each slot - Note: Applicability to multi-TRP can be discussed further Agreement: Further study the following: - For multi-PDSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate TCI state for each scheduled PDSCH - For multi-PUSCH scheduling with a single DCI, study whether or not it is needed to indicate a separate SRI (indication of TCI can be further discussed) for each scheduled PUSCH - Note: the study should take into account DCI overhead aspects - Note: Applicability to multi-TRP can be discussed further

UE provided capability timeDurationForQCL defines the required scheduling offset, i.e. time between the scheduling DCI and scheduled PDSCH, that the UE applies TCI state provided in DCI or TCI state of the scheduling PDCCH (if no TCI field in DCI) for the PDSCH reception. Otherwise the UE applies so called default PDSCH beam which is defined (in single TRP scenario) as follows [3GPP TS 38.214]:

“Independent of the configuration of tci-PresentInDCI and tci-PresentDCI-1-2 in RRC connected mode, if the offset between the reception of the DL DCI and the corresponding PDSCH is less than the threshold timeDurationForQCL and at least one configured TCI state for the serving cell of scheduled PDSCH contains qcl-Type set to 'typeD',

-	the UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId in the latest slot in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE.”

Above can then be described as in the following figure where gNB schedules UE with 8 PDSCHs and assuming timeDurationForQCL being 4 slots (Figure 1):

Figure 1 Impact of UE’s capability timeDurationForQCL to QCL assumptions for multi-PDSCH.

As can be noted from the figure QCL assumption for the first four PDSCHs would be different to the last four PDSCHs. For the first four PDSCHs UE would apply so called “default PDSCH QCL assumption” and for the rest of the PDSCHs either the QCL assumption according to the TCI state of the scheduling PDCCH or the TCI state provided in the DCI. However, it’s considered that the same transmit beam (and thus the same QCL assumption) would be used for all the PDSCHs, at least in single TRP scenario. Of course, the gNB would be able to guarantee by the configuration that the same QCL assumption is in practice used for all the PDSCHs, e.g. by not providing TCI state in DCI and having only the same active TCI state shared by configured CORESET(s). On the other hand, in general while SSB is QCL-TypeD source for CORESET#0 and CSI-RS may be used as QCL-TypeD source for CORESETs other than zero. Thus, allowing flexible configuration of CORESETs and QCL sources it may be beneficial to consider specification work to enable single QCL assumption for all the PDSCHs in multi-PDSCH scheduling when some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

Observation 1: If some of PDSCHs in multi-PDSCH scheduling are allocated with scheduling offset less than timeDurationForQCL the UE would have different QCL assumptions for the PDSCHs allocated with scheduling offset than timeDurationForQCL and for the PDSCH allocated with scheduling offset equal to and greater than timeDurationForQCL.

It’s also to be noted that 3GPP TS 38.214, section 5.1.5, says that in case of scheduling offset is equal to or greater than timeDurationForQCL and multi-slot PDSCH is configured the UE can assume single TCI state for across the slots [3GPP TS 38.214, 5.1.5]:

“The UE may assume that the DM-RS ports of PDSCH of a serving cell are quasi co-located with the RS(s) in the TCI state with respect to the QCL type parameter(s) given by the indicated TCI state if the time offset between the reception of the DL DCI and the corresponding PDSCH is equal to or greater than a threshold timeDurationForQCL, where the threshold is based on reported UE capability [13, TS 38.306]. When the UE is configured with a single slot PDSCH, the indicated TCI state should be based on the activated TCI states in the slot with the scheduled PDSCH. When the UE is configured with a multi-slot PDSCH, the indicated TCI state should be based on the activated TCI states in the first slot with the scheduled PDSCH, and UE shall expect the activated TCI states are the same across the slots with the scheduled PDSCH.”

Proposal 7: Support single TCI state or QCL assumption for the multi-PDSCH transmission in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

In above scenario there was no another search space set within the slots on which the scheduled PDSCHs were allocated at the earlier search space set. However, we can consider another scenario where another search space is configured within timeDurationForQCL. That scenario is depicted in Figure 2. In there it’s assumed that PDCCH in the CORESET defined by the first search space set schedules eight PDSCHs. After four slots there is another monitoring occasion defined by the other search space set. As can be seen there would be effectively three different QCL assumptions for the PDSCH reception while the intention would be to use a single beam for the PDSCHs.

Figure 2 Monitorin occasion of another search space set within the slots of the scheduled PDSCHs.

Then also in this case the gNB can guarantee that actually the same beam, and thus the same QCL-TypeD RS can be assumed by the UE for all the PDSCHs (CORESETs are having the same active TCI state). This can be guaranteed by scheduling restriction such as same QCL assumption is applied any two lowest indexed CORESETs where the offset is less than timeDurationForQCL. Otherwise, gNB should ensure that multi-PDSCH scheduling doesn’t extend the slots of next search space having different QCL assumption.

Observation 2: gNB can by the configuration/scheduling guarantee that the UE may apply the same QCL-TypeD RS for the reception of the multi-PDSCH transmission even though some of the PDSCHs would have scheduling offset less than timeDurationForQCL.

Proposal 8: NW ensures single TCI state or QCL assumption across the slots for the multi-PDSCH transmission.

Similarly, multi-PUSCH transmission, other than mulit-TRP operation, there is no clear reason to support beam switching within short period. Multi-PUSCH transmission is intended for reducing DCI overhead by applying the same UL transmission parameters.

Proposal 9: Support single SRI or a single common UL TCI state(if supported) is used for a multi-PUSCH transmission.

## LBT impact on Beam Management

### TRS

In certain unlicensed scenario regulated maximum allowed EIRP limits (e.g. CEPT scenarios c1 and c2 with 40 dBm max EIRP) are such that larger arrays compared to FR2 may not be needed. Thus, beam dimensioning based on FR2 would be enough, e.g. in terms of maximum number of supported SSBs, and beam management procedures developed in Rel. 15 and Rel. 16 are expected to provide a good baseline for unlicensed operation above 52.6 GHz.

On the other hand, channel access mechanism(s) may have impact on the beam management. Depending on the co-existence scheme, use of periodic reference signals in beam management (e.g. in beam failure detection) may need to be reconsidered. Beam management relies heavily on periodic signals, more specifically on periodic TRS (P-TRS) as QCL source for DL signals and channels. Furthermore, beam failure detection RS and candidate RSs for new beam identification in defined beam failure recovery procedure need to be periodic, and typically failure detection RSs are P-TRSs as being active QCL sources for the PDCCH monitoring in CORESETs. Based on QCL source(s) RS UE prepares channel estimation filters (time and frequency domain estimates like delay spread, doppler spread) and sets its RX beam for the reception of the downlink signals and channels. It can also be noted that in typical deployment the same periodic RSs are used as spatial source for uplink signals and channels, i.e. DL reference signals based on which the UE forms the transmit beam(s) for uplink transmissions. Typical QCL configuration for the downlink signals and channels is provided in Figure 3 (where the RS in the start of the arrow represents the source and the signal in the end of the arrow represents the target).

Figure 3 Typical QCL source configuration for the target signals (signals in the end of the arrows).

One problem is that considered channel access mechanism, e.g. LBT, may prevent transmission of P-TRS that is the main QCL source for different signals and channels. Thus, UE may not have up to date QCL source for coming signals/channels to be received (or to be transmitted) and that would impact negatively on the downlink performance but as well the uplink performance. Furthermore, P-TRS must be validated by gNB in sub-7 NR-U, which is inefficient. Thus, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure within a period. That could be achieved e.g. by defining a P-TRS burst structure that has multiple opportunities within a certain period.

Observation 3: For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure.

Potential solutions could be for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

Proposal 10: Consider solutions to provide robustness for TRS transmission due to LBT failures, for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered for one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

### BFD-RS

There was a proposal for a method for guaranteeing transmission of BFD-RS against LBT failure. In fact, BFD-RSs are perdic CSI-RS resources configured explicitly or implicitly. For explicit configuration, BFD-RS is explicitly configured with a set of periodic CSI-RS resource. For implicit configuration, BFD-RS is determined by periodic CSI-RS resource indicated by TCI-state for respective CORESETs for UE monitoring.

Thus, we don’t need to discuss BFD-RS separately for LBT fauilure handling. In addition, generally, periodic TRS is likely to be used for BFD-RS, if mechanism of handling LBT failure for periodic TRS is supported, this is automatically supported.

If other types of CSI-RS such as CSI-RS for acquisition or CSI-RS for beam management are used for BFD-RS, the same mechanism as TRS can be used.

Observation 4: More transmission opportunities for the BFD-RS against LBT failures can be supported by the same mechanism used for peridic CSI-RS such as TRS.

## Beam Management in the COT

If directional LBT is applied e.g. for the shared COT by the gNB, it is assumed that the subsequent transmissions in the COT can take place using transmit beam(s) confined within the spatial directions/domain sensed by the LBT beam(s). That may have impact on the validity of the configured DL RSs for L1-RSRP measurement and reporting as well as on the beam switching (e.g. MAC CE based TCI state update/switch) during the COT.

E.g. beam switching during the COT may need to differentiate the following cases:

1)	The new beam is within (one of) the LBT beam(s) of the COT, and

2)	The new beam is not QCLed with the LBT beam of the COT meaning that it cannot be seen to be within the spatial directions/domain sensed by the LBT beam(s).

In 2) it would be important to the gNB be able switch the beam for the next COT while still be serving the UE with the old beam during the current COT. Thus, e.g. application time of the beam switching command could take into account whether or not the new beam is within the LBT beam of the COT.

Proposal 11: In case of directional LBT (if applied), consider impacts on beam management in the COT, e.g.

- impact on validity of the configured DL RSs for L1-RSRP measurement and reporting and

- impact on beam switching application time within the COT (e.g. the case when the new beam is or is not QCLed with the LBT beam of the COT).

## Multi-slot CSI-RS

When operating with higher SCS, like 960 kHz SCS, PDCCH monitoring periodicity may likely be in absolute time domain similar to operation at 120 kHz SCS in order to keep PDCCH processing requirements similar to FR2. To reflect that it has been agreed to support multi-slot PDSCH and PUSCH scheduling. The same logic can be considered e.g. for the CSI-RS as well, i.e. to enable multi-slot aperiodic CSI-RS transmission. That can be provided e.g. with one of the following alternatives:

- Alt1: Use slot offset (could reuse the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated

- Alt2: Enable a single triggering state to have multiple CSI-RS resource sets for channel measurements where the sets can be configured with different slot offsets relative to each other

From the above alternatives, the Alt1 would require less standardization work and would be more simple. Thus, if multi-slot CSI-RS is introduced, Alt1 would be preferred approach.

Observation 5: Support of multi-slot CSI-RS can be provided by having a slot offset (could reuse the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated.

# Conclusion

Based on the discussion in the previous chapter we made the following observations and proposals:

Proposal 1: Define parameter values (UE capabilities) for the timeDurationForQCL for the PDSCH scheduling for 480 kHz and 960 kHz SCS and values should be:

- first value: ≤ 56 symbols (≤ 4 slots) and second value: ≤ 112 symbols (≤ 8 slots) with 480 kHz SCS

- first value: ≤ 112 symbols (≤ 8 slots) and second value: ≤ 224 symbols (≤ 16 slots) with 960 kHz SCS

Proposal 2: Define parameter values (UE capabilities) for the beamSwitchTiming for the A-CSI-RS triggering for 480 kHz and 960 kHz SCS and values should be

- {≤ 56 symbols/4 slots, ≤112 symbols/8 slots, ≤192 symbols, ≤64 slots, ≤96 slots) with 480 kHz SCS

- {≤ 112 symbols/8 slots, ≤224 symbols/16 slots, ≤384 symbols, ≤128 slots, ≤192 slots) with 960 kHz SCS

Proposal 3: Define parameter values (UE capabilities) for the beamReportTiming for 480 kHz and 960 kHz SCS and values should be:

- {≤ 56 symbols/4 slots, ≤112 symbols/8 slots, ≤224 symbols/16 slots} with 480 kHz SCS

- {≤ 112 symbols/8 slots, ≤224 symbols/16 slots, ≤448 symbols/32 slots} with 960 kHz SCS

Proposal 4: Values for maxNumberRxTxBeamSwitchDL should be ≥2 for both 480 and 960 kHz SCS.

Proposal 5: No additional time delay d is defined when triggering PDCCH with 120kHz or 480kHz has a smaller subcarrier spacing than AP-CSI-RS.

Proposal 6: No explicit beam switching gap is introduced between DL signals and channels.

Observation 1: If some of PDSCHs in multi-PDSCH scheduling are allocated with scheduling offset less than timeDurationForQCL the UE would have different QCL assumptions for the PDSCHs allocated with scheduling offset than timeDurationForQCL and for the PDSCH allocated with scheduling offset equal to and greater than timeDurationForQCL.

Proposal 7: Support single TCI state or QCL assumption for the multi-PDSCH transmission in case of some of the PDSCHs are having lower scheduling offset than timeDurationForQCL.

Observation 2: gNB can by the configuration guarantee that the UE may apply the same QCL-TypeD RS for the reception of the multi-PDSCH transmission even though some of the PDSCHs would have scheduling offset less than timeDurationForQCL both in single-TRP and multi-TRP (two coresetPoolIndices configured) configurations.

Proposal 8: NW ensures single TCI state or QCL assumption across the slots for the multi-PDSCH transmission.

Proposal 9: Support single SRI or a single common UL TCI state is used for a multi-PUSCH transmission.

Observation 3: For P-TRS transmissions in the cell, it would be beneficial to have a mechanism to be able to transmit P-TRSs dropped due to LBT failure.

Proposal 10: Consider solutions to provide robustness for TRS transmission due to LBT failures, for instance:

- A beam specific (SSB specific) aperiodic TRS transmission that could be triggered for one or multiple UEs at a time to “patch” non-transmitted P-TRS using certain beam (certain SSB as QCL-TypeD source)

- Multiple transmission opportunities for the P-TRS within a time period

Observation 4: If there is need to provide more transmission opportunities for the BFR-RS against LBT failures, the same mechanism should be considered as for TRS.

Proposal 11: In case of directional LBT (if applied), consider impacts on beam management in the COT, e.g.

- impact on validity of the configured DL RSs for L1-RSRP measurement and reporting and

- impact on beam switching application time within the COT (e.g. the case when the new beam is or is not QCLed with the LBT beam of the COT).

Observation 5: Support of multi-slot CSI-RS can be provided by having a slot offset (could reuse the parameter CSI-ResourcePeriodicityAndOffset currently applicable only for periodic and semi-persistent resource) parameter for the aperiodic CSI-RS resource where the offset would be calculated from the slot where the first CSI-RS resource of the same set is allocated.

# References

- RP-210862, “Revised WID: Extending current NR operation to 71 GHz”, CMCC

- TR 38.808, “Study on supporting NR from 52.6GHz to 71 GHz", 3GPP

- RAN1#104-e Chairman Notes