| 3GPP TSG-RAN WG1 Meeting #104b e-Meeting, April 12th – 20th, 2021 | R1-2103175 |
| --- | --- |

Agenda item:	8.6.1.2

Source: 	Qualcomm Incorporated

Title: 	RX Branch Reduction for RedCap UE

Document for:	Discussion/Decision

# Introduction

To meet the generic and use specific requirements of mid-range use cases, potential UE complexity reduction techniques were identified and analyzed for wearables, industry wireless sensors and surveillance cameras [1]. The RAN1 study for R17 RedCap devices was completed in Q4 2020, and a revised WID [2] was approved in RAN-91e meeting. A main objective of the WI is to specify support for the following UE complexity reduction features in RAN1, RAN2 and RAN4:

- Reduced minimum number of Rx branches:

  - For frequency bands where a legacy NR UE is required to be equipped with a minimum of 2 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - For frequency bands where a legacy NR UE (other than 2-Rx vehicular UE) is required to be equipped with a minimum of 4 Rx antenna ports, the minimum number of Rx branches supported by specification for a RedCap UE is 1. The specification also supports 2 Rx branches for a RedCap UE in these bands.

  - A means shall be specified by which the gNB can know the number of Rx branches of the UE.

- Maximum number of DL MIMO layers:

  - For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported.

  - For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported.

In this contribution, we discuss the PHY impacts of RX branch reduction for R17 RedCap UE.

# DL Coverage Recovery

Based on the revised WID for R17 RedCap UE [2], the minimum number of RX branches in all FR1 bands is 1. The NR R17 specification will also support 2 RX branches in these bands.

The following observations have been made in [1] for the reduced number of RX branches and reduced antenna efficiency:

- DL coverage recovery for RedCap UE is needed for FR1 only

- For RedCap UE with 1 Rx branch and reduced antenna efficiency,  the need for coverage recovery depends on the frequency bands and DL PSD:

- For carrier frequency of 4 GHz with DL PSD 24 dBm/MHz, coverage recovery may be needed for the downlink channels of Msg2, Msg4 and PDCCH CSS. A small or moderate compensation can be considered, where the square brackets indicate that the exact amount will depend on the techniques, scenarios, etc.:

-	[1 dB] for PDCCH CSS

-	[2-3 dB] for Msg4

-	[6 dB] for Msg2 without TBS scaling. It is noted that coverage loss for Msg2 can be compensated by using the existing TBS scaling technique.

-	For other carrier frequencies or DL PSD of 33 dBm/MHz, coverage recovery is not needed for the downlink channels if the target for coverage recovery is based on the MIL of the bottleneck channel for the reference NR UE.

- For RedCap UE with 2 Rx branches and reduced antenna efficiency, the need for coverage recovery also depends on the frequency bands and DL PSD:

- For carrier frequency of 4 GHz with DL PSD 24 dBm/MHz, coverage recovery may be needed for the downlink channels of Msg2. A small or moderate compensation can be considered, where the square brackets indicate that the exact amount will depend on the techniques, scenarios, etc.:

-	[1 dB] for Msg2 without TBS scaling. It is noted that coverage loss for Msg2 can be compensated by using the existing TBS scaling technique.

- For other carrier frequencies or DL PSD of 33 dBm/MHz, coverage recovery is not needed for the downlink channels if the target for coverage recovery is based on the MIL of the bottleneck channel for the reference NR UE.

On the other hand, developing DL coverage recovery solutions specific for RedCap UE is out of the scope of R17 WI [2].  Nevertheless, gNB can improve the DL coverage of RedCap UE by using the solutions available to non-RedCap UEs in NR R15/16, which include at least:

- TB scaling for msg2 PDSCH or msgB PDSCH

- Low MCS

- PDSCH repetition

- Power boosting of gNB transmitter

- VRB-to-PRB interleaving

- Large AL for PDCCH

DL coverage recovery for RedCap UE can be triggered by early indication of reduced capabilities, early CSI reporting during initial access, or other UE assistance information carried by PUCCH, PUSCH or UCI multiplexed with PUSCH.

Observation 1: DL coverage of RedCap UE can be improved by solutions available to non-RedCap UEs in NR R15/16, such as:

- TB scaling for msg2 PDSCH or msgB PDSCH

- Low MCS

- PDSCH repetition

- Power boosting of gNB transmitter

- VRB-to-PRB interleaving

- Large AL for PDCCH

Observation 2: DL coverage recovery of RedCap UE can be triggered by early indication of reduced capabilities, early CSI reporting during initial access, or other UE assistance information carried by PUCCH, PUSCH or UCI multiplexed with PUSCH.

# Reduction of PDCCH Blocking Rate

Due to the introduction of RedCap UEs and their needs for DL coverage recovery, the blocking rate of PDCCH can potentially increase. To improve the co-existence of RedCap UE and non-RedCap UE, solutions capable of reducing the PDCCH blocking rate should be considered in NR R17 RedCap WI.

In NR R15/16, compact DCI formats (e.g. 0_2 and 1_2) have been used for the dynamic and semi-static scheduling of PDSCH and PUSCH. Although the PDCCH transmitted in USS is not identified as the bottleneck of DL coverage [1], the payload size reduction of DCI is able to reduce the probability of PDCCH blocking, especially when RedCap UE and non-RedCap UE co-exist in the same cell or same BWP. Therefore, compact DCI formats should be supported by RedCap UE, and the size of DCI fields should be aligned with reduced UE capabilities. For example:

- CA is not supported  carrier indicator field is not included

- BW reduction  range of FDRA is upper bounded by max UE BW of 20 MHz

- Max modulation order relaxation on DL bit-width reduction for MCS field

- DL MIMO layer reduction  bit-width reduction for DCI fields of precoding, MIMO layer indication, antenna port indication, CSI request and etc.

- Latency relaxation and coverage recovery due to reduced capabilities  joint optimization of RV cycling order and bit-width of RV indication field

On the other hand, for RedCap UEs with semi-static or periodic traffic patterns, SPS scheduling and configured grant can be applied to reduce the overhead of dynamic grant. In NR R17, small data transmission (SDT) is specified for RRC inactive UEs. The RACH-based or CG-based transmission can reduce the PDCCH blocking rate in initial DL BWP.

Observation 3: Reduction of PDCCH blocking rate can improve the co-existence of RedCap UE and non-RedCap UE.

Proposal 1: To enable PDCCH blocking rate reduction, the following solutions should be supported or further studied for RedCap UEs in NR R17:

- Compact DCI format aligned with reduced UE capabilities

- Joint optimization of  RV cycling order and number of repetitions

- SPS-based and CG-based transmission in RRC connected state

- RACH-based or CG-based SDT in RRC inactive state

# CSI Measurements and Reporting Procedures

As noted in [1], RX branch number reduction (from 2 to 1, 4 to 2, or 4 to 1) as well as antenna efficiency reduction due to device size limitation are expected to impact the receiver characteristics, demodulation performance requirements, CSI reporting, RRM/RLM measurements, HO and cell (re)selection procedures of RedCap UE.

As a baseline, the CSI resource setting and reporting setting of non-RedCap UE can be re-used. The content of CSI report is specified by reportQuantity, which includes a limited number of combinations including cri-RSRP, ssb-index-RSRP, CRI, RI, LI, CQI, PMI and index of desired precoding matrix. Due to reduced number of RX branches and reduced number of DL MIMO layers [2], simplified CSI measurements and reporting procedures can be supported in NR R17. For RedCap UE with 1 RX branch, the max number of DL MIMO layer is 1. Therefore, it is not necessary for a 1 RX UE to report RI, and the reporting quantities of CSI can be reduced for RedCap UE.

Observation 4: The CSI resource setting and CSI reporting setting of non-RedCap UE can be used as a baseline for RedCap UE. The contents and quantities of CSI report can be reduced as a result of RX branch number reduction and DL MIMO layer reduction.

Proposal 2: For Redcap UEs with reduced number of RX branches and reduced antenna efficiency:

- their impacts on receiver characteristics, RRM/RLM measurement requirements and demodulation performance requirements can be specified by RAN4;

- their impacts on HO and cell (re)selection procedures can be specified by RAN2.

# Conclusions

In this contribution, we discussed the PHY impacts of reduced RX branches for R17 RedCap devices. To summarize, we have the following observations and proposals:

Observation 1: DL coverage of RedCap UE can be improved by solutions available to non-RedCap UEs in NR R15/16, such as:

- TB scaling for msg2 PDSCH or msgB PDSCH

- Low MCS

- PDSCH repetition

- Power boosting of gNB transmitter

- VRB-to-PRB interleaving

- Large AL for PDCCH

Observation 2: DL coverage recovery of RedCap UE can be triggered by early indication of reduced capabilities, early CSI reporting during initial access, or other UE assistance information carried by PUCCH, PUSCH or UCI multiplexed with PUSCH.

Observation 3: Reduction of PDCCH blocking rate can improve the co-existence of RedCap UE and non-RedCap UE.

Observation 4: The CSI resource setting and CSI reporting setting of non-RedCap UE can be used as a baseline for RedCap UE. The contents and quantities of CSI report can be reduced as a result of RX branch number reduction and DL MIMO layer reduction.

Proposal 1: To enable PDCCH blocking rate reduction, the following solutions should be supported or further studied for RedCap UEs in NR R17:

- Compact DCI format aligned with reduced UE capabilities

- Joint optimization of  RV cycling order and number of repetitions

- SPS-based and CG-based transmission in RRC connected state

- RACH-based or CG-based SDT in RRC inactive state

Proposal 2: For Redcap UEs with reduced number of RX branches and reduced antenna efficiency:

- their impacts on receiver characteristics, RRM/RLM measurement requirements and demodulation performance requirements can be specified by RAN4;

- their impacts on HO and cell (re)selection procedures can be specified by RAN2.

# References

- 3GPP TR 38.875, “Study on Support of Reduced Capability NR Devices,” December 2020.

- RP-210918, “Revised WID on Support of Reduced Capability NR Devices,” 3GPP TSG RAN#91e, March 22nd to 26th ,  2021.