3GPP TSG RAN WG1 #104b-e                                         R1-2103879

e-Meeting, April 12th – 20th, 2021‎

Source:	CATT

Title:	Maintenance of Multi-beam enhancement operation

Agenda Item:	7.2.6

Document for:	Discussion and Decision

- Introduction

In this contribution, we deal with remaining issues on multi-beam operation in NR Rel-16 and provide our TPs.

- Text Proposal on default pathloss reference RS

Reason for changes:

In current spec, the higher layer parameter enableDefaultBeamPL-ForPUSCH0-0-r16 is not consistent with those in TS 38.213 that specifies the default pathloss reference signal/beam for PUSCH.

Summary of changes:

In section 7.1.1 of TS 38.213, update the parameter enableDefaultBeamPL-ForPUSCH0-0.

Specs/Sections impacted:

TS38.213 V16.4.0 /Section 7.1.1.

Consequences if not approved:

Higher-layer parameter misalignment between 38.213 and 38.331.

The text proposal for TS38.213 V16.4.0 is

- If - the PUSCH transmission is scheduled by DCI format 0_0 on serving cell c, - the UE is not provided a spatial setting for PUCCH resources on the active UL BWP of the primary cell [11, TS 38.321], and - the UE is provided enableDefaultBeamPL-ForPUSCH0-0-r16 the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the serving cell c

The text proposal for section 7.2.1 of TS38.213 V16.4.0 is

If the UE - is not provided pathlossReferenceRSs, and - is not provided PUCCH-SpatialRelationInfo, and - is provided enableDefaultBeamPL-ForPUCCH-r16, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index qd providing a periodic RS resource configured with with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

The text proposal for section 7.3.1 of TS 38.213 V16.4.0 is

If the UE - is not provided pathlossReferenceRS or SRS-PathlossReferenceRS-Id, - is not provided spatialRelationInfo, and - is provided enableDefaultBeamPL-ForSRS-r16, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index providing a periodic RS resource configured with qcl-Type set to 'typeD' in - the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP, if CORESETs are provided in the active DL BWP of serving cell c

- Editorial issue

Reason for changes:

In section 7.2.1 of TS38.213, there is a typo.

Summary of changes:

In section 7.2.1 of TS 38.213, delete the word “with”.

Specs/Sections impacted:

TS38.213 V16.4.0 /Section 7.2.1.

Consequences if not approved:

The spec is not precise.

The text proposal for TS38.213 V16.4.0 is

If the UE - is not provided pathlossReferenceRSs, and - is not provided PUCCH-SpatialRelationInfo, and - is provided enableDefaultBeamPL-ForPUCCH, and - is not provided coresetPoolIndex value of 1 for any CORESET, or is provided coresetPoolIndex value of 1 for all CORESETs, in ControlResourceSet and no codepoint of a TCI field, if any, in a DCI format of any search space set maps to two TCI states [5, TS 38.212] the UE determines a RS resource index qd providing a periodic RS resource configured with qcl-Type set to 'typeD' in the TCI state or the QCL assumption of a CORESET with the lowest index in the active DL BWP of the primary cell. For a PUCCH transmission over multiple slots, a same qd applies to the PUCCH transmission in each of the multiple slots.

- References

- 3GPP RAN1, TS 38.214 V16.4.0

- 3GPP RAN1, TS38.331 V16.4.0