## Study on NB-IoT/eMTC support for Non-Terrestrial Network

Please refer to RP-202689 for detailed scope of the WI

R1-2102753	FS_LTE_NBIOT_eMTC_NTN work plan	MediaTek Inc.

### Scenarios applicable to NB-IoT/eMTC

R1-2102343	Application scenarios of IoT in NTN	Huawei, HiSilicon

R1-2102422	Discussion on scenarios applicable to NB-IoT/eMTC	OPPO

R1-2102550	Discussion on scenarios applicable to NB-IoT_eMTC for NTN	vivo

R1-2102617	Applicable scenario and initial evaluation result to NB-IoT/eMTC	CATT

R1-2102750	Discussion on NTN-IoT scenario with MEO	Hughes/EchoStar

R1-2102754	Scenarios applicable to IoT NTN	MediaTek Inc.

R1-2102831	Link budget evaluations for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102905	Discussion on scenarios applicable to NB-IoT/eMTC	CMCC

R1-2102916	Discussion on the scenarios and assumption for IoT-NTN	ZTE

R1-2102972	Discussion on the link budget of NBIoT and eMTC over NTN	Xiaomi

R1-2103060	On scenarios and evaluations for NB-IoT and eMTC based NTN	Ericsson

R1-2103070	Scenarios applicable to NB-IoT/eMTC 	Qualcomm Incorporated

R1-2103132	Link Budget Analysis of IoT NTN	Apple

R1-2103266	Initial link budget evaluation for NB-IoT/eMTC	Samsung

R1-2103318	Scenarios for IoT-NTN	Sony

R1-2103716	Link budget analysis for Set-4	Sateliot, Gatehouse, Thales

R1-2103826	Summary #1 of AI 8.15.1 Scenarios applicable to NB-IoT/eMTC Document for: Discussion and Decision		Moderator (MediaTek)

[104b-e-NR-NB_IoT_eMTC-01] Email discussion/approval on scenarios applicable to NB-IoT/eMTC with checkpoints for agreements on Apr-15, Apr-20 – Gilles (MediaTek)

### Enhancements to time and frequency synchronization

Including random access procedure/signals

R1-2102758	"Summary #1 of AI 8.15.2  Enhancements to time and frequency

synchronization"	MediaTek Inc.

R1-2102344	Discussion on time and frequency synchronization enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102423	Discussion on enhancements to time and frequency synchronization	OPPO

R1-2102473	Consideration on enhancements to time and frequency synchronization	Spreadtrum Communications

R1-2102618	Time and frequency synchronization for NB-IoT/eMTC	CATT

R1-2102736	Enhancements to time and frequency synchronization	Asia Pacific Telecom, FGI, ITRI, III

R1-2102755	Enhancements to time and frequency synchronization for IoT NTN	MediaTek Inc.

R1-2102832	Enhancement to time and frequency synchronization for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102906	Enhancements to time and frequency synchronization for IoT  NTN	CMCC

R1-2102917	Discussion on the synchronization for IoT-NTN	ZTE

R1-2102973	Discussion on time and frequency synchronization for IoT NTN	Xiaomi

R1-2103056	On synchronization for NB-IoT and eMTC NTN	Intel Corporation

R1-2103061	On time and frequency synchronization enhancements for IoT NTN	Ericsson

R1-2103071	Enhancements to time and frequency synchronization	Qualcomm Incorporated

R1-2103133	On Time and Frequency Synchronization in IoT NTN	Apple

R1-2103267	On enhancements to time and frequency synchronization	Samsung

R1-2103273	Time/Frequency Synchronization for IoT NTN	InterDigital, Inc.

R1-2103319	Time and frequency synchronisation enhancements for IoT-NTN	Sony

R1-2103528	Time and frequency synchronization for IoT NTN	Lenovo, Motorola Mobility

R1-2102758	Summary #1 of AI 8.15.2 	Enhancements to time and frequency

synchronization	Moderator (MediaTek)

[104b-e-NR-NB_IoT_eMTC-02] Email discussion/approval on enhancements to time and frequency synchronization with checkpoints for agreements on Apr-15, Apr-20 – Gilles (MediaTek)

### Timing relationship enhancements

R1-2102345	Discussion on timing relationship enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102424	Discussion on timing relationship enhancements	OPPO

R1-2102474	Consideration on timing relationship enhancements	Spreadtrum Communications

R1-2102619	Timing relationship enhancement for NB-IoT/eMTC	CATT

R1-2102737	Timing relationship enhancements	Asia Pacific Telecom, FGI, ITRI, III

R1-2102756	Timing relationship enhancements for IoT NTN	MediaTek Inc.

R1-2102800	Timing relationship enhancements to support NB-IoT eMTC in Non-Terrestrial Network	Zhejiang Lab

R1-2102833	Timing relationship enhancements for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102907	Timing relationship enhancements  for IoT  NTN	CMCC

R1-2102918	Discussion on timing relationship for IoT-NTN	ZTE

R1-2102974	Discussion on the timing relationship enhancement for IoT NTN	Xiaomi

R1-2103057	On timing relationship for NB-IoT and eMTC NTN	Intel Corporation

R1-2103062	On timing relationship enhancements for IoT NTN	Ericsson

R1-2103072	Timing relationship enhancements	Qualcomm Incorporated

R1-2103134	On Timing Relationship Enhancement in IoT NTN	Apple

R1-2103268	Timing relationship enhancements	Samsung

R1-2103274	Timing relationship enhancement for IoT NTN	InterDigital, Inc.

R1-2103320	Timing relationships for IoT-NTN	Sony

R1-2103529	Timing relationship enhancements for IoT NTN	Lenovo, Motorola Mobility

[104b-e-NR-NB_IoT_eMTC-03] Email discussion/approval on timing relationship enhancements with checkpoints for agreements on Apr-15, Apr-20 – Sam (Sony)

Agreement:

The following NB-IoT timing relationships need enhancing for essential minimum functionality of IoT NTN:

- NPDCCH to NPUSCH format 1

- RAR grant to NPUSCH format 1

- NPDSCH to HARQ-ACK on NPUSCH format 2

- Timing advance command activation

- FFS: NPDCCH order to NPRACH

- FFS: Other NB-IoT timing relationships

Agreement:

The enhancement based on extending the timing relationship, by e.g. Koffset, adopted in NR NTN should be the starting point for enhancement of NB-IoT timing relationships in IoT NTN. Details can be further discussed considering IoT NTN.

Agreement:

The following eMTC timing relationships need enhancing for essential minimum functionality of IoT NTN:

- MPDCCH to PUSCH

- RAR grant to PUSCH

- MPDCCH to scheduled uplink SPS

- PUSCH to HARQ-ACK on PUCCH

- CSI reference resource timing

- MPDCCH to aperiodic SRS

- Timing advance command activation

- FFS: MPDCCH order to PRACH

- FFS: Other eMTC timing relationships

Agreement:

The enhancement based on extending the timing relationship, by e.g. Koffset, adopted in NR NTN should be the starting point for enhancement of eMTC timing relationships in IoT NTN. Details can be further discussed considering IoT NTN.

### Enhancements on HARQ

R1-2102346	Discussion on HARQ enhancement for IoT in NTN	Huawei, HiSilicon

R1-2102425	Discussion on HARQ enhancements	OPPO

R1-2102475	Consideration on enhancements on HARQ	Spreadtrum Communications

R1-2102551	Discussion on HARQ enhancements on NB-IoT/eMTC for NTN	vivo

R1-2102620	HARQ operation enhancement for NB-IoT/eMTC	CATT

R1-2102738	Enhancements on HARQ	Asia Pacific Telecom, FGI, ITRI, III

R1-2102757	Enhancements on HARQ for IoT NTN	MediaTek Inc.

R1-2102834	HARQ for NB-IoT/eMTC over NTN	Nokia, Nokia Shanghai Bell

R1-2102908	Enhancements on HARQ for IoT  NTN	CMCC

R1-2102919	Discussion on HARQ for IoT-NTN	ZTE

R1-2102975	Discussion on the HARQ enhancement for IoT NTN	Xiaomi

R1-2103063	On HARQ enhancements for IoT NTN	Ericsson

R1-2103073	Enhancements on HARQ	Qualcomm Incorporated

R1-2103135	On HARQ Enhancement in IoT NTN	Apple

R1-2103269	On enhancements on HARQ	Samsung

R1-2103275	HARQ enhancement for IoT NTN	InterDigital, Inc.

R1-2103321	HARQ issues for IoT-NTN	Sony

R1-2103530	HARQ enhancement for IoT NTN	Lenovo, Motorola Mobility

R1-2103803	Summary#1 of enhancements on HARQ	Moderator (Samsung)

[104b-e-NR-NB_IoT_eMTC-04] Email discussion/approval on enhancements on HARQ with checkpoints for agreements on Apr-15, Apr-20 – Carmela (Samsung)

Agreement:

Increasing the number of HARQ processes for NB-IoT and for eMTC in NTN is recommended not to be supported in Rel-17.

### Others

R1-2102426	Discussion on other aspects	OPPO

R1-2102476	Consideration on other design aspects for IOT NTN	Spreadtrum Communications

R1-2102835	Applicability of NB-IoT/eMTC features for NTN	Nokia, Nokia Shanghai Bell

R1-2102920	Discussion on additional enhancement for IoT-NTN	ZTE

R1-2102976	Discussion on the other design aspects for IoT NTN	Xiaomi

R1-2103064	On evaluation assumptions for NB-IoT and eMTC based NTN	Ericsson

R1-2103074	Other aspects for NTN IOT	Qualcomm Incorporated

R1-2103396	Other aspects to support IoT in NTN	Huawei, HiSilicon