3GPP TSG RAN WG1 Meeting #104bis-e		R1-210xxxx

e-Meeting, April 12th – April 20th, 2021

Title:	Session notes for 8.10

(Enhancements to Integrated Access and Backhaul)

Source:	Ad-hoc chair (Samsung)

## Enhancements to Integrated Access and Backhaul

Please refer to RP-210758 for detailed scope of the WI

R1-2103182	Workplan for Rel-17 IAB	Qualcomm Incorporated

### Enhancements to resource multiplexing between child and parent links of an IAB node

R1-2102340	Resource multiplexing between backhaul and access for IAB duplexing enhancements	Huawei, HiSilicon

R1-2102538	Enhancement to resource multiplexing between child and parent links	vivo

R1-2103046	Enhancements to Resource Multiplexing between Child and Parent Links of an IAB Node			Intel Corporation

R1-2103120	Enhanced resource multiplexing within an IAB node	Apple

R1-2103183	Resource management for enhanced duplexing	Qualcomm Incorporated

R1-2103255	Enhancements to Resource Multiplexing for NR IAB	Samsung

R1-2103330	Discussion on IAB resource multiplexing enhancements	ETRI

R1-2103374	Discussions on enhancements to resource multiplexing between child and parent links of an IAB node		CEWiT

R1-2103435	Enhancements for resource multiplexing among IAB backhaul and access links	Nokia, Nokia Shanghai Bell

R1-2103436	Enhancements for IAB resource multiplexing	AT&T

R1-2103497	Enhancements to the IAB resource multiplexing	ZTE, Sanechips

R1-2103591	Resource multiplexing between child and parent links of an IAB node	NTT DOCOMO, INC.

R1-2103608	Resource multiplexing in enhanced IAB systems	Lenovo, Motorola Mobility

R1-2103629	Discussions on IAB resource multiplexing enhancements	LG Electronics

R1-2103712	Resource multiplexing and DC in enhanced IAB	Ericsson

/This one is to use NWM – please use RAN1-104b-e-NWM-NR-eIAB-01 as the document name

[104b-e-NR-eIAB-01] Email discussion on enhancements to resource multiplexing between child and parent links of an IAB node – Thomas (AT&T)

- 1st check point: April 15

- 2nd check point: April 20

3837

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Proposal 3.1.1:

Enhancements to support intra-band inter-carrier dual connectivity (inter-donor and intra-donor) scenarios are introduced in Rel-17

- FFS: Extend Rel-16 CA TDD confliction resolution framework for at least synchronous intra-band NR-DC operation

- FFS: TDD configurations (and usage of DCI Format 2-0 if supported) for the parent nodes and H/S/NA configurations for the child node are coordinated in case of inter-donor operation

- FFS: Support for simultaneous Tx/Rx at the child IAB-MT

### Other enhancements for simultaneous operation of IAB-node’s child and parent links

Including IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed

Void (not be handled during this e-meeting). No contributions please.

R1-2103047	Other Enhancements for Simultaneous Operations	Intel Corporation

### Others

Void (not be handled during this e-meeting). No contributions please.