3GPP TSG RAN WG1 Meeting #104b-e			R1-210xxxx

e-Meeting, April 12-20, 2021

Agenda Item:	7.2.2

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on NR-U HARQ maintenance

Document for:	Discussion and Decision

# Introduction

Corrections on NR-U HARQ have been submitted at RAN1#104b e-meeting. The preparation phase (April 8th – 9th) determined that corrections labelled Type3CB#1 and Type3CB#1 are deemed as essential corrections, and will be discussed at RAN1#104b-e by email discussion.

[104b-e-NR-NRU-04] Email discussion/approval on HARQ enhancements until Apr-16 – David (Huawei)

- Type3CB#1, Type3CB#2

| Issue # | Issue summary | Contributions |
| --- | --- | --- |
| Type3CB#1 | Type-3 HARQ-ACK codebook size ambiguity R1-2102325 Observation: in the pseudo-code of section 9.1.4 of TS38.213v16.5.0, in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall not include a bit for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. Whether the UE reports a bit impacts the Type-3 HARQ-ACK codebook size, so if any UE is implemented based on a different understanding than in the observation above, then codebook mismatch will occur when the gNB interpretation is consistent with the observation above, and vice-versa. A safer (and more robust) approach would be to define/reserve a bit to report. Proposal 1: in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall report NACK for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. (TP provided in R1-2102325) R1-2102367 Observation 1: Type-3 HARQ-ACK codebook is also applicable to frequency band that does not require shared spectrum access. gNB cannot avoid a SPS PDSCH is transmitted before a type 3 CB without enough processing time when multiple SPS PDSCH configurations or SPS PDSCH configuration with short periodicity is configured to a UE. Observation 2: The size of Type-3 HARQ-ACK codebook will be changed when the UE has not obtained HARQ-ACK information for a HARQ process. Proposal 1: For a given HARQ process, if the UE has not obtained HARQ-ACK information, should be reserved in the Type-3 HARQ-ACK codebook. (TP1 provided in R1-2102367) | R1-2102325 R1-2102367 |
| Type3CB#2 | Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook Reason for change: In current specification of multiplexing timeline definition in Clause 9.2.5, DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH is missing for Tproc,2mux calculation. Summary of change: Add the DCI format 1_1 providing the indication of a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH in the paragraphs of Tproc,2mux calculation in Clause 9.2.5. Refer to R1-2102587 for the detailed CR. | R1-2102587 |
| Type2CB#3 | Assumption on NFI value for a PDSCH group not received at UE side when the UL DAI indicates a value not equal to 4 for that group Proposal #2: For the case when a PDSCH group is not received at UE side and the UL DAI in UL grant DCI corresponding to the PDSCH group indicates a value not equal to 4, the following behavior is applied. - NFI value for the PDSCH group is assumed to be non-toggled from the latest value. - Payload size of the HARQ-ACK on PUSCH is determined by the indicated UL DAI itself without accumulating the HARQ-ACKs in the previous PUCCH occasion (similarly as for the case when UE only receives DL fallback DCI and the UE would transmit HARQ-ACK on PUCCH). FL note: Discussed as issue A9 in the past | R1-2103336 |

# Type3CB#1: Type-3 HARQ-ACK codebook size ambiguity

| Type3CB#1 | Type-3 HARQ-ACK codebook size ambiguity R1-2102325 Observation: in the pseudo-code of section 9.1.4 of TS38.213v16.5.0, in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall not include a bit for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. Whether the UE reports a bit impacts the Type-3 HARQ-ACK codebook size, so if any UE is implemented based on a different understanding than in the observation above, then codebook mismatch will occur when the gNB interpretation is consistent with the observation above, and vice-versa. A safer (and more robust) approach would be to define/reserve a bit to report. Proposal 1: in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall report NACK for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. (TP provided in R1-2102325) R1-2102367 Observation 1: Type-3 HARQ-ACK codebook is also applicable to frequency band that does not require shared spectrum access. gNB cannot avoid a SPS PDSCH is transmitted before a type 3 CB without enough processing time when multiple SPS PDSCH configurations or SPS PDSCH configuration with short periodicity is configured to a UE. Observation 2: The size of Type-3 HARQ-ACK codebook will be changed when the UE has not obtained HARQ-ACK information for a HARQ process. An example is provided to illustrate how a missed DCI will result in a codebook size mismatch between the UE and gNB. Proposal 1: For a given HARQ process, if the UE has not obtained HARQ-ACK information, should be reserved in the Type-3 HARQ-ACK codebook. (TP1 provided in R1-2102367) | R1-2102325 R1-2102367 |
| --- | --- | --- |

Given the various observations and examples in R1-2102325 and R1-2102367, which illustrate various conditions under which a codebook size mismatch may occur between the UE and the gNB, it is proposed to ensure that the codebook size does not depend on whether a UE has obtained or not obtained HARQ-ACK information for a HARQ process.

Proposal 1: correct the generation of the Type-3 HARQ-ACK codebook to ensure that HARQ-ACK bit(s) are always included for each of the configured HARQ processes.

- If UE has neither reported nor obtained HARQ-ACK information for a HARQ process, UE reports NACK for the corresponding bit(s).

A corresponding TP1 is provided in Annex 1.

Companies are invited to comment on proposal 1 and TP1 using the table below.

| Company | Comment |
| --- | --- |

# Type3CB#2: Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook

| Type3CB#2 | Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook Reason for change: In current specification of multiplexing timeline definition in Clause 9.2.5, DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH is missing for Tproc,2mux calculation. Summary of change: Add the DCI format 1_1 providing the indication of a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH in the paragraphs of Tproc,2mux calculation in Clause 9.2.5. Refer to R1-2102587 for the detailed CR. | R1-2102587 |
| --- | --- | --- |

Proposal 2: agree on the CR in R1-2102587

Companies are invited to comment on proposal 2 using the table below.

| Company | Comment |
| --- | --- |

# References

R1-2102325	Maintenance on channel access and HARQ procedures for NR Unlicensed	Huawei, HiSilicon

R1-2102367	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2102587	Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook				CATT, Huawei, HiSilicon, OPPO

R1-2103336	Remaining issues of UL channels and HARQ procedure for NR-U	LG Electronics

# Annex 1: TP1

| Reason for change: | The construction of the Type-3 HARQ-ACK codebook may result in a codebook size mismatch between UE and gNB, if the UE determined that it has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception, but the gNB determined otherwise. |
| --- | --- |
| Summary of change: | Specify that if the UE has not yet obtained HARQ-ACK information for a TB and a HARQ process, the UE shall report NACK for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. |
| Consequences if not approved: | UE and gNB may have a different assumptions on the size (number of bits) reported by the UE in the Type-3 HARQ-ACK codebook, leading to HARQ failure. |

================== Start of TP#1 for TS 38.213 v16.5.0 ===================

9.1.4	Type-3 HARQ-ACK codebook determination

If a UE is provided pdsch-HARQ-ACK-OneShotFeedback, the UE determines o0ACK,o1ACK,…,oOACK-1ACK HARQ-ACK information bits, for a total number of OACK HARQ-ACK information bits, of a Type-3 HARQ-ACK codebook according to the following procedure.

Set NcellsDL to the number of configured serving cells

Set NHARQ,cDL to the value of nrofHARQ-ProcessesForPDSCH for serving cell c, if provided; else, set NHARQ,cDL=8

Set NTB,cDL to the value of maxNrofCodeWordsScheduledByDCI for serving cell c if harq-ACK-SpatialBundlingPUCCH is provided and NDIHARQ=0, or if harq-ACK-SpatialBundlingPUCCH is not provided, or if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c; else, set NTB,cDL=1

Set NHARQ-ACK,cCBG/TB,max to the number of HARQ-ACK information bits per TB for PDSCH receptions on serving cell c as described in Clause 9.1.1 if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided; else, set NHARQ-ACK,cCBG/TB,max=0

Set NDIHARQ=0 if pdsch-HARQ-ACK-OneShotFeedbackNDI is provided; else set NDIHARQ=1

Set c=0 – serving cell index

Set h=0 – HARQ process number

Set t=0 – TB index

Set g=0 – CBG index

Set j=0

while c<NcellsDL

while h<NHARQ,cDL

if NDIHARQ=0

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c, if any; else,

j=j+1

g=g+1

end while

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

g=0

j=j+1

t=t+1

end while

else

while t<NTB,cDL

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c, if any; else,

j=j+1

= NDI value indicated in the DCI format corresponding to the HARQ-ACK information bit(s) for TB t for HARQ process number h on serving cell c, if any; else,

j=j+1

t=t+1

end while

end if

t=0

else

if NHARQ-ACK,cCBG/TB,max>0

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c, and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

while g<NHARQ-ACK,cCBG/TB,max

= HARQ-ACK information bit for CBG g of TB t for HARQ process number h of serving cell c

j=j+1

g=g+1

end while

end if

if UE has not obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c

while g<NHARQ-ACK,cCBG/TB,max

j=j+1

g=g+1

end while

end if

g=0

t=t+1

end while

else

while t<NTB,cDL

if UE has reported HARQ-ACK information for TB t for HARQ process number h on serving cell c and has not subsequently detected a DCI format scheduling a PDSCH reception, or received a SPS PDSCH, with TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

end if

if UE has obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c corresponding to a PDSCH reception and has not reported the HARQ-ACK information corresponding to the PDSCH reception

if harq-ACK-SpatialBundlingPUCCH is not provided

= HARQ-ACK information bit for TB t for HARQ process h of serving cell c

else

= binary AND operation of the HARQ-ACK information bits corresponding to first and second transport blocks for HARQ process h of serving cell c. If the UE receives one transport block, the UE assumes ACK for the second transport block

end if

j=j+1

t=t+1

end if

if UE has not obtained HARQ-ACK information for TB t for HARQ process number h on serving cell c

= NACK

j=j+1

t=t+1

end if

end while

end if

t=0

end if

h=h+1

end while

h=0

c=c+1

end while

If NTB,cDL>1, when a UE receives a PDSCH with one transport block, the HARQ-ACK information is associated with the first transport block.

If a UE receives a SPS PDSCH, or a PDSCH that is scheduled by a DCI format 1_0 for a serving cell c and if maxCodeBlockGroupsPerTransportBlock is provided for serving cell c, and pdsch-HARQ-ACK-OneShotFeedbackCBG is provided, the UE repeats NHARQ-ACK,cCBG/TB,max times the HARQ-ACK information for the transport block in the PDSCH.

If a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, the UE determines a PUCCH or a PUSCH to multiplex a Type-3 HARQ-ACK codebook for transmission in a slot as described in Clauses 9.2.3 and 9.2.5. The UE multiplexes only the Type-3 HARQ-ACK codebook in the PUCCH or the PUSCH for transmission in the slot.

If

-	a UE detects a DCI format that includes a One-shot HARQ-ACK request field with value 1, and

-	the CRC of the DCI is scrambled by a C-RNTI or an MCS-C-RNTI, and

-	resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in the DCI format are equal to 0, or

-	resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in the DCI format are equal to 1, or

-	resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in the DCI format are equal to 0 or 1

the DCI format provides a request for a Type-3 HARQ-ACK codebook report and does not schedule a PDSCH reception. The UE is expected to provide HARQ-ACK information in response to the request for the Type-3 HARQ-ACK codebook after N symbols from the last symbol of a PDCCH providing the DCI format, where the value of N for μ=0,1,2 is provided in Clause 10.2 by replacing "SPS PDSCH release" with "DCI format".

If a UE multiplexes HARQ-ACK information in a PUSCH transmission, the UE generates the HARQ-ACK codebook as described in this Clause except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH.

================== End of TP#1 for TS 38.213 v16.5.0 ===================