3GPP TSG RAN WG1 Meeting #104b-e			R1-210xxxx

e-Meeting, April 12-20, 2021

Agenda Item:	7.2.2

Source:	Moderator (Huawei)

Title:	Feature lead summary#1 on NR-U HARQ maintenance

Document for:	Discussion and Decision

# Introduction

Corrections on NR-U HARQ have been submitted at RAN1#104b e-meeting. This first summary asks for companies’ views on the criticality of the proposed corrections during the preparation phase (April 8th – 9th). The corrections proposed to solve 3 issues at RAN1#104b are summarized in the table below.

| Issue # | Issue summary | Contributions |
| --- | --- | --- |
| Type3CB#1 | Type-3 HARQ-ACK codebook ambiguity R1-2102325 Observation: in the pseudo-code of section 9.1.4 of TS38.213v16.5.0, in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall not include a bit for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. Whether the UE reports a bit impacts the Type-3 HARQ-ACK codebook size, so if any UE is implemented based on a different understanding than in the observation above, then codebook mismatch will occur when the gNB interpretation is consistent with the observation above, and vice-versa. A safer (and more robust) approach would be to define/reserve a bit to report. Proposal 1: in case NDI reporting is not configured for Type-3 HARQ-ACK codebook, if the UE has not yet obtained HARQ-ACK information for a TB corresponding to a scheduled PDSCH reception and the UE has detected a DCI format scheduling a PDSCH reception for the TB, the UE shall report NACK for the corresponding TB and HARQ process in the Type-3 HARQ-ACK codebook. (TP provided in R1-2102325) R1-2102367 Observation 1: Type-3 HARQ-ACK codebook is also applicable to frequency band that does not require shared spectrum access. gNB cannot avoid a SPS PDSCH is transmitted before a type 3 CB without enough processing time when multiple SPS PDSCH configurations or SPS PDSCH configuration with short periodicity is configured to a UE. Observation 2: The size of Type-3 HARQ-ACK codebook will be changed when the UE has not obtained HARQ-ACK information for a HARQ process. Proposal 1: For a given HARQ process, if the UE has not obtained HARQ-ACK information, should be reserved in the Type-3 HARQ-ACK codebook. (TP1 provided in R1-2102367) | R1-2102325 R1-2102367 |
| Type3CB#2 | Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook Reason for change: In current specification of multiplexing timeline definition in Clause 9.2.5, DCI format 1_1 indicating a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH is missing for Tproc,2mux calculation. Summary of change: Add the DCI format 1_1 providing the indication of a request for a Type-3 HARQ-ACK codebook report without scheduling PDSCH in the paragraphs of Tproc,2mux calculation in Clause 9.2.5. Refer to R1-2102587 for the detailed CR. | R1-2102587 |
| Type2CB#3 | Assumption on NFI value for a PDSCH group not received at UE side when the UL DAI indicates a value not equal to 4 for that group Proposal #2: For the case when a PDSCH group is not received at UE side and the UL DAI in UL grant DCI corresponding to the PDSCH group indicates a value not equal to 4, the following behavior is applied. - NFI value for the PDSCH group is assumed to be non-toggled from the latest value. - Payload size of the HARQ-ACK on PUSCH is determined by the indicated UL DAI itself without accumulating the HARQ-ACKs in the previous PUCCH occasion (similarly as for the case when UE only receives DL fallback DCI and the UE would transmit HARQ-ACK on PUCCH). FL note: Discussed as issue A9 in the past | R1-2103336 |

# Recommendation from preparation phase

Recommendation from the preparation phase email discussion is to discuss the following issues at RAN1#104b: TBD

# References

R1-2102325	Maintenance on channel access and HARQ procedures for NR Unlicensed	Huawei, HiSilicon

R1-2102367	Discussion on the remaining issues of HARQ enhancements	OPPO

R1-2102587	Correction on multiplexing timeline definition for Type-3 HARQ-ACK codebook				CATT, Huawei, HiSilicon, OPPO

R1-2103336	Remaining issues of UL channels and HARQ procedure for NR-U	LG Electronics