3GPP TSG RAN WG1 #105-e	R1-2104346

e-Meeting, May 10th – 27th, 2021

Source:	vivo

Title:	Further discussion and evaluation on HST-SFN transmission schemes

Agenda Item:	8.1.2.4

Document for:	Discussion and Decision

Introductions

In the previous RAN1 e-meetings, the following agreements about enhancement on HST-SFN deployment and multi-TRP PDCCH were achieved and need further discussion.

Agreement Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both: - UL RS based Doppler estimation by gNB - FFS: Details including UL RS enhancement - DL RS based Doppler feedback by UE - FFS: Details - FFS: Whether UE capability needs to be introduced - Whether to support one or both will be decided later Agreement When the same DMRS port(s) are associated with two TCI states containing TRS as source reference signal, at least one variant is supported for Rel-17 HST-SFN scenario based on further evaluations - Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB) - Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA) - FFS: Indication method to apply QCL, e.g., via new QCL-type, or reuse existing QCL-type while UE to ignore certain QCL properties - Note: Each TCI state in the above variants may be additionally associated with {Spatial Rx parameter} (i.e., QCL-TypeD) - Note: Companies are encouraged to provide evaluation results for the above variants based on agreed EVM from RAN1#102e meeting - Note: Above variants are applicable to scheme 1 and/or TRP based pre-compensation as a reference for evaluation. Agreement For HST-SFN scenario: - Support semi-static (RRC based) switching of scheme 1 (PDSCH) with 2a, 2b, 3, 4 - FFS all other details including RRC signaling, possible RAN4 impact (if any), etc. Agreement Support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a - FFS: Whether dynamic switching is additionally supported Agreement - Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2 - This feature is UE optional - FFS all other details including RRC signalling, possible RAN4 impact (if any), etc. Agreement For PDCCH reliability enhancements, support SFN transmission scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

In this contribution, further analysis of frequency offset pre-compensation for HST-SFN  is provided. Link-level simulation results are given to show the performance of the potential enhancement. Besides, some detailed issues about SFN PDCCH enhancement are discussed, such as default beam and BFD.

Discussion

HST-SFN deployment

In the HST scenario, due to frequent handover between the cells, SFN transmission scheme was introduced in LTE. Multiple TRPs with the same Cell ID are connected to one BBU to overcome frequent handover under HST-SFN deployment. However, one of the main problems of HST-SFN deployment is the opposite Doppler shifts from different TRPs. From UE’s perspective, the received signal is a combination of multiple delayed signals with different power and opposite high Doppler shifts in SFN manner as shown in Figure 1, which is quite challenging for UE demodulation.

HST-SFN deployment

To deal with opposite Doppler shifts, several potential solutions were proposed in RAN1#102 e-meeting, which basically can be classified as the UE-based solution and NW-based solution.

UE-based solution for HST-SFN

Scheme 1: SFN transmission with distributed TRS

In scheme 1, two TRS resource sets TRS1 and TRS2 are transmitted from TRP1 and TRP2 respectively, while PDSCH/PDCCH are transmitted in SFN manner, as shown in Figure 2. UE can estimate the Doppler shifts by measuring two TRS resource sets, and then apply the results for PDSCH/PDCCH DMRS estimation.

SFN transmission with distributed TRS

However, the channel of HST-SFN deployment is generally a LOS channel with multiple Doppler shifts, and the Doppler spread is typically not a U-shape spectrum. Therefore, after estimating the multiple Doppler shifts based on distributed TRS, UE should use them to reconstruct the Doppler spectrum, and then conduct time-domain interpolation for DMRS estimation by using the Wiener filter, instead of the U-shape Doppler spectrum assumption. That means a Rel-17 UE should simultaneously support the advanced time-domain interpolation designed for scheme 1 and the legacy U-shape spectrum time-domain interpolation for DMRS estimation, leading to the increasing of implementation complexity of Rel-17 UE.

Scheme 1 would increase the implementation complexity of Rel-17 UE, due to the additional time-domain interpolation for DMRS estimation.

Scheme 2: SFN transmission with distributed TRS and DMRS

In scheme 2, in addition to two TRS resource sets, two orthogonal PDSCH DMRS ports are also transmitted from TRP1 and TRP2 respectively, while PDSCH is still transmitted in SFN manner, as shown in Figure 3. UE can achieve estimating the channel of each TRP separately with orthogonal PDSCH DMRS ports, and then combine the channel of each TRP to reconstruct a composite channel for PDSCH demodulation.

SFN transmission with distributed TRS and DMRS

However, there are two main drawbacks of scheme 2 as follows.

- Complexity: Separate channel estimation of two TRPs needs a total of twice time-domain filtering to be operated for the LS channel estimation results from two orthogonal DMRS ports. Relatively, scheme 1 only needs once time-domain filtering with two Doppler shifts value estimated by two TRS. Moreover,  the NW-based solution even has no requirement for UE to enhance its DMRS estimation processing. Therefore, from the perspective of UE, the complexity of scheme 2 is higher than other schemes.

- DMRS overhead: Two orthogonal PDSCH DMRS ports from two CDM groups separately would directly affect the overall throughput of PDSCH transmission in scheme 2.

Scheme 2 needs higher UE complexity and more DMRS overhead, compared to other proposed schemes for Rel-17 HST-SFN.

NW-based solution for HST-SFN

TRP-specific frequency offset pre-compensation

In the procedure of frequency offset pre-compensation, the key point is how to know the Doppler shifts that are used to process frequency pre-compensation in the NW side. There are two potential methods proposed in the previous meetings to handle this issue.

- Option 1: Implicit indication of the Doppler shifts using uplink signals

In Option 1, NW would estimate the uplink frequency offset based on the uplink signals such as SRS. The frequency offset pre-compensation process based on Doppler estimation using UL RS is provided in Figure 4.

Frequency offset pre-compensation based on Doppler estimation using UL RS

As described in Figure 4, UE transmits the UL RS signal to TRP1 and TRP2 at the uplink carrier frequency fc  + Δf, where Δf is the frequency adjusting value for uplink. Then TRP1 estimates the uplink frequency offset Δf + Δf1 and TRP2 estimates the uplink frequency offset Δf + Δf2. NW calculates the frequency pre-compensation value Δf1 - Δf2 for TRP2 by subtracting Δf + Δf2 from Δf + Δf1. It can be observed that the final frequency pre-compensation value Δf1 - Δf2 is independent of Δf. That means as long as satisfying the uplink frequency shifts estimate requirement, no matter what the exact value of Δf is, frequency offset pre-compensation can still work.

Therefore, the uplink carrier frequency can be adjusted based on UE implementation, regardless of the downlink frequency shift estimation by TRS. To avoid adjusting the oscillator frequently, after UE modulates the downlink carrier frequency based on TRS, it can transmit uplink signals at the same downlink carrier frequency by UE implementation. In this way, the frequency adjusting value Δf for uplink can be equal to Δf1, which achieves the same purpose as indicating the QCL-like association between uplink signals and TRS.

The frequency pre-compensation value can be derived based on implementation, regardless of the carrier frequency of UL RS.

Support implementation-based frequency offset pre-compensation using uplink signals for Doppler estimation, without defining QCL-like association between uplink signals and TRS.

- Option 2: Explicit reporting of the Doppler shifts using CSI framework

In the previous meetings, some companies had concerns about the feasibility and complexity when using UL RS for Doppler estimation at the NW side. Therefore, one potential way is UE to report the Doppler shifts estimated on TRSs to NW using the CSI framework. A feasible frequency offset pre-compensation process based on Doppler feedback is provided in Figure 5.

Frequency offset pre-compensation based on Doppler feedback

In the above procedure, firstly UE would adjust its downlink carrier frequency indicated by the TCI state associated with TRS1, then UE could directly feed back a pre-compensation value of the Doppler shift, rather than two TRP-specific Doppler values corresponding to each TRP. In this case, same as the pre-compensation based on Doppler estimation using UL RS in Figure 4, there is no need to perform frequency offset pre-compensation at TRP1.

In the current specification, TRS is used to track the Doppler shift which is no need to be reported. However, if Doppler shift needs to be fed back by UE using the CSI framework, some mechanisms should be specified to support it. For instance, the direct or indirect associations of TRS, i.e. NZP-CSI-RS resource sets with trs-info and CSI report setting should be built. Subsequently, UCI mapping and quantization of the Doppler feedback value is a big issue. Moreover, when UE is far from one TRP, the power of received signals would be very low and the estimated Doppler shift corresponding to faraway TRPs would also be inaccurate. It would cause a certain degree of waste of feedback resources if the Doppler shift is still reported by UE in this case. Therefore, potential methods could be studied to save the CSI feedback resources.

Support of Doppler feedback should be discussed under the MTRP CSI framework agreed in Rel-17 considering at least the following issues:

        - Association of TRS and CSI report setting/resources sets/subsets

        - Quantization and UCI mapping of the Doppler feedback

        - Mechanisms for saving CSI feedback overhead

TRP-specific timing offset pre-adjustment

From the perspective of UE, the SFN channel is composed of multiple TRPs’ channels with respective received path power and delay. When UE is located at the middle point of two TRPs, the signals transmitted from two TRPs have the same path power and delay, but different arrival phases. Therefore, two TRP’s SFN signals might cancel out with each other at the middle point of two TRPs as shown in Figure 6 c), resulting in the low SNR of received signals. That would cause the degradation of UE demodulation performance. To solve the above problem, a small delay CDD can be introduced in a certain TRP. For instance, a fixed small delay CDD with the length of CP/16 can be added before signals transmitted from the second TRP. After that, the relative delay of two LOS paths in the received SFN signal would be nearly CP/16, which can effectively avoid SFN signals canceling out with each out.

| a) Channel of TRP1 | b) Channel of TRP2 | c) Composed SFN channel at the UE side |
| --- | --- | --- |

TRP’s channel canceling out with each other when UE is located at 350m

Moreover, when UE is at some locations where the power of signals from two TRPs is similar, but the path delay difference is relatively large, the frequency-domain SFN channel would experience deep and more frequent fading as shown in Figure 7. That would also cause the performance degradation of SFN transmission. In this case, small delay CDD can also be used to reduce the path delay difference between two TRPs’ SFN signal, which can inhibit the deep and frequent fading of the composited SFN channel at the UE side.

| a) Channel of TRP1 | b) Channel of TRP2 | c) Composed SFN channel at the UE side |
| --- | --- | --- |

The deep and frequent fading of the composed SFN channel when UE is located at 250m

SFN signals from two TRPs might cancel out with each other at the middle point of two TRPs.

Lager delay between the SFN signals from two TRPs would cause deep and more frequent fading of the SFN channel, leading to performance degradation of SFN transmission.

However, a fixed small delay CDD at certain TRP is not always optimal when UE is moving along the track. A potential way is to perform a dynamic small delay CDD corresponding to UE’s location. In this case, NW could adjust the Tx timing when transmitting the downlink signals to UE at each TRP. The main point is how does NW get the timing shift value for a certain TRP. Similar to Doppler shifts, timing shifts can also be estimated by UL RS or fed back from UE. To further enhance the performance of HST-SFN, TRP-specific timing offset pre-adjustment is an effective way to solve the mentioned drawbacks of HST-SFN and could be further studied.

| a) SNR=0dB | b) SNR=4dB |
| --- | --- |

Performance comparison of SFN w/s CDD and w/o CDD

Figure 8 shows the simulation results of SFN with small delay CDD and without small delay CDD using frequency offset pre-compensation. Type I codebook with PRG2 is applied to PDSCH which is mapped on 50 PRBs. According to the results, it is obvious that when UE is located at 350m, the performance of SFN without CDD is degraded by the cancellation of SFN signals. After introducing a fixed CDD with the length of CP/16, it can be found that the performance at 350m is improved significantly. However, a fixed CDD cannot improve the performance at every location and even would cause degradation at certain locations. Therefore, if the network can adjust the small delay CDD value dynamically based on UE’s location, the performance at other locations can also be improved. As shown in Figure 8 SFN with dynamic CDD to keep the received path delay difference of SFN signals at every location as CP/16 can generally improve the performance of UE demodulation.

Small delay CDD can improve the UE demodulation performance but is sensitive to delay offset added between the two TRPs.

TRP-specific timing offset pre-adjustment can be considered to further enhance the performance of HST-SFN,  e.g. based on feedback of timing offset between two different TRPs.

New QCL assumption for PDSCH/PDCCH DMRS

While distributed TRS resource sets are transmitted from each TRP respectively, PDSCH and PDCCH are transmitted in SFN manner. Thus, it is natural that the same DMRS port(s) could be QCLed with multiple TRS resource sets simultaneously.

In RAN1#103 e-meeting, several QCL assumptions were proposed for the frequency offset pre-compensation scheme. They can mainly be divided into two types of realization as follows.

- Type1: Define a new QCL-Type

- Variant A: One of the TCI states can be associated with {average delay, delay spread} and another TCI state can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI states can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI states can be associated with {delay spread} and another TCI state can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Type 2: Reuse existing QCL-type but ignore certain QCL properties

  - Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

For type 1, it is easy to observe that the main difference between Variant A and Variant C is whether another TCI state should be associated with average delay property. In the existing specification, the average delay is mainly measured by SSB and TRS and then used for downlink timing. For HST-SFN deployment with two distributed TRSs, there are two feasible ways to measure the average delay.

- 1st and 2nd TRS estimate the average delay separately

In this case, the UE would measure an average delay on each TRS for related TRP’s time-domain channel. However, from the perspective of adjusting downlink timing, UE referring to the average delay measured on one of two TRSs is enough, i.e. the TRS which is referred to adjust the downlink frequency shift simultaneously. If both TCI states related to two TRSs are associated with average delay as provided in Variant A, UE would be confused about which average delay value should be used to adjust the downlink timing.

- 1st and 2nd TRS estimate the average delay together

In this case, the UE can estimate one common average delay on the composit channel of  1st and 2nd TRS. However, due to the different path delays from two TRPs, the calculated average delay based on Variant A using the composit channel would be larger than Variant C as shown in Figure 9, which would affect the accuracy of downlink timing.

Comparison of the average delay estimation between Variant A and Variant C

Therefore, both TCI states associated with average delay would cause ambiguity and imprecision of downlink timing. On the contrary, when only one TCI state is associated with average delay, it would be clear for UE to adjust downlink timing correctly. For instance, in the previous procedure description of frequency offset pre-compensation, the downlink carrier frequency is assumed to be adjusted based on TRS1 as fc  +Δf1, where Δf1 is the Doppler shift estimate result corresponding to the transmission between TRP1 and UE, because TRS1 has stronger receiving power and can provide more precise Doppler shift estimation than TRS2. At the same time, the downlink timing adjustment can also refer to TRS1 with its average delay.

Both TCI states associated with average delay would cause ambiguity and imprecision of downlink timing.

For type 2, reusing existing QCL-type but ignoring certain QCL properties is an equivalent method to achieve the QCL relationship between DMRS and TRS in frequency offset pre-compensation. Assume that both TCI states associated with QCL-TypeA, i.e.,{average delay, delay spread, Doppler shift, Doppler spread} are indicated to UE. The UE should ignore some of the QCL properties of a certain TCI state and only refer to delay spread. In this case, the same QCL relationship as Variant C is achieved. However, before ignoring the QCL properties of a certain TCI state, UE should know which TCI state is the anchor to provide the complete QCL-TypeA reference or which TCI state is used to provide only delay spread. Thus, an implicit or explicit indication should be designed to make sure UE can know it.

Ignoring {average delay, Doppler shift, Doppler spread} properties of QCL-TypeA can achieve the same effect as Variant C, but an implicit or explicit indication should be designed to inform UE which TCI state to provide only {delay spread} reference.

Support Variant C as the QCL assumption for the frequency offset pre-compensation scheme in Rel-17.

Detailed discussion about SFN PDCCH

The combination of transmission schemes for PDCCH and PDSCH

Considering the multifarious cases in the HST deployment, the combination of transmission schemes for PDCCH and PDSCH would be quite flexible, especially for transmission in STRP and SFN manner. Table 1 shows four different combinations of PDCCH and PDSCH transmission in STRP or SFN manner.

The combination of transmission schemes for PDCCH and PDSCH

| PDCCH | PDSCH | Issue |
| --- | --- | --- |
| STRP | STRP | / |
| STRP | SFN | - Whether UE has the capability to dynamically switch the receiving process between STRP transmission for PDCCH and SFN transmission for PDSCH |
| SFN | STRP | - Whether UE has the capability to dynamically switch the receiving process between SFN transmission for PDSCH and STRP transmission for PDCCH - Default beam enhancement |
| SFN | SFN | - How to indicate the transmission of SFN PDSCH/PDCCH - Whether PDCCH is pre-compensated together with PDSCH |

If PDCCH is transmitted in STRP manner while PDSCH is transmitted in SFN manner, whether UE has the capability to dynamically switch the receiving process between STRP PDCCH and SFN PDSCH should be discussed. Similarly, in the case that PDCCH is transmitted in SFN manner while PDSCH is transmitted in STRP manner, whether UE has the capability to dynamically switch the receiving process between SFN PDCCH and STRP PDSCH should also be discussed. Assume UE doesn’t have the capability to dynamically switch the receiving process between STRP transmission for PDCCH and SFN transmission for PDSCH, then the default TCI state of SFN PDSCH scheduled by DCI 1_0 without TCI indication would be contradictory. In other words, if UE doesn’t have the capability to dynamically switch its receiving process between STRP transmission for PDCCH and SFN transmission for PDSCH, UE can’t be scheduled with SFN PDSCH transmission based on DCI 1_0 transmitted from STRP.

If UE doesn’t have the capability to dynamically switch its receiving process between STRP transmission for PDCCH and SFN transmission for PDSCH, the SFN PDSCH transmission can’t be scheduled by the DCI 1_0 transmitted from STRP.

Support to discuss whether UE reports the capability to dynamically switch the receiver between STRP/SFN PDCCH and SFN/STRP PDSCH.

SS-specific TCI state activation for CORESET

In the exsiting specification, one UE can be configured with up to 3 CORESETs and 10 search space sets on each of up to four BWP on a serving cell, and a search space set is associated with only one CORESET. If one CORESET is associated with multiple search spaces, the QCL parameters of all of the PDCCH resources carried in these search spaces are indicated by the TCI state of this CORESET. Therefore, when two TCI states for a CORESET are configured in Rel-17, whether all of the search spaces of this CORESET are associated with both two TCI states could be discussed.

Since different DCI formats can be configured in different search spaces, some DCI formats in the common search space set is not appropriate with SFN transmission or has been configured with other schemes. In this case, these search spaces can be associated with one or both of the TCI states for the CORSET. For instance, if some cell-specific CORESETs configured in SIB1 are indicated with two TCI states, UE can receive STRP-based PDSCH scheduled by the search space associated with one of the two TCI states of the CORESETs, while also receive SFN-based PDSCH scheduled by the search space associated with both of the TCI states.

As shown in Figure 10, even if the CORESET is configured with two TCI states, search space 2 and search space 3 can still be transmitted from a single TRP. gNB can flexibly adjust one PDCCH transmission in the case of STRP or MTRP transmission.

SS-specific TCI State(s) activation for different search spaces

Support to configure/activate one or two TCI states for different search spaces associated with the same CORESET indicated with two TCI states.

Default beam for different channels

- Default beam for PDSCH

UE receives PDSCH with default beam before successfully decoding the DCI payload, when at least one configured TCI state for the serving cell of the scheduled PDSCH contains the 'QCL-TypeD'. In 38.214, the default beam for PDSCH in some cases usually has a relationship with the QCL configuration of the lowest CORESET ID. Therefore, new cases about the scheduling relationship between SFN based PDCCH and corresponding PDSCH should be discussed. There are four possible options as follows:

- Option1: SFN based PDCCH scheduling PDSCH from STRP in Rel-15

- Option2: SFN based PDCCH scheduling PDSCH from MTRP in Rel-16 (including scheme 1a,2a,2b,3,4), and UE is capable of simultaneous reception of two beams

- Option3: SFN based PDCCH scheduling PDSCH from M-TRP in Rel-16 (including scheme 3,4), but UE is not capable of simultaneous reception of two beams

- Option4: SFN based PDCCH scheduling SFN based PDSCH from M-TRP, and UE is capable of simultaneous reception of two beams

Regarding option 2, UE can receive two different beams simultaneously, meanwhile, at least one TCI codepoint indicates two TCI states. The default beam of these PDSCH transmissions are quasi co-located with the RS(s) with respect to the QCL parameter(s) associated with the TCI states corresponding to the lowest codepoint among the TCI codepoints containing two different TCI states, regardless of the number of TCI states of the lowest CORESET ID.

Regarding option 4, there are many common features between SFN based PDSCH and PDSCH transmission schemes 1a, 2a and 2b, so the behavior of the default beam for option 4 can be the same as option 2.

Regarding option 1 and option 3, the default beam of PDSCH should refer to the lowest CORESET ID in the latest slot, in which one or more CORESETs within the active BWP of the serving cell are monitored by the UE. The case that the lowest CORESET ID configured with one TCI state has been supported in the existing specification. However, if the lowest CORESET ID is configured with two TCI states, how to determine the default beam of this PDSCH transmission from S-TRP may include the following alternatives.

- Alt1: gNB ensures the lowest CORESET ID in the latest slot only configured one TCI state by implementation

- Alt2: Modify the definition of the lowest CORESET ID in the latest slot, e.g. the lowest CORESET ID among the CORESETs associated with one TCI state in the latest slot.

- Alt3: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state

- Alt4: QCL assumption associated with both of two TCI states

The pros and cons of these alternatives are listed below.

The pros and cons of different solutions of default beam for PDSCH

|  | Pros | Cons |
| --- | --- | --- |
| Alt1 | No change in the current spec | There may be a certain restriction on RRC configuration. For example, gNB should ensure the lowest CORESET ID in every PDCCH monitoring occasion is always associated with one TCI state. |
| Alt2 | Small spec impact | There may be a certain restriction on RRC configuration, e.g. gNB should ensure at least one CORESET associated with one TCI state in every PDCCH monitoring occasion. |
| Alt3 | Small spec impact | Association with a certain TCI state should be defined. |
| Alt4 | Small spec impact | It will increase the complexity of UE in case of option 1 and cannot support option 3. |

If neither enableDefaultTCIStatePerCoresetPoolIndex nor enableTwoDefaultTCI-States is configured for UE, support Alt3 as the default beam assumption for STRP and Rel-16 scheme 3/4  PDSCH when CORESET are indicated with two TCI states.

- Default beam for A-CSI RS triggering and NZP CSI RS

In Rel-16, for each A-CSI-RS resource in a CSI-RS resource set associated with each CSI triggering state, the UE is indicated with the QCL parameters through higher layer signaling of qcl-info which contains a list of TCI states for the A-CSI-RS resources associated with the CSI triggering state. However, in case that the scheduling offset between the last symbol of the PDCCH carrying the triggering DCI and the first symbol of the A-CSI-RS resources configured without higher layer parameter trs-Info is smaller than the beam-switching threshold which is indicated in reporting of UE capability, the UE applies the QCL assumption used for the CORESET associated with a monitored search space with the lowest CORESET ID in the latest slot, where one or more CORESETs within the active BWP of the serving cell are monitored, and the CORESETs belong to the same CORESET pool index if configured. Therefore, the default beam assumption of A-CSI RS can also refer to the lowest CORESET ID in the latest slot, which is same as the default beam for PDSCH discussed above. For simplifying the spec impact, it is better to share the same design of the default beam for both A-CSI RS and PDSCH.

Using the same mechanism of default beam assumption for A-CSI RS and PDSCH.

Spec 38.214 defines the following: “For a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'on', the UE shall not expect to be configured with CSI-RS over the symbols during which the UE is also configured to monitor the CORESET, while for other NZP-CSI-RS-ResourceSet configurations, if the UE is configured with a CSI-RS resource and a search space set associated with a CORESET in the same OFDM symbol(s), the UE may assume that the CSI-RS and a PDCCH DM-RS transmitted in all the search space sets associated with CORESET are quasi co-located with 'typeD', if 'typeD' is applicable. This also applies to the case when CSI-RS and the CORESET are in different intra-band component carriers, if 'typeD' is applicable. Furthermore, the UE shall not expect to be configured with the CSI-RS in PRBs that overlap those of the CORESET in the OFDM symbols occupied by the search space set(s).”

Therefore, for a CSI-RS resource associated with a NZP-CSI-RS-ResourceSet with the higher layer parameter repetition set to 'off' and overlapping with a search space set associated with a CORESET in time, UE would assume the default beam for the CSI-RS refers to QCL assumption for the CORESET related to the PDCCH. However, if the CORESET is indicated with two TCI states, two alternatives should be considered to determine the default beam of the CSI-RS.

- Alt1: gNB ensures the CORESET ID only be configured one TCI states by implementation

- Alt2: QCL assumption associated with one of TCI states, e.g. always selects the first or the second TCI state

Alt1 may introduce some scheduling restrictions for CSI-RS but without any spec impact, while Alt2 is also a simple scheme to resolve the issue without restricting the scheduling flexibility of gNB.

The default beam of CSI-RS in case of overlapping with CORESET in time domain should be further discussed.

- Default beam for SRS and PUSCH

Default beam of the uplink channel and reference signal including PUSCH and SRS is defined in Rel-16,  which depends on the QCL of the lowest CORESET ID in the active DL BWP in the CC. The same analysis as above makes sense here, and the same approach can be applied to the default beam of downlink and uplink channels and signals.

Reuse the same rule for determination of the default beam of uplink channels and reference signals in case one beam is applied as the default beam.

BFD resource for BFR

Beam failure detection is the first step of the beam failure recovery mechanism, in which UE monitors a set of q0  periodic CSI-RS resources to assess whether the beam failure trigger condition has been met or not. The CSI-RS can be explicitly configured by RRC signaling or implicitly associated by using the same QCL assumption with CORESETs. In current spec, at most three CORESETs are configured for one active BWP and one of them is reserved as special CORESET-BFR, so UE only support up to a set q0 of two CSI-RS resources for BFD.

If the CSI-RS resources for BFD are implicitly indicated and one of CORESETs is configured with two TCI states while the other CORESET is configured with one TCI state, UE may monitor three CSI-RS resources totally, which exceed the number of UE monitoring capability. Furthermore, UE assesses the beam failure trigger condition according to SINR estimation based on CSI-RS against the SINR threshold of target PDCCH BLER. In the case of PDCCH SFN transmission, whether the SINR is estimated based on each CRI-RS separately or based on two CSI-RS resources together should be discussed. Two options can be considered as follows.

- Alt1: The DL RS in the two TCI states are directly used as the BFD RS.

  - Regardless of how many TCI states are configured for the CORESET, BFD is performed in each CSI-RS resource, and beam failure is declared when the signal quality of all CSI-RS resources is lower than the threshold.

- Alt2: UE calculates one SINR in SFN assumption based on two independent CSI-RS resources whose TCI states are associated with the two TCI states of the CORESET independently

  - UE can combine the channel estimation results based on two CSI-RS resources in SFN mode and then estimate the SINR to identify the performance of PDCCH. Regarding the explicit configuration for set q0, when SFN based PDCCH is configured, UE should extract two corresponding CSI-RS resources to combine the channel estimation and derive the SINR.

Since Alt2 considers the combined gain of PDCCH, it will significantly reduce the probability of triggering the BFR procedure unnecessarily. However, if Alt2 is supported, once BFD is reported by UE, then how to find new candidate beams or one group of beams for SFN based PDCCH should be further studied. In Rel-17, BFR for MTRP is being discussed in AI 8.1.2.3 with many common issues. Therefore, one unified scheme to address BFR in MTRP operation is preferred. Apart from SFN based PDCCH, other PDCCH enhancement, e.g. TDM based PDCCH should also be taken into account in BFR procedure enhancement.

Take Alt2 into account with high priority for BFD enhancement.

Indication for SFN PDSCH/PDCCH transmission

Indication for SFN PDSCH

According to the agreements reached in the previous meetings, it has been agreed to support the semi-static switching of scheme 1 and R16 MTRP schemes for PDSCH. That implies one specific RRC parameter would be specified to indicate scheme 1 for PDSCH transmission. Besides, frequency offset pre-compensation also has been agreed in the last meeting. Since scheme 1 and frequency offset pre-compensation are both SFN-based transmission, using a unified RRC parameter, e.g. ‘sfnScheme’ as the indication of them is a more clear way for RRC parameter simplification.

Support a unified RRC parameter, e.g. ‘sfnScheme’ as the indication for both scheme 1 and frequency offset pre-compensation.

Indication for SFN PDCCH

Since there are several MTRP PDCCH schemes studied in the current R17 discussion, how to indicate the transmission scheme for SFN PDCCH is another important issue. Since a RRC parameter would be specified to indicate SFN PDSCH, an additional RRC parameter for indicating SFN PDCCH per CORESET seems redundant.The reasons are as follows.

- MAC CE with two TCI states is a unique feature of SFN PDCCH, which is a feasible way to differentiate SFN PDCCH from other MTRP/STRP PDCCH schemes.

- When switching between STRP PDCCH and SFN PDCCH, assuming a new RRC parameter is used to indicate SFN PDCCH, then RRC reconfiguration is needed. That would cause additonal RRC overhead for switching.

- In HST-SFN deployment, PDCCH and PDSCH are basically both transmitted in SFN manner. Since a RRC parameter would be specified to indicate SFN PDSCH, this RRC parameter can also be used to indicate SFN PDCCH simultaneously to save the RRC overhead.

Support one of the following indication methods for SFN PDCCH, rather than a specific RRC parameter per CORESET:

- Purely using MAC CE with two TCI states as the indication

- Using the same RRC indication as SFN PDSCH

Indication of frequency offset pre-compensation for SFN PDSCH/PDCCH

Frequency offset pre-compensation has been agreed in the last meeting, then whether PDCCH is pre-compensated together with PDSCH could be further discussed. From the perspective of UE complexity, if PDCCH is transmitted by scheme 1, but PDSCH is transmitted by the frequency offset pre-compensation scheme, UE should maintain two receiving processes simultaneously or support dynamic switching between scheme 1 and frequency offset pre-compensation, which would increase the difficulty of UE’s receiving process. In addition, there seems no use case that SFN PDSCH and SFN PDCCH are configured with different SFN schemes.

If frequency offset pre-compensation is enabled, both SFN PDCCH and SFN PDSCH should be pre-compensated.

Conclusions

In summary, the following observations and proposals are made for Rel-17 HST-SFN enhancement.

- Scheme 1 would increase the implementation complexity of Rel-17 UE, due to the additional time-domain interpolation for DMRS estimation.

- Scheme 2 needs higher UE complexity and more DMRS overhead, compared to other proposed schemes for Rel-17 HST-SFN.

- The frequency pre-compensation value can be derived based on implementation, regardless of the carrier frequency of UL RS.

- SFN signals from two TRPs might cancel out with each other at the middle point of two TRPs.

- Lager delay between the SFN signals from two TRPs would cause deep and more frequent fading of the SFN channel, leading to performance degradation of SFN transmission.

- Small delay CDD can improve the UE demodulation performance but is sensitive to delay offset added between the two TRPs.

- Both TCI states associated with average delay would cause ambiguity and imprecision of downlink timing.

- Ignoring {average delay, Doppler shift, Doppler spread} properties of QCL-TypeA can achieve the same effect as Variant C, but an implicit or explicit indication should be designed to inform UE which TCI state to provide only {delay spread} reference.

- If UE doesn’t have the capability to dynamically switch its receiving process between STRP transmission for PDCCH and SFN transmission for PDSCH, the SFN PDSCH transmission can’t be scheduled by the DCI 1_0 transmitted from STRP.

- Support implementation-based frequency offset pre-compensation using uplink signals for Doppler estimation, without defining QCL-like association between uplink signals and TRS.

Support of Doppler feedback should be discussed under the MTRP CSI framework agreed in Rel-17 considering at least the following issues:

        - Association of TRS and CSI report setting/resources sets/subsets

        - Quantization and UCI mapping of the Doppler feedback

        - Mechanisms for saving CSI feedback overhead

- TRP-specific timing offset pre-adjustment can be considered to further enhance the performance of HST-SFN,  e.g. based on feedback of timing offset between two different TRPs.

Support Variant C as the QCL assumption for the frequency offset pre-compensation scheme in Rel-17.

Support to discuss whether UE reports the capability to dynamically switch the receiver between STRP/SFN PDCCH and SFN/STRP PDSCH.

Support to configure/activate one or two TCI states for different search spaces associated with the same CORESET indicated with two TCI states.

If neither enableDefaultTCIStatePerCoresetPoolIndex nor enableTwoDefaultTCI-States is configured for UE, support Alt3 as the default beam assumption for STRP and Rel-16 scheme 3/4  PDSCH when CORESET are indicated with two TCI states.

Using the same mechanism of default beam assumption for A-CSI RS and PDSCH.

The default beam of CSI-RS in case of overlapping with CORESET in time domain should be further discussed.

Reuse the same rule for determination of the default beam of uplink channels and reference signals in case one beam is applied as the default beam.

Take Alt2 into account with high priority for BFD enhancement.

Support a unified RRC parameter, e.g. ‘sfnScheme’ as the indication for both scheme 1 and frequency offset pre-compensation.

Support one of the following indication methods for SFN PDCCH, rather than a specific RRC parameter per CORESET:

- Purely using MAC CE with two TCI states as the indication

- Using the same RRC indication as SFN PDSCH

If frequency offset pre-compensation is enabled, both SFN PDCCH and SFN PDSCH should be pre-compensated.

References

RAN WG1 #104b e-meeting Chairman’s Notes.

Annex

- Evaluation results for HST-SFN solutions

Assuming that UE moves from TRP1 to TRP2, a CDL-based extended channel is used in the simulation. Besides, the SRS resource with the pattern of two consecutive columns is used for uplink Doppler shifts estimation. The evaluation results with and without CFO between TRPs and latency error between frequency estimation and application of frequency offset pre-compensation are given as follows separately.

- Evaluation without CFO and latency error

| a) SNR=8dB | b) SNR=20dB |
| --- | --- |

Evaluation on the performance of frequency offset pre-compensation without CFO and latency error

- Evaluation with CFO and latency error

To present the effect of frequency error on the frequency offset pre-compensation and scheme 1, the latency up to 10ms is considered in the evaluation to show the frequency deviation between the frequency estimation value and application value of frequency offset pre-compensation on PDSCH, as TRS and SRS are transmitted with a typical period of 10ms in the practical HST deployment. Moreover, assume that CFO between two TRPs is a random uniform value between [-0.1ppm, 0.1ppm], i.e. [-350Hz, 350Hz] when the carrier frequency is 3.5GHz.

| a) AMC enabled | b) MSC17 |
| --- | --- |

Evaluation on the performance of frequency offset pre-compensation and scheme 1 with CFO and latency error

- Simulation assumptions

Link-level simulation assumptions

| Parameter | Value |
| --- | --- |
| Duplexing | TDD |
| Carrier frequency | 3.5GHz |
| Subcarrier spacing | 30kHz |
| Propagation condition | CDL-based extension |
| TRP deployment | Ds=700m, Dmin=150m, TRP height=35m, UE height=1.5m |
| gNB antenna port configuration | 2 ports |
| UE antenna port configuration | 2 ports |
| Digital precoding method | Precoding cycling / Type I codebook, PRG=2 |
| TRS periodicity | 10ms, 2 slot TRS |
| DMRS type | Type 1 |
| Number of DMRS symbols | 1+1+1 |
| PDSCH mapping | Type A, Starting symbol 2, Duration 12 |
| Bandwidth | 10RB / 50RB |
| MCS | MCS adaptation / MCS17 |
| Rank | 1 |
| UE speed | 500km/h |