3GPP TSG RAN WG1 #105-e		R1-2104413

e-Meeting, May 10th – 27th, 2021

Agenda Item:     8.1.2.3

Source:	Spreadtrum Communications

Title:	Discussion on enhancements on beam management for multi-TRP

Document for:	Discussion and decision

Introduction

According to R17 FeMIMO WID [1], the following objective is included in the scope:

- Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: c. Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this paper, we will present our opinions on beam management related enhancement for multi-TRP transmission.

Discussion

Enhancement on beam management

For beam measurement/reporting enhancement to facilitate inter-TRP beam pairing, the framework has been formulated as below shown [2].

Agreement On CMR resource configuration for beam reporting option 2, decide in RAN1#105-e whether to adopt “set” or “subset”: - NOTE: the following has been agreed - Two CMR resource sets or subsets, per periodic/semi-persistent CMR resource setting - FFS : extension to aperiodic CMR resource setting if two CMR resource sets are supported - Each reported beam pair in a single CSI -report consists of M = 2 SSBRI/CRI values, where each SSBRI /CRI points to a CMR resource in a different CMR resource set or subset. - FFS : bitwidth of each SSBRI/CRI determined based on the number of SSB/CSI-RS resources from the associated set/subset, or across two sets/subsets Agreement For beam reporting option 2 - On the maximum number of beam pairs/groups (N) that can be reported in a single CSI-report, discuss and down-select from the following two alternatives in RAN1#105-e: - Alt1: Support maximum value N = {1, 2} - Alt2: Support maximum value N = {1, 2, 3, 4} - FFS: Introduce a UE capability Ncap on the maximum value of N in Rel.17 - On the number of beam pairs/groups (N) reported in a single CSI-report, discuss and down select between the following two alternatives in RAN1#105-e - Alt1: The value of N is fixed by RRC configuration - Alt2: The value of N is upper bounded by a maximum value Nmax configured by RRC, and dynamically selected/indicated by UE

For option 2, regarding CMR configuration to achieve one-to-one mapping with TRP, generally both ‘set’ and ‘subset’ could work. However, considering spec workload issue, we slightly prefer ‘set’. In Rel-15/Rel-16, the mechanism of beam measurement and reporting is based on per resource set, e.g., the repetition configuration, scheduling restriction. If introducing ‘subset’, a large of specification should be revised.

Proposal 1: For option 2, regarding CMR configuration, support

- Two CMR resource sets can be configured per periodic/semi-persistent CMR resource setting

- Two CMR resource sets can be simultaneously triggered per aperiodic CMR resource setting

Regarding to the maximum value of N, we have not seen the necessity to support larger than 2, and up to 2 for N is enough.

Proposal 2: For N, not support the value larger than 2.

In Rel-15/Rel-16, group based beam reporting has been supported, where one CSI report only includes one beam group with two beams and different beams within a group can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters. In other words, it is up to UE’s implementation to form beam group based on either a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters. In Rel-17, we should also inherit the principle. In addition, we also have not seen the necessarity.

Proposal 3: Different beams in the same pair/group can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

Regarding whether option 2 could be applied for L1-SINR, on the one hand, we prefer one unified framework for L1-RSRP and L1-SINR. On the other hand, that different beams within a pair/group can be received simultaneously naturally could be applied for inter-beam interference calculation. Thus, we think the above agreement could be applied for L1-SINR.

Proposal 4: For L1-SINR based beam measurement in support of M-TRP simultaneous transmission, support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously.

- M = 2

- N=1 and N=2

Enhancement on BFR

For per-TRP beam failure, last meeting the following agreements are achieved [2].

Agreement - Support S-DCI and M-DCI in TRP-specific BFR in Rel.17 - S-DCI is low priority, M-DCI is high priority - Unified design for S-DCI and M-DCI should not be precluded due to the prioritization Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC. Agreement On BFD-RS of TRP-specific BFR - BFD-RS resource number: - The total number of RSs in two BFR-RS sets per DL BWP is a UE capability - On the maximum number of RS per BFD-RS set, down-select from the following two alternatives in RAN1#105-e - Alt1: max value is 2 - Alt2: max value is a UE capability, including possible candidate value of 1 Agreement For the TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group when beam failure is detected in a one or more CCs in one or more of BFD-RS sets configured in one or more of CCs, - Down select one of the following PUCCH-SR resource selection rules when SR is triggered (or their combinations) for the study, without precluding other alternatives, in RAN1#105-e - Alt-1: PUCCH-SR resource associated with other/non-failed BFD-RS set, association details FFS - Alt-2: PUCCH-SR resource associated with failed BFD-RS set, association details FFS - Alt-3: Leave it up to UE implementation - Note: PUCCH-SR resource is PUCCH resource carrying SR - FFS: Whether two PUCCH-SR resources are under the same or different SR resource configuration or SR configuration (eventual decision may or may not happen in RAN1)

In Rel-15/16, a UE can be provided, for each BWP of a serving cell, a set  of periodic CSI-RS resource configuration indexes by failureDetectionResources. If the UE is not provided  by failureDetectionResources for a BWP of the serving cell, the UE determines the set  to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set  includes RS indexes with QCL-TypeD configuration for the corresponding TCI states. The UE expects the set  to include up to two RS indexes.

Regarding the maximum number of RS per BFD-RS set, considering UE complexity issue, we prefer it is up to UE capability, and candidate value could include 1 and 2.

For M-DCI based M-TRP, other than explicit BFD-RS configuration, BFD-RS set per TRP could also be achieved implicitly by basing on CORESET configurations per TRP. But for S-DCI based M-TRP operation, generally all CORESETs are configured for one TRP. If still following Rel-15/Rel-16 implicit BFD-RS determination rule, then BFD-RS set for another TRP without CORESET configuration could not be achieved. New rule, e.g., BFD-RS set per TRP is determined by basing active TCI states for PDSCH, can be considered. But the benefit should be further justified.

Proposal 5: For the maximum number of RS per BFD-RS set, it is based on UE’s capability, and the candidate values at least include 1, and 2.

Proposal 6: For TRP specific BFR, at least explicit BFD-RS set configuration could be supported.

For BFRQ, one PUCCH-SR resource is enough for FR1. However, for FR2, considering spatial characteristics, two PUCCH-SR resources are needed. For PUCCH non-repetition case, there is no need to introduce new feature to support one PUCCH-SR resource with two spatial relations. In addition, configuring two PUCCH-SR resources for FR2 is very flexible and there is nearly no additional standard work.

Proposal 7: For TRP specific BFR, not support one PUCCH-SR resource configured with 2 spatial relations.

For a UE configured with two PUCCH-SR resources in a cell group, when beam failure is detected in a one or more CCs in one or more of BFD-RS sets, last meeting extensive discussion is carried out, and three alternatives are listed. From our perspective, when it comes to the function of PUCCH-SR resource, on one hand, the PUCCH-SR resource is used to carry SR to request gNB to schedule PUSCH resources for UE to report beam failure information by MAC CE; on the other hand, it could tell gNB there exists DL beam failure, and gNB could quickly adjust its downlink scheduling strategy, e.g., stop DL scheduling for the UE. If considering the above functions, Alt-2 is reasonable. But when UL serving beams for the failed TRP are also failed, Alt-1 is better than Alt-2, due to its quicker reachable to gNB for PUCCH-SR. In summary, Alt-1 is more applicable when the UL beams corresponding to failed TRP is also bad, otherwise, Alt-2 is better. In general, when UE decided to trigger BFR and send information to gNB, whether UL beam failed or not may depend on UE’s judgement. Thus, we prefer PUCCH-SR resource selection is up to UE’s implementation.

Proposal 8: For TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group, when SR is triggered for beam failure procedure, it is up to UE’s implementation to select the PUCCH-SR resource.

Regarding to whether cell-specific and TRP-specific BFR can be configured on the same serving cell, previous meetings have some email discussions but no consensus. For one serving cell configured with M-TRP operation, generally there exists the following three beam failure cases as Fig.1 shows. For case 1 and case 2, beam failure happens in only either one of two TRP, where at least one link could work. TRP-

Figure 1 Beam failure cases for M-TRP

specific BFR is enough. For case 3, both links from TRP1 and TRP2 failed. In this case, generally gNB/UE could try to recover one TRP firstly, then perhaps try to recover another TRP. Or gNB/UE could try to recover both TRP independently. However, the essential still is TRP-specific BFR.

Observation 1:  It is not necessary to configure both cell-specific and TRP-specific BFR.

In RAN1#104e, it has been agreed that R17 BFRQ MAC-CE can convey the information of failed CC indices, and at least a single TRP failed information. In our opinion, R17 BFRQ MAC CE could be utilized to convey cell level bam failure event by simple method, e.g., one bit field in the MAC CE to indicate the validity of TRP failed information. When failure status is different across cells, it also could use R17 BFRQ MAC CE to convey both cell level and TRP level beam failure events.

Propose 9:  Support R17 BFRQ MAC CE to convey cell level beam failure event.

Simultaneous reception of channels/RSs with different QCL-TypeD

Although simultaneous reception of channels/RSs with different QCL-TypeD has been listed as low priority, we still think it could be worth of further study for the sake of performance improvement for multi-TRP operation. Next, we will further discuss about multiple PDCCHs monitoring and multiple DL SPS PDSCH reception.

Multiple PDCCHs monitoring

In R16, for multi-DCI based multi-TRP transmission, different TRPs use different CORESETs with different value of high layer signaling index in PDCCH-Config, and CORESETs with same value of high layer signaling index belongs to the same TRP.

In R15/R16, if a UE is configured for single cell operation or for operation with carrier aggregation in a same frequency band, and monitors PDCCH candidates in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) of one or more cells, the UE monitors PDCCHs only in a CORESET, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties as the CORESET, on the active DL BWP of a cell from the one or more cells, which corresponds to the CSS set with the lowest index in the cell with the lowest index containing CSS, if any; otherwise, to the USS set with the lowest index in the cell with lowest index, and the lowest USS set index is determined over all USS sets with at least one PDCCH candidate in overlapping PDCCH monitoring occasions.

When UE is capable of simultaneously receiving control/data from multi-TRP, in overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) from different TRP, it is natural and simple that UE could monitor PDCCHs in two CORESETs with different QCL-TypeD properities with different value of high layer signaling index, and in any other CORESET from the multiple CORESETs having same QCL-TypeD properties and same value of high layer signaling index  as either one of the two CORESETs. The priority rules of monitoring PDCCHs in R15 could be applied within the CORESETs with the same value of CORESETPoolIndex. Obviously it will enable more flexible scheduling for each TRP and benefit for performance improvement.

Proposal 10: Support to enhance on PDCCH reception for multi-DCI based multi-TRP case.

Proposal 11: In overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) from different TRPs, priority rule of monitoring in Rel-15 should be applied separately for each TRP.

Multiple DL SPS PDSCH reception

In R16, for IIOT scenario with single TRP transmission, multiple semi-persistent scheduling PDSCHs could be configured to meet with the latency requirement. Furthermore, it could also lower overhead. Generally multi-TRP transmission also is one important feature for IIOT. Thus, in our opinion, DL SPS PDSCH for multi-TRP transmission should be supported.

Proposal 12: Support to enhance on DL SPS PDSCH reception for multi-DCI based multi-TRP case.

In Rel-16, part of work has been done to support DL SPS PDSCH for multi-TRP transmission, e.g., the determination of repetition number, the association between SPS PDSCH and TRP where SPS PDSCH is associated with the same TRP as the CORESET activated PDCCH lies in. However, R16 only allows UE receives one PDSCH with lowest configured sps-ConfigIndex when more than one PDSCH(s) without corresponding PDCCH transmission overlapping.

When UE is capable of simultaneously receiving control/data from multi-TRP, it is natural and simple two overlapping PDSCHs each from different TRP could be received by UE. For each TRP, only one PDSCH with lowest configured sps-ConfigIndex could be received in overlapping PDSCH occasions.

Proposal 13: In overlapping PDSCH without corresponding PDCCH transmissions receiving occasions from multiple TRP, one PDSCH with lowest configured sps-ConfigIndex for each TRP could be received.

Regarding how to associate SPS PDSCH with TRP, one natural and simple way is that like dynamic PDSCH, SPS PDSCH associates with the same value of CORESETPoolIndex as the PDCCH which activates the SPS PDSCH.

Proposal 14: PDSCH without corresponding PDCCH transmission associates with the same value of CORESETPoolIndex as CORESET where PDCCH activating the PDSCH lies in.

Conclusion

In this paper, we provide our views on enhancement on beam management for multi-TRP, and have the following proposals.

Observation 1:  It is not necessary to configure both cell-specific and TRP-specific BFR.

Proposal 1: For option 2, regarding CMR configuration, support

- Two CMR resource sets can be configured per periodic/semi-persistent CMR resource setting

- Two CMR resource sets can be simultaneously triggered per aperiodic CMR resource setting

Proposal 2: For N, not support the value larger than 2.

Proposal 3: Different beams in the same pair/group can be received simultaneously by the UE either with a single spatial domain receive filter, or with multiple simultaneous spatial domain receive filters.

Proposal 4: For L1-SINR based beam measurement in support of M-TRP simultaneous transmission, support a single CSI-report consisting of N beams pairs/groups and M (M>1) beams per pair/group, and different beams within a pair/group can be received simultaneously.

- M = 2

- N=1 and N=2

Proposal 5: For the maximum number of RS per BFD-RS set, it is based on UE’s capability, and the candidate values at least include 1, and 2.

Proposal 6: For TRP specific BFR, at least explicit BFD-RS set configuration could be supported.

Proposal 7: For TRP specific BFR, not support one PUCCH-SR resource configured with 2 spatial relations.

Proposal 8: For TRP specific BFR, for a UE configured with two PUCCH-SR resources in a cell group, when SR is triggered for beam failure procedure, it is up to UE’s implementation to select the PUCCH-SR resource.

Propose 9:  Support R17 BFRQ MAC CE to convey cell level beam failure event.

Proposal 10: Support to enhance on PDCCH reception for multi-DCI based multi-TRP case.

Proposal 11: In overlapping PDCCH monitoring occasions in multiple CORESETs that have same or different QCL-TypeD properties on active DL BWP(s) from different TRPs, priority rule of monitoring in Rel-15 should be applied separately for each TRP.

Proposal 12: Support to enhance on DL SPS PDSCH reception for multi-DCI based multi-TRP case.

Proposal 13: In overlapping PDSCH without corresponding PDCCH transmissions receiving occasions from multiple TRP, one PDSCH with lowest configured sps-ConfigIndex for each TRP could be received.

Proposal 14: PDSCH without corresponding PDCCH transmission associates with the same value of CORESETPoolIndex as CORESET where PDCCH activating the PDSCH lies in.

Reference

- RP-202024, Further enhancements on MIMO for NR

- Draft Report of 3GPP TSG RAN WG1 #104b-e