3GPP TSG RAN WG1 Meeting #105-e	R1-2104550

e-Meeting, May 19th – 27th, 2021

Agenda item:		8.12.1

Source:	Nokia, Nokia Shanghai Bell

Title:	Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED UEs

Document for:		Discussion and Decision

# Introduction

The Rel-17 WI on the support of NR Multicast and Broadcast Services [1], includes two RAN1 lead objectives to:

- Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

  - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

- Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided. [RAN1, RAN2]

This document, which covers the first objective related to group scheduling, mainly focuses on the topics for further study indicated as part of the agreements that were made as part of the RAN1-104bis-e meeting.

In section 2, we discuss our views on the above-mentioned agreements, mainly focusing on how to progress with the topics that are currently being further studied.

In section 3, we conclude the document by presenting the summary.

# Discussion on Group Scheduling Mechanism for 5G Multicast / Broadcast

## MBS Common Frequency Region

In this section we will discuss aspects related to MBS common frequency region, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

The down-selection of Option 2A and Option 2B for CFR for multicast of RRC-CONNECTED UEs will be made before the end of RAN1#105-e.

Agreement:

One CFR is supported per dedicated unicast BWP for multicast of RRC-CONNECTED UEs.​

FFS: Whether more than one CFR is supported per dedicated unicast BWP​

FFS: Whether multicast can be supported or not in a dedicated unicast BWP when no CFR is configured for that BWP​

BWP Configuration Options 2A vs. 2B

As presented in [2], the key difference between MBS CFR options 2A and 2B is that: option 2A would require additional RRC signaling for the definition of the MBS specific BWP, possibly reusing parameters currently defined for BWP configurations; and option 2B would require possible RRC signaling indicating the MBS frequency region – in terms of the starting PRB and length of PRBs within each UE’s dedicated unicast BWP. Here RRC signaling is assumed to be used, since it is currently used for BWP related configurations.

Option 2A has the advantage that different PDSCH configurations such as DMRS and resource allocation types, PDSCH aggregation factor, rate match patterns, etc., could be configured specifically for the MBS CFR as compared to the unicast region, but also has the additional complexity of BWP switching in scenarios where the center frequencies of the MBS common frequency region is not aligned within the unicast BWP. Aligning the center frequencies of two bandwidth parts imposes significant scheduling restrictions on the gNB. For option 2A, it needs to be studied further whether the MBS specific BWP would impact the currently defined limits in terms of the total number of BWPs that can be configured. Due to the need for BWP switching, the previous agreements related to the simultaneous reception of unicast and multicast traffic within the same slot – depending on UE capabilities would not be applicable. The need for BWP switching is not required for option 2B, since the MBS frequency region is simply a set of PRBs that are confined within the dedicated unicast BWP and is not classified as a dedicated BWP. As mentioned earlier, if required the PDSCH/PDCCH-config information could be configured separately for the MBS CFR for option 2B. Option 2B also satisfies all the requirements defined for the CFR, requires minimal specification effort and does not require further clarifications from other RAN working groups, and hence would be straightforward to be agreed upon.

Observation-1: The key difference between option 2A and 2B is related to the RRC signaling of the common frequency resources:

- Option 2A requires the signaling of MBS specific BWP with parameters possibly taken from current BWP configurations and would possibly require BWP switching based on center frequency alignment.

- Option 2B requires the signaling of the MBS frequency region – in terms of the starting PRB and length of PRBs within each UE’s dedicated unicast BWP, and PDCCH/PDSCH-config parameters.

- The impact of option 2A on the number of BWPs that can be configured for a UE needs to be studied and clarified.

Observation-2: Currently it is not clear whether simultaneous reception of unicast and multicast traffic within the same slot is possible with option 2A.

Observation-3: Option 2B satisfies all the requirements defined for the CFR, requires minimal specification effort and does not require further clarifications from other RAN working groups.

Proposal-1: Agree on selecting option 2B for configuring multicast common frequency resources, due to the additional complexities involved in the use of option 2A related to BWP switching.

Multiple MBS CFR per UE:

The configuration of multiple MBS CFRs per UE, per UE dedicated unicast BWP was yet another topic that was discussed in the last meeting, which we believe requires separate considerations for multicast and broadcast traffic.

Figure-3: Multiple MBS CFRs per UE for different MBS services.

For multicast traffic, the common frequency resources which are shared across multiple UEs receiving the MBS traffic could be located non-contiguously within a UE-dedicated BWP, as shown in Figure-3. Here these multiple CFRs could be considered as different MBS BWPs based on previously discussed option 2A – where the maximum limit could be based on the limits set for MBS BWP, or simply different sets of contiguous PRBs with a start PRB and CFR size parameters defined for different MBS services. As we mentioned in [2], the motivation for having multiple dedicated BWPs for MBS traffic is still not clear to us, and this perhaps requires further discussions.

Observation-4: Multiple common frequency resources can be configured per UE based on gNB implementation – even though the motivations for doing so are not clear, with the maximum limit dependent on UE capabilities and available system resources.

The configuration of multiple CFRs for broadcast traffic, with non-contiguous resources in the frequency domain would introduce significant complexity for the idle and inactive mode UEs, since the configuration of CFR for such UEs would require either a wider initial BWP or utilizing other solutions. But, as discussed in [3], there are potential benefits for idle/inactive UEs receiving broadcast traffic to be configured with multiple fully overlapped CFRs in terms of power saving and only utilize larger bandwidth when required.

Observation-5: For multicast traffic, the motivation for configuring multiple CFRs per UE requires further clarification, and for broadcast traffic, there are potential benefits in terms of power savings from having multiple overlapping CFRs configured per UE, depending on UE capabilities and traffic characteristics.

Proposal-2: Agree to limit CFRs to one per UE per BWP, considering factors such as additional signaling required for configuring multiple CFRs, and that a single CFR could be utilized to configure multiple MBS services.

Optionality of MBS CFR Configurations:

One of the key issues raised during RAN1-104bis-e is related to whether the MBS CFR configurations are required when the CFR is equal to the unicast BWP. Based on our understanding, the MBS CFR configurations are mainly required for enabling the UE to interpret the frequency domain resource assignment field within the DCI and to configure different parameters for group-common PDCCH/PDSCH if they are different from the configurations used for unicast. If the MBS CFR is equal to the unicast BWP and there is no intention for the gNB to schedule unicast traffic on the BWP, then there is no need to signal the MBS CFR configurations. Also, within a cell, there would be a multitude of users receiving either no multicast traffic or only a subset of the available multicast services. In all of these scenarios, we believe that all the MBS CFR configurations need not be received by all the UEs, and hence could be considered as optional.

Proposal-3: Agree that CFR for multicast defaults to the UE-dedicated unicast BWP, and when there is no explicit unicast traffic scheduled within the BWP.

## MBS CORESET

In this section we will discuss aspects related to MBS coreset, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

If a CFR is configured for multicast in RRC-CONNECTED state and confined within a dedicated unicast BWP, further study the following options.​

Option 1: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.​

Option 2: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.​

Option 3: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR cannot be used for unicast transmission.​

Option 4: the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP cannot be used for multicast transmission even if the CORESET is fully contained in the CFR in frequency domain, but the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.​

As presented in [2], regarding the agreement on the CORESET for PTM transmission scheme 1, that the group-common PDCCH is configured within the CFR for group-common PDSCH, a key open question is related to the number of CORESET(s) for group-common PDCCH that should be configured within the CFR for group-common PDSCH. Currently there are up to 10 search space (SS) sets that could be configured per UE per BWP, where each SS set is associated with a single CORESET, with a maximum of three CORESETs configured per BWP. In order to minimize UE and gNB complexity and to ensure backward compatibility, it would be beneficial to maintain these limits for MBS as well. Thus, it would also be preferable to not define any explicit limits in terms of the number of CORESETs for group-common PDCCH that is allowed within the CFR for group-common PDSCH. The number of CORESETs configured within the MBS CFR could be left to gNB implementation, to be determined based on the control channel load of the MBS traffic.

Observation-6: It would be beneficial to maintain currently defined limits for the total number of CORESETs within PDCCH-config for unicast and MBS, in order to minimize UE and gNB complexity and to ensure backward compatibility.

Proposal-4: The existing limits on the total number of CORESETs for UE-specific BWPs are also applied to those BWPs with MBS CFR, and the number of CORESETs configured within the MBS CFR should be left to gNB implementation.

Regarding the CORESET configuration using PDCCH-config for unicast and MBS, we think that the gNB should have the flexibility to decide whether to use the CORESETs within the dedicated unicast BWP and CFR for both unicast and MBS. Restricting CORESETs to a particular type of traffic would limit the overall availability of PDCCH resources within the BWP, which would be detrimental to the operation of the network.

Observation-7: The gNB should have the flexibility to decide whether to use the CORESETs within CFR of the dedicated unicast BWP for both unicast and MBS, since restricting CORESETs to a particular type of traffic would limit the overall availability of PDCCH resources within the BWP, which would be detrimental to the operation of the network.

Proposal-5: Agree to support option 1 where the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

## Semi-Persistent Scheduling for MBS

In this section we will discuss aspects related to MBS support of SPS, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

Confirm the working assumption:

For activation/deactivation of SPS group-common PDSCH for MBS in RRC_CONNECTED state,

At least group-common PDCCH is supported

FFS: Whether and how to address the missed activation and deactivation

FFS: Whether UE-specific PDCCH is supported for activation/deactivation

Agreement:

The retransmission scheme for a given SPS group-common PDSCH can be either PTM scheme 1 or PTP.

FFS: Whether PTM scheme 1 retransmission and PTP retransmission can be used simultaneously for different UEs in the same MBS group

Agreement:

Define G-CS-RNTI at least for SPS group-common PDSCH and activation/deactivation of SPS group-common PDSCH, different from CS-RNTI for unicast SPS PDSCH.

G-CS-RNTI is used for PTM scheme 1 based dynamic retransmission of SPS group-common PDSCH

FFS: Whether CS-RNTI can be used for PTP retransmission of SPS group-common PDSCH.

FFS: Number of G-CS-RNTI.

Missed Activation/Deactivation:

One of the key issues with the use of group-common PDCCH for activating or deactivating SPS group-common PDSCH is the possibility of missed PDCCH signaling messages. This issue could be handled using blind repetition of the SPS group-common PDCCH activation / deactivation messages – in case NACK-only feedback is utilized, or using UE-specific PDCCH for SPS group-common PDSCH – if ACK/NACK feedback option is used. If the UE-specific PDCCH is used for SPS group-common PDSCH, there needs to be an association between the CS-RNTI and group-common G-CS-RNTI using higher layer signaling.

Proposal-6: Missed SPS activation/deactivation could be handled using blind repetition of the SPS group-common PDCCH activation / deactivation messages – in case NACK-only feedback is utilized, and/or using UE-specific PDCCH for SPS group-common PDSCH – if ACK/NACK feedback option is used.

Observation-8: If the UE-specific PDCCH is used for SPS group-common PDSCH, there needs to be an association between the CS-RNTI and group-common G-CS-RNTI using higher layer signaling.

Proposal-7: RAN2 to consider how to associate CS-RNTI and group-common G-CS-RNTI using higher layer signaling when UE-specific PDCCH is used for SPS group-common PDSCH.

UE-specific PDCCH for SPS and Dynamic Scheduling:

One of the key issues raised from the working assumption from RAN1-104e was related to the support of UE-specific PDCCH for SPS. As discussed in our previous contribution on group scheduling [2], we believe that there is a strong motivation to provide flexibility to the gNB to either use PTM scheme 1 or 2, depending on various implementation-specific criteria. Similar arguments could be applied to SPS as well, where both group-common and UE-specific PDCCH could be used to active the SPS configuration. As discussed in [2], for the scenario where two UEs are receiving the same high-data rate, high-reliability virtual reality traffic, it would make perfect sense to utilize the group-common PDSCH. On the other hand, in this scenario the number of UEs that are served is low, and it is a possible for scheduling independent uplink feedback resources for each UE, as well as UE-specific downlink retransmissions that may be sent on UE-specific PDSCH resources. Thus, the benefits of using group-common PDCCH as compared to UE-specific PDCCH, might be limited in such a scenario. The selection among the options in this scenario could also depend on the overall PDCCH load experienced by the gNB.

Other motivations for supporting both options could include:

(a) seamless mobility – when a UE is handed over from a cell which currently has an ongoing multicast session for an MBS service to a cell where the UE would anyway be scheduled unicast transmission due to lack of ongoing multicast session, and

(b) in case current active BWP of a UE is not the same as the BWP where MBS PDSCH resources are scheduled, UE-specific PDCCH would enable the gNB to instruct the UE to switch its BWP, and to schedule MBS PDSCH resources simultaneously on the same DCI.

Observation-9: Having a UE-specific PDCCH that can schedule UEs to use a group-common PDSCH is desirable for the following reasons:

- In scenarios where there is a low density of users receiving multicast traffic with high data rates and requiring uplink feedback, gNB will have the flexibility to choose the appropriate control channel signaling mechanism

- Enables the support of seamless mobility and switching from multicast to unicast

- Enables simultaneous BWP switching and scheduling of MBS PDSCH resources using the same DCI

- For SPS, it ensures the reliable reception of the SPS activation, deactivation and modification messages.

Observation-10: In order to support both signaling options to access the same group-common PDSCH, new signaling mechanisms will be required to allow the network to configure and modify on a dynamic basis the use of either PTM schemes 1 or 2.

Taking these factors into account, it would be beneficial to have both PTM schemes 1 and 2 available to the gNB for scheduling the multicast transmission, where the gNB can determine the appropriate way of signaling the PDCCH information associated with the group-common PDSCH in an optimal manner.

Proposal-8: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a C-RNTI for dynamic scheduling and CS-RNTI for SPS, to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on a common RNTI.

Proposal-9: The same group-common PDSCH for PTM transmission can be simultaneously accessed by:

- A set of UEs using the same group-common PDCCH with CRC scrambled by a common RNTI, or

- A set of UEs, where each UE uses a UE-specific PDCCH with CRC scrambled by a C-RNTI or CS-RNTI

Proposal-10: The network can dynamically modify the signaling used to configure a UE to access a group-common PDSCH.

Simultaneous PTM Scheme 1 Retransmission and PTP Retransmission:

The simultaneous support for PTM scheme 1 and PTP based retransmissions would lead to some complexities – since the PTM scheme 1 based retransmission could be received by all the UEs whereas PTP based retransmission could be received by only those UEs for which the retransmission is being sent. Due to this consideration, it is beneficial to avoid simultaneous support for PTM scheme 1 and PTP based retransmissions. For PTP based retransmissions, it would be beneficial to confirm whether CS-RNTI or C-RNTI would be used. Also, if UE-specific RNTI is utilized, how could the UE be aware of the association with the SPS group-common PDSCH.

Observation-11: Simultaneous support for PTM scheme 1 and PTP based retransmissions of the same transport block would lead to additional complexities – since the PTM scheme 1 based retransmission could be received by all the UEs whereas PTP based retransmission could be received by only those UEs for which the retransmission is being sent.

Observation-12: If UE-specific RNTI is utilized, the UE needs to be aware of the association between the retransmitted data with the SPS group-common PDSCH.

Proposal-11: Avoid simultaneous support and use of PTM scheme 1 and PTP based retransmissions of the same transport block.

Proposal-12: For PTP based retransmissions, confirm whether CS-RNTI or C-RNTI would be used.

## DCI Formats

In this section we will discuss aspects related to DCI formats for MBS, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

For group-common PDCCH of Rel-17 MBS, support at least two DCI formats.

DCI format 1_0 is used as the baseline for the first DCI format with CRC scrambled with G-RNTI.

DCI format 1_1 or 1_2 is used as the baseline for the second DCI format with CRC scrambled with G-RNTI

FFS(1): Which of DCI format 1_1 or 1_2 is used as the baseline

FFS(2): Details of the reuse (or not) of DCI format 1_0, 1_1 or 1_2

Baseline DCI Format 1_1 vs. 1_2:

Both DCI formats 1_1 and 1_2 have been defined for the scheduling of PDSCH in one cell. Some of the key differences observed based on comparing these formats with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI are as follows [4]:

- MCS, NDI and RV: Format 1_1 has different interpretations for MCS, NDI and RV for TB 1 and 2, whereas format 1_2 has a variable size for RV depending on higher layer parameters with fixed interpretations for MCS and NDI.

- HARQ process number: fixed size of 4 bits for 1_1 whereas 0-4 bits for 1_2 – depending on higher layer configuration

- Downlink assignment index: 0, 2, 4 or 6 bits for 1_1 whereas 0, 1 2, or 4 bits for 1_2

- PUCCH resource indicator: 3 bits for 1_1 vs. 0-3 bits for 1_2

- One-shot HARQ-ACK request, PDSCH group index, Number of requested PDSCH group(s): 0-1 bit for 1_1 vs. field not being present in 1_2

- New feedback indicator: 0-2 bits for 1_1 vs. field not being present in 1_2

Considering these differences between the formats, it would be beneficial to consider format 1_2 as a baseline for the second DCI format for MBS, with CRC scrambled with G-RNTI. Since the baseline format is also used for CS-RNTI, it could be straightforward to assume this format for the SPS case where the CRC is scrambled with G-CS-RNTI.

Observation-13: Format 1_2 has lower number of fields and reduced complexity in terms of the interpretations of various fields based on higher layer configurations.

Observation-14: Format 1_2 is currently used for C-RNTI and CS-RNTI, hence a new baseline format for MBS could consider support for CRC scrambled by both G- and G-CS-RNTI.

Proposal-13: Agree on DCI format 1_2 as the baseline for the second DCI format with CRC scrambled with G-RNTI and G-CS-RNTI.

Reuse of DCI Format 1_0 Fields:

While most of the fields could be reused directly for MBS, some of the key differences are as follows:

FDRA: The field is currently interpreted based on the current UE active DL BWP. But as discussed earlier, this field needs to be interpreted based on the MBS CFR - if configured. If the DCI is scheduled within the CFR, it is up to gNB implementation to match the size of this field with the unicast DCI, if format 1_0 is used to schedule both unicast and MBS traffic.

ChannelAccess-CPext: This field could be assumed to be set to 0 bits if the CRC is scrambled by G- or G-CS-RNTI

Observation-15: Reuse the existing fields in DCI format 1_0 with the following exceptions:

- FDRA field interpreted based on the CFR rather than the unicast DL BWP.

- ChannelAccess-CPext assumed to be set to 0 bits if the CRC is scrambled by G- or G-CS-RNTI

Proposal-14: Agree to reuse existing fields in DCI format 1_0.

- FFS: Interpretation of FDRA based on CFR and ChannelAccess-CPext fields.

Reuse of DCI Format 1_2 Fields:

The following fields are not required for MBS, and hence need not be reused:

Carrier indicator: This field is used for cross-carrier scheduling and is assumed to be 0 bits, since the DCI will be scheduled using CSS [5].

Bandwidth part indicator: This field could be assumed to be 0 bits since there would be no BWP switching required for MBS with the DCI scheduled within the CFR.

FDRA: Similar interpretation as with format 1_0, based on the CFR size.

Observation-16: For format 1_2, fields such as carrier indicator, BWP indicator could be assumed to be set to 0 bits and FDRA field interpretation could be done based on CFR size similar to format 1_0.

For some of the fields such as: TDRA, VRB-to-PRB mapping, PRB bundling size indicator, Rate matching indicator, ZP CSI-RS trigger, RV, HARQ process number, DAI, TPC command for scheduled PUCCH, PRI, priority indicator, etc., the size of the field is determined based on higher layer configuration. For MBS, this would imply that either the higher-layer configurations for all of these fields are aligned for all the UEs receiving MBS, or fixed size assumptions could be applied.

Observation-17: Some of the fields for format 1_2 such as: TDRA, VRB-to-PRB mapping, PRB bundling size indicator, Rate matching indicator, ZP CSI-RS trigger, RV, HARQ process number, DAI, TPC command for scheduled PUCCH, PRI, priority indicator, etc., the size of the field is determined based on higher layer configuration which could be different for different UEs receiving the same multicast service which might lead to different DCI size estimation.

Proposal-15: Discuss further whether fields with variable size depending on higher-layer configuration should be assumed to have fixed size for multicast or use current interpretation with synchronized higher layer configurations across all UEs receiving a particular multicast service.

- FFS: Interpretation of FDRA based on CFR

- FFS: Whether fields such as carrier indicator, BWP indicator could be assumed to be set to 0 bits

- FFS: Issues related to DCI size alignment

## Multiplexing Options

In this section we will discuss aspects related to multiplexing options for MBS, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

At least support the following cases for PDSCH reception for MBS in a slot based on UE capability for RRC_CONNECTED UEs

Case 1: support TDM between M (M>1) TDMed unicast PDSCHs and one group-common PDSCH in a slot per CC

FFS: the value(s) of M

Case 2: support TDM among N (N>1) group-common PDSCHs in a slot per CC

FFS: the value(s) of N

Case 3: support TDM between K (K>1) TDMed unicast PDSCHs and L (L>1) TDMed group-common PDSCHs in a slot per CC

FFS: the value(s) of K and L

Assuming that different values and upper limits of M, N, K and L could be supported depending on UE capabilities, it would be beneficial to leave the values of these parameters to gNB implementation, as compared to defining specific values.

Proposal-16: Agree not to define M, N, K and L parameters in specification and leave the upper limits of these parameters to gNB implementation.

## MBS SS Type and Monitoring Priority

In this section we will discuss aspects related to MBS SS types and monitoring priority related issues, specifically addressing further details, open issues and FFSs from the following agreements from RAN1-104bis-e meeting.

Agreement:

For CSS of group-common PDCCH of PTM scheme 1 for multicast in RRC_CONNECTED state, down-select from the following alternatives (to be decided in RAN1#105):

Alt 1: support Type-3 CSS

The monitoring priority of Type-3 CSS for group-common PDCCH is the same as existing Rel-15/16 CSS, regardless of which DCI format of group-common PDCCH is configured in Type-3 CSS

Alt 2: support a new Type-x CSS

The monitoring priority of new Type-x CSS is determined based on the search space set indexes of the new Type-x CSS set and USS sets, regardless of which DCI format of group-common PDCCH is configured in the new Type-x CSS.

Alt 3: support both Alt 1 and Alt 2

During RAN1-104e, it was agreed to at least support CSS for the SS set of group-common PDCCH for PTM scheme 1 while receiving multicast traffic in RRC_CONNECTED. A key related question is whether to reuse the existing Rel-15/16 CSS type(s) or whether a new CSS type needs to be defined for multicast. If an existing CSS type is reused for this purpose, then one possibility could be Type3-PDCCH which supports the use of C-RNTI and CS-RNTI(s) in the primary cell. However, assuming that similar configurations are used for configuring the CSS, it would be challenging for the UE to apply a different monitoring priority for multicast traffic, as compared to the currently defined ones. Thus, all multicast traffic scheduled using PTM scheme 1 would have higher monitoring priority as compared to traffic scheduled over USS. Since we would like to avoid such a situation, it is proposed to define a new CSS type for multicast.

Another key factor to be considered is the DCI format utilized for scheduling MBS traffic. As mentioned earlier, it has already been agreed that DCI format 1_1 or 1_2 could be used as the second format for scheduling MBS. Since currently these formats are defined only for USS and not CSS, it would be beneficial from simplicity and backward compatibility perspective to define a new type-x CSS for group-common PDCCH of PTM scheme 1. Currently CSS is configured with a longer periodicity as compared to USS, which would also be another key reason to support a new type of CSS as compared to reusing an existing format which would impact existing search space monitoring occasions / periodicities.

Observation-18: Reusing an existing CSS type would imply that all multicast traffic scheduled using PTM scheme 1 would have higher monitoring priority as compared to traffic scheduled over USS.

Observation-19: If DCI format 1_1 or 1_2 is used to schedule group-common PDCCH for multicast, the use of type-3 CSS could lead to backward compatibility issues and added UE complexity.

Observation-20: CSS is configured with a longer periodicity as compared to USS, hence a new type of CSS as compared to reusing an existing format would have minimal impact on existing search space monitoring occasions / periodicities.

Proposal-17: Agree on Alt 2 to define a new type-x CSS for multicast, rather than reusing an existing Rel-15/16 CSS type.

The above proposal and observations would imply that we would prefer Option 2 for monitoring priority, which should be determined based on the SS set indexes for the new CSS type for multicast and USS.

Proposal-18: Agree that the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

## Other Topics

In this section we will discuss aspects related to other topics which were not discussed or agreed during the RAN1-104bis-e meeting.

Configurations Related to MBS Frequency Region:

Starting PRB and the number of PRBs

As presented in [2], since the MBS frequency region is defined essentially as a set of contiguous PRBs within the dedicated unicast BWP, the UE needs to be informed about the starting PRB and length of the MBS frequency region. Here the starting PRB could be defined as a frequency resource offset (FRO) relative to the starting PRB of the UE dedicated BWP. The length of PRBs – which indicate the size of the MBS CFR – could also be signaled along with the frequency resource offset parameter. The UE can utilize these UE-specific parameters in combination with the UE-dedicated unicast BWP to compute the MBS frequency resources. These parameters also enable the use of group-common PDCCH signaling, since the FDRA parameter can be adapted using the UE-specific parameters to compute the exact frequency domain resource assignments.

Figure-1: MBS CFR configurations for two UEs.

Figure-2: Constructing UE-specific FDRA from group-common PDCCH using FRO / Start PRB and MBS CFR Size.

Considering the scenario shown in Figure-1 where UEs 1 and 2 have different UE-dedicated BWP configurations with the MBS CFR configured within BWP-2 for UE-1 and BWP-3 for UE-2. Using the start PRB / FRO and MBS CFR size parameter, the UEs can adapt the FDRA configurations received using the group-common PDCCH DCI into UE-dedicated BWP specific resource allocations, as illustrated in Figure-2. Here we have considered both resource allocation type-0 and type-1, and we can observe that with these two parameters the UE will be able to determine appropriate resource assignments based on the MBS CFR size and starting PRB information.

There are multiple ways in which this information could be signaled to the UE, depending on the scenario considered:

- In the scenario where there is a single MBS frequency region defined per UE or even per cell, the starting PRB and length of the MBS frequency region could be signaled to the UE as part of the BWP (re-)configuration message.

- In the scenario where there are multiple common frequency regions – e.g., depending on the multicast service – the parameters could be signaled together with the group-common RNTI configuration.

Proposal-19: The key requirement for receiving multicast data using group common PDCCH is to signal the starting PRB relative to the UE-dedicated BWP as a frequency resource / PRB offset parameter, and the length of PRBs or CFR size for the MBS CFR.

Note: The signaling details of these parameters could be RAN2 decision.

Currently there are two options being considered for the starting PRB reference point: Option 1: Point A and Option 2: the starting PRB of the dedicated unicast BWP. As illustrated in Figure 2, the easiest option would be option 2, where the starting PRB is referenced to the starting PRB of the dedicated unicast BWP, since this would enable each UE to interpret the FDRA with respect to its own dedicated unicast BWP. Since the UE is already configured with a UE-dedicated BWP and MBS CFR is located within the CFR, and currently defined FDRA configurations are relative to the starting PRB of the BWP rather than point A, we think option 2 would be relatively easier to implement as compared to option 1.

Proposal-20: The starting PRB should be referenced to the starting PRB of the dedicated unicast BWP (option 2 for MBS CFR option 2B).

Unified broadcast CFR design for idle, inactive, and connected:

Since the same broadcast traffic can be received by users in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED mode, it is important to define a unified design framework for broadcast reception, irrespective of the UE RRC state. Depending on the characteristics of the broadcast CFR, it can be further studied and agreed whether there can be commonalities identified between the multicast and broadcast CFR design. Since multicast CFR would not be used for delivering data to users in RRC_IDLE mode, there would be some inherent differences between this CFR and the one for broadcast data. In the scenario where the UE is receiving both multicast and broadcast traffic, it would be logical to have two separate CFRs even if the design framework would be similar, due to how the scheduling information would be signaled to the UE. For multicast, group-common PDCCH could be utilized, whereas for broadcast traffic system information broadcast could be one way of delivering the signaling information.

Proposal-21: Further study and agree on the commonalities identified between the multicast and broadcast CFR design.

Uplink Feedback for SPS:

In our opinion, there is no necessity to specify anything specifically for HARQ feedback for semi-persistently scheduled MBS deviating from what is specified for SPS for unicast, irrespective of whether ACK / NACK or NACK-only feedback is used and whether this is transmitted on UE-specific or group-common PUCCH resources, cf. [3]. The indication of a PUCCH resource for PTM HARQ feedback should as usual be based on n1PUCCH-AN and be timed via PDSCH-to-HARQ_feedback timing indicator (K1) or dl-DataToUL-ACK field relative to the corresponding PDSCH transmission. However, mechanisms to reliably activate / deactivate SPS for MBS are required and discussed in [3].

Proposal-22: Inherit uplink HARQ feedback configuration for SPS-based MBS from SPS for unicast in combination with uplink feedback configuration for non-SPS-based MBS, augmented by mechanisms for reliable SPS activation / deactivation.

Another aspect regarding SPS-based MBS that was marked as for further study in the previous meeting is the handling of retransmissions. Conventionally, SPS only provides pre-allocated resources for first HARQ transmissions while retransmissions are scheduled dynamically, i.e., by sending PDCCHs to indicate individual retransmissions.

However, our studies have shown that PTM in NR can be much more spectrally efficient, when operating with several HARQ transmissions per transport block, cf. Figure-4 below for some results on spectral efficiency (SE) in a dense-urban (IMT-2020 evaluation) scenario with 20 UEs per cell and a maximum of 8 HARQ transmissions per transport block.

The green dotted line shows the reference case of running adaptive modulation and coding (AMC) to meet a target BLER of 1% without any retransmissions, i.e., e.g., using SPS only and no dynamic scheduling of HARQ retransmissions at all. The blue dash-dotted line shows the SE performance for AMC configured with conventional settings from unicast. Even here, it is to be noted that the BLER of 30% applies to the worst UE in the cell, however, in total—due to uncorrelated error events of different UEs—the overall rate of retransmissions in a cell will be typically considerably higher than 30%. Accordingly, while SE of the PTM transmission itself is improved considerably, PDCCH savings of SPS for first transmissions only would already be quite limited. Further considerable improvements in spectral efficiency are achieved with an AMC scheme that relies heavily on HARQ retransmissions, cf. red solid curve. Here, approximately 80% of all PDSCH transmissions are in fact HARQ retransmissions, i.e., for every transport block there are on average 4 HARQ retransmissions. It is to be emphasized that this high rate of retransmissions—when averaged over a time in the order of 100ms—is in fact quite stable due to the fact that multiple UEs with uncorrelated error events contribute to this.

Observation-21: Significantly higher spectral efficiency can be achieved when relying heavily on HARQ retransmissions compared to operation with conventional first HARQ transmission BLER targets for the worst UE in the cell.

Accordingly, it is highly desirable to harvest these SE gains, but it is also clear, that the conventional approach of using SPS only for first transmissions and dynamic scheduling for all HARQ retransmissions will not be an efficient solution due to the large share of HARQ retransmissions. Instead, a scheme that allows for HARQ retransmissions to be sent on SPS-allocated resources would be highly desirable to fully exploit SPS’s potential savings on PDCCH resource consumption. At the same time, dynamic allocation of additional resources for first transmissions or HARQ retransmissions should not be precluded for the sake of flexibility and efficiency.

Figure-4 Spectral Efficiency of PTM with different AMC strategies.

Proposal-23: Support HARQ retransmissions on SPS-allocated resources.

Specifically, we imagine that splitting the downlink grant signaling into two parts as follows would be beneficial:

- The SPS grant, as known already from unicast, is sent to allocate resources that might be used for first HARQ transmissions or HARQ retransmissions or possibly blind retransmissions. This is conventionally transmitted on the PDCCH when there is a need to activate / deactivate / modify the SPS grant.

- Additional information, e.g., a new data indicator, is transmitted along with each MBS data transmission on the PDSCH such that the UEs can decide whether to

  - Start a fresh decoding attempt based on this transmission only, this being the first transmission for a transport block, or

  - Combine this retransmission with previous (re-)transmission(s) for the same transport block and attempt decoding, or

  - Avoid unnecessary decoding attempts, because this transmission belongs to a transport block that this UE has already decoded

Proposal-24: Possibilities to add in-band control signaling on PDSCH to facilitate retransmissions on SPS-allocated PDSCH resources should be studied.

Additional discussions related to the reliability and uplink feedback topics have been presented in [3].

# Conclusions

In this contribution, we have discussed various aspects of group scheduling for this WI.  From those discussions we have the following observations and proposals:

Observation-1: The key difference between option 2A and 2B is related to the RRC signaling of the common frequency resources:

- Option 2A requires the signaling of MBS specific BWP with parameters possibly taken from current BWP configurations and would possibly require BWP switching based on center frequency alignment.

- Option 2B requires the signaling of the MBS frequency region – in terms of the starting PRB and length of PRBs within each UE’s dedicated unicast BWP, and PDCCH/PDSCH-config parameters.

- The impact of option 2A on the number of BWPs that can be configured for a UE needs to be studied and clarified.

Observation-2: Currently it is not clear whether simultaneous reception of unicast and multicast traffic within the same slot is possible with option 2A.

Observation-3: Option 2B satisfies all the requirements defined for the CFR, requires minimal specification effort and does not require further clarifications from other RAN working groups.

Observation-4: Multiple common frequency resources can be configured per UE based on gNB implementation – even though the motivations for doing so are not clear, with the maximum limit dependent on UE capabilities and available system resources.

Observation-5: For multicast traffic, the motivation for configuring multiple CFRs per UE requires further clarification, and for broadcast traffic, there are potential benefits in terms of power savings from having multiple overlapping CFRs configured per UE, depending on UE capabilities and traffic characteristics.

Observation-6: It would be beneficial to maintain currently defined limits for the total number of CORESETs within PDCCH-config for unicast and MBS, in order to minimize UE and gNB complexity and to ensure backward compatibility.

Observation-7: The gNB should have the flexibility to decide whether to use the CORESETs within CFR of the dedicated unicast BWP for both unicast and MBS, since restricting CORESETs to a particular type of traffic would limit the overall availability of PDCCH resources within the BWP, which would be detrimental to the operation of the network.

Observation-8: If the UE-specific PDCCH is used for SPS group-common PDSCH, there needs to be an association between the CS-RNTI and group-common G-CS-RNTI using higher layer signaling.

Observation-9: Having a UE-specific PDCCH that can schedule UEs to use a group-common PDSCH is desirable for the following reasons:

- In scenarios where there is a low density of users receiving multicast traffic with high data rates and requiring uplink feedback, gNB will have the flexibility to choose the appropriate control channel signaling mechanism

- Enables the support of seamless mobility and switching from multicast to unicast

- Enables simultaneous BWP switching and scheduling of MBS PDSCH resources using the same DCI

- For SPS, it ensures the reliable reception of the SPS activation, deactivation and modification messages.

Observation-10: In order to support both signaling options to access the same group-common PDSCH, new signaling mechanisms will be required to allow the network to configure and modify on a dynamic basis the use of either PTM schemes 1 or 2.

Observation-11: Simultaneous support for PTM scheme 1 and PTP based retransmissions of the same transport block would lead to additional complexities – since the PTM scheme 1 based retransmission could be received by all the UEs whereas PTP based retransmission could be received by only those UEs for which the retransmission is being sent.

Observation-12: If UE-specific RNTI is utilized, the UE needs to be aware of the association between the retransmitted data with the SPS group-common PDSCH.

Observation-13: Format 1_2 has lower number of fields and reduced complexity in terms of the interpretations of various fields based on higher layer configurations.

Observation-14: Format 1_2 is currently used for C-RNTI and CS-RNTI, hence a new baseline format for MBS could consider support for CRC scrambled by both G- and G-CS-RNTI.

Observation-15: Reuse the existing fields in DCI format 1_0 with the following exceptions:

- FDRA field interpreted based on the CFR rather than the unicast DL BWP.

- ChannelAccess-CPext assumed to be set to 0 bits if the CRC is scrambled by G- or G-CS-RNTI

Observation-16: For format 1_2, fields such as carrier indicator, BWP indicator could be assumed to be set to 0 bits and FDRA field interpretation could be done based on CFR size similar to format 1_0.

Observation-17: Some of the fields for format 1_2 such as: TDRA, VRB-to-PRB mapping, PRB bundling size indicator, Rate matching indicator, ZP CSI-RS trigger, RV, HARQ process number, DAI, TPC command for scheduled PUCCH, PRI, priority indicator, etc., the size of the field is determined based on higher layer configuration which could be different for different UEs receiving the same multicast service which might lead to different DCI size estimation.

Observation-18: Reusing an existing CSS type would imply that all multicast traffic scheduled using PTM scheme 1 would have higher monitoring priority as compared to traffic scheduled over USS.

Observation-19: If DCI format 1_1 or 1_2 is used to schedule group-common PDCCH for multicast, the use of type-3 CSS could lead to backward compatibility issues and added UE complexity.

Observation-20: CSS is configured with a longer periodicity as compared to USS, hence a new type of CSS as compared to reusing an existing format would have minimal impact on existing search space monitoring occasions / periodicities.

Observation-21: Significantly higher spectral efficiency can be achieved when relying heavily on HARQ retransmissions compared to operation with conventional first HARQ transmission BLER targets for the worst UE in the cell.

Proposal-1: Agree on selecting option 2B for configuring multicast common frequency resources, due to the additional complexities involved in the use of option 2A related to BWP switching.

Proposal-2: Agree to limit CFRs to one per UE per BWP, considering factors such as additional signaling required for configuring multiple CFRs, and that a single CFR could be utilized to configure multiple MBS services.

Proposal-3: Agree that CFR for multicast defaults to the UE-dedicated unicast BWP, and when there is no explicit unicast traffic scheduled within the BWP.

Proposal-4: The existing limits on the total number of CORESETs for UE-specific BWPs are also applied to those BWPs with MBS CFR, and the number of CORESETs configured within the MBS CFR should be left to gNB implementation.

Proposal-5: Agree to support option 1 where the CORESET configured in PDCCH-config for unicast in the dedicated unicast BWP can be used for multicast transmission if the CORESET is fully contained in the CFR in frequency domain, and the CORESET configured in PDCCH-config for MBS in the CFR can be used for unicast transmission.

Proposal-6: Missed SPS activation/deactivation could be handled using blind repetition of the SPS group-common PDCCH activation / deactivation messages – in case NACK-only feedback is utilized, and/or using UE-specific PDCCH for SPS group-common PDSCH – if ACK/NACK feedback option is used.

Proposal-7: RAN2 to consider how to associate CS-RNTI and group-common G-CS-RNTI using higher layer signaling when UE-specific PDCCH is used for SPS group-common PDSCH.

Proposal-8: For RRC_CONNECTED UEs, support UE-specific PDCCH with CRC scrambled by a C-RNTI for dynamic scheduling and CS-RNTI for SPS, to schedule a group-common PDSCH, where the scrambling of the group-common PDSCH is based on a common RNTI.

Proposal-9: The same group-common PDSCH for PTM transmission can be simultaneously accessed by:

- A set of UEs using the same group-common PDCCH with CRC scrambled by a common RNTI, or

- A set of UEs, where each UE uses a UE-specific PDCCH with CRC scrambled by a C-RNTI or CS-RNTI

Proposal-10: The network can dynamically modify the signaling used to configure a UE to access a group-common PDSCH.

Proposal-11: Avoid simultaneous support and use of PTM scheme 1 and PTP based retransmissions of the same transport block.

Proposal-12: For PTP based retransmissions, confirm whether CS-RNTI or C-RNTI would be used.

Proposal-13: Agree on DCI format 1_2 as the baseline for the second DCI format with CRC scrambled with G-RNTI and G-CS-RNTI.

Proposal-14: Agree to reuse existing fields in DCI format 1_0.

- FFS: Interpretation of FDRA based on CFR and ChannelAccess-CPext fields.

Proposal-15: Discuss further whether fields with variable size depending on higher-layer configuration should be assumed to have fixed size for multicast or use current interpretation with synchronized higher layer configurations across all UEs receiving a particular multicast service.

- FFS: Interpretation of FDRA based on CFR

- FFS: Whether fields such as carrier indicator, BWP indicator could be assumed to be set to 0 bits

- FFS: Issues related to DCI size alignment

Proposal-16: Agree not to define M, N, K and L parameters in specification and leave the upper limits of these parameters to gNB implementation.

Proposal-17: Agree on Alt 2 to define a new type-x CSS for multicast, rather than reusing an existing Rel-15/16 CSS type.

Proposal-18: Agree that the monitoring priority is determined based on the search space set indexes of search space set(s) for multicast and USS sets.

Proposal-19: The key requirement for receiving multicast data using group common PDCCH is to signal the starting PRB relative to the UE-dedicated BWP as a frequency resource / PRB offset parameter, and the length of PRBs or CFR size for the MBS CFR.

Note: The signaling details of these parameters could be RAN2 decision.

Proposal-20: The starting PRB should be referenced to the starting PRB of the dedicated unicast BWP (option 2 for MBS CFR option 2B).

Proposal-21: Further study and agree on the commonalities identified between the multicast and broadcast CFR design.

Proposal-22: Inherit uplink HARQ feedback configuration for SPS-based MBS from SPS for unicast in combination with uplink feedback configuration for non-SPS-based MBS, augmented by mechanisms for reliable SPS activation / deactivation.

Proposal-23: Support HARQ retransmissions on SPS-allocated resources.

Proposal-24: Possibilities to add in-band control signaling on PDSCH to facilitate retransmissions on SPS-allocated PDSCH resources should be studied.

# References

- RP-201038, “Revised Work Item on NR Multicast and Broadcast Services” Huawei, RAN#88-e, June 29 – July 3, 2020

- R1-2102655, “Group Scheduling Mechanisms to Support 5G Multicast / Broadcast Services for RRC_CONNECTED UEs,” Nokia, Nokia Shanghai Bell, RAN1#104bis-e, April 12-20, 2021.

- R1-2104551, “Reliability Improvements for RRC_CONNECTED UEs,” Nokia, Nokia Shanghai Bell, RAN1#1045-e, May 19th – 27th, 2021.

- 3GPP TS 38.212, “NR; Multiplexing and channel coding,” v16.5.0, March 2021.

- 3GPP TS 38.213, “NR; Physical layer procedures for control,” v16.5.0, March 2021