| 3GPP TSG-RAN WG1 Meeting #105e e-Meeting, May 10th – 27th, 2021 | R1-2104687 |
| --- | --- |

Agenda item:	8.8.1.3

Source: 	Qualcomm Incorporated

Title: 	 Joint channel estimation for PUSCH

Document for:	Discussion/Decision

# Introduction

RAN 90e agreed to specify joint channel estimation as a technique to extend PUSCH coverage [1]:

- Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4] - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4] - Potential optimization of DMRS location/granularity in time domain is not precluded - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

This document discusses our views on joint channel estimation for PUSCH. More specifically, we shall discuss its use cases, time domain window, interaction between PUSCH DMRS bundling and frequency hopping, relation to PTRS, and DMRS optimization.

# Applicability of Joint Channel Estimation

RAN1 provided the following feedback to RAN4 on possible use cases for joint channel estimation across PUSCH repetitions [2]:

For PUSCH transmission, the following use cases are considered in RAN1. Among the following cases, RAN1 suggest RAN4 to deprioritize use case 5a for PUSCH transmission.  Use case 1: back-to-back PUSCH transmissions within one slot.  Use case 2: non-back-to-back PUSCH transmissions within one slot. ‐ Use case 2a: no uplink transmission in the middle of two PUSCH transmissions ‐ Use case 2b: other uplink transmissions in the middle of two PUSCH transmissions  Use case 3: back-to-back PUSCH transmissions across consecutive slots.  Use case 4: non-back-to-back PUSCH transmissions across consecutive slots. ‐ Use case 4a: no uplink transmission in the middle of two PUSCH transmissions ‐ Use case 4b: other uplink transmissions in the middle of two PUSCH transmissions  Use case 5: PUSCH transmissions across non-consecutive slots. ‐ Use case 5a: no uplink transmission in the middle of two PUSCH transmissions ‐ Use case 5b: other uplink transmissions in the middle of two PUSCH transmissions Note: RAN1 assumes “back-to-back PUSCH transmission” has zero gap in-between adjacent PUSCH transmissions.

RAN4 had provided feedback on the below scenarios in [3]:

- For back-to-back transmissions with zero gap in-between adjacent transmissions

- For non-back-to-back transmission with non-zero gap in-between adjacent transmissions where no downlink reception in-between the PUSCH or PUCCH repetition in the same band for TDD case

However, the following scenarios related to use case 4 (non-back-to-back PUSCH transmissions across consecutive slots) and use case 5 (PUSCH transmissions across non-consecutive slots) are under RAN4 study:

- In scenario of no more than X un-scheduled OFDM symbols in-between the PUSCH or PUCCH repetition (e.g., X = 0, 1, 2, …, 14), and scenario of other physical signals/channels in-between PUCCH or PUSCH repetitions from the UE perspective, e.g., SRS or PUCCH transmission in-between the PUSCH repetition for the UE, RAN4 is still discussing if X can be non-zero value and UE can maintain phase continuity

Further RAN4 feedback in [4]:

- For non-zero un-scheduled gap case for a gap less than 14 symbols, RAN4 confirms the feasibility of phase continuity and power consistency when UE is not required to meet the existing off power requirements. Whether new or existing off power requirements for shorter duration than 1 msec as well as the maximum value of X un-scheduled symbols will be introduced are pending on further RAN4 discussions.

- For the case with other UL channels in between repetitions, at least if the other scheduled signals/channels during the non-zero gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels, it is feasible to maintain the phase continuity and power consistency across the repetitions.

Based on RAN4 feedback, we think RAN1 can provide support for non-zero un-scheduled gap case. In addition, for coverage enhancement, the motivation for use case 2 is not clear. Hence, the following proposals are made:

Proposal 1: Support for the following cases:

- Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

‐   Use case 4a: no uplink transmission in the middle of two PUSCH transmissions

- Use case 5: PUSCH transmissions across non-consecutive slots.

‐   Use case 5a: no uplink transmission in the middle of two PUSCH transmissions

Proposal 2: Do not support the following case:

- Use case 2: non-back-to-back PUSCH transmissions within one slot.

Lastly, the scope of joint channel estimation for PUSCH in the coverage enhancement WID is not only restricted to PUSCH repetition but may also be appliable to PUSCHs carrying different transport blocks. From coverage enhancement perspectives, it makes sense to enable PUSCH repetitions and then apply joint channel estimation across repetitions to achieve desired gain. However, it is not clear on whether joint channel estimation across PUSCHs scheduled to carry different transport blocks should be supported for coverage enhancement scenarios. Therefore, we have made the following observation:

Observation 1: Motivation on having joint channel estimation across PUSCHs scheduled to carry different transport blocks is unclear for coverage enhancement.

# Time Domain Window and UE Indication

## Time Domain Window

The UE is required to maintain phase continuity across bundled PUSCH transmissions to enable promising gain of joint channel estimation at gNB. At first glance, it may seem that the network can assume the UE to maintain the phase continuity across the scheduled PUSCH transmissions subject to the phase continuity conditions. However, from a UE perspective, the UE typically performs some (periodic) events such as uplink tracking loop (e.g., frequency tracking), UE calibration, antenna virtualization etc., at slot boundary. Such events are transparent to the network and expected to impact the phase continuity. If the UE must maintain the phase continuity across scheduled PUSCH transmissions for a very long period, the UE may have to forgo such events leading to possible performance degradation. For example, if the UE forgoes the frequency tracking, the frequency drift would get accumulated, at least impacting UL performance. Therefore, it is beneficial to define bundling windows over which the PUSCH transmissions may be bundled, and RAN1#104b-e made the following agreements on time domain window for joint channel estimation:

Agreements: - For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements. - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window - FFS the units the time domain windown (e.g. repetitions, slots, and/or symbols) - FFS : association between the potential use case(s) and units of the time window - FFS: single or multiple time domain windows - FFS: relation with UE capability - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms - FFS whether or not to further consider impacting of timing advance

Figure 1: Time domain window illustration

Another aspect is when the UE can determine the windows. From UE perspectives, UE implementation would be simpler if the UE could determine the windows before the first PUSCH repetition transmission. In order to enable such determination, the windows could be determined based on simi-static slot format configuration for TDD deployments. Furthermore, based on the slot format, some rules can be defined for UE to determine the start of each window. For instance, the window n can start from the first available slot after window (n-1) for PUSCH repetition transmission that is bundled with subsequent PUSCH repetition transmissions in the nth window as illustrated in Figure 1.

Proposal 3: Support one or multiple non-overlapping time domain windows for joint channel estimation over PUSCH repetitions (for the same TB).

- Window is determined based on semi-static slot format configuration.

- Window duration is in unit of physical slots

- All windows have the same window duration

- FFS: determine start of a window.

## Bundling Indication by UE

Having the UE to indicate whether a PUSCH transmission is coherent with respect to the other PUSCH transmission should be useful for cases where some change on the UE side may impact the phase continuity but such change may not be known to the gNB, for example:

- PUSCH transmit power control depends on pathloss measurement. If the pathloss measurement changes, the transmit power should be changed accordingly, but such power change may not be known to the gNB.

- Another example is that the UE may need to perform some (periodic) events as discussed in Section 3.1. Such event(s) are expected to impact phase continuity across PUSCH transmissions, but gNB is not aware of such event(s).

How to enable UE indication can be further discussed. One possible approach is to allow UE to signal indication in the UCI multiplexing with PUSCH transmission. The indication can indicate whether the PUSCH transmission is coherent with respect to the previous PUSCH transmission or whether the PUSCH transmission is coherent with respect to the next PUSCH transmission as illustrated in Figure 2.

Figure 2: Examples of bundling indication by UE via UCI multiplexed with PUSCH

Proposal 4: For each PUSCH transmission, the UE signals a bundling indication in the PUSCH transmission.

# Interaction with Inter-slot Frequency Hopping

It was agreed to consider inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation for PUSCH. In general, we can have two cases as follows:

- Non-interleaving case (as illustrated in Figure 3): The bundled PUSCH transmissions in one frequency hop are first transmitted and then another bundled PUSCH transmissions are transmitted in the other frequency hop.

- Interleaving case (as illustrated in Figure 4): PUSCH transmissions are interleaved between the first frequency hop and the second frequency hop.

Figure 3: Non-interleaving case

Figure 4: Interleaving case

RAN4 LS reply [3] provided answer to question 1 on what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions. One of the conditions is that intra-slot and inter-slot frequency hopping is not enabled within a repetition bundle:

- Question 1: Under what conditions UE can keep phase continuity cross PUCCH or PUSCH repetitions - RAN4 Answer for question 1: If the following conditions are met - Modulation order does not change. - RB allocation in terms of length and frequency position should not be changed, and intra-slot and inter-slot frequency hopping is not enabled within a repetition bundle. - No change on transmission power level of its own CC, i.e., no change on the power control parameters specified in TS 38.213, and also when own CC is not impacted by other concurrent CC(s) that are configured for inter-band CA or DC for same UE with dynamic power sharing and no change in any configured CC s that are part of configured intra-band uplink CA or DC. - No UL beam switching for FR2 UE occurs

Observation 2: Within a single time domain DMRS bundling window, intra-slot and inter-slot frequency hopping is to be disabled based on RAN4 feedback.

Proposal 5: When inter-slot frequency hopping is configured with DMRS bundling, all PUSCH transmissions in a single time domain DMRS bundling window belong to the same hop.

Furthermore, RAN1 104b-e made the following agreements:

Agreements: - For inter-slot frequency hopping with inter-slot bundling, down select on the following two options: - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size. - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size. - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined. - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD. - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

Therefore, if multiple non-overlapping time domain windows are supported, the frequency hopping can be tied to the window to avoid additional signalling. Hence, we are making the following proposal:

Proposal 6: For inter-slot frequency hopping with inter-slot bundling, the bundle size (time domain hopping interval) equals to the time domain window size (Option 1).

# Joint Channel Estimation in presence of PTRS

In 5G, PTRS was introduced to track phase noise (especially for FR2). Compared to DMRS, PTRS has much less frequency density, which results in much smaller overhead (compared to DMRS). In presence of DMRS, gNB may be able to not only track the continuous phase noise inside a time slot, but also estimate the phase jumps across slots. In this, by compensating the estimated phase jumps across slots, gNB may be able to apply DMRS bundling even for the case that UE is not able of maintaining phase continuity or when the phase continuity is not perfect. However, relying on PTRS for estimating the phase jumps and compensating them may require a higher density for PTRS (e.g., higher time density). Currently, the presence of PTRS and its time density is determined based on configurable thresholds for MCS (as shown in Table 1).

Table 1. Determining presence of PTRS and its time density based on configured MCS thresholds.

| Scheduled MCS | Time density |
| --- | --- |
| IMCS < ptrs-MCS1 | PT-RS is not present |
| ptrs-MCS1 IMCS < ptrs-MCS2 | 4 |
| ptrs-MCS2 IMCS < ptrs-MCS3 | 2 |
| ptrs-MCS3 IMCS < ptrs-MCS4 | 1 |

It would be useful to allow a different set of MCS thresholds to determine the presence of PTRS (and its time density), when DMRS bundling is applied.

Proposal 7: Support different criteria for activation of PTRS or its density for the case of joint channel estimation.

Figure 4 shows the performance of PUSCH repetition over two time slots, when a random phase jump (glitch) is applied at the slot boundary (i.e. the worst case which means no phase continuity). In absence of phase jump, no PTRS is needed for PUSCH reception for MCS 7, while it is useful for MCS 16. However, when DMRS bundling is applied in presence of the phase jump, PTRS becomes necessary for both MCS 7 and MCS 16. This shows that different MCS thresholds are useful to determine presence of PTRS (or its time density). The simulation results also show that when gNB can rely on PTRS for estimating the phase jumps, the phase continuity requirement can be relaxed.

Figure 5. Reception of PUSCH repetition over two time slots, in presence of worst random phase jump at the slot boundary and using PTRS to estimate and compensate it (with PTRS with time density of every other symbol)

# DMRS Optimization

RAN1#104e made the following agreements on DMRS optimization in junction with joint channel estimation:

Agreements: - Companies are encouraged to study optimization of DMRS granularity in time domain with joint channel estimation, including: - Use cases - Simulations results - Enhanced schemes, e.g., - Different DMRS density for different PUSCH transmissions - No DMRS for some PUSCH transmissions - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method. - Companies are encouraged to study optimization of DMRS location in time domain with joint channel estimation, including: - Use cases - Simulations results - Enhanced schemes, e.g., - DMRS equally spaced among PUSCH transmissions - DMRS located in special slots - Orphan symbol used for DMRS - If applicable, impact of dynamic changes, e.g., cancellation of a repetition and companies report the evaluation method. - Note: the simulation assumptions for DM-RS in TR 38.830 are used as baseline for performance evaluation on optimization of DMRS location/granularity in time domain. - Take into account impairments such as frequency offset, and report corresponding parametrization together with the results. Further discuss impairment details. Agreements: - For joint channel estimation. - Take into account the residual frequency error, e.g., +/- 0.1 ppm as upper bound. - Companies can report other values and frequency error model.

We are not in favor of any DMRS granularity optimizations. In particular, if DMRS bundling is enabled across K repetitions, we must be prepared to accommodate scenarios where one or more of the K repetitions are not transmitted due to cancellations or prioritizations. For example, if PUSCH is scheduled for 4 repetitions over 4 consecutive slots, and say we completely eliminated DMRS from the middle two slots. Now if for some reason, the last repetition is cancelled, we are then left with 3 actual repetitions, with just one of the three repetitions having DMRS symbols. This clearly is not a desired outcome.

Additionally, we prefer to keep the same granularity across all repetition so that if a gNB wishes to perform per-slot DMRS processing, this option remains feasible without suffering a big impact on performance.

Further, if UCI is multiplexed with PUSCH, then there can be instances where the gNB prefers to decode the multiplexed UCI before all remaining repetitions arrive. This serves as another motivation to not overly handicap per-slot DMRS processing.

With these considerations in mind, we make the following proposal:

Proposal 8: Maintain the same DMRS granularity across all PUSCH transmissions that are configured for DMRS bundling.

Based on the above discussion, since preserving per-slot DMRS processing is important, we don’t see a strong need to change DMRS locations in time domain. DMRS locations must be chosen so that per-slot processing isn’t overly handicaped. This leads us to the following proposal:

Proposal 9: No change in DMRS locations compared to R15/ R16 for PUSCH transmissions that are configured with DMRS bundling.

DMRS Symbols in S Slot

Additional proposals were made in the last meeting on introducing DMRS in the S slot, i.e. consider TDD configurations such as DDDSU slot pattern, with PUSCH occupying the U slot and the resources in the S slot being used as additional DMRS locations for this PUSCH. It is not surprising that additional DMRS resources that do not reduce data resources can improve performance. However the availability of these resources and the mechanics of enabling such a mode of operation warrants further investigation.

Figure 6 DMRS in S slot

In particular, a DMRS Symbol that precedes a PUSCH transmission has a significant impact on PUSCH timeline. The UE now needs to prepare to transmit this PUSCH one additional slot ahead in time. If this PUSCH is dynamically triggered, then the slot offset indicated by the DCI may no longer line up with the timeline that the UE has to operate on in order to accommodate this additional DMRS. Further, dedicated signaling mechanisms will need to be developed to indicate to the UE when an additional DMRS symbol needs to be placed in the S slot. This could entail a large effort to specify this feature.

Further note that the uplink resources in the S slot are typically set aside to carry either SRS or PUCCH. Thus they may only be sporadically available for PUSCH. Further, when PUSCH is configured with multiple repetitions and DMRS bundling is enabled across these repetitions, the presence of an additional DMRS in the S slot may not provide any significant benefits.

Based on these considerations, we make the following proposal:

Proposal 10: Due to potentially large specification impact and decreasing gains as PUSCH repetitions increase (with large bundling window), DMRS locations in S slot can be considered with a lower priority.

# Conclusion

This document discusses our views on joint channel estimation as a technique to extend PUSCH coverage. In particular, we have made the following observations and proposals:

Observation 1: Motivation on having joint channel estimation across PUSCHs scheduled to carry different transport blocks is unclear for coverage enhancement.

Observation 2: Within a single time domain DMRS bundling window, intra-slot and inter-slot frequency hopping is to be disabled based on RAN4 feedback.

Proposal 1: Support for the following cases:

- Use case 4: non-back-to-back PUSCH transmissions across consecutive slots.

‐   Use case 4a: no uplink transmission in the middle of two PUSCH transmissions

- Use case 5: PUSCH transmissions across non-consecutive slots.

‐   Use case 5a: no uplink transmission in the middle of two PUSCH transmissions

Proposal 2: Do not support the following case:

- Use case 2: non-back-to-back PUSCH transmissions within one slot.

Proposal 3: Support one or multiple non-overlapping time domain windows for joint channel estimation over PUSCH repetitions (for the same TB).

- Window is determined based on semi-static slot format configuration.

- Window duration is in unit of physical slots

- All windows have the same window duration

- FFS: determine start of a window.

Proposal 4: For each PUSCH transmission, the UE signals a bundling indication in the PUSCH transmission.

Proposal 5: When inter-slot frequency hopping is configured with DMRS bundling, all PUSCH transmissions in a single time domain DMRS bundling window belong to the same hop.

Proposal 6: For inter-slot frequency hopping with inter-slot bundling, the bundle size (time domain hopping interval) equals to the time domain window size (Option 1).

Proposal 7: Support different criteria for activation of PTRS or its density for the case of joint channel estimation.

Proposal 8: Maintain the same DMRS granularity across all PUSCH transmissions that are configured for DMRS bundling.

Proposal 9: No change in DMRS locations compared to R15/ R16 for PUSCH transmissions that are configured with DMRS bundling.

Proposal 10: Due to potentially large specification impact and decreasing gains as PUSCH repetitions increase (with large bundling window), DMRS locations in S slot can be considered with a lower priority.

# Reference

- RP-202928, New WID on coverage enhancements, RAN 90e, December 2020.

- R1-2104119, Reply LS on PUCCH and PUSCH repetition

- R4-2103393, Reply on LS on PUCCH and PUSCH repetition.

- R4-2105417, Reply LS on PUCCH and PUSCH repetition