3GPP TSG RAN WG1 #105-e		R1-2104731

e-Meeting, May 10th – 27th, 2021

Source:	OPPO

Title:	Text Proposals for Tx Switching between Two Uplink Carriers

Agenda Item:	7.2.12

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss a remaining issue for the current specification of UL Tx switching between two uplink carriers and present the corresponding text proposal to address.

Although the similar proposal/CR is proposed in RAN1#104-e meeting, some companies suggested to discuss it in the NR CA/DC session. In RAN1#104bis-e meeting, similar discussions on the RRC parameter were carried out in NR CA/DC session, and RAN1 agreed a CR to update RRC parameter as below [R1-2104019 (TS38.213, CR0216, Rel-16)]

RAN1#104bis-e output for power control [R1-2104019 (TS38.213, CR0216, Rel-16)] - tdm-PatternConfig-r15 is updated to tdm-PatternConfig - tdm-PatternConfig-r16 is updated to tdm-PatternConfig2

However, the above modification/CR approved by NR CA/DC session is only applicable for the power control part of TS 38.213, and not applicable for the UL Tx switching part of TS 38.214. Thus, the same issue still exists for UL Tx switching.

Correction of RRC parameter “tdm-PatternConfig-r16”

| Reason for change: | The RRC parameter “tdm-PatternConfig-r16” used in the current version of TS 38.214, whereas the RRC parameter “tdm-PatternConfig2-r16” is used in TS 36.331. Thus, the RRC parameters in TS 38.214 and TS 36.331 are not aligned. “tdm-PatternConfig-r15” should also be modified as “tdm-PatternConfig” |
| --- | --- |
| Summary of change: | Align the RRC parameters of TS 38.214 and TS 36.331. - tdm-PatternConfig-r15 is changed to tdm-PatternConfig - tdm-PatternConfig-r16 is changed to tdm-PatternConfig2 |
| Consequences if not approved: | Misalignment between TS 38.214 and TS 36.331 |
| Clauses affected: | 6.1.6.1 |

The corresponding part in TS 36.311 is copied as below

RRCConnectionResume-v1610-IEs ::=	SEQUENCE {

idleModeMeasurementReq-r16			ENUMERATED {true}				OPTIONAL,	-- Need ON

restoreMCG-SCells					ENUMERATED {true}				OPTIONAL,	-- Need ON

restoreSCG							ENUMERATED {true}				OPTIONAL,	-- Cond EarlySec

sCellToAddModList-r16				SCellToAddModList-r16			OPTIONAL,	-- Cond EarlySec

sCellToReleaseList-r16				SCellToReleaseListExt-r13		OPTIONAL,	-- Need ON

sCellGroupToReleaseList-r16			SCellGroupToReleaseList-r15		OPTIONAL,	-- Need ON

sCellGroupToAddModList-r16			SCellGroupToAddModList-r15		OPTIONAL,	-- Cond EarlySec

nr-SecondaryCellGroupConfig			OCTET STRING					OPTIONAL,	-- Cond RestoreSCG

p-MaxEUTRA-r16						P-Max							OPTIONAL,	-- Cond SCG

p-MaxUE-FR1-r16						P-Max							OPTIONAL,	-- Cond SCG

tdm-PatternConfig-r16				TDM-PatternConfig-r15			OPTIONAL,	-- Cond FDD-PCell

tdm-PatternConfig2-r16				TDM-PatternConfig-r15			OPTIONAL,	-- Need OR

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

tdm-PatternConfig This field is used when power control or IMD issues require single UL transmission in (NG)EN-DC as specified in TS 38.101-3 [101] and TS 38.213 [88]. tdm-PatternConfig2 This field is used for dual UL transmission in EN-DC with LTE FDD PCell and for single UL transmission in EN-DC with LTE FDD/TDD PCell, as specified in TS 38.101-3 [101] and TS 38.213 [88]. The network sets at most one of tdm-PatternConfig and tdm-PatternConfig2 to setup. When this field is configured in EN-DC with LTE TDD PCell, it is not applicable if TDD configuration is sa0 or sa6 in SIB1.

Thus, we propose to have the following proposals:

Proposal 1: For UL Tx switching in TS 38.214, correct the corresponding RRC parameters as below:

- tdm-PatternConfig-r15 is changed to tdm-PatternConfig

- tdm-PatternConfig-r16 is changed to tdm-PatternConfig2

Thus, we propose the following text proposal to align the RRC parameters of TS 38.214 and TS 36.331:

In TS 38.214 Section 6.1.6 <omitted text> - 6.1.6.1 Uplink switching for EN-DC For a UE indicating a capability for uplink switching with BandCombination-UplinkTxSwitch for a band combination, and if it is for that band combination configured with a MCG using E-UTRA radio access and with a SCG using NR radio access (EN-DC), if the UE is configured with uplink switching with parameter uplinkTxSwitching, - for the UE configured with switchedUL by the parameter uplinkTxSwitchingOption, when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - when the UE is to transmit an NR uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - when the UE is to transmit an E-UTRA uplink that takes place after an NR uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - the UE is not expected to transmit simultaneously on the NR uplink and the E-UTRA uplink. If the UE is scheduled or configured to transmit any NR uplink transmission overlapping with an E-UTRA uplink transmission, the NR uplink transmission is dropped, - for the UE configured with uplinkTxSwitchingOption set to 'dualUL', when the UE is to transmit in the uplink based on DCI(s) received before T0-Toffset or based on a higher layer configuration(s): - when the UE is to transmit an NR two-port uplink that takes place after an E-UTRA uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - when the UE is to transmit an E-UTRA uplink that takes place after an NR two-port uplink on another uplink carrier then the UE is not expected to transmit for the duration of NTx1-Tx2 on any of the two carriers. - the UE is not expected to transmit simultaneously a two- port transmission on the NR uplink and the E-UTRA uplink. - in all other cases the UE is expected to transmit normally all uplink transmissions without interruptions. - when the UE is configured with tdm-PatternConfig-r15 or by tdm-PatternConfig-r16 tdm-PatternConfig2 - for the E-UTRA subframes designated as uplink by the configuration, the UE assumes the operation state in which one-port E-UTRA uplink can be transmitted. - for the E-UTRA subframes other than the ones designated as uplink by the configuration, the UE assumes the operation state in which two-port NR uplink can be transmitted. <omitted text>

Conclusion

In this contribution, we discuss one remaining issue on the current specification of UL Tx switching between two uplink carriers and also propose the corresponding text proposal to fix it.

References

- TS 38.214, NR; Physical layer procedures for data

- TS 36.331, E-UTRA; Radio Resource Control (RRC) protocol specification