3GPP TSG RAN WG1 #105-e																			R1-2104919

e-Meeting, May 10th – 27th, 2021

Source: 	Intel Corporation

Title:	Enhancements on PUSCH repetition type A

Agenda item:	8.8.1.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#104-e meeting, the following agreements and conclusions were made for enhancements on PUSCH repetition type A [1]:

Agreements:

Select one of the following alternatives, considering the aspect whether or not the determination of all the available slots should be done prior to the first actual transmission of the repetitions (other alternatives are not precluded)

- Alt1: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and does not depend on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

- Alt2: Whether or not a slot is determined as available for UL transmissions depends on RRC configurations (at least tdd_ul_dl configuration, FFS: other RRC configurations) and also depends on dynamic signaling (at least SFI, FFS: other dynamic signaling e.g. CI, PUSCH priority for URLLC).

Agreements:

The maximum number of repetitions for DG-PUSCH is also applicable to CG-PUSCH.

Agreements:

For defining available slots: a slot is determined as unavailable if at least one of the symbols indicated by TDRA for a PUSCH in the slot overlaps with the symbol not intended for UL transmissions

- FFS details

Agreements:

Rel-17 PUSCH repetition Type A supports the increase of maximum number of repetitions with repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI.

- FFS: increasing the maximum number of repetitions with repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

Conclusion:

Discuss further to select one of the following alternatives:

- Alt-a: The determination of all the available slots has to be done prior to the first actual transmission of the repetitions.

- Alt-b: The determination of all the available slots does not have to be done prior to the first actual transmission of the repetitions. The timeline requirement is per repetition basis.

In the contribution, we discuss enhancements on PUSCH repetition type A. Our views on TB processing over multi-slot PUSCH and joint channel estimation for PUSCH are described in our companion contributions [2] and [3], respectively. In addition, our view on PUCCH enhancements is presented in our companion contribution [4].

# Maximum number of repetitions for PUSCH

In NR Rel-15, multi-slot based transmission for PUSCH was supported, which can be used to improve the coverage. More specifically, the number of slots for PUSCH repetitions can be 2, 4 or 8. Further, in Rel-16, the number of repetitions can be dynamically indicated in the DCI for scheduling PUSCH. In addition, the maximum number of PUSCH repetitions is increased to 16.

Figure 1 illustrates link level simulation results for PUSCH with different number of repetitions. In the simulations, it is assumed DFT-s-OFDM waveform and intra-slot frequency hopping for PUSCH. In addition, it is assumed TBS = 136, MCS = 0, 2 DMRS symbols are allocated in each slot and moving speed of 3km/h. From the figure, it can be observed that link level performance for PUSCH can be improved by increasing the number of repetitions. More specifically, ~2dB performance gain can be observed when doubling the repetition levels for PUSCH.

Figure 1. Simulation results for PUSCH with repetitions

To further improve the coverage, especially when considering TDD configuration with limited UL slots, maximum number of repetitions can be increased for PUSCH repetition type A. More specifically, the maximum number of repetitions for PUSCH may be increased from 16 to 32. Note that further increase of maximum repetition levels for PUSCH may not be desirable given that the NR coverage enhancement is mainly targeted for non-LPWA services, where UEs are not in deep coverage scenarios.

Observation 1

- ~2dB performance gain can be observed when the repetition level for PUSCH is doubled.

Proposal 1

- Maximum number of repetitions for PUSCH repetition type A can be increased to 32.

At the RAN1#104e meeting, it was agreed that the increased maximum number of repetitions is supported for repetition factors configured in a TDRA list with a row index indicated either by the configured grant configuration or by TDRA field in a DCI. It is FFS whether to support the increased maximum number of repetitions for the repetition factor configured in PUSCH-Config and/or ConfiguredGrantConfig.

As defined in the specification, when numberofrepetitions is not presented in the TDRA table and if pusch-AggregationFactor is configured in PUSCH-Config, the number of repetitions is determined based on pusch-AggregationFactor. In this regard, it is more desirable to also support pusch-AggregationFactor with the increased maximum number of repetitions. In particular, when UE is not configured with number of repetitions in the TDRA table, sufficient number of repetitions for PUSCH can still be possible, which can help meet the coverage enhancement target. The same mechanism can also apply for repK in ConfiguredGrantConfig.

Proposal 2

- Rel-17 PUSCH repetition Type A supports the increased maximum number of repetitions with repetition factor for pusch-AggregationFactor configured in PUSCH-Config and repK in ConfiguredGrantConfig.

# PUSCH repetition based on available UL slots

In Rel-15, for PUCCH repetition in TDD system, PUCCH is only transmitted in a slot when the number of available symbols is greater than or equal to the configured or indicated number of symbols for PUCCH transmission. Further, in case of cancellation due to collision with semi-static configured DL symbols or SSB transmission, UE would postpone the PUCCH repetition until the configured number of repetitions is reached.

For coverage enhancement of PUSCH with repetition type A, similar mechanism can be considered for unpaired spectrum to allow UE to postpone the PUSCH transmission until all the configured or indicated number of repetitions is reached. In this regard, the number of actual repetitions for PUSCH transmission can be guaranteed, especially in case of cancellation due to collision with semi-static configured DL symbols, SSB transmissions, CORESET0 with Type0-PDCCH CSS set and invalid UL symbols, which can ensure the coverage performance for PUSCH. As defined in Rel-16, invalid UL symbols are semi-statically configured after the last symbol in a consecutive number of symbols that are indicated as DL, which can be used as DL to UL switching purpose.

Note that it may not be desirable to consider dynamic indication including SFI indicated in the DCI format 2_0, or priority indicator to determine the availability of UL slots for PUSCH repetition. This is primarily due to the fact that if UE mis-detects the DCI carrying dynamic indication, UE may have different understanding from gNB on which slots to be used for PUSCH repetitions. With postponement mechanism for PUSCH repetition, this may introduce undesirable interference or even cross-link interference in the network.

Figure 2 illustrates one example of PUSCH with repetition type A on the basis of available UL slots. In the figure, the number of UL symbols in the 2nd slot is not sufficient for PUSCH transmission. In this case, UE would transmit the PUSCH in the 3rd slot assuming the number of repetitions is 2.

Figure 2. PUSCH repetition on the basis of available UL slots

Based on the discussions above, determination of available UL slots for PUSCH repetition type A should only depend on RRC configurations and not depend on dynamic signalling. In addition, symbols not intended for UL transmissions for RRC configurations include semi-static configured DL symbols, SSB transmissions, CORESET0 with Type0-PDCCH CSS set and invalid UL symbols. Further, determination of all the available slots is performed prior to the first actual transmission of the repetitions, which can help simplify UE implementation, i.e., without recounting the number of repetitions due to dynamic signalling.

Proposal 3

- For PUSCH repetition type A, determination of available UL slots depends on RRC configurations and does not depend on dynamic signalling.

  - Symbols not intended for UL transmissions for RRC configurations include semi-static configured DL symbols, SSB transmissions, CORESET0 with Type0-PDCCH CSS set and invalid UL symbols.

- Determination of all the available slots is performed prior to the first actual transmission of the repetitions.

Note that the above criteria would apply for both DG-PUSCH and CG-PUSCH with repetition type A. Further, in case when the number of repetitions is large, it may not be desirable to postpone the PUSCH repetitions after a time window, especially for CG-PUSCH. To avoid excessive resource waste and improve system level spectral efficiency, the time window may be configured to allow UE to postpone the PUSCH repetitions to a certain number of slots.

Proposal 4

- For PUSCH with repetition type A, a time window may be configured to allow UE to postpone the PUSCH repetitions to a certain number of slots.

# Conclusions

In this contribution, we discussed enhancements on PUSCH repetition type A. Further, we summarize the observations and proposals as follows:

Observation 1

- ~2dB performance gain can be observed when the repetition level for PUSCH is doubled.

Proposal 1

- Maximum number of repetitions for PUSCH repetition type A can be increased to 32.

Proposal 2

- Rel-17 PUSCH repetition Type A supports the increased maximum number of repetitions with repetition factor for pusch-AggregationFactor configured in PUSCH-Config and repK in ConfiguredGrantConfig.

Proposal 3

- For PUSCH repetition type A, determination of available UL slots depends on RRC configurations and does not depend on dynamic signalling.

  - Symbols not intended for UL transmissions for RRC configurations include semi-static configured DL symbols, SSB transmissions, CORESET0 with Type0-PDCCH CSS set and invalid UL symbols.

- Determination of all the available slots is performed prior to the first actual transmission of the repetitions.

Proposal 4

- For PUSCH with repetition type A, a time window may be configured to allow UE to postpone the PUSCH repetitions to a certain number of slots.

# References

- Chairman’s notes, RAN1 #104 e-Meeting, January 2021

- R1-2104920, “Discussion on TB processing over multi-slot PUSCH”, Intel Corporation, e-Meeting, May 2021

- R1-2104921, “Discussion on joint channel estimation for PUSCH”, Intel Corporation, e-Meeting, May 2021

- R1-2104922, “Discussion on PUCCH enhancements”, Intel Corporation, e-Meeting, May 2021