3GPP TSG RAN WG1 #105-e																			R1-2104921

e-Meeting, May 10th – 27th, 2021

Source: 	Intel Corporation

Title:	Discussion on joint channel estimation for PUSCH

Agenda item:	8.8.1.3

Document for:	Discussion and Decision

# Introduction

At the RAN1#104b-e meeting, the following agreements and conclusions were made regarding joint channel estimation for PUSCH [1]:

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain window (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

  - FFS: relation with UE capability

  - FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

  - FFS whether or not to further consider impacting of timing advance

Agreements:

- A new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17.

Agreements:

- For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

Conclusion:

- For optimization of DMRS granularity in time domain with joint channel estimation, the proponents are encouraged to provide more simulation results in next meeting

Agreements:

- For the time domain window for joint channel estimation, down select on the following two options:

  - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

  - Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

Agreement:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

  - Only for single layer transmissions

  - Subject to UE capability

  - FFS: Over back-to-back PUSCH transmissions with different TBs

In the contribution, we discuss joint channel estimation for PUSCH coverage enhancement, with primary focus on potential optimization of DMRS patterns and inter-slot frequency hopping with inter-slot bundling. Our views on enhancement on PUSCH repetition type A, TB processing over multi-slot PUSCH, PUCCH enhancements, and Msg3 PUSCH repetition are presented in our companion contributions [2], [3], [4] and [5], respectively.

# Discussion on joint channel estimation for PUSCH

At the RAN1#104e meeting, it was agreed that joint channel estimation can be at least supported for back-to-back PUSCH transmissions for repetition type A. In addition, it was agreed as working assumption that joint channel estimation can be enabled for TBoMS when back to back transmission over consecutive slots is considered [6]. Further, at the RAN1#104b-e meeting, it was agreed that joint channel estimation can be supported for back-to-back PUSCH transmissions for repetition type B [1].

Note that joint channel estimation can help in improving the channel estimation performance, and hence overall link budget of uplink transmission. This is of primary importance as coverage enhancement solutions are mainly targeted for low SNR regime where channel estimation is typically a performance bottleneck. However, for PUSCH transmission with different TBs, it is expected that UE is in good channel conditions, which implies that joint channel estimation may not be necessary for improving channel estimation accuracy. Hence, in our view, joint channel estimation is not supported for PUSCH with different TBs.

Proposal 1

- Joint channel estimation is not supported for PUSCH transmission with different TBs.

Further, at the RAN1#104b-e meeting, it was agreed that a time domain window for joint channel estimation is specified during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements [1]. In our view, the time domain window may be explicitly configured by higher layers via RRC signalling. In case when inter-slot frequency hopping with inter-slot bundling is applied, the time domain window for joint channel estimation can be determined by the bundle size.

Given the fact that a number of repetitions are employed for PUSCH transmission to improve coverage, it is expected the granularity of joint channel estimation can be considered as repetition or slot level. In this case, the units of the time domain window can be defined according to the number of repetitions or slots for PUSCH repetitions.

In addition, for certain scenarios where channel conditions for a UE are improved, the UE may not need coverage enhancement for PUSCH. In this case, gNB may not need to perform joint channel estimation for PUSCH, which implies that the conditions for UE to maintain power consistency and phase continuity can be relaxed accordingly. In this regard, it may be more desirable to allow gNB to enable or disable the time domain window for PUSCH so as to adapt the channel conditions. In our view, enabling or disabling the time domain window may be configured by higher layers via RRC signaling, which is similar to the agreement as made for joint channel estimation for PUCCH, i.e., support enabling PUCCH repetitions with DMRS bundling via RRC configuration.

In order to maintain power consistency during the time domain window, UE shall transmit the multiple PUSCHs using same Tx power in order to enable joint channel estimation at the receiver. Similarly, for phase continuity, same precoder is needed for multiple PUSCH transmissions in order to enable joint channel estimation. Hence, in our view, same Tx power and precoder are required within the time domain window for joint channel estimation over multiple PUSCHs.

Proposal 2

- The time domain window for joint channel estimation is defined based on the number of repetitions or slots.

  - The time domain window can be configured by higher layers.

  - When inter-slot frequency hopping with inter-slot bundling is applied, the time domain window is determined by the bundle size.

  - Enabling or disabling the time domain window can be configured by higher layers.

- Within the time domain window, UE needs to maintain same Tx power, precoder and frequency resource for joint channel estimation over multiple PUSCHs.

# Discussion on potential DMRS enhancement

At the RAN1#104e meeting, it was agreed to further study the optimization of DMRS location in time domain with joint channel estimation, which includes DMRS equally spaced among PUSCH transmissions, DMRS located in special slots or orphan symbol used for DMRS [6]. Further, it was agreed at the RAN1#104e meeting that a new DMRS pattern equally spaced among PUSCH transmissions is not considered for joint channel estimation in Rel-17 [1]. In this section, we present simulation results for additional DMRS symbol located in the special slots.

Figure 1 illustrates an example of additional DMRS located in the special slot. In the figure, for TDD system, additional DMRS symbols may be inserted in the uplink symbols of special slots before the actual transmission of PUSCH, which can help in improving the channel and frequency offset estimation, and overall decoding performance for PUSCH repetition.

Figure 1. Additional DMRS located in the special slot

Figure 2 illustrates link level simulation results by using an additional DMRS symbol in the special slot. In the simulation, it was assumed that TDD frame structure with DDDSU pattern, TBS = 288, 4 PRBs, 2 and 4 repetitions, 2 DMRS symbols in each UL slot and inter-slot frequency hopping with bundle size of 2 slots. In addition, joint channel estimation is employed with bundle size of 2 slots and the DMRS symbol in special slot. Further, CFO is 0.1ppm of 4GHz carrier frequency, and ML based CFO estimation algorithm is employed at receiver.

From the figure, it can be observed that for PUSCH with 2 and 4 repetitions and joint channel estimation with bundle size of 2 slots, ~0.5dB performance gain can be achieved when an additional DMRS symbol is inserted in the special slot for PUSCH repetition. This is expected as this extra DMRS symbol can be utilized together with other DMRS symbols for improvement of channel and frequency offset estimation accuracy.

Figure 2. Simulation results for additional DMRS symbol in the special slot

Observation 1

- For PUSCH with 2 and 4 repetitions and joint channel estimation with bundle size of 2 slots, ~0.5dB performance gain can be achieved when an additional DMRS symbol is inserted in the special slot for PUSCH repetition.

Proposal 3

- FFS: additional DMRS symbols located in the special slot for PUSCH enhancement.

# Enhancement on inter-slot frequency hopping

To facilitate the joint channel estimation, frequency resource for uplink transmission during the repetitions may remain the same for certain number of slots in order to allow inter-slot interpolation for channel estimation improvement. This inter-slot frequency hopping with inter-slot bundling can exploit the benefit of frequency diversity while maximizing the gain from joint channel estimation.

Figure 3 illustrates one example of enhanced inter-slot frequency hopping pattern for coverage enhancement. In the example, PUSCH transmission occupies the same frequency resource for two slots before it switches to other frequency resources. In this case, the bundle size for inter-slot frequency hopping is 2 slots.

Figure 3. Inter-slot frequency hopping with inter-slot bundling for PUSCH

Figure 4 illustrates link level simulation results for PUSCH with enhanced frequency hopping (FH) patterns. In the simulations, it is assumed TBS = 136, 4 PRBs, 4 repetitions and 2 DMRS symbols are allocated in each slot. For enhanced frequency hopping pattern, inter-slot frequency hopping with bundle size of 2 slots is assumed. In addition, joint channel estimation with bundle size of 2 slots is employed in the simulations. Further, CFO is uniformly distributed within [-0.1, 0.1] ppm of 700MHz carrier frequency, and ML based CFO estimation algorithm is employed at receiver. From the figure, it can be observed that when employing joint channel estimation, ~1.0dB performance gain can be achieved by inter-slot frequency hopping with inter-slot bundling, compared to the case without frequency hopping.

Figure 4. Simulation results for inter-slot frequency hopping with inter-slot bundling

Observation 2

- For PUSCH with 4 repetitions, when employing joint channel estimation with bundle size of 2 slots, ~1.0dB performance gain can be achieved by inter-slot frequency hopping with inter-slot bundling of 4 slots, compared to the case without frequency hopping.

At the RAN1#104b-e meeting, it was agreed to down-select the following two options for inter-slot frequency hopping with inter-slot bundling [1]:

- Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

- Option 2: The bundle size (time domain hopping interval) can be different from the time domain

In our view, Option 2 can provide more flexibility compared to Option 1 on the operation of joint channel estimation and inter-slot frequency hopping with inter-slot bundling. In particular, the time domain window size for joint channel estimation can be larger than or equal to the bundle size for inter-slot frequency hopping. For instance, UE may maintain the power consistency and phase continuity over 4 consecutive slots. When 4 slots are allocated for PUSCH repetitions, the bundle size can be 2 slots, where gNB may perform joint channel estimation in two bundled slots.

For inter-slot frequency hopping with inter-slot bundling, the bundle size may be configured by higher layers, or implicitly determined based on the number of repetitions for PUSCH. As mentioned above, in order to enable joint channel estimation, UE needs to maintain same Tx power, precoder and frequency resource within the bundled slots. In this regard, when inter-slot frequency hopping with inter-slot bundling is applied, the time domain window can be determined by the bundle size.

Proposal 4

- For inter-slot frequency hopping with inter-slot bundling, the bundle size can be different from the time domain window size.

  - The bundle size may be configured by higher layers or determined based on the number of repetitions for PUSCH.

# Conclusions

In this contribution, we discussed joint channel estimation for PUSCH coverage enhancement, with primary focus on potential optimization of DMRS patterns and inter-slot frequency hopping with inter-slot bundling. Further, we summarize the observations and proposals as follows:

Observation 1

- For PUSCH with 2 and 4 repetitions and joint channel estimation with bundle size of 2 slots, ~0.5dB performance gain can be achieved when an additional DMRS symbol is inserted in the special slot for PUSCH repetition.

Observation 2

- For PUSCH with 4 repetitions, when employing joint channel estimation with bundle size of 2 slots, ~1.0dB performance gain can be achieved by inter-slot frequency hopping with inter-slot bundling of 4 slots, compared to the case without frequency hopping.

Proposal 1

- Joint channel estimation is not supported for PUSCH transmission with different TBs.

Proposal 2

- The time domain window for joint channel estimation is defined based on the number of repetitions or slots.

  - The time domain window can be configured by higher layers.

  - When inter-slot frequency hopping with inter-slot bundling is applied, the time domain window is determined by the bundle size.

  - Enabling or disabling the time domain window can be configured by higher layers.

- Within the time domain window, UE needs to maintain same Tx power, precoder and frequency resource for joint channel estimation over multiple PUSCHs.

Proposal 3

- FFS: additional DMRS symbols located in the special slot for PUSCH enhancement.

Proposal 4

- For inter-slot frequency hopping with inter-slot bundling, the bundle size can be different from the time domain window size.

  - The bundle size may be configured by higher layers or determined based on the number of repetitions for PUSCH.

# References

- Chairman’s notes, RAN1 #104b e-Meeting, April 2021

- R1-2104919, “Enhancements on PUSCH repetition type A”, Intel Corporation, e-Meeting, May 2021

- R1-2104920, “Discussion on TB processing over multi-slot PUSCH”, Intel Corporation, e-Meeting, May 2021

- R1-2104922, “Discussion on PUCCH enhancements”, Intel Corporation, e-Meeting, May 2021

- R1-2104923, “On Msg3 PUSCH repetition”, Intel Corporation, e-Meeting, May 2021

- Chairman’s notes, RAN1 #104 e-Meeting, January 2021