3GPP TSG RAN WG1 #105-e			R1-2105129

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.12.2

Source:	Apple Inc.

Title:	Discussion on MBS reliability improvement for RRC_CONNECTED UEs

Document for:	Discussion/Decision

# Introduction

In RAN#86 meeting, new work item on NR Multicast and Broadcast Services [1] was agreed. Some of objectives of this study item are showing below,

- Specify RAN basic functions for broadcast/multicast for UEs in RRC_CONNECTED state [RAN1, RAN2, RAN3]:

  - Specify a group scheduling mechanism to allow UEs to receive Broadcast/Multicast service [RAN1, RAN2]

    - This objective includes specifying necessary enhancements that are required to enable simultaneous operation with unicast reception.

  - Specify support for dynamic change of Broadcast/Multicast service delivery between multicast (PTM) and unicast (PTP) with service continuity for a given UE [RAN2, RAN3]

  - Specify support for basic mobility with service continuity [RAN2, RAN3]

  - Assuming that the necessary coordination function (like functions hosted by MCE, if any) resides in the gNB-CU, specify required changes on the RAN architecture and interfaces, considering the results of the SA2 SI on Broadcast/Multicast (SP-190625) [RAN3]

  - Specify required changes to improve reliability of Broadcast/Multicast service, e.g. by UL feedback. The level of reliability should be based on the requirements of the application/service provided.[RAN1, RAN2]

  - Study the support for dynamic control of the Broadcast/Multicast transmission area within one gNB-DU and specify what is needed to enable it, if anything [RAN2, RAN3]

Some MBS reliability related agreements were made in RAN1#104bis-e meeting [2]. In this contribution, we discuss the MBS reliability enhancement aspects, including the HARQ-ACK feedback mechanism, repetition enhancement, etc.

# MBS reliability improvements for RRC_CONNECTED UEs

HARQ-ACK feedback mode

According to last RAN1 meeting agreement, NACK-only based HARQ-ACK feedback is supported. To serve the purpose of resource saving, the PUCCH resource need to separately configured in cell specific manner, the resource is shared among the users in the group. In addition, different from ACK/NACK based feedback, PUCCH resource for NACK-only based feedback need to configure in a common frequency region, which is similar as the CFR defined in the DL.

If UE is configured with NACK-only based feedback mode, UE would expect the re-transmission is not via PTP.

Agreement: [2] Support NACK-only based HARQ-ACK feedback for RRC_CONNECTED UEs receiving multicast.

Proposal 1: For NACK-only based feedback, PUCCH resources are shared among users in the group and are configured in a common frequency region.

HARQ-ACK feedback resource indication

One of the left issues related the PUCCH resource is how to indicate the PUCCH resource.

- Alt.1: the last DCI for unicast

The benefit of this Alternative is the unicast DCI could directly indicate UE specific PUCCH resource for HARQ-ACK feedback for users in the group. But it seems not a complete solution, if there is no unicast PDSCH is scheduled in the feedback window, then no PUCCH resource could be indicated for feedback.

- Alt.2: the last DCI across unicast and multicast;

For this Alternative, there were concerns that one PRI indicating resource to all users could cause PUCCH resource collision or reserve more PUCCH resources. Generally, PUCCH resources for MBS could not be shared among users, due to same k1 value is applied. So if the last DCI is multicast,  it’s up to gNB to configure orthogonal PUCCH resources among users within the same group. If the PUCCH overhead is the concern, HARQ-NACK based feedback mode can be configured.

Agreement:[2] For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, determining the PUCCH resources for transmission is based on the PRI indicated in the “last DCI”, where the “last DCI” refers to, down-select the following alternatives: - Alt.1: the last DCI for unicast; - Alt.2: the last DCI across unicast and multicast;

Proposal 2: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

HARQ codebook

For Type 2 HARQ-ACK codebook, one open issue is whether to support concatenating more than one Type-2 codebook for multicast. The issue is related to discussion in another section whether multiple CFRs are supported in design. To support multiple CFRs, the DAI for different CFR would be counted separately, the sub-codebook can be constructed for each CFR. In addition, a CORESET would be configured for a CFR. If MBS BWP, i.e., Option 2A, is not preferred, then the number of CORESET is still restricted to three for UE dedicated BWP, it’s hard to support multiple CFRs.

Agreement:[3] For ACK/NACK based feedback if supported for multicast, for Type-2 HARQ-ACK feedback construction for PTM scheme 1, - DAI for unicast and DAI for multicast are separately counted. - Concatenation of Type-2 HARQ-ACK codebook for unicast and multicast is supported. - FFS details on concatenating the codebooks. - FFS whether to support concatenating more than one Type-2 HARQ-ACK codebook for multicast. Agreement: [3] - FFS whether to support more than one common frequency resources per UE / per dedicated unicast BWP subjected to UE capabilities

Proposal 3: Whether to support more than one Type-2 codebook for MBS  is waiting for the outcome of the CFR design.

If FDM between multiple TDMed unicast PDSCHs and multiple TDMed group-common PDSCHs in a slot is supported, then HARQ-ACK codebooks can be constructed separately for multicast PDSCH and unicast PDSCH, sub-codebooks for unicast and multicast are concatenated. For Type-1 HARQ-ACK codebook construction, the following four options were discussed in last RAN1 meeting.

•     Opt 1: HARQ-ACK bits for all the PDSCH occasions over all the slots for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for multicast, for a given serving cell.

èin the order of PDSCH occasions first, and then C-RNTI before G-RNTI, and then serving cell index, and then CORESETPoolIndex (if configured for mTRP)

  - Opt 1a: HARQ-ACK bits over all the slots for unicast, precede, HARQ-ACK bits for over all the slots for multicast for a given serving cell with multicast reception.

•     Opt 2: HARQ-ACK bits for all PDSCH occasions for unicast, precede, HARQ-ACK bits for all PDSCH occasions for multicast, within the same slot.

è in the order of C-RNTI before G-RNTI first, and then PDSCH occasions, and then serving cell index, and then CORESETPoolIndex (if configured for mTRP)

•     Opt 3: HARQ-ACK bits for unicast precedes HARQ-ACK bits for multicast within one SLIV group where PDSCH occasions are overlapping.

è in the order of C-RNTI before G-RNTI first, and then SLIV group of overlapped PDSCH occasions, and then PDSCH occasions, and then serving cell index, and then CORESETPoolIndex (if configured for mTRP)

•     Opt 4: HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for unicast, precede, HARQ-ACK bits for all the PDSCH occasions over all the slots for all serving cells for multicast. (This is similar to the joint Type-1 codebook for mTRP).

è in the order of PDSCH occasions first, and then serving cell index, and then CORESETPoolIndex (if configured for mTRP), and then C-RNTI before G-RNTI.

For the HARQ-ACK bit ordering, the Option 1 HARQ-ACK bit ordering has a variant “for a given serving cell with multicast reception”, it actually changes the CC index, and gives the CC with MBS high priority in HARQ-ACK ordering. This kind of reordering will have large standard impacts and is not necessary. The standard impact of Option1 is introducing the MSB HARQ-ACK order pseudo-code after unicast HARQ-ACK ordering in the same CC. Option 4 is similar as current specification, just replace the TRP1, TRP2 with unicast PSCH and MBS respectively, which has less standard impacts.  The block in Figure 1 means the HARQ-ACKs for PDSCH occasion over all slots.

| Option 1 | Option 1a | Option 4 |
| --- | --- | --- |

Figure 1: HARQ-ACK bits ordering

Proposal 4: For FDM between unicast PDSCH and a CFR in a slot and type 1 HARQ-ACK feedback, HARQ-ACK bits over all the slots for all serving cells for unicast, precede, HARQ-ACK bits over all the slots for all serving cells for multicast.

HARQ feedback enabling/disabling

The following agreements were reached for enabling/disabling HARQ-ACK feedback. There are mainly two options on the table. For option3, it’s combination of RRC signaling and DCI indication, the benefit is the HARQ-ACK feedback can be enabled or disabled dynamically. But this flexibility is really limited for PTM scheme 1, the group common DCI will apply to multiple UEs, it’s hard to predict that all the UEs in the group will receive the group-common PDSCH correctly at the same time. For Option 2, it’s UE specific configuration, gNB could get the UE channel information from CSI report to determine whether HARQ-ACK feedback is enabled or disabled.

For PTP transmission, the HARQ-ACK feedback is always enabled which is the same as the unicast PDSCH reception.

Agreement:[3] For enabling/disabling HARQ-ACK feedback for RRC_CONNECTED UE receiving multicast, - Option 3: RRC signalling configures the enabling/ disabling function of DCI indicating the enabling /disabling HARQ-ACK feedback. - If RRC signalling configures the function, DCI indicates (explicitly or implicitly) whether HARQ-ACK feedback is enabled/disabled - FFS details on RRC signalling and DCI indicating. - If RRC signalling does not configure the function, DCI does not indicate enabling/disabling the HARQ-ACK feedback. - FFS whether enabling or disabling the feedback is the default mode. - Option 2: RRC indicates enabling/disabling. - FFS: whether down-selection between option 3 and option 2 is needed or support the both options. - FFS: enabling/disabling by MAC-CE.

Proposal 5: HARQ-ACK feedback is only enabled or disabled by RRC signaling for PTM scheme 1.

# Summary

In this contribution, we discuss MBS HARQ feedback mechanism and potential reliability improvement techniques, and have the following observation and proposals:

Proposal 1: For NACK-only based feedback, PUCCH resources are shared among users in the group and configured in a common frequency region.

Proposal 2: For multiplexing the ACK/NACK-based HARQ-ACK feedback for multicast and unicast, the PRI in last DCI across the unicast and multicast is used for PUCCH resource indication.

Proposal 3: Whether to support more than one Type-2 codebook for MBS  is waiting for the outcome of the CFR design.

Proposal 4: For FDM between unicast PDSCH and a CFR in a slot and type 1 HARQ-ACK feedback, HARQ-ACK bits over all the slots for all serving cells for unicast, precede, HARQ-ACK bits over all the slots for all serving cells for multicast.

Proposal 5: HARQ-ACK feedback is only enabled or disabled by RRC signaling for PTM scheme 1.

# References

- RP-193248, “New Work Item on NR Multicast and Broadcast Services”, Huawei, HiSilicon

- R1-2104151, “Report of RAN1#104bis-e meeting”, ETSI MCC

- R1-2102281, “Report of RAN1#104-e meeting”, ETSI MCC