3GPP TSG-RAN WG1 #105-e		R1-2105133

e-Meeting, May 10th – 27th, 2021

Source: 	Apple Inc.

Title:                     On efficient SCell Activation/Deactivation

Agenda item:	8.13.3

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 104-e meeting, the following was agreed for efficient activation/deactivation mechanism for one SCG and SCells [1]:

Working Assumption For efficient SCell activation with assistance of temporary RS, a SSB of the to-be-activated SCell can be indicated as a QCL source for the temporary RS in case of known SCell - FFS: QCL type - FFS: the case of unknown SCell - FFS: other QCL source, e.g., the SSB/P-TRS of another active cell R1-2101933 Summary#2 of efficient SCell activation/de-activation mechanism of NR CA Moderator (Huawei) Agreement For efficient activation of SCells, down select at least one option from below: - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS - Option 1b: A single DCI to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including potential impact on SCell activation related procedures and, e.g. timeline design for SCell activation and for receiving temporary RS - FFS: The same DCI for SCell deactivation - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding temporary RS(s) with enhancement of timeline - Details FFS including timeline design for receiving a DCI trigger of temporary RS, and for receiving temporary RS - Note: Companies are encouraged to provide complete solutions for fast SCell activation. - Note: the previous agreement on the definitions of Alt 1 and Alt 2 is still effective

In RAN1 104-bis e-meeting, good progress was made on the signaling designs for efficient SCell activation operation and the following was agreed:

Agreement For efficient activation of SCells - Option 1a: MAC CE(s) contained in a single PDSCH to trigger both SCell activation and corresponding temporary RS(s) - Details FFS including timeline design for receiving temporary RS Note: Separate from the support of Option 1a, it is up to RAN4 whether or not to consider an activation time enhancement for Option 2 without requiring further RAN1 work - Option 2: A Rel-15/16 SCell activation MAC-CE to trigger SCell activation and a Rel-15/16 DCI to trigger corresponding Rel-15/16 A-TRS(s) Send an LS to RAN4. The LS is endorsed in R1-2104110

In this contribution, we present discuss remaining open issues regarding the timeline, QCL information as well as the BWP location.

# 2. Discussion

For both LTE and Rel-15/Rel-16 NR, MAC CE signaling is used to activate/deactivate the SCell(s) to provide the increased capability to handle the instantaneous traffic volume variation. The required delay was defined in [3] as sum of the following three components as depicted in FIG.1 below:

- THARQ: The timing between DL data transmission and acknowledgement as specified in TS 38.213

- TActivation_time: The SCell activation delay in millisecond. Table 1 summarizes the TActivation_time values required for different cases in FR1 depending on the SCell status and measurement cycle configuration.

Table 1: TActivation_time values for FR1

| Case | Description |  | TActivation_time |
| --- | --- | --- | --- |
| 1 | If the SCell is known | The SCell measurement cycle <=160ms | TFirstSSB+5ms |
| 2 |  | The SCell measurement cycle >160ms | TFirstSSB_MAX+Trs+5ms |
| 3 | If the SCell is unknown and Ês/Iot ≥ -2dB |  | TFirstSSB_MAX+TSMTC_MAX+2Trs+5ms |

- TCSI-reporting: The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331

Figure 1: MAC-CE based SCell Activation Delay in Rel-15/16 NR

The key distinction between FR1 and FR2 system is that the beamforming operation was used in both gNB and UE. Correspondingly, delay caused by necessary beam management operations was added for SCell activation delay in FR2. According to the section 8.3.2 in [2], for both FR1 and FR2, the longest delay occurs for the case of unknown at the UE side, e.g. the target SCell is in a band pair that is operated with independent QCL assumptions or beam management compared to PCell/PSCell or other active SCells on the same FR2 band .

In RAN1 102 e-meeting, TRS was agreed as temporary RS for SCell activation, which at least provides the functionalities of ACG setting and T/F tracking procedure to reduce the latency. It was further agreed in RAN1 103 e-meeting as working assumption that the existing Rel-15/16 TRS structure is reused for temporary RS for the efficient SCell activation purpose. One LS was sent to RAN4 to request the number of TRS burst/symbols to fulfil the targeted functionalities i.e., ACG settling and T/F tracking.

One FFS aspect is w.r.t the timeline of A-TRS transmission. Assuming HARQ-ACK feedback on PUCCH is supported for MAC-CE triggering command, it is beneficial to define the A-TRS offset relative to the last symbol of PUCCH so as to reduce the RRC signaling overhead. Considering the SCell activation time instances maybe varied for different UEs, it is desirable to allow indicating the A-TRS offset in triggering command such that a single A-TRS transmission can be utilized by multiple UEs even with different SCell activation instances over time, as illustrated in FIG.2.

Figure 2: The timeline of A-TRS transmission

Proposal 1: The timing of A-TRS transmission is defined relative to the PUCCH transmission that carries the HARQ-ACK for triggering command.

Proposal 2: The offset value of TRS transmission is indicated in triggering command.

On the TCSI-reporting, the value depends on the periodicity configuration of P-CSI-RS or SP-CSI-RS used for CSI reporting. It was widely observed that 20ms/40ms periodicity is typically configured in NR system, which makes TCSI-reporting dominant for SCell activation procedure. As one example, if we count the additional delay from periodic report, there could still be up to 25ms delay even with A-TRS to expedite TO/FO/AGC refinement.

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

Proposal 3: The new MAC-CE command that triggers the SCell activation and A-TRS transmission is used to additionally trigger A-CSI-RS transmission to reduce the latency.

Another FFS aspect related to A-TRS is which BWP should be considered for measurement. Two options were identified in RAN1 104 bis e-meeting:

- Option 1: The BWP configured by “firstActiveDownlinkBWP-Id”

- Option 2: gNB indicates the BWP along with the indication of triggering the temporary RS

For a SCell, RRC signaling for SCell configuration/reconfiguration indicates the first active DL BWP when the SCell is activated. In our view, it provides the full flexibility to select proper BWP as first active DL for efficiently TRS transmission and others such as power consumption. The motivation and benefit of introducing additional RRC signaling to configure another BWP for TRS transmission is unclear. More importantly, transmission TRS on the first active DL BWP avoids BWP switching after TRS measurement and reduces the SCell activation latency. Considering all these discussions, our preference is Opt.1.

Proposal 4: TRS is transmitted on the BWP that is configured by “firstActiveDownlinkBWP-Id”.

# 3. Conclusion

In this contribution, we have presented our views with the following proposal and observations:

Observation 1: The CSI measurement and reporting are one of dominant factors for the latency of SCell activation procedure.

Proposal 1: The timing of A-TRS transmission is defined relative to the PUCCH transmission that carries the HARQ-ACK for triggering command.

Proposal 2: The offset value of TRS transmission is indicated in triggering command.

Proposal 3: The new MAC-CE command that triggers the SCell activation and A-TRS transmission is used to additionally trigger A-CSI-RS transmission to reduce the latency.

Proposal 4: TRS is transmitted on the BWP that is configured by “firstActiveDownlinkBWP-Id”.

# References

- RAN1 103 e-meeting Chairman notes

- RAN1 104 bis e-meeting Chairman notes

- TS 38.133 		Requirements for support of radio resource management