3GPP TSG RAN WG1 #105-e		R1-2105214

e-Meeting, May 10th – 27th, 2021

Agenda Item:	8.4.2

Source:	Lenovo, Motorola Mobility

Title:	Discussion on NTN uplink time synchronization

Document for:	Discussion

- Introduction

In RAN1#104bis-e, the agreements related to uplink synchronization are as follows:

In this contribution, we discuss the issues related to time synchronization.

- Discussion

The formula to determine TA in RRC connected state is agreed in last meeting, and the granularity of the common timing offset NTA,common is still FFS.

There are two options regarding the granularity:

- Option 1: The granularity is same as legacy TA, i.e. 16*64/2u*Tc

- Option 2: The granularity can be separately configured.

The pros of Option 1 is that accuracy can be kept same as in legacy release, and the pros of Option 2 is that signaling overhead reduction can be achieved with separate configuration. The indicated NTA,common is corresponding to the RTT between the satellite and the reference point. The reference point is where the DL and UL frame are aligned. The selection of TA command unit in legacy release is to address UE moving within the cell coverage area. The NTA,commonindication is due to moving of satellite. From signaling perspective, we prefer Option 2 to separate configure the unit of NTA,common. However, it seems most companies prefer Option 1 from accuracy point of view. For sake of progress, we are also fine to support Option 1.

Proposal 1: Support the granularity of NTA,common to be same as granularity of TA command.

Combination of open loop and close loop TA control is supported per previous meeting agreements, and the details of NTA update are still FFS. From our perspective, the issue related to contradiction between close loop and open loop TA should be resolved. For UE specific TA estimation, the UE may estimate NTA,UE-specific-1 based on satellite’s position  and UE’s positions in a first time instance, and as the satellite is moving, the UE may estimate an updated NTA,UE-specific-2 based on satellite’s position  and UE’s positions in a later second time instance. Between the first and the second time instance, uplink signal may be transmitted from UE to gNB to determine TAC in MAC CE. In this case, the impact of satellite moving are counted twice by both the UE specific TA calculation and the TAC determination, so the total TA value by adding these two values is not correct.

In legacy releases, there is close loop TA control by MAC CE, and there is also open loop TA control by SSB or DL signal reception. Actually there is also contradiction between close-loop and open-loop TA control, and there is a sentence in TS 38.213 to resolve the contradiction: “If the received downlink timing changes and is not compensated or only partly compensated by the uplink timing adjustment without timing advance command as described in TS 38.133, the UE changes N_TA accordingly”. Whether there is contradiction between open loop and close loop TA control is determined by the time domain order of the nearest SSB reception when applying the MAC CE and uplink signal reception associated with MAC CE. If the SSB reception is after the uplink signal reception, then the received downlink timing change is not fully compensated, otherwise, it is compensated. As the cell coverage area in TN is relatively small, it is not necessary to differentiate the time instance between transmission and reception, so at gNB side, it can be determined by the time domain order of SSB transmission and uplink signal reception, and at UE side, it can be determined by time domain order of SSB reception and uplink signal transmission. However, there is large propagation delay in NTN, so it is necessary to differentiate the time instance for DL/UL transmission and reception. Additionally, open loop TA in NTN is performed autonomously by UE and in NR SSB transmission/reception is known by both gNB and UE, so there should be some mechanisms to maintain common understanding between gNB and UE.

There can be several alternatives to resolve the contradiction issue:

- Alt1: Predefine a rule to determine the time instance to determine TAC in MAC CE;

  - e.g. Suppose the MAC CE is received at T_mac, and a predefined duration is T_duration, then the time instance can be determined to be the latest UL reception time instance at satellite side before T_mac – T_duration. T_duration is used to address the propagation delay between gNB and UE, and it can be configured or same as K-offset for DL reception and UL transmission timing relationship.

  - At UE side, the UL reception time instance can be determined based on the UE’s position and satellite’s position.

- Alt2: Explicit indicate a time instance to determine TAC in MAC CE;

  - The explicit indication can be a time domain info. And the time domain info can be carried together with the MAC CE;

  - The explicit indication can be also a DL signaling. The DL signaling indicates UE to determine a time instance based on reception of the DL signaling.

With either Alt 1 or Alt 2, the time instance to determine TAC in MAC CE can be determined, so that UE can calculate the NTA,UE-specific for the determined time instance, and apply TAC to the determined NTA,UE-specific, so there is no contradiction. Meanwhile, common understanding between gNB and UE is also achieved.

Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE.

Regarding whether TA margin indication is necessary, as common TA indication is already agreed, and the common TA can absorb the maximum TA uncertainty, so TA margin indication is not need.

Proposal 3: TA margin indication is not supported.

There are also discussions on whether a drift rate of common timing offset indication is necessary. The common timing offset is corresponding to RTT between satellite and the reference point. As the satellite moves, the common timing offset is also changed. If there is no drift rate indication, the change of common timing offset can only be maintained by frequent common timing offset indications, which imposes large signaling overhead. Therefore, we propose to support indication of the drift rate of common timing offset.

In addition, the drift rate as a linear function of time can have significant variations, hence demanding indication of drift rate variations. If the drift rate variation is described by a constant, the result is TA indication as a quadratic function of time, which provides further degree of freedom and, as a result, a further reduction of the signaling overhead.

Proposal 4: Support indication of common timing offset drift rate and indication of drift rate variation.

Furthermore, for NTN based on regenerative payload, the TA change rate is proportional to the satellite velocity’s component along the UE-satellite line, which is also proportional to the Doppler shift observed by the UE. Therefore, the UE can use Doppler measurements to estimate a TA change rate and update the TA value with lower-overhead signaling.

Finally, there have been discussions on configuring a validity timer for closed-loop TA update. However, a semi-static configuration of timeAlignmentTimer would not take into account the significant variation of the duration that a timer value remains valid as the satellite moves with respect to the horizon. For example, when the satellite is close to the zenith with respect to the UE’s location, the duration of validity is significantly longer than the case that the satellite is closer to the horizon, in which case the TA timer must expire significantly faster. Therefore, instead of a semi-static configuration, a timer value can be included in the TA indication message, hence allowing the satellite to indicate shorter validity duration when the relative velocity between the satellite and the UE is large.

Proposal 5: Support indication of TA validity timer value in the TA indication signaling.

- Conclusion

In this contribution, we discussed the issues related to TA in NTN network, and our proposals are as following:

Proposal 1: Support the granularity of NTA,common  to be same as granularity of TA command.

Proposal 2: Study mechanisms to resolve the contradiction between open loop and close loop TA control. The mechanism can be to define a time instance to determine TAC in the MAC CE.

Proposal 3: TA margin indication is not supported.

Proposal 4: Support indication of common timing offset drift rate and indication of drift rate variation.

Proposal 5: Support indication of TA validity timer value in the TA indication message.

- References

RP-201256, RANP#88e, “Solutions for NR to support non-terrestrial networks (NTN)”

TR 38.821 v16.0.0

RAN1#104bise chairman notes

R1-2103990, Thales, “FL Summary on enhancements on UL time and frequency synchronization for NR NTN”