3GPP TSG RAN WG1 #105-e Meeting	R1-2105276

e-Meeting, May 19th – 27th, 2021

Agenda item:		8.1.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Enhancements for HST-SFN deployment

Document for:		Discussion and Decision

# Introduction

The Rel-17 work item for enhancements on MIMO for NR includes an objective to extend specification support for enhancements on multi-TRP/panel transmission. In RAN #104bis-e, the following agreements were made on the HST-SFN topic [1]:

Agreement

Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission

- The corresponding MAC CE includes at least the following fields

  - Serving cell ID

  - CORESET ID

  - Two TCI state IDs

- FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE

- FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex

Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17.

R1-2103902	Draft LS on TCI states indication for PDCCH	Intel Corporation

Decision: The draft LS is endorsed. Final version is approved in R1-2104064.

Agreement

Specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both:

- UL RS based Doppler estimation by gNB

  - FFS: Details including UL RS enhancement

- DL RS based Doppler feedback by UE

  - FFS: Details

  - FFS: Whether UE capability needs to be introduced

- Whether to support one or both will be decided later

Agreement

- Support dynamic (DCI-based) switching of scheme 1 (PDSCH) with single-TRP scheme by TCI state field in DCI format 1_1/1_2

  - This feature is UE optional

- FFS all other details including RRC signalling, possible RAN4 impact (if any), etc.

Working Assumption

All QCL source RS resource types as defined in TCI state for Rel-16 multi-TRP are supported for scheme 1

Agreement

Support semi-static (RRC-based) switching of scheme 1 (PDSCH) with Rel-16 scheme 1a

- FFS: Whether dynamic switching is additionally supported

For future meeting:

Companies to consider Proposal #3-8a in FL summary (R1-2104020) for future meetings.

Companies to consider Proposal #3-10 in FL summary (R1-2104020) for future meetings.

Agreement

Scheme 1 for PDSCH is identified by

- New RRC parameter and the number of TCI states indicated by DCI

  - FFS RRC configuration details, e.g., per BWP or per CC

  - FFS whether or not restriction to a single CDM group for DM-RS is also supported

In this contribution, we discuss enhancements to support high speed train SFN scenario and make some observations and proposals.

# Discussion

## HST-SFN transmission

Downlink performance with baseline HST-SFN transmission is degraded from the effects of multiple Doppler shifts experienced by the signals received from the TRPs at the UE. The demodulation performance suffers from the ability of the UE to estimate only a single Doppler shift and perform channel estimation based on U-shaped Doppler spectrum. Doppler shift estimation based on SFN TRS or DMRS in the UE receiver provides a single, composite Doppler shift due to the inability to separate the transmissions from different TRPs. Therefore, UE-based and network-based solutions are considered for specification in Rel-17 for DL HST-SFN performance enhancement.

## UE-based solution for HST-SFN enhancement

### Signalling for indication of scheme 1

It was agreed in RAN1 #104bis-e that scheme 1 for PDSCH is identified by a new RRC parameter and the number of TCI states indicated by DCI. Scheme 1 requires configuration of distributed TRS. As such, if scheme 1 is configured per component carrier, then transmission of TRS in each BWP of the component carrier is necessary to enable the UE to obtain the QCL parameters for the receiver. Likewise, it is also necessary for the UE to track the TRS in all the BWPs. In practice, however, the network may use only a single bandwidth part for scheme 1, which results in unnecessary network overhead and UE complexity. On the other hand, with configuration of scheme 1 per BWP, the network can choose to provide a configuration in only those BWPs in which scheme 1 will be used.

Proposal 1: For RRC configuration of scheme 1, configuration per BWP is supported.

It is necessary to support semi-static switching between Rel-17 scheme 1 and Rel-16 schemes without allowing both types of schemes to be enabled at the same time. This is because Rel-16 schemes and Rel-17 scheme 1 cannot be distinguished based only on DCI parameters. Thus, the new RRC parameter for identifying scheme 1 should preclude simultaneous configuration of Rel-16 schemes using the scheme enabler parameter.

Proposal 2: Configuration of Rel-17 scheme 1 using a new RRC parameter cannot be simultaneously enabled with configuration of Rel-16 FDM-TDM schemes using the scheme enabler parameter.

In RAN1 #104bis-e, it was agreed to support semi-static (RRC-based) switching of scheme 1 (for PDSCH) with Rel-16 scheme 1a. In our view, the scenarios in which Rel-16 scheme 1a and Rel-17 scheme 1 would be supported are different. While Rel-16 scheme 1a may be supported in areas with dense TRP deployment, such as in a city or near a train station, scheme 1 is likely to be supported on a high-speed train where TRPs are deployed with a certain separation. As such, when a user transits through a train station and on to train, support of semi-static switching between Rel-16 1a and Rel-17 scheme is sufficient, and we do not see a use case for dynamic switching.

Proposal 3: Do not support dynamic switching of Rel-17 scheme 1 (PDSCH) with Rel-16 scheme 1a.

### Additional source RS for scheme 1

Based on Rel-15 specification, for the DM-RS of PDSCH, the UE shall expect that a TCI-State indicates one of the following quasi co-location type(s):

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with the same CSI-RS resource, or

-	'typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured with higher layer parameter trs-Info and, when applicable, 'typeD' with a CSI-RS resource in an NZP-CSI-RS-ResourceSet configured with higher layer parameter repetition,or

-	typeA' with a CSI-RS resource in a NZP-CSI-RS-ResourceSet configured without higher layer parameter trs-Info and without higher layer parameter repetition and, when applicable, 'typeD' with the same CSI-RS resource.

The third type listed above allows for indication of CSI-RS as the QCL source for QCL-TypeA in the TCI for DM-RS of PDSCH. With the current agreements, however, only TRS is supported as QCL source for QCL-TypeA in both the TCI states. If this restriction is maintained, then the specification must be changed for Rel-17 SFN-based multi-TRP transmission schemes. While current consideration of TRS as the QCL source is based on the better tracking performance from TRS, the issue is whether such a restriction is necessary. It is noted that CSI-RS can be configured as the source for QCL-TypeD. Since the network can anyway configure only TRS as the QCL source for QCL-TypeA, imposing the restriction may not be necessary.

Proposal 4: Confirm working assumption. All QCL source RS resource types as defined in TCI state of Rel-16 multi-TRP are supported for scheme 1.

### Support of scheme 2

In RAN1 #102-e, it was agreed that the following two DL transmissions schemes will be studied for enhanced estimation of Doppler frequency shifts and demodulation at the UE in HST-SFN deployments.

- Scheme 1:

  - TRS is transmitted in TRP-specific / non-SFN manner

  - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner

- Scheme 2:

  - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner

  - PDSCH from TRPs is transmitted in SFN manner

Figure 2 shows the throughput performance obtained for the following schemes.

- Scheme 1

- Scheme 2

- Full SFN transmission

We consider downlink performance at different locations on the track between the two TRPs. The figure shows the performance obtained assuming (a) an SNR of 8 dB and (b) an SNR of 16 dB. For each scheme, the best MCS is selected for each track location.

(a)

(b)

Figure 1. Downlink performance of scheme 1 and scheme 2 at different track locations between two TRPs for a train speed of 500 kmph with SNR (a) SNR 8 dB and (b) SNR 16 dB.

The antenna element pattern at each TRP is assumed to be pointed towards the midpoint on the track between the two adjacent TRPs. Per the agreed assumptions, a 16-to-1 mapping is used to virtualize the 16 elements in the two adjacent columns. However, the weights are assumed to always be configured to point the beam formed by this array towards the UE location on the track. This corresponds to implementing a beam switching as the UE moves along the track. We found that the alternative approach of using fixed weights to always point a fixed beam towards the midpoint results in an unreasonably large difference in the total antenna gain between the midpoint and the track location closest to the TRP. Besides, at the closest point to the TRP, the line-of-sight ray would not be the dominant ray anymore in angles of departure different from the line-of-sight as a result of the array gain. The net effect leads to strange results close to the TRP. For this reason, the assumption of using a beam pointed towards the UE is adopted. Other simulation assumptions are provided in the Appendix.

It is seen that the scheme based on full SFN transmission experiences poor performance at locations that are more than approximately 75 m away from any TRP. At these locations, both scheme 1 and scheme 2 provide vastly superior performance compared with baseline SFN transmission, with bigger gains observed for SNR of 16 dB compared with SNR of 8 dB. The performance of both the schemes also degrades at locations approaching the midpoint. However, scheme 1 is consistently superior to scheme 2 at all locations and at both SNRs since scheme 2 suffers from higher DMRS overhead. As such there is no benefit from supporting scheme 2 if scheme 1 is also supported.

Observation 1: Scheme 1 consistently yields better performance compared with scheme 2.

Proposal 5: Scheme 2 is not supported in Rel-17.

## Network-based frequency pre-compensation scheme

In RAN1 #104bis-e, it was agreed that specification-based TRP Doppler pre-compensation scheme is supported in Rel-17 for FR1 with one or both of (a) UL RS based Doppler estimation by gNB and (b) DL RS based Doppler feedback by UE.

### QCL types/assumptions to support frequency pre-compensation

The following variants have been agreed for consideration to be used with the frequency pre-compensation scheme.

- Variant A: One of the TCI state can be associated with {average delay, delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant B: One of the TCI state can be associated with {average delay, delay spread} and another TCI state with {Doppler shift, Doppler spread} (i.e., QCL-TypeB)

- Variant C: One of the TCI state can be associated with {delay spread} and another TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

- Variant E: Both TCI states can be associated with {average delay, delay spread, Doppler shift, Doppler spread} (i.e., QCL-TypeA)

We discuss below our understanding of how these variants may be used. Variant A and Variant C are intended to be used similarly. The two TCI states described by the variants are used to indicate QCL of the DMRS ports with the two distributed TRSs. In both variants, one of the TCI states indicates QCL of the DMRS with the TRS with respect to {average delay, delay spread, Doppler shift, Doppler spread}, which corresponds to QCL-TypeA. This TCI state indicates QCL of the DMRS with what can be referred to as the anchor TRS, the Doppler shift of which corresponds to the Doppler shift of the frequency pre-compensated signals. The difference between variants A and C is in the QCL indication provided by the other TCI state – while variant A corresponds to association with both average delay and delay spread, variant C corresponds to association with only delay spread. Variant A is proposed with the understanding that both average delay and average delay spread parameters must be obtained for both TRPs. On the other hand, variant C is based on the understanding that the average delay parameter is useful only for the anchor TRP since the timing correction is performed with respect to only the TRP transmitting the anchor TRS.

In variant B, the first TCI state indicates QCL of the DMRS ports with the SFN (non-pre-compensated) TRS with respect to {average delay, delay spread} while the second TCI state is used to indicate QCL of the DMRS ports with the anchor TRS with respect to {Doppler shift, Doppler spread}, which corresponds to QCL-TypeB. The average delay and delay spread parameters for the composite channel from both TRPs can be obtained from the SFN TRS. In practice, however, transmission of the non-anchor TRS is also necessary to enable the UE to track it such that it can apply the QCL appropriate QCL assumption when the network changes the indicated TCI state corresponding to a change in the anchor TRS as the UE crosses the mid-point.

Among the above variants, variant B requires the transmission of three TRSs and entails additional UE complexity. Our understanding is that the average delay should be based on transmissions from both TRPs, as supported by variant A.

Proposal 6: Support variant A for QCL-type assumptions for network-based frequency pre-compensation.

### Indication of the carrier frequency for UL

The frequency pre-compensation scheme based on implicit indication uses UL RS such as SRS for estimation of the compensation frequencies. The TRS is usually configured to be periodically transmitted, with typical periodicities of 10 ms to 40 ms. While TRS is used for time and frequency tracking by the UE, there are several alternatives for UL frequency offset estimation by the TRP.

In general, UL frequency offset estimation requires a reference signal and is performed using adjacent DM-RS symbols or PT-RS during UL demodulation, i.e., there is no separate measurement for frequency offset estimation. Therefore, there is additional complexity for the TRP to measure UL frequency offset when no PUSCH transmission is scheduled. Also, PUSCH is only sent to a single TRP; as such, without introducing multi-TRP PUSCH transmission, PUSCH-based frequency offset estimation is not applicable for HST-SFN scheme.

Using SRS is an alternative solution, but SRS is generally used for DL/UL CSI. Hence, to apply the frequency pre-compensation scheme, the UL SRS configuration must be optimized for frequency offset estimation. Because SRS has intra-symbol repetitions for comb-type RE mapping, we can use time-domain estimation. However when SNR of UL SRS signal is low, the frequency offset estimation error is distributed over a wide range. Figure 2 shows simple evaluation results for CDF of frequency offset estimation error when applying (a) one symbol SRS transmission with time-domain estimation (b) and two consecutive SRS symbols with frequency-domain estimation. We see that the frequency offset estimation error variability is lower in the case of two SRS symbols (frequency-domain estimation) but still quite large. In a practical deployment, the SRS link budget is low because of lower UE power than TRP (23 dBm vs. 46 dBm) and wider SRS bandwidth than other UL signal such as PRACH, PUCCH, and so on. Assuming low SNR of SRS signal, the estimation error can be upto 20% or 40% of the real frequency offset. Simulation results indicate that some of the performance gain of the frequency pre-compensation scheme at the mid-point track location is eroded due to large frequency offset estimation error.

(a)

(b)

Figure 2. Frequency offset estimation error (AWGN) with (a)	one SRS symbol (time-domain estimation) (b) two SRS symbols (frequency-domain estimation).

As an example of SRS optimization, the time domain span of SRS resource should be multiple symbols similar to TRS time domain pattern. There are two options for SRS transmission: the first option is using existing SRS structure with symbol repetition. The second option is introducing new SRS pattern composed of two SRS symbols with an interval of 2-3 symbols. The first option can be supported without specification impact while higher SRS overhead is expected. The second option requires additional specification effort such as new SRS type. The feasibility of each option should be studied, and regardless of the option to apply, the SRS overhead is very high.

Observation 2: SRS-based frequency offset estimation introduces higher SRS overhead and complexity in gNB.

If SRS spatial relation is configured with a TRS, it can be both for spatial relation and path loss calculation. However, for supporting SRS-based frequency pre-compensation, the SRS transmission should have different spatial relation RS and pathloss RS, which cannot be supported by the existing SRS transmission. If the SRS is configured with a spatial relation to the primary TRP, received SNR of the SRS signal in the other TRP is expected to be very poor. In addition, for FR2, there is no option to support simultaneous two-panel transmission, so SRS should be sent to two TRPs separately.

Observation 3: Single SRS transmission toward one TRP may degrade the frequency offset estimation in the other TRP.

Proposal 7: For SRS based frequency offset estimation, if supported, UE should be configured with two SRS resources to different TRPs, where each SRS resource is associated with different TRP.

Depending on the rate at which the pre-compensation frequency needs to be updated, the SRS could have different periodicities. Nevertheless, if frequency pre-compensation is separately applied for each UE, the SRS overhead can be significant. In addition, the approach would require association of SRS configuration with TRS along with dedicated triggering. Methods based on prediction at the network can also be used to reduce the overhead, where again triggering of SRS can be based on conditions observed either by the UE or by the network indicating poor prediction.

Proposal 8: To reduce the overhead of SRS-based frequency offset estimation, if supported, efficient triggering method for SRS transmission should be supported.

On the other hand, the explicit indication would also entail additional overhead for reporting the Doppler shifts estimated for the two TRPs. The approach would require enhancement of reporting to include Doppler shift estimates for two TRPs. However, the accuracy of frequency offset estimation by TRS at UE side is higher than that of UL SRS based measurement. In addition, this CSI-based explicit report only requires UL transmission to a single TRP, which is much easier than sending multiple SRS to two TRPs.

For explicit indication, a new reporting quantity of frequency offset shall be defined. In addition, the triggering of UE’s measuring frequency of offset by TRS shall be supported. A potential disadvantage is the impact of frequency offset estimation errors, which would be more dominant at lower SNRs, e.g., for the TRP that is relatively far away from the UE. Mitigation of this impact may therefore be necessary.

While both approaches have advantages and disadvantages, we have a slight preference for the explicit approach. Although the agreement is to support one or both of the approaches, the specification impact of supporting both approaches would be high. Therefore, down-selecting one approach is preferable.

Proposal 9: Support only explicit indication of the Doppler shifts through the CSI framework.

For the implicit approach for carrier frequency indication, the carrier frequency of UL RS transmitted by the UE for frequency offset estimation need not have an exactly known relationship with the carrier frequency at the TRPs. Theoretically, the carrier frequency of the UL RS may have an arbitrary offset since the frequency compensation value for one TRP can be zero while the frequency pre-compensation value for the other TRP can be equal to the difference between the Doppler shifts estimated at the two TRPs. In practice, however, the frequency offset of the UL RS received by either of the TRPs must be within the estimation range, which can be handled through implementation. Therefore, there is no need to specify a QCL-like association between the DL TRS and UL signal used for carrier frequency indication.

Proposal 10: QCL-like association of the TRS with UL signal transmitted for carrier frequency indication is supported by implementation without specification impact.

### Signalling for indication of frequency pre-compensation scheme

It has already been agreed that Rel-17 scheme 1 is configured with a new RRC parameter and that semi-static switching between Rel-17 scheme 1 and Rel-16 schemes is supported. For maintaining consistency between scheme 1 and the frequency pre-compensation scheme and due to similarities in UE operations with respect to the two schemes due to both schemes being based on SFN transmission, RRC configuration of the frequency pre-compensation scheme is preferred.

Proposal 11: The frequency pre-compensation is configured using an RRC parameter.

To avoid the potential performance issue with frequency offset estimation error, it may be preferable to use pre-compensation based SFN only for the TRP having good link quality. For example, according to the expected frequency estimation error (DL/UL RSRP/SINR), gNB can disable pre-compensation scheme and fall back to single-TRP operation. This would require support for dynamic switching between single-TRP operation and pre-compensation scheme, which can be based on TCI indication.

Proposal 12: Support dynamic switching between network-based frequency per-compensation scheme and single-TRP operation.

## SFN transmission of PDCCH

### MAC-CE indication of TCI states for CORESET

In RAN1 #104bis-e, we agreed to support new MAC-CE for PDCCH activating two TCI states for SFN-based PDCCH. One remaining issue is whether SFN PDCCH can be supported for the CORESET configured with CORESETPoolIndex. Since the UE is assumed to be able to simultaneously receive signals from up to two TRPs, if a CORESET is configured with CORESETPoolIndex, it is already supporting the maximum number of TRPs for reception, and therefore has no further capability to receive SFN PDCCH.

Observation 4: SFN PDCCH transmission cannot be supported when a CORESET is configured with CORESETPoolIndex.

Proposal 13: SFN PDCCH transmission is not supported for M-DCI based M-TRP transmission.

### Default TCI state for PDSCH and aperiodic CSI-RS

The default TCI state for PDSCH with SFN PDCCH, where the CORESET is indicated with two TCI states, can be studied under two cases.

Case 1: PDSCH is scheduled with Rel-15 single TRP or Rel-16 (TDM) scheme 3/4

In our view, there is no justification for SFN PDCCH scheduling PDSCH with single-TRP PDSCH or Rel-16 TDM schemes. When PDCCH contains fallback DCI, CORESET indicated with a single TCI state for non-SFN PDCCH can be used. Otherwise, there is no point of using SFN PDCCH and to schedule non-SFN PDSCH.

Observation 5: There is no need to consider default TCI state for non-SFN PDSCH scheduled by SFN PDCCH.

Case 2: PDSCH is scheduled with SFN PDSCH

When enableTwoDefaultTCI-States is not configured, UE applies the Rel-15 default QCL assumption, which is to follow the TCI States or QCL assumption applied to the lowest indexed CORESET. Therefore, if the CORESET is indicated with two TCI states for SFN transmission of PDCCH, PDSCH is also transmitted with the SFN scheme.

When enableTwoDefaultTCI-States is configured, to support two TCI states for PDSCH, UE shall be indicated by MAC-CE where at least one code point includes two TCI states. In this case, UE shall apply the lowest codepoint of TCI state in the MAC-CE as the default TCI state for PDSCH. We think that HST-SFN shall be used after activation of TCI state by MAC-CE. We don’t see any need for further specification support other than Rel-16 default QCL assumption.

If the UE is scheduled with a DCI format not having TCI field present, applicability of Rel-17 SFN transmission scheme to PDSCH is according to the applied scheme to PDCCH, using the same principle as the existing specification.

Observation 6: Rel-16 default TCI/QCL assumption related to S-DCI Multi-TRP can be applied for Rel-17 SFN PDSCH.

Proposal 14: For SFN PDCCH transmission with PDSCH transmission,

- if UE is not configured with enableTwoDefaultTCI-States, UE assumes that PDSCH is transmitted with SFN, which is the same as PDCCH, i.e. Rel-15 default TCI states is supported;

- if UE is configured with enableTwoDefaultTCI-States, UE assumes Rel-16 default TCI/QCL is supported for the PDSCH.

Another issue is related to default TCI state for AP CSI-RS when it is scheduled by PDCCH in a CORESET indicated with two TCI states, the scheduling offset for the AP CSI-RS is less than the threshold, and there is no other overlapping DL signal with the indicated TCI state. In this case, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states. Therefore, it is not necessary assume only of the two TCI states as the default beam. However, our preference is to delay this discussion until further clarification of the basic principle from the ongoing parallel discussion on multi-TRP CSI in the other agenda item.

Observation 7: When AP CSI-RS is scheduled by PDCCH in a CORESET indicated with two TCI states, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states.

For aperiodic CSI-RS, we can support similar rules as proposed for PDSCH. Though only a single TCI state is used for aperiodic CSI-RS transmission, QCL assumption can be two TCI states to prevent failure to receive either of the two beams. Since the UE is receiving PDCCH with two beams, the UE is not required to turn-off one of RX beams toward the two beams. One or two beams of PDCCH beams can be used for transmitting aperiodic CSI-RS.

Proposal 15: For SFN PDCCH transmission with AP CSI-RS transmission,

- if UE is not configured with enableTwoDefaultTCI-States, UE assumes that AP CSI-RS is transmitted with one of beams for PDCCH (i.e. Rel-15 default QCL assumption);

- if UE is configured with enableTwoDefaultTCI-States, UE assumes that AP CSI-RS is transmitted with one of beams for two default TCI states (i.e. Rel-16 default QCL assumption)

### BFR support

For BFR, a periodic reference signal is used for BFD-RS. When implicit method is applied for determining BFD-RS, the periodic CSI-RS associated with current CORESET is used as BFD-RS. If scheme 1 is applied for PDCCH, there is an ambiguity for determining BFD-RS for the CORESET.

There are two alternative options. The first option is corresponding to Rel-15 principle, using both CSI-RS as the q0 BFD-RSs and determining beam failure when RSRP of both CSI-RS are below threshold. The second option is applying Rel-17 multi-TRP BFR principle: defining two sets of BFD-RSs for two TRP and deriving BFD-RS per TRP based on further clarification of BFD-RS to TRP association.

Because the multi-TRP BFR is not completed, this issue can be postponed until after completion of multi-TRP BFR.

Observation 8: BFD-RS for SFN PDCCH can be discussed after completion of Rel-17 BFR.

However, in RAN1 #104bis-e, it is concluded that the following proposal for BFR should be considered. Thus, if we assume Rel-15/16 BFR framework, we can have following discussion.

Proposal #3-8a:

- When two TCI states are configured activated for a CORESET, study the following enhancements to BFR procedures

  - Configuration of RS for BFD

    - Implicit configuration

      - Alt 1-1: RS of CORESETs with only single active TCI states are used

      - Alt 1-2: RS of CORESETs with both single and two TCI states are used

      - Alt 1-3: RS of CORESET(s) with only two TCI states are used

    - Explicit configuration

      - Alt 2-1 Support defining CSI-RS resource or SSB pairs

        - FFS other details

      - Alt 2-2 Reuse the existing approach for BFD RS configuration

  - Assumptions for hypothetical BLER calculation for PDCCH

      - Alt 3-1: RS in the two TCI states or CSI-RS / SSB are directly used as the BFD RS

      - Alt 3-2: UE calculates one hypothetical BLER under SFN assumption of BFD RS pairs

  - Configuration of NBI RS

      - Alt 4-1: Reuse the existing Rel-15 NBI configuration based on single CSI-RS resource

      - Alt 4-2: Introduce two new beam identification CSI-RS resource sets or new beam identification CSI-RS resource pairs

  - FFS applicability of the BFR enhancements above to

    - Rel-15 BFR

    - Rel-16 BFR

    - Rel-17 BFR

  - FFS UE behavior on monitoring the PDCCH candidate after BFD

- FFS applicability of some enhancements to RLM procedures

- FFS other aspects

In order to avoid conflicting with the discussion in the other agenda items, we consider BFR only when two TCI states are activated for PDCCH transmission. Thus, Alt-1/2 for BFD-RS configuration should not be considered. In addition, we don’t see requirement of any specification support for the explicit configuration.

For the issue of hypothetical BLER calculation, both alternatives can be used as UE implementation, and no specification is required.

For NBI-RS configuration, existing specification already supports configuration and indication of multiple NBI-RSs, and the network can schedule one or two TCI states for PDCCH transmission, and no specification support is required.

Observation 9: With Rel-15/16 BFR framework, if SFN PDCCH is supported, the existing specification can be applied for BFR by implementation. For per-TRP BFR in Rel-17, further study is required.

### Default spatial relation for PUCCH/SRS/PUSCH

Regarding the default spatial relation when DCI format 0_0 is used and PDCCH is transmitted with scheme 1, a simple option is using the RS associated with the lowest indexed TCI state as the reference RS. Or, UE can be configured or activated a default spatial relation. Also, gNB can handle this by implementation/scheduling. Since this is related to Rel-17 common TCI framework, we prefer postponing the discussion until later.

Observation 10: Default spatial relation/PL-RS is related to Rel-17 beam management framework.

# Conclusion

In this contribution, we discuss the sub-objective of multi-TRP/panel transmission related to enhancement for HST-SFN. The following observations and proposals are made.

For UE-based solution

Proposal 1: For RRC configuration of scheme 1, configuration per BWP is supported.

Proposal 2: Configuration of Rel-17 scheme 1 using a new RRC parameter cannot be simultaneously enabled with configuration of Rel-16 FDM-TDM schemes using the scheme enabler parameter.

Proposal 3: Do not support dynamic switching of Rel-17 scheme 1 (PDSCH) with Rel-16 scheme 1a.

Proposal 4: Confirm working assumption. All QCL source RS resource types as defined in TCI state of Rel-16 multi-TRP are supported for scheme 1.

Observation 1: Scheme 1 consistently yields better performance compared with scheme 2.

Proposal 5: Scheme 2 is not supported in Rel-17.

For NW-based solution

Proposal 6: Support variant A for QCL-type assumptions for network-based frequency pre-compensation.

Observation 2: SRS-based frequency offset estimation introduces higher SRS overhead and complexity in gNB.

Observation 3: Single SRS transmission toward one TRP may degrade the frequency offset estimation in the other TRP.

Proposal 7: For SRS based frequency offset estimation, if supported, UE should be configured with two SRS resources to different TRPs, where each SRS resource is associated with different TRP.

Proposal 8: To reduce the overhead of SRS-based frequency offset estimation, if supported, efficient triggering method for SRS transmission should be supported.

Proposal 9: Support only explicit indication of the Doppler shifts through the CSI framework.

Proposal 10: QCL-like association of the TRS with UL signal transmitted for carrier frequency indication is supported by implementation without specification impact.

Proposal 11: The frequency pre-compensation is configured using an RRC parameter.

Proposal 12: Support dynamic switching between network-based frequency per-compensation scheme and single-TRP operation.

For SFN PDCCH transmission

Observation 4: SFN PDCCH transmission cannot be supported when a CORESET is configured with CORESETPoolIndex.

Proposal 13: SFN PDCCH transmission is not supported for M-DCI based M-TRP transmission.

Observation 5: There is no need to consider default TCI state for non-SFN PDSCH scheduled by SFN PDCCH.

Observation 6: Rel-16 default TCI/QCL assumption related to S-DCI Multi-TRP can be applied for Rel-17 SFN PDSCH.

Proposal 14: For SFN PDCCH transmission with PDSCH transmission,

- if UE is not configured with enableTwoDefaultTCI-States, UE assumes that PDSCH is transmitted with SFN, which is the same as PDCCH, i.e. Rel-15 default TCI states is supported;

- if UE is configured with enableTwoDefaultTCI-States, UE assumes Rel-16 default TCI/QCL is supported for the PDSCH.

Observation 7: When AP CSI-RS is scheduled by PDCCH in a CORESET indicated with two TCI states, both the TCI states indicated for the CORESET can be assumed as the default beam by the UE for reception even if the AP CSI-RS transmission is scheduled on only one of the two TCI states.

Proposal 15: For SFN PDCCH transmission with AP CSI-RS transmission,

- if UE is not configured with enableTwoDefaultTCI-States, UE assumes that AP CSI-RS is transmitted with one of beams for PDCCH (i.e. Rel-15 default QCL assumption);

- if UE is configured with enableTwoDefaultTCI-States, UE assumes that AP CSI-RS is transmitted with one of beams for two default TCI states (i.e. Rel-16 default QCL assumption)

Observation 8: BFD-RS for SFN PDCCH can be discussed after completion of Rel-17 BFR.

Observation 9: With Rel-15/16 BFR framework, if SFN PDCCH is supported, the existing specification can be applied for BFR by implementation. For per-TRP BFR in Rel-17, further study is required.

Observation 10: Default spatial relation/PL-RS is related to Rel-17 beam management framework.

# References

- Draft Report of 3GPP TSG RAN WG1 #104bis-e, v0.1.0.

# Appendix

The simulation assumptions used for performance evaluation are as show in Table 1.

Table 1. Simulation assumptions

| Parameter | Value |
| --- | --- |
| TRP layout | Ds=700m, Dmin=150m |
| Carrier frequency | 3.5 GHz |
| Subcarrier spacing | 30 kHz |
| Propagation channel | CDL-D, DS=100ns |
| UE antenna configuration | 2 ports: [1, 1, 1, 1, 2] |
| UE antenna pattern | Omni-directional |
| TRP antenna configuration | 2 ports: [1, 1, 8, 2, 2] , maximum gain 8 dBi, downtilt angle 5 degrees. |
| TRP antenna pattern | Directional (Table 5) Azimuth: Pointed towards the midpoint on the track between adjacent TRPs Downtilt: 5 degrees |
| UE speed | 500 kmph |
| TRS configuration | 10 ms, 2-slot pattern |
| DMRS configuration | DMRS type 1, Number of symbols: 1 + 1 + 1 |
| PDSCH mapping | Type A, start symbol 2, duration 12, rank 1 |
| MCS | MCS adaptation (MCS 13 to MCS 23) |
| Number scheduled RBs | 50 |