3GPP TSG RAN WG1 #105-e	                                                        R1-2105314

e-Meeting, May 10th – May 27th, 2021

Agenda Item:		8.5.1

Source:				Samsung

Title:					Discussion on potential enhancements of information reporting from UE and gNB for multipath/NLOS mitigation

Document for:	Discussion

# Introduction

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

In the approved new WI for positioning enhancement [1], one possible direction is to study the enhancement of information reporting for multi-path or NLOS mitigation.

This contribution discusses the LOS/NLOS related aspects and potential enhancement.

# Discussion on NLOS/LOS Indication

In TR38.875, simulation results showed that there is a degradation in localization accuracy when NLOS links are present during the localization estimation process. It was shown that for the InF-DH scenario, sub-meter level requirement for horizontal positioning accuracy was not achieved in both FR1 and FR2 bands due to the presence of NLOS paths. When compared with the case for LOS path only, the received signal corresponding to the NLOS path has a delay offset, which is due to additional reflections and scattering, and a different angle of arrival or departure. The identification and elimination of NLOS paths is therefore beneficial for both time-based and angular-based positioning techniques.

If the LOS/NLOS could be reliably differentiated, then one promising method for NLOS mitigation in positioning is to have the UE/TRP report whether the measurements are based on LOS/NLOS conditions. Then, the LMF can consider this information when derive the positioning. Thus, it is useful to have  a LOS/NLOS indicator reported with the measurements.

Proposal 1: Support UE/TRP sending to the LMF an NLOS/LOS indication associated with the measurements for positioning if the LOS/NLOS could be reliably differentiated.

The reported LOS/NLOS indicator can be either a hard decision (binary values) or a soft decision (real values such as the probability of the path being NLOS/LOS). The hard decision indicator relies on the performance of UE/TRP LOS/NLOS detection, and requires less complexity at the LMF. However, the soft decision can provide higher accuracy for positioning performance.  For example, consider the DL-TDOA positioning as shown in Figure 1. If a hard decision indication is reported, it does not take into account the likelihood of the NLOS detection. As shown in Figure 1, some TRP-UE paths are “almost” LOS (i.e., the reflectors are close to the UE or TRP), whereas others are very NLOS (i.e., the reflector are far from the UE/TRP). This shows that not all NLOS conditions are equal, and LMF has the advantage in selecting the reported measurements that are corresponding to better channel conditions. Thus, if a soft decision indication is reported, the reported RSTD measurements can be sorted according to a certain metric at the location server. The location server can choose some of the reported measurements, which are more likely to be LOS paths, or assign different weights to all the measurements according to the associated NLOS conditions for location calculation.

Figure 1: DL-TDOA positioning

Proposal 2: The NLOS/LOS indicator can be either a hard decision (binary values) or a soft decision (real values such as the probability of the path being NLOS/LOS).

# Uncertainty range indication for NLOS mitigation

In order to derive a reliable LOS/NLOS path to facilitate the NLOS mitigation, the uncertainty range of the first arrival LOS path could be indicated to UE/TRP if such information could be derived e.g., at LMF. Such uncertainty range could be represented by a time window formation. For example, the UE can be configured with a timing window per TRP for searching the DL-PRS signal corresponding to first arrival LOS path. The configuration of the search window requires a priori rough knowledge of the UE location at the LMF. From signaling perspective, this method is similar to the configuration of nr-DL-PRS-ExpectedRSTD. The timing window approach can be configured for both and the UE and TRP, and for both timing based and angle based positioning methods. Given the timing window, the UE/TRP can make measurements for positioning based on paths that arrive within the window.

Proposal 3: Support the indication of the uncertainty range of the first arrival LOS path to UE/TRP.

# Conclusion

This contribution discusses the LOS/NLOS related aspects and potential enhancement. Observations and proposals are summarized as follows:

Proposal 1: Support UE/TRP sending to the LMF an NLOS/LOS indication associated with the measurements for positioning if the LOS/NLOS could be reliably differentiated.

Proposal 2: The NLOS/LOS indicator can be either a hard decision (binary values) or a soft decision (real values such as the probability of the path being NLOS/LOS).

Proposal 3: Support the indication of the uncertainty range of the first arrival LOS path to UE/TRP.

# References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.