3GPP TSG-RAN WG1#105-e			R1-2105591

e-Meeting, May 10th – 27th, 2021

Agenda item:	8.1.2.4

Title:	On Enhancements for HST-SFN deployment

Source:	Convida Wireless

Document for:	Discussion

# Introduction

Rel-17 FeMIMO WI will extend the multi-TRP support to PDCCH and to the high-speed train (HST) deployment using single frequency network (SFN) DL transmission [1].

- Extend specification support in the following areas [RAN1] - Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Enhancement to support HST-SFN deployment scenario: - Identify and specify solution(s) on QCL assumption for DMRS, e.g. multiple QCL assumptions for the same DMRS port(s), targeting DL-only transmission - Evaluate and, if the benefit over Rel.16 HST enhancement baseline is demonstrated, specify QCL/QCL-like relation (including applicable type(s) and the associated requirement) between DL and UL signal by reusing the unified TCI framework

In agenda item 8.1.2.1 (RAN1#102-e and RAN1#103-e), it was agreed to support SFN transmission of PDCCH:

Agreement To enable a PDCCH transmission with two TCI states, study pros and cons of the following alternatives: - Alt 1: One CORESET with two active TCI states - Alt 2: One SS set associated with two different CORESETs - Alt 3: Two SS sets associated with corresponding CORESETs - At least the following aspects can be considered: multiplexing schemes (TDM / FDM/ SFN / combined schemes), BD/CCE limits, overbooking, CCE-REG mapping, PDCCH candidate CCEs (i.e. hashing function), CORESET / SS set configurations, and other procedural impacts. Agreement For Alt 1 (one CORESET with two active TCI states), study the following - Alt 1-1: One PDCCH candidate (in a given SS set) is associated with both TCI states of the CORESET. - Alt 1-2: Two sets of PDCCH candidates (in a given SS set) are associated with the two TCI states of the CORESET, respectively - Alt 1-3: Two sets of PDCCH candidates are associated with two corresponding SS sets, where both SS sets are associated with the CORESET and each SS set is associated with only one TCI state of the CORESET - Note 1: A set of PDCCH candidates contain a single or multiple PDCCH candidates, and a PDCCH candidate in a set corresponds to a repetition or chance - Note 2: How one or more PDCCH candidates are counted for monitoring (for BD limit) is FFS - The note is applicable also to other alternatives Agreement For PDCCH reliability enhancements, support SFN scheme + Alt 1-1. - FFS: TCI state activation for CORESET, impact on default beam, BFD resource for BFR

In RAN1#102-e and RAN1#103-e, it was also agreed in agenda item 8.1.2.4 to support SFN transmission of PDCCH:

Agreement For the discussion purpose consider the following categorization of the enhanced DL transmission schemes - Scheme 1: - TRS is transmitted in TRP-specific / non-SFN manner - DM-RS and PDCCH/PDSCH from TRPs are transmitted in SFN manner - Scheme 2: - TRS and DM-RS are transmitted in TRP-specific / non-SFN manner - PDSCH from TRPs is transmitted in SFN manner Agreement Support at least the following configuration for HST scenario in Rel-17 - The same DMRS port(s) can associate with multiple TCI states - FFS other details Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner Agreement At most two TCI states are supported for HST scenario in Rel-17 - FFS: Whether to support more than two TCI states for FR2 - FFS configuration/signalling details of the TCI states Note: DMRS and PDCCH/PDSCH from different TRPs are transmitted in SFN manner

A further agreement was reached in RAN1#104-bis:

Agreement Introduce enhanced MAC CE signaling for PDCCH activating two TCI states for SFN-based PDCCH transmission - The corresponding MAC CE includes at least the following fields - Serving cell ID - CORESET ID - Two TCI state IDs - FFS whether for CA scenario additionally support RRC configured set of the serving cells which can be addressed by a single MAC CE - FFS whether or not enhanced MAC CE signaling is applicable to a CORESET configured with CORESETPoolindex Send LS to RAN2 to inform about agreement on support of enhanced MAC CE for CORESET in Rel-17. LS is endorsed in R1-2104064

# Discussion

## 2.1 Default TCI when CORESET has Two Activated TCI States

In Rel-15/16, there is a number of cases in which the default TCI state for PDSCH reception is based on a CORESET TCI state (in the latest monitored slot). Since a CORESET with two activated TCI states is supported in Rel-17, the default TCI for PDSCH needs to be discussed.

- If one of the two activated TCI states is applied to PDSCH, which one should be clarified.

- If both of the two activated TCI states are applied to PDSCH, this needs to be described.

At least the following cases are relevant:

- Case 1: If the offset between the DCI and the corresponding PDSCH is less than the threshold:

  - The UE is not configured with enableTwoDefaultTCI-States.

  - The UE is configured with enableTwoDefaultTCI-States, but no TCI codepoint indicates two TCI states.

- Case 2: If the TCI field is not present in the DCI and the offset is equal to or larger than the threshold:

  - Case 2.1: The scheduled PDSCH is single-TRP.

  - Case 2.2: The scheduled PDSCH is multi-TRP (single-DCI based scheme).

Proposal 1: Define default TCI for the cases that the offset between the DCI and the corresponding PDSCH is less than the threshold, and the UE is either not configured with enableTwoDefaultTCI-States or the UE is configured with enableTwoDefaultTCI-States, but no TCI codepoint indicates two TCI states.

The mode of operation when the TCI field isn’t present in DCI is to use the same TCI state for the scheduled PDSCH as for the PDCCH that scheduled the PDSCH (after the threshold). As the beam for PDCCH is updated using MAC CE based activation for the CORESET, so is the beam for PDSCH. Since Rel-16 CORESETs are activated with a single TCI state, Case 2.2 above isn’t supported in Rel-16. With the introduced support of two activated TCI states for a CORESET in Rel-17, Case 2.2. should be supported for multi-TRP PDSCH schemes in both Rel-16 and Rel-17.

Proposal 2: Support single-DCI based multi-TRP PDSCH transmission (Rel-16/17 schemes) scheduled by a DCI without TCI field that is carried on a CORESET with two activated TCI states.

## 2.2 Beam Failure Recovery (BFR) when CORESET has Two Activated TCI States

Reference signals for beam failure detection (BFD-RS) may be explicitly configured or implicitly derived from the activated TCI states of the CORESETs in the active BWP. In Rel-15/16, in which a single set of BFD-RS is used in a BWP, 38.213 [2] describes the implicit derivation as follows:

If the UE is not provided by failureDetectionResourcesToAddModList for a BWP of the serving cell, the UE determines the set to include periodic CSI-RS resource configuration indexes with same values as the RS indexes in the RS sets indicated by TCI-State for respective CORESETs that the UE uses for monitoring PDCCH and, if there are two RS indexes in a TCI state, the set includes RS indexes configured with qcl-Type set to 'typeD' for the corresponding TCI states.

In other words, the UE determines the BFD-RS set from the RS sets, where the RS sets are indicated by the RRC IE TCI-State for the CORESETs. Note that the singular form of TCI-State refers to the IE name, while the plural form of RS sets refers to the multiple TCI states. With this reading and interpretation, the current specification does not need to be revised to support implicit BFD-RS determination of a BFD-RS set based on a CORESET with two activated TCI states.

Observation 1: The current specification can support implicit determination of a set of BFD-RS from multiple TCI states, e.g. two activated TCI states for one CORESET.

The Rel-17 per-TRP BFD and BFR, including the support of explicit and implicit BFD-RS set determination, is currently being discussed in agenda item 8.1.2.3. It is preferable to discuss the topic in one agenda item at a time and postpone the discussions for per-TRP BFD-RS determination for Scheme 1 until after completion of Rel-17 multi-TRP BFR.

Proposal 3: Per-TRP BFD for Scheme 1 can be discussed after completion of Rel-17 BFR.

# Conclusion

In this contribution, we considered aspects of multi-TRP PDCCH/PUCCH/PUSCH enhancement. We have the following proposals:

Proposal 1: Define default TCI for the cases that the offset between the DCI and the corresponding PDSCH is less than the threshold, and the UE is either not configured with enableTwoDefaultTCI-States or the UE is configured with enableTwoDefaultTCI-States, but no TCI codepoint indicates two TCI states.

Proposal 2: Support single-DCI based multi-TRP PDSCH transmission (Rel-16/17 schemes) scheduled by a DCI without TCI field that is carried on a CORESET with two activated TCI states.

Observation 1: The current specification can support implicit determination of a set of BFD-RS from multiple TCI states, e.g. two activated TCI states for one CORESET.

Proposal 3: Per-TRP BFD for Scheme 1 can be discussed after completion of Rel-17 BFR.

# References

- RP-202024, “Revised WID: Further enhancements on MIMO for NR”, Samsung, RAN#89e, Sep. 2020

- 3GPP TS 38.213, “Physical layer procedures for control”, V16.5.0, March 2021