3GPP TSG RAN WG1 #105-e		R1-2105605

e-Meeting, May 10th – 27th, 2021

Title: 	Performance Evaluation Results for XR

Source: 	ZTE, Sanechips

Agenda item:	8.14.3

Document for:	Discussion and decision

Introduction

In 3GPP TSG RAN1#104bis-e meeting, some evaluation methodologies/assumptions and parameters of traffic model were discussed and the corresponding agreements are given in [1].

Based on the agreed traffic model and evaluation methodologies/assumptions, we present some performance evaluation results for XR in terms of capacity and power saving gain in different scenarios. Moreover, the critical parameters have been reported in results template.

FR1 Evaluations

In this section, the system performance of capacity, power consumption of FR1 are evaluated.

In RAN1#104e meeting, the average data rate and air interface PDB for DL video stream are agreed as follows.

Agreements - Average data rate for DL video stream: - VR/AR: 30, 45 Mbps @60fps (baseline) - 30, 60 Mbps @60fps (optional) - Note: this is the aggregated data rate when applicable - CG: 8, 30 Mbps @60fps (baseline) - 8, 45 Mbps @60fps (optional) Agreements - Air interface PDB for DL video stream - VR/AR: - 10ms - Other values, e.g., 5ms, 20 ms can be optionally evaluated. - CG: - 15ms - Other values, e.g., 10ms, 30ms can be optionally evaluated. - FFS whether or not to have more than one mandatory value

Therefore, 2 basic traffic models for VR/AR and 2 basic traffic models for CG can be obtained in Table 1 as follows, respectively.

Table 1 Summary of baseline traffic model

| Traffic Model No. | Configurations |
| --- | --- |
| TM1 | 30Mbps, 10ms |
| TM2 | 45Mbps, 10ms |
| TM3 | 8Mbps, 15ms |
| TM4 | 30Mbps, 15ms |

Furthermore, in RAN1#104bis-e meeting, the relationships between the variance/maximum/minimum packet size w.r.t the mean of truncated Gaussian distribution have been determined as follows.

Agreement: Parameters of Truncated Gaussian distribution for packet size of DL video stream in case of single stream evaluation (note: these parameter values are those before the truncation): - [STD, Max, Min]: [10.5, 150, 50]% of Mean packet size - Other values that can be used for evaluation: [STD, Max, Min] = [4, 112, 88] % of Mean for single eye buffer, [3, 109, 91] % of Mean for dual eye buffer - FFS: Whether and how to evaluate single eye and dual eye buffer - Note: Companies report the values used in their simulation results. - Note: There is no consensus that the [10.5, 150, 50]% of mean packet size is the best set of parameters

In our evaluations, both baseline and optional relationship of standard deviation/maximum/minimum packet size w.r.t mean packet size, that is [10.5, 150, 50]% of mean packet size and [3,109,91]% of mean packet size, have been considered.

For the KPI of single stream per UE in DL, RAN1 has the following agreement in RAN1#104bis-e meeting.

Agreement: In case of single stream per UE in DL, a UE is declared a satisfied UE if more than X (%) of packets are successfully delivered within a given air interface PDB. - The baseline X value is 99. - Other values of X can be optionally evaluated, e.g., X < = 95, X=99.9. - Additional combinations of (X, PDB) values can be optionally evaluated, e.g., - (99, 7), (95, 13) for VR/AR - (99, 12), (95, 18) for CG FFS: Different values for I-frame and P-frame if evaluation of them is agreed.

Hence, in our evaluations, baseline value X = 99, has been considered as packet success rate.

In addition, jitter was also considered in our downlink capacity evaluation. According to the agreement of jitter in RAN1#104e meeting, parameters of jitter distribution are shown in Table 2.

Table 2 Jitter model

| Mean | STD | Max delay | Min delay |
| --- | --- | --- | --- |
| 0(ms) | 2(ms) | 4(ms) | -4(ms) |

The parameters of FR1 evaluation are listed in Appendix A.

Capacity

The traffic models TM1,TM2,TM3,TM4 are used for downlink capacity evaluations, while traffic models for Pose/Control and AR video are considered in uplink capacity evaluation.

    - 2.1.1  Single Stream for DL FR1

- [10.5, 150, 50]% * mean packet size

Table 3 Summary of parameters for truncated Gaussian distribution of packet size using [10.5, 150, 50]% relationship.

| Traffic Model No. | Mean (Byte) | STD (Byte) | Maximum (Byte) | Minimum (Byte) |
| --- | --- | --- | --- | --- |
| TM1 | 62500 | 6563 | 93750 | 31250 |
| TM2 | 93750 | 9844 | 140625 | 46875 |
| TM3 | 16667 | 1750 | 25001 | 8333 |
| TM4 | 62500 | 6563 | 93750 | 31250 |

The simulation parameters are listed in Table A.1 in Appendix A. As a result, the system simulation results in indoor Hotspot and Dense Urban are presented in Figure 1 and Figure 2, respectively.

| - Capacity | - RU |
| --- | --- |

Figure 1 Capacity simulation results in indoor Hotspots scenario ([10.5, 150, 50]%)

| - Capacity | - RU |
| --- | --- |

Figure 2 Capacity simulation results in Dense Urban scenario ([10.5, 150, 50]%)

According to the results in Figure 1, it is found out there are 11.4 UEs for TM1, 7.2 UEs for TM2 and 12.9 UEs for TM4 per cell meet the capacity requirement in Indoor Hotspot, respectively.

According to the results in Figure 2, it is found out there are 13.1 UEs for TM1, 7.8 UEs for TM2 and 14.8 UEs for TM4 per cell meet the capacity requirement in Dense Urban, respectively.

Based on the analysis above, we can summarize the following observation of capacity results:

For the traffic model with (STD, MAX, MIN) = [10.5, 150, 50]% *mean packet size, the capacity results can be summarized as follows

| Traffic model No. | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Indoor Hotspot | Dense Urban |
| TM1 | 11.4 | 13.1 |
| TM2 | 7.2 | 7.8 |
| TM3 | - | - |
| TM4 | 12.9 | 14.8 |

- (STD, MAX, MIN) = [3, 109, 91]% *mean packet size

Table 4 Summary of parameters for truncated Gaussian distribution of packet size using [3, 109, 91]% relationship

| Traffic Model No. | Mean (Byte) | STD (Byte) | Maximum (Byte) | Minimum (Byte) |
| --- | --- | --- | --- | --- |
| TM1 | 62500 | 1875 | 68125 | 56875 |
| TM2 | 93750 | 2813 | 102188 | 85313 |
| TM3 | 16667 | 500 | 18167 | 16532 |
| TM4 | 62500 | 1875 | 68125 | 56875 |

The simulation parameters are listed in Table A.1 in Appendix A. As a result, the system simulation results in indoor Hotspot and Dense Urban are presented in Figure 3 and Figure 4, respectively.

| - Capacity | - RU |
| --- | --- |

Figure 3 Capacity simulation results in indoor Hotspots scenario ([3,109,91]%)

| - Capacity | - RU |
| --- | --- |

Figure 4 Capacity simulation results in Dense Urban scenario ([3, 109, 91]%)

According to the results in Figure 3, it is found out there are 11.8 UEs for TM1, 7.3 UEs for TM2, >15 UEs for TM3 and 13.3 UEs for TM4 per cell meet the capacity requirement in Indoor Hotspot, respectively.

According to the results in Figure 4, it is found out there are 13.3 UEs for TM1, 7.9 UEs for TM2 and 14.8 UEs for TM4 per cell meet the capacity requirement in Dense Urban, respectively.

Based on the analysis above, we can summarize the following observation of capacity results:

For the traffic model with (STD, MAX, MIN) = [3, 109, 91]% *mean packet size, the capacity results can be summarized as follows

| Traffic model No. | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Indoor Hotspot | Dense Urban |
| TM1 | 11.8 | 13.3 |
| TM2 | 7.3 | 7.9 |
| TM3 | >15 | - |
| TM4 | 13.3 | 14.8 |

According to the simulation results above, it can be observed that capacity performance based on the traffic model with (STD, MAX, MIN) = [3, 109, 91]% *mean packet size is slightly better than that based on the traffic model with (STD, MAX, MIN) = [10.5, 150, 50]% *mean packet size.

    - 2.1.2  Multiple Streams for DL FR1

In RAN1#104bis-e meeting, there are three options for multi-stream modelling for DL, which are shown as follows.

Agreement: In addition to single stream per UE in DL which is baseline, two streams can be optionally evaluated for DL - Option 1: I-frame + P-frame - Option 1A: slice-based traffic model - Option 1B: Group-Of-Picture (GOP) based traffic model - Option 2: video + audio/data - Option 3: FOV + omnidirectional stream - Companies should report detailed assumptions in their simulations on packet size distribution for each stream, packet arrival interval (or fps) for each stream, PDB for each stream, PER requirement for each stream, criteria for being satisfied. - Companies should strive to align the parameter values for the options chosen as much as possible - FFS: Whether audio stream is separate or aggregated with the data stream in option 2 (Intention of option 2 is not to create a 3 stream option)

According to our contribution [2], we think that “Option 1: I-frame + P-frame” should be regarded as start point for multi-stream modelling. In this part, our evaluations on group of picture (GoP)-based traffic model at 30Mbps and PDB = 10ms/15ms, which is the same as TM1 and TM4, in Indoor Hotspot scenario have been provided. The parameters of group of picture (GoP)-based traffic model is given as follows.

Table 5 Parameters of GoP-based traffic model

| Application | AR/VR/CG |
| --- | --- |
| Two stream data | I-frame / P-frame |
| Structure | A GoP consist of 8 frames with: 1 I-frame & 7 P-frames |
| Frame per second | I-frame + P-frame = 60FPS |
| Average packet size ratio | I-frame : P-frame = 2:1 |
| Mean packet size | - frame: 111.11K bytes P-frame: 55.56K bytes |
| Relationship between STD/MAX/MIN and mean | [10.5, 150, 50]% * mean packet size |
| (PER, PDB) | AR/VR: - I-frame: (1%, 10ms) - P-frame:(5%, 10ms) CG: - I-frame:(1%, 15ms) - P-frame(5%, 15ms) |

| - Capacity | - RU |
| --- | --- |

Figure 5 Capacity simulation results of AR/VR 30Mbps GoP-based traffic model in Indoor Hotspot scenario

| - Capacity | - RU |
| --- | --- |

Figure 6 Capacity simulation results of CG 30Mbps GoP-based traffic model in Indoor Hotspot scenario

According to the results in Figure 5 and Figure 6, it can be observed that the capacity performance of two-stream model is quite worse than that of single stream model, especially at the high system load. This is because that the bottleneck of the GoP-based traffic model is I-frame, whose mean packet size is 2 times larger than that of single stream but with the same PER and PDB as that of single stream. It seems that there is space for improvement in capacity simulation of multi-stream.

For the evaluation of multi-stream model, the capacity results in indoor Hotspot can be summarized as follows

| Traffic model | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Single stream | GoP-based traffic model |
| AR/VR 30Mbps | 11.4 | 10.2 |
| CG 30Mbps | 12.9 | 11.7 |

    - 2.1.3  Single Stream for UL Traffic

- Pose / Control

In uplink simulation, interaction/pose information delivering is evaluated. According to [1], traffic model of uplink interaction/pose information has been determined as follows.

- Traffic model for Pose/control - Periodic: 4ms (no jitter) - Other values can be optionally evaluated. - Fixed: 100 bytes (SA4 input) - PDB: 10 ms

The simulation parameters are listed in Table A.2 in Appendix A. The capacity evaluation results in Indoor Hotspot scenario are shown in Figure 7.

Figure 7 Capacity simulation results of pose/control traffic model in UL Indoor Hotspot scenario

According to uplink simulation results in Figure 7, it can be observed that larger than 40 UEs per cell are able to achieve the capacity requirement in Indoor Hotspot scenario, when pose/control traffic model is considered.

For uplink evaluation of interaction/pose information delivering, larger than 40 UEs per cell could be supported in Indoor Hotspot scenario.

- Video for UL AR

According to the agreement for UL traffic in RAN1#104bis-e meeting, we think that Option 2: single stream should be the start point for capacity evaluation.The single stream model for UL has been determined as follows.

- Option 2 (Optional for power evaluation and baseline for capacity evaluation): Single stream as defined below - Packet size: Truncated Gaussian distribution with the parameter values same as for DL - Periodicity: 60 fps - Jitter (optional): same model as for DL - Data rate: 10 Mbps (baseline), 20 Mbps (optional) - PDB: [60] ms (baseline), [10/15] ms (optional)

Hence, parameters of the single stream model are listed in Table 6.

Table 6 Single Stream model parameters

| Parameters | Values |
| --- | --- |
| Data rate | 20Mbps |
| Periodicity | 60fps |
| Mean packet size | 41667 Byte |
| Standard deviation | 4375 Byte |
| Maximum packet size | 62501 Byte |
| Minimum packet size | 28034 Byte |
| Packet delay budget | 10ms |
| Bandwidth | 100 MHz |

| - Capacity | - RU |
| --- | --- |

Figure 8 Capacity simulation results of traffic model in Table 6 in UL Indoor Hotspot scenario

According to uplink simulation results in Figure 8, it can be observed that 4.5 UEs per cell are able to achieve the capacity requirement in Indoor Hotspot scenario, when pose/control traffic model is considered.

For uplink evaluation of interaction/pose information delivering, 4.5 UEs per cell could be supported in Indoor Hotspot scenario.

Power consumption

Table 7 Traffic models for power consumption evaluation

| Traffic Model No. | Mean (Byte) | STD (Byte) | Maximum (Byte) | Minimum (Byte) |
| --- | --- | --- | --- | --- |
| TM1-1 | 62500 | 6563 | 93750 | 31250 |
| TM1-2 | 62500 | 1875 | 68125 | 56875 |
| TM2-1 | 93750 | 9844 | 140625 | 46875 |
| TM2-2 | 93750 | 2813 | 102188 | 85313 |

The traffic models used for power consumption evaluation are presented in Table 7.

In the following power consumption evaluation, the drx-HARQ-RTT-Timer is set to 0 and drx-RetransmissionTimer is set to 2ms. Other DRX settings are captured in Table 8.

One thing should be noted is that in this power consumption evaluation, DL and UL are simulated separately. Power of UL slots are obtained by UL simulation using pose/control traffic model and UE can transmit UL data in UL slots even if UE is in “DRX off” state. The power consumption of CSI and SRS are not considered. The power models are shown in Table B.1 and B.2 in Appendix B.

Table 8 DRX settings

|  | DRX cycle | Drx-onDurationTimer | Drx-inactivityTimer |
| --- | --- | --- | --- |
| DRX setting 1(ms) | 16 | 5 | 2 |

In this section, the following two schemes are evaluated:

- Baseline: No power saving technique is applied, that is, UE is always in Active Time.

- Enhanced DRX mechanism: The enhanced CDRX is a scheme which drx-startoffset changes dynamically. The dynamic change of drx-startoffset change can be triggered by 1) UE does not receive any data during the DRX ON period,  2) UE receives a change indication.

Jitter impact

In previous RAN1 meetings, jitter may should be considered in XR services. Jitter will affect the packet arrival time and may have influence on power saving schemes.

The Figure 9 and Table B.3 show the capacity and power saving gain (%) for TM2-2 with or without jitter in indoor scenario.

Figure 9 Capacity results

According to the simulation results, the capacity of baseline with or without jitter is almost same. But the capacity of eCDRX will be greatly impacted by jitter.

Jitter will decrease the capacity of power saving scheme.

To reduce the impact of jitter on capacity, UE may need have more time on “DRX On” state. For example, drx-onduration timer should be longer which will decrease the power saving gain.

Capacity and Power saving gain for eCDRX

In previous RAN1 meetings, the bitrate of AR/VR traffic model is 45Mbps and 30Mbps. In this section, these XR services are evaluated. The PDB is set to 10ms and the packets exceed PDB will be discard. The simulation results are shown in Figure 10-11 and Table 9. More simulation results are provided in Table B.3 in Appendix B. Jitter is considered in this section.

| (a)Capacity | (b) Mean power saving gain |
| --- | --- |

Figure 10 capacity and mean power saving gain for 45Mbps

| (a)Capacity | (b) Mean power saving gain |
| --- | --- |

Figure 11 capacity and mean power saving gain for 30Mbps

Table 9 Capacity and power saving gain for eCDRX

| Indoor | Capacity(90%) | C = floor(Capacity) | % of satisfied Ues @ C | Power saving gain @ C |
| --- | --- | --- | --- | --- |
| TM1-1 | 7.9 | 7 | 94 | 25.8% |
| TM1-2 | 7.4 | 7 | 92.8 | 25.8% |
| TM2-1 | 3.5 | 3 | 94 | 25.6% |
| TM2-2 | 4 | 4 | 90 | 24.3% |

According to the simulation results, the capacity decreases compared with baseline. The power saving gain is 25% for VR/AR in indoor scenario.

In indoor scenario, dynamic change drx-startoffset scheme can provide 25% power saving gain.

FR2 Evaluations

For FR2, TM1 and TM2 are used for downlink capacity evaluation. The simulation parameters are listed in Table A.3 in Appendix A. As a result, the capacity evaluation results for Indoor Hotspot scenarios in FR2 are shown in Figure 12.

Figure 12 Capacity simulation results in indoor Hotspots scenario

According to simulation results in Figure 12, it can be observed that 8.2 UEs per cell for TM1 and 3.4 UEs per cell for TM2 are able to achieve the capacity requirement in Indoor Hotspot scenario.

Based on the analysis above, we can summarize the following observation of capacity results for FR2:

For DL XR traffic in FR2, the capacity results can be summarized as follows

| Traffic model No. | #% of satisfied UE in Indoor Hotspot |
| --- | --- |
| TM1 | 8.2 |
| TM2 | 3.4 |

Conclusion

In this contribution, capacity results for single stream and multiple stream as well as power saving results are presented, and we have the following observations:

Observation 1: For the traffic model with (STD, MAX, MIN) = [10.5, 150, 50]% *mean packet size, the capacity results can be summarized as follows

| Traffic model No. | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Indoor Hotspot | Dense Urban |
| TM1 | 11.4 | 13.1 |
| TM2 | 7.2 | 7.8 |
| TM3 | - | - |
| TM4 | 12.9 | 14.8 |

Observation 2: For the traffic model with (STD, MAX, MIN) = [3, 109, 91]% *mean packet size, the capacity results can be summarized as follows

| Traffic model No. | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Indoor Hotspot | Dense Urban |
| TM1 | 11.8 | 13.3 |
| TM2 | 7.3 | 7.9 |
| TM3 | >15 | - |
| TM4 | 13.3 | 14.8 |

Observation 3: For the evaluation of multi-stream model, the capacity results in indoor Hotspot can be summarized as follows

| Traffic model | # % of Satisfied UE |  |
| --- | --- | --- |
|  | Single stream | GoP-based traffic model |
| AR/VR 30Mbps | 11.4 | 10.2 |
| CG 30Mbps | 12.9 | 11.7 |

Observation 4: For uplink evaluation of interaction/pose information delivering, larger than 40 UEs per cell could be supported in Indoor Hotspot scenario.

Observation 5: For uplink evaluation of interaction/pose information delivering, 4.5 UEs per cell could be supported in Indoor Hotspot scenario.

Observation 6: Jitter will decrease the capacity of power saving scheme.

Observation 7: In indoor scenario, dynamic change drx-startoffset scheme can provide 25% power saving gain.

Observation 8: For DL XR traffic in FR2, the capacity results can be summarized as follows

| Traffic model No. | #% of satisfied UE in Indoor Hotspot |
| --- | --- |
| TM1 | 8.2 |
| TM2 | 3.4 |

References

“Chair’s note”, 3GPP TSG RAN WG1 Meeting #104b-e, e-meeting, April 12th-20th, 2021.

R1-2105603. “Remaining Issues of XR Traffic Model”, ZTE, Sanechips. 3GPP TSG RAN WG1 Meeting#105-e, May 10th – 27th, 2021.

- Appendix

- A Simulation Parameters

Table A.1 simulation parameters for FR1 DL

| Parameters | Value |  |
| --- | --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m | Scenario-2: Dense Urban hexagonal layout with 7, 3 Sectors |
| Inter-BS distance | 20m | 200m |
| Carrier frequency | 4 GHz |  |
| Duplex Mode / Simulation bandwidth | 100 MHz |  |
| SCS | 30KHz |  |
| TDD pattern | DDDSU |  |
| BS Antenna Configuration | 32 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 32TX:(M, N, P, Mg, Ng; Mp, Np) = (4,4, 2, 1, 1; 4, 4); The antenna tilt is 90 degrees. | 64 Tx antenna ports, and (dH, dV) = (0.5λ, 0.5λ); 64TX:(M, N, P, Mg, Ng; Mp, Np) = (8,8, 2, 1, 1; 4, 8); The antenna tilt is 12 degrees. |
| UE Antenna Configuration | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); | 4 Rx antenna ports, and Panel model 1: dH = 0.5λ 4 Rx: (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1; 1,2); |
| Transmit Power | 24 dBm per 20M at TRP, and 23 dBm at UE | 44 dBm per 20M at TRP, and 23 dBm at UE |
| BS Height | 3 m | 25 m |
| UE Height | 1.5m | Outdoor UEs: 1.5 m Indoor UTs: 3(nfl – 1) + 1.5; nfl ~ uniform(1,Nfl) where Nfl ~ uniform(4,8) |
| Antenna Element Gain | 5dBi for BS and 0 dBi for UE | 8dBi for BS and 0 dBi for UE |
| Receiver Noise Figure | 5 dB for BS and 9 dB for UE |  |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel | 80% of users are indoor,20%of users are outdoor; Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 15 |  |
| Scheduling Algorithm | MU-MIMO+ PF |  |
| HARQ/repetition | HARQ retransmission |  |
| Channel estimation | Realist |  |
| Target BLER | 10% for first transmission |  |
| PHY processing delay | UE PUSCH processing Capability #1,N1=10 |  |
| PDCCH overhead | 1/7(2symbols per 14symbol) |  |
| DMRS overhead | 1 symbol per 14symbol |  |
| BS receiver | MMSE-IRC |  |

Table A.2 Simulation assumption for FR1 UL

| Parameter | value |
| --- | --- |
| Scenarios | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m |
| Carrier frequency | 4GHz |
| Bandwidth | 100 MHz |
| Subcarrier spacing | 30 KHz |
| Frame structure | DDDSU (S: 10D:2G:2U) for UL interaction/pose information delivering |
| BS Antennas (M,N,P,Mg,Ng;Mp,Np) | For 32R: (4,4,2,1,1;4,4) (dH,dV) = (0.5, 0.5)λ |
| UE Antennas (M,N,P,Mg,Ng;Mp,Np) | For 2T: (1,1,2,1,1;1,1) (dH, dV)=( 0.5, N/A)λ |
| BS antenna pattern | Ceiling-mount pattern, 5 dBi |
| UE antenna pattern | Omnidirectional, 0 dBi |
| UE max Power | 23 dBm |
| Power control | P0 = -80 dBm, alpha = 0.8 |
| Noise Figure | 5dB for BS and 9dB for UE |
| Scheduler | MU-MIMO+PF |
| Device deployment | 100% indoor |
| Down-tilt | 90 degrees |
| BS receiver | MMSE-IRC |
| UE receiver | MMSE-IRC |
| Channel estimation | Realistic |
| Target BLER | 10% for first transmission |
| UE speed | 3 km/h |
| HARQ/repetition | HARQ retransmission,Maximum number of retransmissions is 3 |
| PHY processing delay | UE PUSCH processing Capability #1,N2=12 |
| DMRE overhead | 4RE |

Table A.3 simulation parameters for FR2 DL

| Parameters | Value |
| --- | --- |
| Scenario | Scenario-1: Indoor Hotspot 12 nodes in 50 m x 120 m |
| Inter-BS distance | 20m |
| Carrier frequency | 30 GHz |
| Duplex Mode / Simulation bandwidth | 100 MHz |
| SCS | 120KHz |
| TDD pattern | DDDSU |
| BS Antenna Configuration | For 2T: (16,8,2,1,1;1,1) (dH, dV)=(0.5, 0.5)λ |
| UE Antenna Configuration | For 4R/panel: (2,4,2,1,2;1,2) (dH,dV) = (0.5, 0.5)λ (dg,V,dg,H) = (0, 0)λ |
| Transmit Power | 23 dBm per 80M at TRP, EIRP should not exceed 58 dBm |
| Antenna Height | 3 m for BS and 1.5 m for UE |
| Antenna Element Gain | 5dBi for BS and 5 dBi for UE |
| Receiver Noise Figure | 7 dB for BS and 13dB for UE |
| UE distribution | 100% of users are indoor Use 3km/h for modeling fading channel |
| Number of UEs per cell | up to 10 |
| Scheduling Algorithm | SU-MIMO+ PF |
| HARQ/repetition | HARQ retransmission，Maximum number of retransmissions is 3 |
| Channel estimation | Realist |
| Target BLER | 10% for first transmission |
| PHY processing delay | UE PUSCH processing Capability #1,N1=20 |
| PDCCH overhead | 1/7(2symbols per 14symbol) |
| DMRS overhead | 1 symbol per 14symbol |
| BS receiver | MMSE-IRC |

- B Power consumption results and power model

Table B.1 Power model for DL

| Power state | Relative Power(1 slot) |
| --- | --- |
| PDCCH-only | 100 |
| PDCCH+PDSCH | 300 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Table B.2 Power model for UL

| Power state | Relative Power(1 slot) |  |  |
| --- | --- | --- | --- |
|  | 0dBm | 23dBm | Other Tx power |
| UL(long PUCCH or PUSCH) | 250 | 700 | Obtained by linear function or step function |
| PDCCH+PUSCH | Sum(PDCCH-only, PUSCH) |  |  |
| PDCCH+PDSCH+PUSCH | Sum(PDCCH+PDSCH, PUSCH) |  |  |

Note: Power model of “PDCCH+PUSCH” is the sum of “PDCCH-only” power consumption and “PUSCH” power consumption. Power model of “PDCCH+PDSCH+PUSCH” is the sum of “PDCCH+PDSCH” power consumption and “PUSCH” power consumption.

Table B.3 Mean power saving gain in indoor scenario

|  | Number of UEs per cell | All UE |  | Only satisfied UE |  | Satisfied UE percentage |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | linear | Step | linear | step |  |
| eCDRX(jitter) TM1-1 | 3 | 27% | 27% | 27% | 27% | 100% |
|  | 5 | 26.2% | 26.2% | 26.3% | 26.3% | 98.3% |
|  | 7 | 25.8% | 25.8% | 26% | 26% | 94% |
| eCDRX(jitter) TM1-2 | 3 | 27% | 27% | 27% | 27% | 100% |
|  | 5 | 26.2% | 26.2% | 26.2% | 26.2% | 100% |
|  | 7 | 25.8% | 25.8% | 26% | 26% | 92.8% |
| eCDRX(jitter) TM2-1 | 1 | 25.8% | 25.8% | 26.2% | 26% | 100% |
|  | 3 | 25.4% | 25.5% | 25.6% | 25.6% | 94% |
|  | 4 | 24.3% | 24.3% | 24.8% | 24.8% | 87% |
|  | 5 | 23.5% | 23.5% | 24.4% | 24.4% | 70% |
| eCDRX(jitter) TM2-2 | 1 | 27% | 27% | 27% | 27% | 100% |
|  | 3 | 25% | 25% | 25% | 25% | 97.2% |
|  | 4 | 24.4% | 24.3% | 24.3% | 24.3% | 90% |
|  | 5 | 24% | 24% | 24% | 24% | 77% |
| eCDRX(no jitter) TM2-2 | 1 | 26% | 26% | 25.5% | 26% | 100% |
|  | 3 | 24% | 24% | 24% | 24% | 100% |
|  | 5 | 25% | 24.6% | 24.7% | 25% | 96.7% |

Note: “All UE” means all UEs are considered for evaluating the power saving gain. “Only satisfied UE” means only satisfied UEs are considered. “Linear” means linear method is used for UL UE power consumption evaluation. “Step” means only two Tx power values as defined in TR 38.840 are used for UL UE power consumption evaluation.