3GPP TSG RAN WG1 #105-e		R1-2105613

e-Meeting, May 10th – 27th, 2021

Title: 	Miscellaneous corrections of TS38.212_214

Source: 	ZTE, Sanechips

Agenda item:	7.2.4

Document for:	Discussion and decision

Introduction

This contribution provides miscellaneous corrections of TS38.212 and TS38.214.

Draft CRs

Corrections of TS 38.212, section 7.3.1.4.1

In section 7.3.1.4.1 of TS 38.212, the following two DCI field description refer to clause 16.4 of [5, TS 38.213]. But in that clause, there is no relevant description on ‘HARQ process number’ ‘ or ‘New data indicator’. the correct reference should be clause 5.22.1.3.1 of [8, TS 38.321].

- 7.3.1.4.1 Format 3_0 DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Resource pool index –log2I bits, where I is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling. - Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214] - HARQ process number – 4 bits as defined in clause 16.4 of [5, TS 38.213] - New data indicator – 1 bit as defined in clause 16.4 of [5, TS 38.213]

The following TP of TS 38.212 is proposed to correct the reference of ‘HARQ process number’ ‘ and ‘New data indicator’:

- 7.3.1.4.1 Format 3_0 <Unchanged parts omitted> - HARQ process number – 4 bits as defined in clause 5.22.1.3.1 of [8, TS 38.321] - New data indicator – 1 bit as defined in clause 5.22.1.3.1 of [8, TS 38.321] <Unchanged parts omitted>

Corrections of TS 38.214, section 8.1.2.1

An editorial error needs to be corrected in section 8.1.2.1 as below:

- 8.1.2.1 Resource allocation in time domain <Unchanged parts omitted> - The slot of the first sidelink transmission scheduled by the DCI is the first SL slot of the corresponding resource pool that starts not earlier than TDL-TTA2+KSL×Tslot where TDL is starting time of the downlink slot carrying the corresponding DCI, TTA is the timing advance value corresponding to the TAG of the serving cell on which the DCI is received and KSL is the slot offset between the slot DCI and the first sidelink transmission scheduled by DCI and Tslott is the SL slot duration. <Unchanged parts omitted>

An editorial error Tslott in section 8.1.2.1 should be corrected to Tslot.

Conclusion

This paper concludes with the following proposals:

Proposal 1: The following TP of TS 38.212 is proposed to correct the quotation of ‘HARQ process number’ ‘ and ‘New data indicator’:

- 7.3.1.4.1 Format 3_0 <Unchanged parts omitted> - HARQ process number – 4 bits as defined in clause 5.22.1.3.1 of [8, TS 38.321] - New data indicator – 1 bit as defined in clause 5.22.1.3.1 of [8, TS 38.321] <Unchanged parts omitted>

Proposal 2: An editorial error Tslott in section 8.1.2.1 should be corrected to Tslot.

References

- TS 38.212 g50

- TS 38.214 g50