3GPP TSG RAN WG1#105-e	 	 	R1-2105699

e-Meeting, May 10th – 27th, 2021

Source:	NTT DOCOMO, INC.

Title:	Discussion on mitigating UE and gNB Rx/Tx timing delays

Agenda Item:	8.5.1

Document for: 	Discussion and Decision

- Introduction

At the RAN#91-e meeting, updated WID on NR positioning was agreed [1]. The work item includes RAN1-led objectives as follows:

The objective of this work item is to specify solutions to enable RAT dependent (for both FR1 and FR2) and RAT independent NR positioning enhancements for improving positioning accuracy, latency, network and/or device efficiency. The objective of this work item also includes the support of GNSS enhancements.

The specific objectives of this work are:

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4]

  - DL, UL and DL+UL positioning methods

  - UE-based and UE-assisted positioning solutions

- Specify the procedure, measurements, reporting, and signalling for improving the accuracy of [RAN1, RAN2, RAN3]

  - UL AoA for network-based positioning solutions.

  - DL-AoD for UE-based and network-based (including UE-assisted) positioning solutions.

- Specify the enhancements of signalling, and procedures for improving positioning latency of the Rel-16 NR positioning methods, for DL and DL+UL positioning methods, including:

  - Latency reduction related to the request and response of location measurements or location estimate and positioning assistance data; [RAN2, RAN3, RAN1]

  - Latency reduction related to the time needed to perform UE measurements; [RAN1, RAN4]

  - Latency reduction related to the measurement gap; [RAN1, RAN4, RAN2]

- Study and specify, if agreed, the enhancements of information reporting from UE and gNB for multipath/NLOS mitigation [RAN1, RAN2, RAN3]

Notes:

Solutions for RAT-dependent positioning enhancements are designed to operate in both frequency ranges (i.e. FR1 & FR2)

RAN working groups will take into account the work from other working group SA/CT which involves same objectives.

- The WID is subject to further update in RAN #91 for RAN1/2/3/4 scoping.

In this contribution, we present our view on mitigating UE and gNB Rx/Tx timing delays for Rel-17 NR positioning.

- Mitigating Rx/Tx timing errors for UL-TDOA

Regarding mitigating timing error for UL-TDOA, the following agreement was made at the last meeting [3].

Agreement:

Support the following for mitigating UE Tx timing errors and/or TRP Rx timing errors for UL TDOA

- Support a TRP to provide the association information of RTOA measurements with TRP Rx TEG(s) to the LMF when the TRP reports the RTOA measurements to the LMF if the TRP has multiple Rx TEGs

- Support a UE to provide under capability the association information of UL SRS resources for positioning with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether to support a UE to provide the association information of UL SRS resources for MIMO with Tx TEGs to the LMF if the UE has multiple Tx TEGs

  - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF

- FFS: the details of the Signaling, procedures, and UE capability

The current NR supports UL positioning measurement (e.g. UL-RTOA, AoA and RSRP) using SRS resources for MIMO. From the perspective of RS overhead, positioning measurement using SRS resources for MIMO would be beneficial. We think the mitigation enhancements should be introduced for UL positioning using SRS resources for MIMO, since UE Tx timing error would impact on accuracy of UL positioning using not only SRS resources for positioning but also SRS resources for MIMO. According to the TS 38.331, SRS-Config does not have usage = positioning (i.e. usage = {beamManagement, codebook, nonCodebook, antennaSwitching} only). Therefore, when a gNB measures UL positioning metric based on SRS resources for MIMO, the positioning purpose of SRS resources for MIMO is transparent to the UE. If the UE supports to provide the association information of SRS resources for MIMO with UE Tx TEGs to the LMF, we may need to consider how to report the association information (e.g. whether the information should be report for each usage or not).

Observation 1:

- Regarding mitigating UE Tx timing errors for UL-TDOA, the mitigation enhancements should be introduced for UL positioning using SRS resources for MIMO

- Conclusion

In this contribution, we discussed on mitigating UE and gNB Rx/Tx timing delays. Based on the discussion, we made following observation.

Observation 1:

- Regarding mitigating UE Tx timing errors for UL-TDOA, the mitigation enhancements should be introduced for UL positioning using SRS resources for MIMO

References

- RP-210903, 3GPP RAN#91-e, “Revised WID on NR Positioning Enhancements”, Electronic Meeting, Mar. 2021.

- 3GPP RAN1#104-e, Chairman’s Notes, Feb. 2021.

- 3GPP RAN1#104b-e, Chairman’s Notes, Apr. 2021.

Appendix

At the RAN1#104-e meeting, the following definitions of word in this agenda item were agreed [2].

Agreement:

The following definitions are used for the purpose of discussion of internal timing errors (these terms are not agreed to be included in the specifications):

- Tx timing error: From a signal transmission perspective, there will be a time delay from the time when the digital signal is generated at baseband to the time when the RF signal is transmitted from the Tx antenna. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Tx time delay for the transmission of the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Tx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Tx time delay after the calibration, or the uncalibrated Tx time delay is defined as Tx timing error.

- Rx timing error: From a signal reception perspective, there will be a time delay from the time when the RF signal arrives at the Rx antenna to the time when the signal is digitized and time-stamped at the baseband. For supporting positioning, the UE/TRP may implement an internal calibration/compensation of the Rx time delay before it reports the measurements that are obtained from the DL PRS/UL SRS signals, which may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE. The compensation may also possibly consider the offset of the Rx antenna phase center to the physical antenna center. However, the calibration may not be perfect. The remaining Rx time delay after the calibration, or the uncalibrated Rx time delay is defined as Rx timing error.

- UE Tx ‘timing error group’ (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS resources for the positioning purpose, which have the Tx timing errors within a certain margin.

- TRP Tx ‘timing error group’ (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL PRS resources, which have the Tx timing errors within a certain margin.

- UE Rx ‘timing error group’ (UE Rx TEG): A UE Rx TEG is associated with one or more DL measurements, which have the Rx timing errors within a certain margin.

- TRP Rx ‘timing error group’ (TRP Rx TEG): A TRP Rx TEG is associated with one or more UL measurements, which have the Rx timing errors within a margin.

- UE RxTx ‘timing error group’ (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, and one or more UL SRS resources for the positioning purpose, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.

- TRP RxTx ‘timing error group’ (TRP RxTx TEG): A TRP RxTx TEG is associated with one or more gNB Rx-Tx time difference measurements and one or more DL PRS resources, which have the ‘Rx timing errors+Tx timing errors’ within a certain margin.