3GPP TSG RAN WG1 #105	R1-2105728
e-Meeting, May 10th – 27th, 2021

Source:				Panasonic

Title:		Aspects related to reduced number of Rx branches

Agenda item:	8.6.1.2

Document for:			Discussion and Decision

Introduction

This document discusses the aspects related to reduced number of Rx branches for Reduced Capability NR devices. The aspects are as follows:

Indication of the number of Rx branches

DCI format

Discussion

Indication of the number of Rx branches

The following agreement was made in RAN1 #104bis-e:

Agreements: - At least using UE capability report according the existing framework to indicate (implicitly or explicitly) the number of Rx branches - FFS: whether/how to support earlier indication of Redcap UEs with # Rx branches by Msg1 and/or Msg3, and MsgA - FFS: Network configurability of early indication of the number of Rx branches via SIB1, if supported

Regarding the first FFS point, we don’t see the need to support earlier indication of # Rx branches (Msg1/3/A) because the coverage recovery of Msg2/4 is not required according to the RAN discussion.

Regarding the antenna related information, some companies proposed indication of # of panels or polarization. But we don’t see the clear benefit in the context of RedCap. If necessary, it should be discussed in MIMO agenda. From RedCap WI, it should be precluded in order to reduce the amount of the specification work.

Proposal 1:	Earlier indication of # Rx branches (Msg1/3/A) is not supported.

Proposal 2:	Reporting of UE antenna related information to gNB beyond the number of Rx branches is not discussed in RedCap WI.

DCI format

The following agreement was made in RAN1 #104bis-e:

Agreements: - Reuse at least the existing DCI formats 0_x/1_x (including Rel-16 DCI format 0_2/1_2) applicable to Redcap devices as a starting point. - FFS Whether and how potential modification on fields of existing DCI formats is considered to reduce PDCCH block issue, if any. - FFS: Which DCI formats are mandatory for the RedCap UEs to support.

We think DCI formats 0_2/1_2 are beneficial for PDCCH blocking mitigation and should be mandatory for RedCap. DCI formats 0_0/1_0 is mandatory, not different from Rel.15/16.

We don’t prefer optimization on DCI formats 0_2/1_2 intended for PDCCH blocking mitigation. However, there may be the DCI field which is not necessary for the RedCap UEs. Each field in current DCI 0_2/1_2 can be studied and may be removed if not necessary.

On the other hand, as discussed in [1], we don’t think the enhancement on DCI 0_0/1_0 is necessary because it may cause the issue of DCI payload size mismatch between RedCap and non-RedCap UEs when initial BWP is shared with non-RedCap UEs.

Proposal 3:	DCI formats 0_2/1_2 with minimum restriction/adjustment (if needed) and DCI formats 0_0/1_0 with no enhancement should be mandatory for RedCap.

Conclusion

Regarding indication of the number of Rx branches:

Proposal 1:	Earlier indication of # Rx branches (Msg1/3/A) is not supported.

Proposal 2:	Reporting of UE antenna related information to gNB beyond the number of Rx branches is not discussed in RedCap WI.

Regarding DCI format:

Proposal 3:	DCI formats 0_2/1_2 with minimum restriction/adjustment (if needed) and DCI formats 0_0/1_0 with no enhancement should be mandatory for RedCap.

Reference

[1] R1-2103541, Panasonic, RAN1 #104bis, e-Meeting, April 2021