3GPP TSG RAN WG1 #105-e		R1-2105775

e-meeting, May 10th –  27th, 2021

Source:	Lenovo, Motorola Mobility

Title: 	Enhancements for joint channel estimation for multiple PUSCH

Agenda Item:   8.8.1.3

Document for:	Discussion

# 1	Introduction

In RAN#91-e, revised work item for NR coverage enhancements in Rel-17 has been approved with following objectives for in RP-210855 [1]:

- Specification of PUSCH enhancements [RAN1, RAN4]

  - Specify the following mechanisms for enhancements on PUSCH repetition type A [RAN1]

    - Increasing the maximum number of repetitions up to a number to be determined during the course of the work.

    - The number of repetitions counted on the basis of available UL slots.

  - Specify mechanism(s) to support TB processing over multi-slot PUSCH [RAN1]

    - TBS determined based on multiple slots and transmitted over multiple slots.

  - Specify mechanism(s) to enable joint channel estimation [RAN1, RAN4]

    - Mechanism(s) to enable joint channel estimation over multiple PUSCH transmissions, based on the conditions to keep power consistency and phase continuity to be investigated and specified if necessary by RAN4 [RAN1, RAN4]

      - Potential optimization of DMRS location/granularity in time domain is not precluded

    - Inter-slot frequency hopping with inter-slot bundling to enable joint channel estimation [RAN1]

- Specification of PUCCH enhancements [RAN1, RAN4]

  - Specify signaling mechanism to support dynamic PUCCH repetition factor indication [RAN1]

  - Specify mechanism to support DMRS bundling across PUCCH repetitions [RAN1, RAN4]

    - When applicable, based on similar mechanism(s) for enabling joint channel estimation for PUSCH

- Specify mechanism(s) to support Type A PUSCH repetitions for Msg3 [RAN1]

In this contribution, we provide our views on following aspects related to joint channel estimation across multiple PUSCHs for improved coverage:

- Use cases for joint channel estimation

- Inter-slot frequency hopping with inter-slot bundling

- Time domain window for joint channel estimation

- Configuration/indication of joint channel estimation

# 2	Use cases for joint channel estimation

In RAN1#104b-e, following agreement related to use cases have been discussed for joint channel estimation [2]:

Agreement:

- For back-to-back PUSCH transmissions across consecutive slots, support necessary design aspects (under the condition of power consistency and phase continuity) to enable joint channel estimation for the following cases:

  - Over back-to-back PUSCH transmissions (of the same TB) for repetition type B scheduled by dynamic grant or configured grant, if it reuses only those joint channel estimation specification enhancements defined to support repetition Type A.

    - FFS: additional specification enhancements on top of that defined to support repetition Type A

  - Only for single layer transmissions

  - Subject to UE capability

  - FFS: Over back-to-back PUSCH transmissions with different TBs

Based on the above agreement, joint channel estimation will be specified for at least the following use cases under back-to-back PUSCH transmissions:

- PUSCH repetition type A with same TB (joint channel estimation across multiple slots)

- PUSCH repetition type B with same TB (with similar support as for type A repetition).

Further discussion is needed on following aspects:

- Back-to-back PUSCH transmissions

  - Specific enhancements, if any, for PUSCH repetition type B with same TB

  - Different TB transmission with PUSCH repetition type A and type B

- Non back-to-back PUSCH transmissions

  - Within slot

  - Across slots

For the use cases with non-back-to-back PUSCH transmissions for both inter-slot and intra-slot, it should be up to RAN4’s feedback whether the phase continuity can be maintained and the maximum allowed gap between adjacent PUSCH transmissions. Based on LS reply [3], RAN4 confirms the feasibility of phase continuity and power consistency for non-zero un-scheduled gap case for a gap less than 14 symbols when UE is not required to meet the existing off power requirements. Furthermore, it is clarified that for the case with other UL channels in between repetitions, at least if the other scheduled signals/channels during the non-zero gap have the same settings in antenna port, occupied PRBs and UL power than the repeated transmission signals/channels, it is feasible to maintain the phase continuity and power consistency across the repetitions. Based on the reply, it seems, at least in certain scenarios within the slot, non-back-to-back PUSCH transmissions within a slot could be supported. However, since discussion is on-going in RAN4 and still some further clarifications will be need, we suggest deprioritizing discussion on non-back-to-back PUSCH transmissions in RAN1#105-e for both intra-slot and inter-slot joint channel estimation.

Proposal 1: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, use cases with non-back-to-back PUSCH transmission within one slot and across multiple slots should be deprioritized in RAN1#105, pending further discussion in RAN4.

Based on proposal 1, it could also simplify the discussion on specific enhancements same TB transmission with PUSCH repetition type B. Assuming that (at least currently) joint channel estimation is only supported for back-to-back PUSCH transmissions, then we think that no specific enhancements need to be discussed for type B repetition. Basically, once the time-domain window is configured/indicated for PUSCH repetition type B, then joint channel estimation should be applied only for the back-to-back PUSCH actual transmissions within the time-domain window.

Proposal 2: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 for transmission of same TB with PUSCH repetition type B, joint channel estimation should be applied across the actual transmissions that are back-to-back within a time-domain window

- No specific enhancements need to be discussed if the actual transmissions within a time-domain window are non-back-to-back, at least until, the requirements are fully clarified by RAN4.

On the remaining use case of applying joint channel estimation when different TBs are transmitted using PUSCH repetition type A or type B mechanisms, we think that similar agreements as for same TB transmission can be applied, as no additional specification impact is needed.

Proposal 3: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 for transmission of different TBs, agreements made for same TB transmission could be applied.

# 3	Inter-slot frequency hopping with inter-slot bundling

Another aspect related to joint channel estimation using DM-RS bundling is for multiple PUSCH transmissions with frequency hopping. In RAN1#104b-e, following agreement related to joint channel estimation with frequency hopping have been made [2]:

Agreements:

- For inter-slot frequency hopping with inter-slot bundling, down select on the following two options:

  - Option 1: The bundle size (time domain hopping interval) equals to the time domain window size.

  - Option 2: The bundle size (time domain hopping interval) can be different from the time domain window size.

    - FFS: Whether the bundle size (time domain hopping interval) is explicitly configured or implicitly determined.

    - FFS: Whether/How the bundle size (time domain hopping interval) is defined separately for FDD and TDD.

    - FFS: relation between the bundle size (time domain hopping interval) and the time domain window size

In our view, if frequency hopping is enabled for multiple PUSCH transmissions, then the DM-RS bundling should be contained within a single hop. Alternatively, DM-RS bundle duration could be same as frequency hop duration. Otherwise, phase continuity might be an issue for joint channel estimation across multiple frequency hops. However, if such limitation is applied, then the DM-RS bundling is not really beneficial as currently only intra-slot or inter-slot frequency hopping is supported. Therefore, multi-slot frequency hopping should be supported. This would provide better opportunity for joint channel estimation with DM-RS bundling. As illustrated in Figure 3, frequency hop duration is across 3 slots and therefore, the DM-RS bundling is also applied in multiples of 3 slots for each of the frequency hop. Based on this, we support option 1 where the bundle size i.e. the time-domain hopping interval is equal to the time domain window size. This option has additional benefit that the UE will not be required to be indicated with both these parameters and only indication of one parameter should suffice.

Figure 3: Example multi-slot frequency hopping with DM-RS bundling for multiple PUSCHs

Proposal 4: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 with inter-slot frequency hopping, support the option where the bundle size (time domain hopping interval) equals to the time domain window size and only one parameter needs to be indicated to the UE.

# 4	Time domain window for joint channel estimation

In RAN1#104b-e, following agreement related to definition of time-domain window for joint channel estimation has been made [2]:

Agreements:

- For joint channel estimation, specify a time domain window during which a UE is expected to maintain power consistency and phase continuity among PUSCH transmissions subject to power consistency and phase continuity requirements.

  - FFS how the time domain window is determined (e.g., via explicit configuration and/or implicitly derived) and whether or not to have the possibility of enabling/disabling the time domain window

  - FFS the units the time domain windown (e.g. repetitions, slots, and/or symbols)

    - FFS : association between the potential use case(s) and units of the time window

  - FFS: single or multiple time domain windows

- FFS: relation with UE capability

- FFS: whether the term "time domain window" is used in the specification or replaced by other technical terms

- FFS whether or not to further consider impacting of timing advance

Based on above agreements, as it has been agreed to specify time-domain window, then further following details should also be considered for joint channel estimation:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (for maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be additionally configured/indicated (duration value could be smaller than the maximum duration) and the UE is expected to maintain phase continuity over at least each successive non-overlapping time-domain window duration starting from the first PUSCH transmission

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- DM-RS bundling duration could be possibly considered as a term to be included in specifications.

Proposal 5: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, following details related to time domain window should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (depend on maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be configured/indicated (duration value could be smaller than the maximum duration)

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- DM-RS bundling duration could be possible considered as a term to be included in specifications.

Furthermore, following agreement related to options on the definition of unit for time domain window has been made in [2]:

Agreements:

- For the time domain window for joint channel estimation, down select on the following two options:

  - Option 1: The unit of the time domain window is defined separately for the following PUSCH transmissions:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

  - Option 2: The unit of the time domain window is the same for the following PUSCH transmission:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

In our view, we don’t see any specific need on defining time domain window unit separately depending up on different use case. Defining separate units could complicate the associated signaling procedure and unnecessarily increase specification impacts. The main criteria for defining time domain window should be simply based on phase continuity and power consistency requirements. Based on this criteria, it should be sufficient to specify a single unit for all use cases. In order to support different use cases, different set of values (using same unit) could be configured/indicated by network.

Proposal 6: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, same time unit of the time domain window should be specified for all the following use cases:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

# 5	Configuration/indication of joint channel estimation

In RAN1#104-e [4], there has been discussion on how the joint channel estimation can be configured/indicated. Two possibilities have been considered:

- Dedicated signaling to enable or disable joint channel estimation

- Implicit/joint signaling based on other parameters

In our view, if configuration/indicated related to time domain window is introduced, then separate configuration/indication of joint channel estimation can be avoided. Further discussion could be on whether dynamic indication or semi-static indication of time domain window is needed. Considering the availability of UL symbols/slots is based on dynamic SFI and cancellation indication (in addition to semi-static TDD configuration), the duration for the time domain window (i.e. enable/disable of joint channel estimation) should be dynamically signaled such that it can be based on actual duration of PUSCH transmission.

Proposal 7: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, enabling or disabling of joint channel estimation can be jointly indicated by the presence of signalling for time domain window duration

- Dynamic signalling of time domain window duration should be supported

# 6	Conclusion

In this section, we summarize the observation/proposals from above section:

Proposal 1: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, use cases with non-back-to-back PUSCH transmission within one slot and across multiple slots should be deprioritized in RAN1#105, pending further discussion in RAN4.

Proposal 2: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 for transmission of same TB with PUSCH repetition type B, joint channel estimation should be applied across the actual transmissions that are back-to-back within a time-domain window

- No specific enhancements need to be discussed if the actual transmissions within a time-domain window are non-back-to-back, at least until, the requirements are fully clarified by RAN4.

Proposal 3: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 for transmission of different TBs, agreements made for same TB transmission could be applied.

Proposal 4: For specifying joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17 with inter-slot frequency hopping, support the option where the bundle size (time domain hopping interval) equals to the time domain window size and only one parameter needs to be indicated to the UE.

Proposal 5: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, following details related to time domain window should be supported:

- Maximum duration for the time-domain window should be determined based on the minimum of following two durations:

  - Maximum duration for which power consistency and phase continuity can be maintained

  - Maximum duration of PUSCH transmissions (depend on maximum value of repetition factor)

- Depending upon coverage requirements, the duration of the time-domain window can be configured/indicated (duration value could be smaller than the maximum duration)

- For a burst of PUSCH transmissions with joint channel estimation, only a single duration of the time domain window should be configured/indicated

- DM-RS bundling duration could be possible considered as a term to be included in specifications.

Proposal 6: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, same time unit of the time domain window should be specified for all the following use cases:

    - PUSCH repetition type A

    - PUSCH repetition type B, if agreed

    - TBoMS, if agreed

    - Different TB, if agreed

Proposal 7: For supporting joint channel estimation with DM-RS bundling across multiple PUSCHs for coverage enhancements in NR Rel-17, enabling or disabling of joint channel estimation can be jointly indicated by the presence of signalling for time domain window duration

- Dynamic signalling of time domain window duration should be supported

# 7	References

[1]	3GPP RP-210855, “Revised WID on NR coverage enhancements”, China Telecom

[2] 	3GPP RAN1#104b-e. “Chairman Notes”

[3]	3GPP R1-2104168 (R4-2105417), “Reply LS on PUCCH and PUSCH repetition”, Qualcomm

[4] 	3GPP RAN1#104-e. “Chairman Notes”