3GPP TSG-RAN WG1 Meeting #105e	R1-2105811

Online, May 10th – 27th, 2021

Agenda Item:	5

Source:	Ericsson

Title:	Discussion regarding LS reply on fallback applicability for UE FeatureSetDownLinkPerCC capability fields

Document for:	Discussion

# 1	Introduction

In R1-2104164, a RAN2 LS asks RAN1 the following:

For Rel-16, RAN2 would like to confirm whether the following Rel-16 capability (present in FeatureSetDownlinkPerCC) is also applicable to the fallback concept:

multiDCI-MultiTRP;

RAN2 understands that there is no impact to consider the above capability as part of the fallback concept.

To RAN1:  	RAN2 respectfully asks RAN1 to confirm RAN2 interpretation on the Rel-16 capabilities mentioned above.

This discussion document aims at providing some input to the discussion on the RAN2 LS.

# 2	Discussion

In 38.306, the fallback concept is described as follows:

Fallback per CC feature set: A feature set per CC that has lower capabilities of UE supported MIMO layers and BW while keeping the numerology and other parameters the same from the reported feature set per CC for a given carrier per band.

In our view, the ‘multi-DCI-MultiTRP’ capability should also be applicable to the Fallback per CC feature set.  However, there is an issue regarding how the parameter maxNumberMIMO-LayersPDSCH should be interpreted for the ‘multi-DCI-MultiTRP’ feature.  Given that Multi-DCI Multi-TRP involves two PDSCHs, there could be two possible interpretations:

- Interpretation 1: maxNumberMIMO-LayersPDSCH is the maximum number of layers across both PDSCHs

- Interpretation 2:  maxNumberMIMO-LayersPDSCH is the maximum number of layers per PDSCHs

Our interpretation is Interpretation 1.  However, this needs to be clarified in the description of 38.306.  Hence, we propose the following answer to RAN2:

Response to RAN2:	RAN1 would like to confirm that the ‘multi-DCI-MultiTRP’ capability should also be applicable to the Fallback per CC feature set.  However, some clarification is needed in TS 38.306 on how the parameter ‘maxNumberMIMO-LayersPDSCH’ should be interpreted for the ‘multi-DCI-MultiTRP’ feature.  RAN1’s interpretation is that ‘maxNumberMIMO-LayersPDSCH’ is the maximum number of layers across both PDSCHs with regards to the ‘multi-DCI-MultiTRP’ feature.