3GPP TSG RAN WG1 Meeting #105-e	R1-2105935

e-Meeting, May 10th – 27th, 2021

Agenda Item:	5

Source:	Huawei, HiSilicon

Title:	Discussion on RI bit width for Cat5 UE in EN-DC mode

Document for:	Discussion and Decision

Introduction

RAN2 has sent an LS [1] to ask about the RI bit width for Cat. 5 UEs in EN-DC mode. This contribution discusses the understanding from RAN1 spec point of view.

Discussion

For EN-DC, UE can be allowed to support two layer MIMO for TM3/TM4 as indicated in TS 36.331 shown below. This means the maximum number of layers would also depends on the configuration of EN-DC mode.

FeatureSetDL-PerCC-r15 ::= SEQUENCE { fourLayerTM3-TM4-r15 ENUMERATED {supported} OPTIONAL, supportedMIMO-CapabilityDL-MRDC-r15 MIMO-CapabilityDL-r10 OPTIONAL, supportedCSI-Proc-r15 ENUMERATED {n1, n3, n4} OPTIONAL }

| fourLayerTM3-TM4 (in FeatureSetDL-PerCC) Indicates whether the UE supports 4-layer spatial multiplexing for TM3 and TM4 for MR-DC within the indicated feature set. If this field is absent, UE supports two layer MIMO for TM3/TM4. | - |
| --- | --- |

In current RAN1 spec, the maximum number of layers are mainly used for determination of RI bit-width and soft buffer size per HARQ process. For the RI bit-width mentioned in [1], the following is specified in current spec TS 36.212, with details in the Appendix.

TS 36.212, section For rank indication (RI) (RI only, joint report of RI and i1, joint report of CRI and RI, joint report of CRI, RI and i1, joint report of CRI, RI, and PTI, joint report of RI and i1,p-2, and joint report of RI and PTI) or CRI - The corresponding bit widths for CRI feedback for PDSCH transmissions are given by Tables 5.2.2.6.1-2A, 5.2.2.6.1-2C, 5.2.2.6.1-2G, 5.2.2.6.2-3A, 5.2.2.6.2-3C, 5.2.2.6.2-3G, 5.2.2.6.3-3A, 5.2.2.6.3-3C, 5.2.2.6.3-3G, 5.2.3.3.1-3E, 5.2.3.3.1-3H, 5.2.3.3.2-4E, and 5.2.3.3.2-4H. - The corresponding bit widths for RI feedback for PDSCH transmissions are given by Tables 5.2.2.6.1-2, 5.2.2.6.1-2B, 5.2.2.6.1-2D, 5.2.2.6.1-2E, 5.2.2.6.1-2F, 5.2.2.6.2-3, 5.2.2.6.2-3B, 5.2.2.6.2-3D, 5.2.2.6.2-3E, 5.2.2.6.2-3F, 5.2.2.6.3-3, 5.2.2.6.3-3B, 5.2.2.6.3-3D, 5.2.2.6.3-3E, 5.2.2.6.3-3F, 5.2.3.3.1-3, 5.2.3.3.1-3A, 5.2.3.3.1-3B, 5.2.3.3.1-3B-1, 5.2.3.3.1-3C, 5.2.3.3.1-3D, 5.2.3.3.1-3F, 5.2.3.3.1-3G, 5.2.3.3.1-3I, 5.2.3.3.1-3J, 5.2.3.3.1-5, 5.2.3.3.2-4, 5.2.3.3.2-4A, 5.2.3.3.2-4B, 5.2.3.3.2-4C, 5.2.3.3.2-4D, 5.2.3.3.2-4F, 5.2.3.3.2-4G and 5.2.3.3.2-4I which are determined assuming the maximum number of layers as follows: - If the maxLayersMIMO-r10 is configured for the DL cell, the maximum number of layers for subframe operation is determined according to maxLayersMIMO-r10 for the DL cell. - If the UE is configured with ShortTTI and if the higher layer parameter maxLayersMIMO-sTTI-r15 is configured for the DL cell, the maximum number of layers for slot/subslot operation is determined according to maxLayersMIMO-sTTI-r15 for the DL cell. - Else, - If the UE is configured with transmission mode 9, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the configured number of CSI-RS ports and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. …… - Otherwise the maximum number of layers is determined according to the minimum of the number of PBCH antenna ports and ue-Category (without suffix).

From the above spec, it can be observed that the bit widths for RI feedback for PDSCH transmissions are given assuming the maximum number of layers determined by the following, which does not mention the configuration of EN-DC mode.

- Configuration of maxLayersMIMO-r10 or maxLayersMIMO-sTTI-r15,

- Configuration/activation/selection of CSI-RS and the reported UE downlink MIMO capabilities,

- Configuration of CSI-RS and UE category (without suffix),

- PBCH antenna ports and UE category (without suffix).

Therefore, we have the following observation:

Observation 1: The bit widths for RI feedback for PDSCH transmissions are given assuming that the maximum number of layers are determined by following factors:

- Configuration of maxLayersMIMO-r10 or maxLayersMIMO-sTTI-r15,

- Configuration/activation/selection of CSI-RS and the reported UE downlink MIMO capabilities,

- Configuration of CSI-RS and UE category (without suffix),

- PBCH antenna ports and UE category (without suffix).

The above observation is aligned with the RAN2 understanding for RI bit-width of Cat. 5 UE as following:

Hence, RAN2 interpretation is that the RI bit width for a Cat5 UE is NOT affected by the number of MIMO layers it supports in EN-DC mode but only by the network configuration parameter maxLayersMIMO-r10, PBCH antenna ports and the UE category (without suffix), as in the legacy LTE (or simply, according to TS 36.213 only).

Hence, the following is proposed:

Proposal 1: Send a Reply LS to RAN2 to confirm that their assumption on RI bit width for Cat5 UEs in EN-DC mode is correct

Conclusions

We discussed the RI bit-width determination related to the question in [1], and have the following observation and proposal.

Observation 1: The bit widths for RI feedback for PDSCH transmissions are given assuming that the maximum number of layers are determined by following factors:

- Configuration of maxLayersMIMO-r10 or maxLayersMIMO-sTTI-r15,

- Configuration/activation/selection of CSI-RS and the reported UE downlink MIMO capabilities,

- Configuration of CSI-RS and UE category (without suffix),

- PBCH antenna ports and UE category (without suffix).

Proposal 1: Send a Reply LS to RAN2 to confirm that their assumption on RI bit width for Cat5 UEs in EN-DC mode is correct

- References

R1-2104161 (R2-2104538), LS on RI bit width for Cat5 UE in EN-DC mode, From: RAN2, To: RAN1, May, 2021, RAN1#105-e.

- Appendix

TS 36.212, section For rank indication (RI) (RI only, joint report of RI and i1, joint report of CRI and RI, joint report of CRI, RI and i1, joint report of CRI, RI, and PTI, joint report of RI and i1,p-2, and joint report of RI and PTI) or CRI - The corresponding bit widths for CRI feedback for PDSCH transmissions are given by Tables 5.2.2.6.1-2A, 5.2.2.6.1-2C, 5.2.2.6.1-2G, 5.2.2.6.2-3A, 5.2.2.6.2-3C, 5.2.2.6.2-3G, 5.2.2.6.3-3A, 5.2.2.6.3-3C, 5.2.2.6.3-3G, 5.2.3.3.1-3E, 5.2.3.3.1-3H, 5.2.3.3.2-4E, and 5.2.3.3.2-4H. - The corresponding bit widths for RI feedback for PDSCH transmissions are given by Tables 5.2.2.6.1-2, 5.2.2.6.1-2B, 5.2.2.6.1-2D, 5.2.2.6.1-2E, 5.2.2.6.1-2F, 5.2.2.6.2-3, 5.2.2.6.2-3B, 5.2.2.6.2-3D, 5.2.2.6.2-3E, 5.2.2.6.2-3F, 5.2.2.6.3-3, 5.2.2.6.3-3B, 5.2.2.6.3-3D, 5.2.2.6.3-3E, 5.2.2.6.3-3F, 5.2.3.3.1-3, 5.2.3.3.1-3A, 5.2.3.3.1-3B, 5.2.3.3.1-3B-1, 5.2.3.3.1-3C, 5.2.3.3.1-3D, 5.2.3.3.1-3F, 5.2.3.3.1-3G, 5.2.3.3.1-3I, 5.2.3.3.1-3J, 5.2.3.3.1-5, 5.2.3.3.2-4, 5.2.3.3.2-4A, 5.2.3.3.2-4B, 5.2.3.3.2-4C, 5.2.3.3.2-4D, 5.2.3.3.2-4F, 5.2.3.3.2-4G and 5.2.3.3.2-4I which are determined assuming the maximum number of layers as follows: - If the maxLayersMIMO-r10 is configured for the DL cell, the maximum number of layers for subframe operation is determined according to maxLayersMIMO-r10 for the DL cell. - If the UE is configured with ShortTTI and if the higher layer parameter maxLayersMIMO-sTTI-r15 is configured for the DL cell, the maximum number of layers for slot/subslot operation is determined according to maxLayersMIMO-sTTI-r15 for the DL cell. - Else, - If the UE is configured with transmission mode 9, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the configured number of CSI-RS ports and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the maximum of number of antenna port of the configured CSI-RS resources and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the configured number of CSI-RS ports and ue-Category (without suffix). - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the maximum of number of antenna port of the configured CSI-RS resources and ue-Category (without suffix). - If the UE is configured with transmission mode 9, and higher layer parameter semiOpenLoop, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of 2 and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9, and higher layer parameter semiOpenLoop, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of 2 and ue-Category (without suffix). - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1 with activatedResources>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the maximum of number of antenna ports of the activated CSI-RS resources and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1 with activatedResources>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the maximum of number of antenna ports of the activated CSI-RS resources and ue-Category (without suffix). - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with activatedResources=1 or numberActivatedAperiodicCSI-RS-Resources>0, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the number of antenna port of the activated or selected CSI-RS resource and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with activatedResources=1 or numberActivatedAperiodicCSI-RS-Resources>0, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the number of antenna ports of the activated or selected CSI-RS resource and ue-Category (without suffix). - If the UE is configured with transmission mode 9 and higher layer parameter eMIMO-Type and eMIMO-Type2, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the configured number of CSI-RS ports of eMIMO-Type2 and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 9 and higher layer parameter eMIMO-Type and eMIMO-Type2, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers is determined according to the minimum of the configured number of CSI-RS ports of eMIMO-Type2 and ue-Category (without suffix). - If the UE is configured with transmission mode 10, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the configured number of CSI-RS ports for that CSI process and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the maximum of number of antenna port of the configured CSI-RS resources in that CSI process and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the configured number of CSI-RS ports for that CSI process and ue-Category (without suffix). - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the maximum of number of antenna port of the configured CSI-RS resources in that CSI process and ue-Category (without suffix). - If the UE is configured with transmission mode 10, and higher layer parameter semiOpenLoop, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of 2 and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10, and higher layer parameter semiOpenLoop, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of 2 and ue-Category (without suffix). - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1 with activatedResources>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the maximum of number of antenna ports of the activated CSI-RS resources in that CSI process and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with K>1 with activatedResources>1, and RI and CRI are transmitted in the same reporting instance, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the maximum of number of antenna port of the activated CSI-RS resources in that CSI process and ue-Category (without suffix). - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with activatedResources=1 or numberActivatedAperiodicCSI-RS-Resources>0, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the number of antenna ports of the activated or selected CSI-RS resource in that CSI process and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10, and higher layer parameter eMIMO-Type, and eMIMO-Type is set to 'CLASS B' with activatedResources=1 or numberActivatedAperiodicCSI-RS-Resources>0, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the number of antenna port of the activated or selected CSI-RS resource in that CSI process and ue-Category (without suffix). - If the UE is configured with transmission mode 10 and higher layer parameter eMIMO-Type and eMIMO-Type2, and the supportedMIMO-CapabilityDL-r10 field is included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the configured number of CSI-RS ports of eMIMO-Type2 for that CSI process and the maximum of the reported UE downlink MIMO capabilities for the same band in the corresponding band combination. - If the UE is configured with transmission mode 10 and higher layer parameter eMIMO-Type and eMIMO-Type2, and the supportedMIMO-CapabilityDL-r10 field is not included in the UE-EUTRA-Capability, the maximum number of layers for each CSI process is determined according to the minimum of the configured number of CSI-RS ports of eMIMO-Type2 for that CSI process and ue-Category (without suffix) - Otherwise the maximum number of layers is determined according to the minimum of the number of PBCH antenna ports and ue-Category (without suffix).