3GPP TSG RAN WG1 Meeting #106-e	R1-2106494

E-meeting, August 16th – 27th, 2021

Agenda Item:	8.16

Source:	Huawei, HiSilicon, China Unicom

Title:	Enhancements on the scheduling of PUSCH over multiple slots

Document for:	Discussion and Decision

Introduction

In Rel-15 and Rel-16, to maintain the single carrier metric of uplink transmission, it is not allowed to transmit PUCCH and PUSCH simultaneously within a PUCCH group. If PUCCH and PUSCH overlap in time and have same priority level, the UCI (except SR) from PUCCH is multiplexed on the PUSCH. In order to alert the DCI missing and align the payload size of HARQ information carried on PUSCH, total DAI is included in DCI format 0_1 /0_2 to imply how many HARQ information bits should be piggybacked on the PUSCH. On the other hand, a restriction on the scheduling of PUCCH with HARQ is introduced to guarantee the DAI mechanism works properly. As described in the TS 38.213 Clause 9 [1], the restriction is copied as below:

“A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission.”

Another interpretation of this restriction is gNB cannot schedule a PUCCH transmission to carry HARQ information in a slot after a UL grant scheduling PUSCH transmission in that slot, and the PUCCH and PUSCH would overlap in time. In this contribution, it will mainly analyze the impact of the scheduling restrictions when PUSCH repetitions are configured, discuss how to optimize the scheduling of PUSCH over multiple slots in this situation and provide solution  to calculate the number of HARQ information bits on the PUSCH after optimizations.

Scheduling restriction on HARQ multiplexing on PUSCH

Following the timing order of DL/UL DCI formats, as shown in Figure 1, the first DL format in slot n schedules PDSCH_1 with corresponding HARQ feedback in slot n+k, if there is a UL DCI format transmitted in slot n+1 scheduling a PUSCH transmission in slot n+k, thus the second DL DCI format in slot n+2 is no longer allowed to schedule PDSCH_2 corresponding to HARQ information transmitted in slot n+k and overlaps with the scheduled PUSCH by the UL DCI format.

Figure 1. Timing restriction on UL/DL scheduling for HARQ information multiplexed on PUSCH

The same scheduling restriction, although not described explicitly in the spec, is also applied to the PUSCH with repetitions. However, due to the length of PUSCH transmissions in time, the restriction introduces more strict constrains on gNB scheduling for the case of PUSCH repetition.

Take an example for FDD system as shown in Figure 2, UL DCI in slot n schedules a PUSCH transmission over slots from n+2 to n+5, and the number of symbols for each PUSCH repetition is 14. In this case, PUCCH conveying the HARQ feedback corresponding to PDSCH in slot n+1 is not allowed to transmit in the slots of PUSCH repetition, i.e. slot n+2 ~ n+5. So the earliest opportunity for HARQ reporting is slot n+6, which results in a large k1 value and delay for HARQ feedback. The delay would become larger if more repetition times are assigned for PUSCH, and the k1 value could be as large as (K2 + number of PUSCH repetition) slots.

Figure 2. Scheduling of HARQ feedback with timing restriction in FDD system

Observation 1: If PUSCH repetition is configured, the timing restriction on scheduling HARQ after UL grant introduces large delay for HARQ feedback, which could be as large as (k2 + number of PUSCH repetition) slots.

In Rel-16, the number of PUSCH repetitions are counted by configured transmissions, the canceled repetitions caused by DL/UL collision will not be deferred. However, in Rel-17, the repetitions of PUSCH would be enhanced to transmit in available UL slots only, PUSCH repetitions may keep occupying contiguous UL slots and leave no opportunity to transmit PUCCH for a long period, especially for TDD system. Illustrated in Figure 3, a DL domain frame is configured as DDDSU pattern. In the slot 0 of frame N, UL DCI triggers PUSCH to repeat 4 times and each repetition occupies 14 symbols like the example in FDD system. Consequently, for the consecutive UL slots for frame N and N+1, gNB cannot schedule PUCCH to transmit the HARQ information associate with the PDSCHs scheduled in the DL slots of both frames. In other word, due to the PUCCH scheduling restriction, the DL traffic is blocked for lots of dedicated k1 values. For example, if the value of k1 is set as 1< k1<7, PDSCHs cannot be scheduled within any slots of frame N and frame N+1.

Figure 3. Scheduling of HARQ feedback with timing restriction in FDD system

Observation 2: If PUSCH repetition is configured, the timing restriction on scheduling HARQ after UL grant causes PDSCH blockage for dedicated k1 values.

As the analysis above, if repetition is enabled for PUSCH, the scheduling restriction for HARQ feedback after UL grant will introduce a large delay for HARQ reporting. The PDSCH scheduling is also blocked due to lack of PUCCH resource and the DL data rate is also slowed down in the meanwhile. Therefore, optimizations for PUSCH repetitions on the scheduling restriction should be studied to overcome the performance loss caused by the restriction.

Proposal 1: Optimization of timing restriction on scheduling HARQ after UL grant should be supported for the case of PUSCH repetition.

Optimization of the scheduling restriction

Based on further analysis, two alternatives can be considered for the optimization of the scheduling restriction. One alternative is to only apply the timing restriction to initial PUSCH repetition. That is, it is allowed to schedule PDSCH after UL grant with the corresponding HARQ-ACK multiplexed on non-initial PUSCH repetition(s) to avoid additional latency for HARQ feedback and blockage of DL data, as shown in Figure 4. For this alternative, the scheduling of first transmission is similar as that of the single PUSCH transmission, thus a uniform design could be applied for both cases which has less standards impact especially for TEI.

Another alternative is to release the restrictions for all the PUSCH repetitions. No matter initial or non-initial PUSCH repetitions, all of them can convey the HARQ bits for PDSCHs indicated after the UL DCI. This will bring a higher level of flexibility for gNB scheduling, but gNB has to treat single slot and multiple slots PUSCH separately. On the other hand, the removal of scheduling restriction will cause invalidation of total DAI in the UL DCI. More investigation and impact analysis on DAI mechanism are needed, which means lots of standard efforts are required as well. So considering the limited time for TEI discussion, the first alternative is more preferable due to less standards impact.

Proposal 2: The time restriction on scheduling HARQ after UL grant is only applied to the initial PUSCH repetition, and HARQ information bits corresponding to the PDSCH(s) scheduled after the UL grant which triggers the PUSCH transmission are allowed to be multiplexed on the non-initial repetitions.

Figure 4. Apply the timing restriction to the initial PUSCH repetition only

DAI enhancements

The optimization of timing restriction in section 2.1 relaxes the scheduling of PUCCH and makes it possible to piggyback the HARQ information corresponding to PDSCHs scheduled later than UL DCI. However, in this case the total DAI in UL grant cannot reflect the number of scheduled PDSCH(s) after the UL grant exactly, which would have impact on the HARQ-ACK codebook size determination. Therefore, some enhancements are needed here.

As shown in Figure 5, if the scheduling restriction is maintained for initial repetition but relaxed for non-initial ones, i.e. the first alternative in Section 2.1, the total DAI in the UL DCI format can be still applied to the first PUSCH repetition. For the non-initial PUSCH repetitions, one simple way is to use the DAI in the last DL DCI, which is same as the HARQ feedback piggybacked on CG PUSCH. One potential problem raised by some companies in the last two RAN1 meetings is the impact due to last DCI missing. Considering the reliability requirement of PDCCH decoding, the probability of DCI missing is relatively low and thus the impact might not be a very serious issue.

Figure 5. Update total DAI in UL DCI by the DAI in DL DCI

Further enhancements on the DAI mechanism  to address the impact from DCI missing is to still use the total DAI in the UL DCI format, but take the DL scheduling after the UL grant into account. Assuming the total DAI covers both the number of PDCCHs sent before the UL DCI and the ones would be delivered after the UL grant. Although in the PHY layer, gNB cannot anticipate how many PDSCHs will be scheduled in the future, gNB may set an upper bound of HARQ bits as the total DAI in UL grant to cover all the possible PDSCH(s) receptions, as shown in Figure 6. The challenge of this solution is the uncertainty for the future scheduling from gNB side. If the upper bound is set too large, additional resources are wasted.  If the bound is set too small, it will also limit the potential PDSCH receptions so that to degrade the downlink data rate.

Figure 6. Total DAI in UL DCI cover all past and future DL grants

Another method is to update the total DAI by other signaling. For example, a new DCI can be sent to UE to update the total DAI value just before the PUSCH transmission subject to the timeline conditions, similar operation as DCI format 2_4 which used to cancel the PUSCH transmission scheduled previously. As shown in Figure 7, UL DCI_2 is transmitted to UE to update the total DAI value which has been notified by UL DCI_1 in slot n+1, to incorporate the HARQ information corresponding to the PDSCH_2 scheduled in slot n+2. The shortage of this method is also obvious, additional DAI update signaling will bring more scheduling complexity and resources waste.

Figure 7. New UL DCI delivered to update DAI value

Considering above three methods to determine the HARQ information bits on PUSCH comprehensively, it seems the first option (i.e. rely on the DAI in last DL DCI) is more appropriate for TEI from the specification impact perspective. It has been applied for HARQ codebook carried in PUCCH and multiplexing HARQ on CG PUSCH.  Therefore, following proposal is made.

Proposal 3: When the timing restriction on scheduling HARQ after UL grant is released for the non-initial PUSCH repetitions, DAI in the last DCI is applied to determine the number of HARQ information bits multiplexed on the non-initial PUSCH repetitions.

Text proposal: To capture the enhancements on scheduling restriction, following changes on the specification should be adopted.

9 UE procedure for reporting control information <Unchanged parts omitted> A UE does not expect to detect a DCI format scheduling a PDSCH reception or a SPS PDSCH release, a DCI format 1_1 indicating SCell dormancy, or a DCI format including a One-shot HARQ-ACK request field with value 1, and indicating a resource for a PUCCH transmission with corresponding HARQ-ACK information in a slot if the UE previously detects a DCI format scheduling a PUSCH transmission with the number of repetitions K=1 or the first PUSCH repetition with the number of repetitions K>1 in the slot and if the UE multiplexes HARQ-ACK information in the PUSCH transmission. <Unchanged parts omitted>

9.1.2.2 Type-1 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field or other than the first repetition of a PUSCH scheduled by a DCI format includes a DAI field with the number of repetitions K>1, then - if the UE has not received any PDSCH or SPS PDSCH release that the UE transmits corresponding HARQ-ACK information in the PUSCH, based on a value of a respective PDSCH-to-HARQ_feedback timing indicator field in a DCI format scheduling the PDSCH reception or the SPS PDSCH release or on the value of dl-DataToUL-ACK if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_1 or on the value of dl-DataToUL-ACK-ForDCI-Format1-2 if the PDSCH-to-HARQ_feedback timing indicator field is not present in DCI format 1_2, in any of the Mc occasions for candidate PDSCH receptions by a DCI format or SPS PDSCH on any serving cell c, as described in clause 9.1.2.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else the UE generates the HARQ-ACK codebook as described in clause 9.1.2.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH, unless the UE receives only a SPS PDSCH release, or only SPS PDSCH reception, or only a PDSCH that is scheduled by DCI format 1_0 with a counter DAI field value of 1 on the PCell in the Mc occasions for candidate PDSCH receptions in which case the UE generates HARQ-ACK information only for the SPS PDSCH release or only for the PDSCH reception as described in clause 9.1.2. <Unchanged parts omitted>

9.1.3.2 Type-2 HARQ-ACK codebook in physical uplink shared channel If a UE would multiplex HARQ-ACK information in a PUSCH transmission that is not scheduled by a DCI format or is scheduled by a DCI format that does not include a DAI field or other than the first repetition of a PUSCH scheduled by a DCI format includes a DAI field with the number of repetitions K>1, then - if the UE has not received any PDCCH within the monitoring occasions for DCI formats scheduling PDSCH receptions, or SPS PDSCH release, or DCI format 1_1 indicating SCell dormancy on any serving cell c and the UE does not have HARQ-ACK information in response to a SPS PDSCH reception, or in response to a detection of a DCI format 1_1 indicating SCell dormancy, to multiplex in the PUSCH, as described in clause 9.1.3.1, the UE does not multiplex HARQ-ACK information in the PUSCH transmission; - else, the UE generates the HARQ-ACK codebook as described in clause 9.1.3.1, except that harq-ACK-SpatialBundlingPUCCH is replaced by harq-ACK-SpatialBundlingPUSCH. <Unchanged parts omitted>

Conclusions

Based on the discussion above, following observations and proposals are obtained:

Observation 1: If PUSCH repetition is configured, the timing restriction on scheduling HARQ after UL grant introduces large delay for HARQ feedback, which could be as large as (k2 + number of PUSCH repetition) slots.

Observation 2: If PUSCH repetition is configured, the timing restriction on scheduling HARQ after UL grant causes PDSCH blockage for dedicated k1 values.

Proposal 1: Optimization of timing restriction on scheduling HARQ after UL grant should be supported for the case of PUSCH repetition.

Proposal 2: The time restriction on scheduling HARQ after UL grant is only applied to the initial PUSCH repetition, and HARQ information bits corresponding to the PDSCH(s) scheduled after the UL grant which triggers the PUSCH transmission are allowed to be multiplexed on the non-initial repetitions.

Proposal 3: When the timing restriction on scheduling HARQ after UL grant is released for the non-initial PUSCH repetitions, DAI in the last DCI is applied to determine the number of HARQ information bits multiplexed on the non-initial PUSCH repetitions.

Text proposal in Section 2.2.

- References

3GPP, TS 38.213, V16.6.0, Physical layer procedures for control