3GPP TSG RAN WG1 Meeting #106-e                                          R1-2106549

e-Meeting, Aug 16th – Aug 27th, 2021

Source:         ZTE

Title:           Positioning accuracy improvement by mitigating timing delay

Agenda item:    8.5.1

Document for:   Discussion and Decision

Introduction

After intensive discussion in 3GPP RAN1#105-e meeting[1], some solutions about UL-TDOA and multi-RTT method have been achieved. In this contribution, we provide our views on further details for mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays based on the agreements and conclusions in last meeting and previous discussion.

UL-TDOA positioning

In RAN1#105e meeting, the following agreements were achieved:

- Agreement: - For mitigating UE Tx timing errors for UL TDOA, support one of the following options: - Option 1: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF if the UE has multiple Tx TEGs. - FFS: Support LMF to forward the association information provided by the UE to the serving and neighboring gNBs - Option 2: - Subject to UE’s capability, support a UE providing the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB if the UE has multiple Tx TEGs. - Support the serving gNB to forward the association information provided by the UE to the LMF - FFS: Support LMF to forward the association information from the serving gNB for the UE to the neighboring gNBs - FFS: UE should be able to report capability information related to Tx TEGs to LMF via LPP signaling - Support gNB to report the associated SRS resource ID/resource set ID of the RTOA measurement to LMF

- Figure 1. option 1                               Figure 2. option 2

- Figure 1 and Figure 2 illustrate option 1 and option 2, respectively. The solid line indicates the signaling transfer, and the dash line indicates the FFS within 2 options. The detailed analysis of advantages and drawback of option 1 and option 2 are as follows:

- Option 1

- In UL-TDOA, it is LMF’s duty to select and combine RTOA measurements to calculate UE position. In this case, serving gNB or neighboring gNB doesn’t need to know the association relationship between SRS resources and UE Tx TEGs, only LMF acquires this information is enough, which makes option 1 an effective solution. However, there is no LPP procedure for UL positioning so far, so a new skeleton configuration may be needed if option 1 is adopted, including the UL-TDOA measurement request and the UL-TDOA measurement report.

- Option 2

- In option 2, UE should transmit the association relationship firstly to the serving gNB, then forward to LMF. because the UL-TDOA is LMF-based positioning method, we don’t see the need that neighboring gNB should acquire the UE Tx TEG information for the use of mitigating UE Tx timing error. In addition, it may be better for serving gNB to acquire all the SRS information for better scheduling, since serving gNB is in control of SRS resource configuration at air interface. Also, reporting the association using option 2 takes less spec impact than option 1, for example, UE can report the SRS resource ID and their associated Tx TEG ID through RRC signaling.

- Observation 1: Option 2 takes the advantage over option 1 when UE doesn’t support LPP protocol.

- Based on above analysis, both of options have advantages and drawback, determination between option 1 and option 2 is a trade-off. For the consideration of less spec impact, we slightly prefer option 2, i.e., UE sends the association relationship firstly to the serving gNB ,then serving gNB forwards it to LMF.

- Proposal 1: For mitigating UE Tx timing errors for UL TDOA, support option 2, i.e., UE sends the association relationship firstly to the serving gNB, then serving gNB forwards it to LMF.

DL-TDOA positioning

- In last meeting, a proposal of DL-TDOA was basically formed but didn’t agreed due to the lack of time:

- Proposal: - Subject to UE’s capability, support a UE to - measure the same DL PRS resource from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements. - measure different DL PRS resources from a TRP with the same UE RX TEG, and report corresponding RSTD measurements. - measure different DL PRS resources from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements. FFS: indicate whether the measured RX TEG timing error difference has been compensated at UE side. FFS: Whether the TRP can be both ‘RSTD’ reference TRP and neighbor TRP FFS: Details of the Signaling, procedures, and UE capability Note: All RSTD measurements are relative to a single reference timing

- Different DL PRS resources from a TRP should be received freely at UE side, therefore, if UE supports the feature of UE Rx TEG, the second bullet and the third bullet are the basis that should be supported without doubt. Therefore, we don’t need to have a specific UE capability to emphasize the two bullets are supported.

- For the first bullet, this is the case that UE can simultaneously receive the same DL PRS resource by multiple UE Rx TEGs. UE can measure the same DL PRS resource from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements so that LMF may utilize the RSTD measurements to get UE Rx timing error difference between different UE Rx TEGs. Although we think if UE can get timing error difference of different UE Rx TEGs, there is no need to report multiple RSTD measurements, UE is able to compensate the timing error difference locally. However, we see some arguments that UE receives the same DL PRS resource arrives the UE in different path, which could be the case since two UE Rx TEGs may be linked to two UE Rx antenna panels toward different directions. In this case, the timing error difference cannot be acquired by UE since different UE Rx TEGs may see different delay profiles. Therefore we can accept that multiple RSTD measurements should be reported to LMF entirely. It’s up to LMF to decide which RSTD measurement should be excluded when conducting location computation.

- In current spec, the maximum number of measured and reported RSTD measurements per TRP pair is 4, and these RSTD measurements are derived from different PRS resources. If this proposal is adopted, 4 may not be enough since a same PRS may derive multiple RSTD measurements. Therefore, increase the maximum number of measured and reported RSTD measurements per TRP pair in DL-TDOA is necessary in the future enhancement.

- Proposal 2: For DL-TDOA, subject to UE’s capability, support a UE to

- Measure the same DL PRS resource from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements.

- the TRP can be both ‘RSTD’ reference TRP and neighbor TRP

- all RSTD measurements share the same time stamp

- Proposal 3: For DL-TDOA, increase the maximum number of measured and reported RSTD measurements per TRP pair, e.g. 8.

DL+UL positioning

Agreement: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, one or both of the following options: - Option 1: Reporting of UE RxTx TEG ID is supported by the UE - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements. - Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported. - In either option, a Tx TEG ID is associated with (downselection needed) - Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement - Alt. 2: the Tx timing of the Rx-Tx measurement - Alt. 3: one or more UL SRS resources for positioning - Note: An Rx TEG ID is associated with one DL PRS resource (or more DL PRS resources) corresponding to the Rx time of the measurement - FFS: How to resolve potential mismatch between UE and gNB Rx-Tx time difference measurements (e.g. UE provides the UE Rx-Tx measurements associated with a Tx TEG with SRS1, while gNB provides the gNB Rx-Tx measurements with a Rx TEG associated with SRS2). - FFS: The potential impact and modification on the definition of Rx-Tx time difference measurements

We think both options above can be supported according to UE capability. That is, if UE has self-calibration capability, UE reports RxTx TEG ID; if UE doesn’t have the capability, UE reports Rx TEG ID and Tx TEG ID separately. Therefore, there is no necessary to choose between option 1 and option 2.

For the three alternatives, they actually indicate how does UE determine Tx timing for a UE Rx-Tx time difference measurement. The detailed analyies are given as follows:

- Alt. 1: an UL SRS resource for positioning corresponding to the Tx timing of the Rx-Tx measurement

- The UE Rx-Tx difference measurement has no connection with which SRS resource is transmitted according to the definition in TS 38.215. However, this alternative means the definition UE Rx-Tx time difference may need to be modified. To reduce the spec efforts, we prefer to deprioritize this discussion.

- Alt. 2: the Tx timing of the Rx-Tx measurement

- In this alternative, the definition of UE Rx-Tx time difference measurement remains the same with Rel-16, which is not relevant with the SRS resource. Since the Tx timing is defined at Tx antenna connector of the UE (i.e. RF side), UE may calibrate its UE Tx timing error with some remaining margin before reporting the UE Rx-Tx time difference to LMF. Therefore, alt 2 is trying to say that the remaining margin can be grouped to which UE Tx TEG.

- Alt. 3: one or more UL SRS resources for positioning

- Alt 3 is similar with alt 2 that the definition of UE Rx-Tx time difference measurement remains the same with Rel-16. In addition, UE may provide a separate report on the relationship between SRS resources and Tx TEGs in multi-RTT method, where multiple SRS resources may be associated with the same UE Tx TEGs. This method provides more flexibility to avoid the mismatch between UE and gNB on SRS resource .Furthermore, it can not only benefit Multi-RTT but also UL-TDOA positioning method. Therefore, alt 3 is our first preference.

Proposal 4: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, both of the following options:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE

  - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

- Proposal 5: For Tx timing decision, support alt 3, i.e. a Tx TEG ID is associated with one or more UL SRS resources for positioning.

Multiple measurement instances in a report

Agreement: Support enabling - A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and - A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and - Each measurement instance is reported with its own timestamp - FFS: The measurement instances are within a [configured] measurement time window - FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set - FFS: N (including N=1) - FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions - FFS: M (including M=1) - FFS: details of signalling, procedures, and UE capability if any - FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric - Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s). - Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

For a periodic measurement report based on periodic DL PRS, there may be multiple instances of DL PRS to be measured between two consecutive occasions of the measurement report. For a measurement value reported based on the current design, the measurement value may be an average value from several measurement results based on the same DL PRS in different instances and/or different RS. As such, in the current design, the average operation is up to the UE’s implementation, and the LMF is missing a large amount of information regarding the time occasions and measurement results. In addition, even for the same DL PRS in different instances, the measurement results could be quite different due to the timing error variation of the device, blockage, movement of the UE, or other impairments. Therefore, the above agreement was approved in RAN1#104-e meeting so that UE/gNB can provide a positioning measurement report with multiple measurement instances to track aforementioned impairments. The intention of defining measurement instance is to ensure the static timing error of the measurement results in a measurement instance. Meanwhile, different measurement instances may have different timing errors. So, it’s enough to determine a TEG based on the dimension of spatial (panel/RF chain/antenna) and frequency, since timing error shift over time can already be reflected in different measurement instances.

Observation 2: It’s enough to determine a TEG based on the dimension of spatial (panel/RF chain/antenna) and frequency, since timing error shift over time can already be reflected in different measurement instances.

Based on the agreement above, at least the following remaining details need to be discussed in this meeting ( take UE side as an example, it can be also used for gNB/TRP side when applicable),

- The configuration level of N (i.e. the number of instances of DL PRS resource set in a UE measurement instance)

- The association between measurement instances and UE measurement report

- Measurement window and time stamp for UE measurement instances

Configuration level of N

As we know, the periodicity of DL PRS is configured per DL PRS resource set. What’s more, the assistance data provided by LMF may include different periodicities of DL PRS resource sets. Thus, the number of DL PRS resource set instances between two consecutive occasions of a measurement report may also be different for different DL PRS resource sets. Therefore, there could be multiple alternatives to configure the value of N depending on configuration level,

- Alt 1: configured by LMF per DL PRS resource set. The value may depend on the periodicity of corresponding DL PRS resource set

- Alt 2: configured by LMF per TRP. The value may depend on the periodicities of DL PRS resource sets associated with corresponding TRP.

- Alt 3: configured by LMF per positioning frequency layer. The value may depend on the periodicities of DL PRS resource sets associated with corresponding positioning frequency layer.

- Alt 4: configured by LMF per measurement report. The value may depend on the periodicities of DL PRS resource sets provided by LMF via assistance data.

Proposal 6: Further discuss how to configure the number of instances of DL PRS resource set in a UE measurement instance (i.e. the value N), at least consider one of the following alternatives,

- Alt.1: configured by LMF per DL PRS resource set.

- Alt.2: configured by LMF per TRP.

- Alt.3: configured by LMF per positioning frequency layer.

- Alt.4: configured by LMF per measurement report.

Association between measurement instances and UE measurement report

Figure 1 shows a skeleton example of Rel-16 UE reporting structure: a measurement report includes several positioning methods; each positioning method includes several measurement elements; each measurement element includes measurement results from a certain TRP.

Figure 1. Rel-16 UE reporting structure

Based on current design, how to support the association between measurement instances and UE measurement report needs to be further discussed. In general, there could be multiple options to associate multiple measurement instances in a measurement report,

- Option 1: For each indicated DL PRS resource in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource.

- Option 2: For each indicated DL PRS resource set in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource set.

- Option 3: For each indicated measurement element (i.e. TRP) in a measurement report, multiple measurement instances are associated with the indicated measurement element.

- Option 4: For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method.

- Option 5: Multiple measurement instances are directly associated with a measurement report.

Figure 2 shows a skeleton example of UE reporting structure for option 3, where each measurement element may include multiple measurement instances. In addition, the value of N may have impact on the number of measurement instances in a measurement report. For instance, as shown in Figure 2, the number of measurement instances associated with a measurement element (i.e. a TRP) may depend on the value of N and periodicities of DL PRS resource sets associated with the TRP.

Figure 2. UE reporting structure for option 3

Proposal 7: Further discuss the association between measurement instances and UE measurement report, at least consider one of the following options,

- Option 1: For each indicated DL PRS resource in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource.

- Option 2: For each indicated DL PRS resource set in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource set.

- Option 3: For each indicated measurement element (i.e. TRP) in a measurement report, multiple measurement instances are associated with the indicated measurement element.

- Option 4: For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method.

- Option 5: Multiple measurement instances are directly associated with a measurement report.

FFS: The relationship between the value N and the association between measurement instances and UE measurement report.

Time stamp of measurement instance

Measurement instance contains the measurement results derived from one PRS resource set shown N times in a period of time. A detailed example of option 3 is shown in figure 3, where a single TRP contains 2 PRS resource sets, measurement instance 1 contains the measurement results derived from 3 times of PRS resource set 1, and measurement instance 2 contains the measurement results derived from 3 times of PRS resource set 2.

Figure 3. Detailed example of option 3

- Due to the different periodicities of DL PRS resource sets, the duration for different measurement instances may be different. Also, since UE is well aware of its own timing error shift, it is better to let UE determine the length of measurement instance instead of configuring by network in advance. As we agreed, each measurement instance is reported with its own time stamp. In current specification, the time stamp specifies the time instance at which the measurement is performed. However, for multiple measurement instances in a measurement report, we can assume that the timing error is almost static within the duration of a measurement instance, so there is no need to report time stamp for each measurement results within a measurement instance. That’s to say, each measurement instance will only have one time stamp.

- In last meeting, some discussion about time stamp of a measurement instance caused a heated discussion. Some companies believe that only a start or an end time stamp is enough. However, the value N(instances of PRS in a measurement instance) may be configured per resource set, per TRP or per frequency layer. For example, if N is configured per TRP, then different PRS resource sets in this TRP may have different periodicities. In this case, it is hard to decide whether the window is to cover N instances corresponding to the small DL PRS periodicity or N instances corresponding to the large DL PRS periodicity. That is to say, if time stamp is only about a first or last reception time, and N can not be guaranteed, the length of each measurement instance can not be settled. Therefore, one solution to achieve the derivable length of measurement instance is to enhance the time stamp defined in Rel-16. The enhanced time stamp for a measurement instance should indicate a time period, wherein the corresponding measurement instance is conducted in this time period. For example, the time stamp can include,

- A starting time instance that corresponds to a reception time of the first reference signal for determining a measurement instance, and

- An ending time instance that corresponds to a reception time of the last reference signal for determining the measurement instance.

Proposal 8: Define the time stamp for a measurement instance, where the time stamp is a time window indicated by,

- A starting time instance that corresponds to a reception time of the first reference signal for determining a measurement instance, and

- An ending time instance that corresponds to a reception time of the last reference signal for determining the measurement instance.

Conclusions

In this contribution, we discuss positioning accuracy improvement by mitigating timing delay. Based on the discussion, we have the following observations and proposals:

- Observation 1: Option 2 takes the advantage over option 1 when UE doesn’t support LPP protocol.

Observation 2: It’s enough to determine a TEG based on the dimension of spatial (panel/RF chain/antenna) and frequency, since timing error shift over time can already be reflected in different measurement instances.

- Proposal 1: For mitigating UE Tx timing errors for UL TDOA, support option 2, i.e., UE sends the association relationship firstly to the serving gNB ,then serving gNB forwards it to LMF.

- Proposal 2: For DL-TDOA, subject to UE’s capability, support a UE to

- Measure the same DL PRS resource from a TRP with different UE Rx TEGs, and report corresponding RSTD measurements.

- the TRP can be both ‘RSTD’ reference TRP and neighbor TRP

- all RSTD measurements share the same time stamp

- Proposal 3: For DL-TDOA, increase the maximum number of measured and reported RSTD measurements per TRP pair, e.g. 8.

Proposal 4: For mitigating UE Tx/Rx timing errors for DL+UL positioning, a UE may support, up to UE capability, both of the following options:

- Option 1: Reporting of UE RxTx TEG ID is supported by the UE

  - FFS: Further details on how the RxTx TEG IDs are related/associated to Tx TEG IDs and/or Rx TEG IDs and to the Rx-Tx measurements.

- Option 2: Reporting of UE RxTx TEG ID is not supported by the UE; reporting of Rx TEG ID and Tx TEG ID is supported.

- Proposal 5: For Tx timing decision, support alt 3, i.e. a Tx TEG ID is associated with one or more UL SRS resources for positioning.

Proposal 6: Further discuss how to configure the number of instances of DL PRS resource set in a UE measurement instance (i.e. the value N), at least consider one of the following alternatives,

- Alt.1: configured by LMF per DL PRS resource set.

- Alt.2: configured by LMF per TRP.

- Alt.3: configured by LMF per positioning frequency layer.

- Alt.4: configured by LMF per measurement report.

Proposal 7: Further discuss the association between measurement instances and UE measurement report, at least consider one of the following options,

- Option 1: For each indicated DL PRS resource in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource.

- Option 2: For each indicated DL PRS resource set in a measurement report, multiple measurement instances are associated with the indicated DL PRS resource set.

- Option 3: For each indicated measurement element (i.e. TRP) in a measurement report, multiple measurement instances are associated with the indicated measurement element.

- Option 4: For each indicated positioning method in a measurement report, multiple measurement instances are associated with the indicated positioning method.

- Option 5: Multiple measurement instances are directly associated with a measurement report.

FFS: The relationship between the value N and the association between measurement instances and UE measurement report.

Proposal 8: Define the time stamp for a measurement instance, where the time stamp is a time window indicated by,

- A starting time instance that corresponds to a reception time of the first reference signal for determining a measurement instance, and

- An ending time instance that corresponds to a reception time of the last reference signal for determining the measurement instance.

References

- Draft Report of 3GPP TSG RAN WG1 #105-e, RAN1 Chairman Notes