3GPP TSG RAN WG1 #106-e		R1-2106580

e-Meeting, August 16th – 27th, 2021

Source:	vivo

Title:	Discussions on PDCCH monitoring enhancements for NR from 52.6-71GHz

Agenda Item:	8.2.2

Document for:	Discussion and Decision

- Introduction

The following objective is included in the agreed work item [1]:

- Physical layer aspects including [RAN1]:

  - Support enhancement to PDCCH monitoring, including blind detection/CCE budget, and multi-slot span monitoring, potential limitation to UE PDCCH configuration and capability related to PDCCH monitoring.

In this contribution, PDCCH monitoring capability and BD/CCE budget are discussed.

- Discussion

PDCCH monitoring capability

Background in existing NR Rel15&16 operation

In NR operation [2], for a DL BWP with any numerology, UE is expected to be mandatory to monitor PDCCH in every slot where the PDCCH monitoring occasions satisfy the following conditions:

- The duration of coreset associated with the PDCCH monitoring occasions is 1-3 symbols;

- For type 1 CSS with dedicated RRC configuration, type 3 CSS, and USS, the monitoring occasion is within the first 3 OFDM symbols of a slot;

- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions within a single span of three consecutive OFDM symbols within a slot.

For NR Rel-15 capability [3], UE is optionally to indicate the following capability to gNB which is more flexible than the above mandatory one:

- pdcch-MonitoringAnyOccasions: indicate to gNB that PDCCH monitoring occasions could be located in any symbol in the slot for type 1 CSS with dedicated RRC configuration, type 3 CSS, and USS:

  - withDCI-gap indicates there should be minimum time separation between two consecutive transmissions of PDCCH scrambled with C-RNTI or CS-RNTI

  - withoutDCI-gap indicates no minimum time separation limit

- pdcch-MonitoringAnyOccasionsWithSpanGap: indicate to gNB that PDCCH monitoring occasions could be located in any symbol in the slot by meeting the following condition:

  - The span pattern arrangement should satisfy at least one (X, Y) in the UE reported candidate value set, which is determined according to search space configuration and reporting (X, Y) as described below

    - In order to determine a suitable span pattern, first a bitmap b(l), 0<=l<=13 is generated, where b(l)=1 if symbol l of any slot is part of a monitoring occasion, b(l)=0 otherwise. The first span in the span pattern begins at the smallest l for which b(l)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(l)=1. The span duration is max {maximum value of all CORESET durations, minimum value of Y in the UE reported candidate value} except possibly the last span in a slot which can be of shorter duration.

For NR Rel-16 capability [4], UE is optionally to indicate the following capability to gNB:

- pdcch-Monitoring-r16: indicate to gNB that whether the UE supports PDCCH search space monitoring occasions in any symbol of the slot with minimum time separation between two consecutive transmissions of PDCCH with span up to two OFDM symbols for two OFDM symbols or span up to three OFDM symbols for four and seven OFDM symbols. The different value can be reported for PDSCH processing type 1 and PDSCH processing type 2, respectively.

- pdcch-MonitoringMixed-r16: indicate to gNB that UE supports Rel-15 monitoring capability and pdcch-Monitoring-r16 on different serving cells.

It is clearly observed that span patterns are defined for both NR Rel-15 and Rel-16 capability. Their difference lies in the following two aspects:

- Span pattern determination: the span pattern in Rel-15 is repeated in every slot while it may be different in every slot for Rel-16. One example is given below for better understanding:

Figure 1  Example for NR Rel-15 and Rel-16 span pattern

- BD/CCE budget calculation: For maximum number of PDCCH candidates and non-overlapping CCEs, it is calculated per slot for Rel-15 capability and per span for Rel-16 capability.

It is clearly that two types of PDCCH monitoring capability exist in NR Rel-16, i.e. slot-level or span-level capability. UE is required to be able to monitor PDCCH in every slot for either of the above capabilities.

Multi-slot-based capability definition

In RAN#90e [1], it is agreed that (120, 480, 960) KHz SCS are all supported for data/control. When PDCCH is configured in one DL BWP with 480/960KHz SCS, UE needs to monitor PDCCH every slot (i.e. ~15/30 us) if following the mandatory capability defined in existing NR operation as described in Section 2.1.1. This is quite challenging for UE implementation especially in such high frequency band. So, the mandatory capability on PDCCH monitoring in NR FR1&FR2 should be relaxed for NR operation from 52.6-71GHz, e.g. UE only needs to monitor in certain slot group instead of each slot within one subframe.

Proposal 1: For NR operation from 52.6-71GHz, PDCCH monitoring capability in FR1&FR2 should be relaxed from slot level to multi-slot level granularity.

To support multi-slot-based granularity, the most important issue is to define the multi-slot span that PDCCH monitoring capability is based on, i.e. the allowed slot(s) that UE could monitor. In RAN1#104bis [6], the following agreement is made:

Agreement:

Previous agreement is modifed as follows:

Choose one of the following alternatives for defining the multi-slot PDCCH monitoring capability

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive [symbols or slots] in each slot group separately

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y [symbols or slots] within a slot group, e.g. the Y [symbols or slots] always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - The capability indicates the BD/CCE budget within a span of at most Y consecutive [symbols or slots]

  - Y <= X

  - FFS: Exact values of X and Y and units in which they are defined (e.g., symbols, slots), including cases where a span is longer than one slot or crosses a slot boundary.

  - FFS: What is a span pattern, how it is defined and whether it is supported. If it is supported, whether number of slots within which the span pattern is repeated is needed, and if needed, the value of the number of slots.

  - FFS: Further definition of capabilities

- Alt 3: Use a sliding window of X slots as the baseline to define the new capability.

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is [1] slot.

  - FFS: Further definition of capabilities

- Specific numbers for X, Y may depend on UE capability and gNB configuration

  - Examples:

    - X = [4] slots for 480 kHz SCS and X = [8] slots for 960 kHz SCS

To be more specific, the above alternatives in the agreement is further expanded below with additional sub-alternatives:

- Alt. 1: Use a fixed pattern of slot groups as the baseline to define the new capability, i.e. each slot group consists of X slots and Slot groups are consecutive and non-overlapping

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - BD/CCE budget is defined within each slot group for each X or (X, Y)

  - The following alternatives to define allowed monitoring slots

    - Alt. 1.1: The first Y consecutive slots (Y<X) (one example is given in Figure 2)

    - Alt. 1.2: Any Y consecutive slots (Y<X) within a slot group

    - Alt. 1.3: Any Y slots (Y<X) within a slot group

    - Alt. 1.4: Y slots within a slot group (i.e. Y=X in Alt. 1.1/1.2/1.3)

Figure 2  Example for Alt. 1.1 with fixed slot-level pattern

- Alt 2: Use an (X, Y) span as the baseline to define the new capability

  - X is the minimum time separation between the start of two consecutive spans

  - BD/CCE budget is defined within a Y-symbol/slot span for each X or (X, Y)

  - Alt. 2.1: X and Y in terms of slot-level granularity (e.g. X=4, Y=2 as shown in Figure 3)

    - Alt. 2.1.1: Repeated slot level span pattern within each M slots

      - First a bitmap b(s), 0<=s<=M-1 is generated, where b(s)=1 if slot s within any M slots is configured, b(s)=0 otherwise. The first span in the span pattern begins at the smallest s for which b(s)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(s)=1. Then span pattern is repeated in every M slots (e.g. slot 0-15, 16-31 in Figure 3).

    - Alt. 2.1.2: Non-repeated slot level span pattern using M slots as container granularity

      - For each M slots, a bitmap b(s), 0<=s<=M-1 is generated, where b(s)=1 if slot s within the M slots is configured, b(s)=0 otherwise. For each M slots, the first span in the span pattern begins at the smallest s for which b(s)=1. The next span in the span pattern begins at the smallest l not included in the previous span(s) for which b(s)=1. Then span pattern is different in different M slots (e.g. slot 0-15, 16-31 in Figure 3).

Figure 3  Example for Alt. 2.1 with flexible slot-level span

  - Alt. 2.2: X and Y in terms of symbol-level granularity (e.g. X=48, Y=3)

    - Alt. 2.2.1: Repeated symbol level span pattern as NR Rel-15 within the slots that have PDCCH monitoring occasion or each M slots

    - Alt. 2.2.2: Non-repeated symbol level span pattern as NR Rel-16

- Alt. 3: BD/CCE budget using a sliding window of X slots

  - The capability indicates the BD/CCE budget within the sliding window

  - The sliding unit of the sliding window is 1 slot.

  - Any slot can be configured as monitoring occasion

For Alt. 1.2-1.4, there are the following two problems:

- Problem 1: the BD/CCE budget can’t be guaranteed for any X consecutive slots which may exceed UE implementation capability.

- Problem 2: no enough gap between consecutive PDCCH monitoring occasions can be guaranteed for UE to go to sleep and thus save the power consumption.

Although Alt. 1.1 have no such problems, the configuration of PDCCH monitoring is limited in certain fixed slot and gNB has no enough flexibility to configure CSS and USSs for multiple UEs. For Alt. 3, problem 1 is solved but problem 2 is still existing since any slot could be configured as PDCCH monitoring occasion. Alt. 2 could solve the above-mentioned problems with more configuration flexibility.

Proposal 2: Support Alt. 2 to define multi-slot based PDCCH monitoring capability, i.e. use (X, Y) span as baseline to define the capability.

Comparing Alt. 2.1 and Alt. 2.2 in Alt. 2, Alt. 2.1 is more flexible than Alt. 2.2 since UE could be configured with PDCCH monitoring occasions for Y consecutive slots. For Alt. 2.2, Y consecutive symbols within only one slot is allowed to be configured with PDCCH monitoring occasions, i.e. Alt. 2.2 is a special case of Alt. 2.1 assuming Y=1. In this sense, Alt. 2.1 could provide more complexity for gNB in PDCCH monitoring configuration.

Proposal 3: Using slot-level (X, Y) span (i.e. Alt. 2.1) to define multi-slot PDCCH monitoring capability is preferred compared to symbol-level (X, Y) span (i.e. Alt. 2.2).

After determination of the allowed slots for PDCCH monitoring, the allowed symbols within the slot could be referring to Rel-15 capability, e.g. the first 3 symbols within the slot are allowed to be configured with PDCCH monitoring occasions.

Application of multi-slot-based capability

In RAN1#104bis-e meeting, the following conclusion is made:

Conclusion:

For 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed.

According to the above conclusion, the PDCCH monitoring capability is the same as NR Rel-15&16 for other SCSs such as 120K. In other words, multi-slot-based capability applies to BWP with 480K and 960K SCS only.

Another question is whether to support slot-based capability for BWP with 480K/960K. The complexity of slot-based capability is quite challenging to UE implementation and there seems no benefit for UE to support slot-based capability. Thus, there is no need to support slot-based capability for BWP with 480K and 960K. Besides, multi-slot level capability should be the mandatory capability for BWP with 480K/960K.

Proposal 4: Multi-slot level capability is the mandatory capability for BWP with 480K and 960K SCS where slot-based capability is not supported.

It is clearly that UE will report UE capability on whether to support 480/960KHz SCS, e.g.

| scs-480kHz Indicates whether the UE supports 480kHz subcarrier spacing for data channel in FR2-2. | UE | No | No | FR2-2 only |
| --- | --- | --- | --- | --- |
| scs-960kHz Indicates whether the UE supports 960kHz subcarrier spacing for data channel in FR2-2. | UE | No | No | FR2-2 only |

According to Proposal 4, there is no need to report UE capability on whether to support multi-slot-based capability explicitly, i.e.  reporting support of 480K/960K SCS implies support of multi-slot-based capability.

Proposal 5: Reporting support of 480K/960K in UE capability implies support of multi-slot-based capability at UE side.

Mixed capability in multi-cell scenario

For NR Rel-16 UEs, there exists two types of PDCCH monitoring capability (i.e. slot-based and span-based) if it reports pdcch-Monitoring-r16. In this case, gNB may configure the capability type for a serving cell. If not configured, Rel-15 slot-based capability if the default one to be used. Then the following two types of serving cell may exist for one UE:

- Cell Type 1 (FR1/FR2): Serving cell with slot-based PDCCH monitoring capability;

- Cell Type 2 (FR1 only): Serving cell with span-based PDCCH monitoring capability.

Thus, the following cases may occur for one UE:

- Case 1: All serving cells belongs to cell type 1;

- Case 2: All serving cells belongs to cell type 2;

- Case 3: At least one serving cell belongs to cell type 1 and at least one serving cell belongs to cell type 2.

In general, the following table summarizes the relation of UE reporting capability and allowed gNB configuration cases:

| UE type | Reporting capability | Allowed Operation |
| --- | --- | --- |
| Rel-16 UEs | - | Case 1 only |
| Rel-16 UEs | pdcch-Monitoring-r16 | Case 1/Case 2 |
| Rel-16 UEs | pdcch-Monitoring-r16 pdcch-MonitoringMixed-r16 | Case 1/Case 2/Case 3 |

Besides, NR Rel-16 UEs will report capability related parameters (e.g. Ncellscap) for each case respectively.

If multi-slot-based capability is introduced for NR Rel-17 UEs, how to configure or determine the capability type needs to be considered. As long as multi-slot-based capability is the mandatory one for BWP with 480K/960K SCS according to Proposal 4, configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP, which means PDCCH monitoring capability should be defined per BWP.

Proposal 6: For NR Rel-17 UEs, PDCCH monitoring capability is defined per BWP and configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP.

After defining the PDCCH monitoring capability per BWP, the capability for one serving cell should also be determined for calculation of BD/CCE budget in multiple serving cell case. The straight forward way is to adopt the PDCCH monitoring capability for active BWP or configured first active BWP as the capability of the serving cell.

Proposal 7: PDCCH monitoring capability for a serving cell is the capability for its active BWP or configured first active BWP when it is deactivated.

Compared to NR Rel-16, one additional cell type occurs:

- Cell Type 3 (FR2-2 only): Serving cell with multi-slot-based PDCCH monitoring capability.

In addition to the operation cases in NR Rel-16, there may be more cases as listed below:

- Case 4: All serving cells belongs to cell type 3;

- Case 5: At least one serving cell belongs to cell type 1 and at least one serving cell belongs to cell type 3;

- Case 6: At least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3;

- Case 7: At least one serving cell belongs to cell type 1, at least one serving cell belongs to cell type 2 and at least one serving cell belongs to cell type 3.

Observation 1: More additional cases are brought by introduction of multi-slot-based PDCCH monitoring capability.

BD/CCE budget and overbooking

Background in existing NR operation

In NR operation, BD/CCE budget is defined to limit the complexity on PDCCH monitoring from UE side, i.e. UE is expected to monitor PDCCH candidates that satisfies certain conditions per slot. Here BD budget refers to the maximum number of PDCCH candidates and CCE budget refers to non-overlapped CCEs. The conditions are developed for single cell operation and multi-cell operation respectively.

BD/CCE budget in single cell operation scenario

For single cell operation with slot-based capability, the BD and CCE budget value are given in Table 1 and Table 2 [6] respectively. It is clearly observed that the budgets are defined in granularity of slot level in time domain. In this case, UE is not expected to monitor more than MPDCCHmax,slot,μ PDCCH candidates with more than CPDCCHmax,slot,μ non-overlapped CCEs per slot in the single serving cell.

Table 1: Maximum number MPDCCHmax,slot,μ of monitored PDCCH candidates per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of monitored PDCCH candidates per slot and per serving cell MPDCCHmax,slot,μ |
| --- | --- |
| 0 | 44 |
| 1 | 36 |
| 2 | 22 |
| 3 | 20 |

Table 2:  Maximum number CPDCCHmax,slot,μ of non-overlapped CCEs per slot for a DL BWP with SCS configuration μ∈0, 1,2,3 for a single serving cell

| μ | Maximum number of non-overlapped CCEs per slot and per serving cell CPDCCHmax,slot,μ |
| --- | --- |
| 0 | 56 |
| 1 | 56 |
| 2 | 48 |
| 3 | 32 |

For single cell operation with span-based capability, the BD and CCE budget value are given in Table 3, i.e. the value depends different (X, Y).

Table 3:  Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates in a span for combination (X, Y) for a DL BWP with SCS configuration μ∈0, 1 for a single serving cell

|  | Maximum number MPDCCHmax,X,Y,μ of monitored PDCCH candidates per span for combination X,Y and per serving cell |  |  |
| --- | --- | --- | --- |
| μ | (2, 2) | (4, 3) | (7, 3) |
| 0 | 14 | 28 | 44 |
| 1 | 12 | 24 | 36 |

BD/CCE budget in multi-cell operation scenario

For Case 1 defined in 2.1.4, the BD/CCE budget limit is defined below:

When the total number of cells is less than or equal to NcellsCap, the UE is not required to monitor, on the active DL BWP of the scheduling cell, more than  PDCCH candidates or more than  non-overlapped CCEs per slot for each scheduled cell.  Here MPDCCHmax,slot,μ and CPDCCHmax,slot,μ are provided in Table 1 and Table 2 respectively for single cell operation.

When the total number of cells is larger than NcellsCap, there are two different kind of limits:

1) Total limit of each scheduling cell group with SCS µ, i.e. the UE is not required to monitor more than  PDCCH candidates or more than  non-overlapped CCEs per slot on the active DL BWP(s) of scheduling cell(s) from the  downlink cells;

2) Respective limit of the scheduling cell for each scheduled cell, i.e. the UE is not required to monitor on the active DL BWP with SCS configuration  of the scheduling cell more than  PDCCH candidates or more than  non-overlapped CCEs per slot.

For Case 2 defined in 2.1.4, the BD/CCE budget limit is defined as similar way as that for Case 1 by replacing with span-based limit. Note that NcellsCap for this case is different with that for Case 1.

For Case 3 defined in 2.1.4 with mixed capability, the serving cells with the same capability are grouped together. For each group of serving cells, the same BD/CCE budget limit is defined as that for Case 1 or Case 2 depending on the capability type of the cell group. Note that there are two NcellsCap values for different group with different capability type respectively that are different with that for Case 1 and Case 2.

PDCCH overbooking

In NR Rel-15&16, it is allowed to have PDCCH overbooking for scheduling Pcell. Since BD/CCE budget is defined per slot, PDCCH candidates are allocated slot by slot from search space set with lower ID to that with higher ID until it reaches the BD/CCE budget per slot.

BD/CCE budget value for multi-slot-based capability

For Alt. 1/Alt. 3 in Section 2.1.2, the BD/CCE budget is defined per fixed or sliding slot group, i.e. PDCCH candidates and non-overlapped CCEs among the Y slots within the slot group should be less than MPDCCHmax,multi-slot,μ  and CPDCCHmax,multi-slot,μ.

For Alt. 2 in Section 2.1.2, the BD/CCE budget is defined per Y-slot/symbol span, i.e. PDCCH candidates and non-overlapped CCEs among the Y slots within the slot group should be less than MPDCCHmax,multi-slot,μ  and CPDCCHmax,multi-slot,μ.

The following alternatives are the candidate method to determine the value of MPDCCHmax,multi-slot,μ  and CPDCCHmax,multi-slot,μ:

- Alt. 1: MPDCCHmax,multi-slot,μ=X*MPDCCHmax,slot,μ, CPDCCHmax,multi-slot,μ= X*CPDCCHmax,slot,μ, where per slot limit MPDCCHmax,slot,μ and CPDCCHmax,slot,μ should be defined first;

- Alt. 2: Use the value in 120K as the reference, MPDCCHmax,multi-slot (X, Y),μ=X2μ-3∙MPDCCHmax,slot,3 and CPDCCHmax,multi-slot (X, Y),μ=X2μ-3∙CPDCCHmax,slot,3

- Alt. 3: Determine each value for supported (X, Y) using the following table:

|  | Maximum number CPDCCHmax,multi-slot X,Y,μ of non-overlapped CCEs per slot group for combination X,Y and per serving cell |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| μ | (4, 1) | (4, 2) | (8, 1) | (8, 2) | (8, 4) |
| 5 |  |  |  |  |  |
| 6 |  |  |  |  |  |

Among the above alternatives, Alt. 3 is a similar way with that for span-based capability, which clearly provides the BD/CCE budget value as well as supported (X, Y) value.

Proposal 8: For NR Rel-17 UEs, supported (X, Y) value should be determined first and then decide corresponding BD/CCE budget value for each (X, Y) case by case.

BD/CCE budget calculation in multi-cell scenario

In multi-cell operation scenario, BD/CCE budget calculation becomes more complex by introducing such multi-slot-based BD/CCE budget definition, i.e. more additional cases as described in 2.1.4.

As one straightforward way, the BD/CCE budget calculation adopts the same way for NR Rel-16, i.e. serving cells with the same PDCCH monitoring type are grouped together for further handling.  Particularly, the follows steps apply:

- Determination of NcellsCap: UE needs to report respective NcellsCap for different cases, i.e. Case 1-7 as described in 2.1.4. For the case with mixed capability, L NcellsCap values need to be reported where L is the number of capability types in that case (e.g. 3 in case 7);

- Determination of total limit for each group of serving cells:

  - If the group adopts slot-based or span-based capability, legacy way is used;

  - If the group adopts multi-slot-based capability, further divide the cell group into different parts depending on SCS and/or value of X/Y. Then BD/CCE budget for the serving cells will follow one total limit. Note that there may have certain limits in the group or part of serving cells.

As another alternative, the serving cell with SCS µ and multi-slot-based capability can be transformed to an equivalent virtual serving cell with SCS µ’ and slot-based capability, e.g. e.g. cell A with 480KHz SCS and BD/CCE budget per 4 slots is equivalent to a virtual cell A’ with 120KHz and BD/CCE budget per slot.

Proposal 9: For multi-cell operation, the following alternatives could be considered:

Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation;

Alt. 2: Transfer the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation.

Overbooking in NR operation from 52.6-71GHz

In NR operation from 52.6-71GHz, BD/CCE budget will be defined for multiple slot as proposed by Proposal 3. In this case, PDCCH candidates should be allocated for multiple slot Hs in overbooking case. In existing NR operation, PDCCH candidates are allocated per slot in granularity of SS. However, in multi-slot-based PDCCH monitoring capability case, PDCCH candidates could be allocated to multiple slots in granularity of SS and slot. How to allocate the PDCCH candidates in two dimensions should be considered.

Proposal 10: In multi-slot-based PDCCH monitoring capability case, PDCCH candidates could be allocated to multiple slots in granularity of SS and slot.

Search space configuration

In NR Rel-15&16, the allowed SS period configuration is from 1 slot to 2560 slots as shown below [7]. When PDCCH uses 480/960K SCS, there are the following two issues to be considered:

- Smaller SS period (e.g. 1 or 2 slots) is not needed for 480/960K SCS with multi-slot-based capability;

- The largest configurable SS period, i.e. 2560 slots=80/40ms for 480/960K SCS respectively, is not enough for SS configuration.

Proposal 11: Search space configuration should be improved for 480K/960K SCS.

- Conclusion

In this contribution, we focus on PDCCH monitoring capability and BD/CCE budget, and have the following proposals:

Proposal 1: For NR operation from 52.6-71GHz, PDCCH monitoring capability in FR1&FR2 should be relaxed from slot level to multi-slot level granularity.

Proposal 2: Support Alt. 2 to define multi-slot based PDCCH monitoring capability, i.e. use (X, Y) span as baseline to define the capability.

Proposal 3: Using slot-level (X, Y) span (i.e. Alt. 2.1) to define multi-slot PDCCH monitoring capability is preferred compared to symbol-level (X, Y) span (i.e. Alt. 2.2).

Proposal 4: Multi-slot level capability is the mandatory capability for BWP with 480K and 960K SCS while slot-based is not supported.

Proposal 5: Reporting support of 480K/960K in UE capability implies support of multi-slot-based capability at UE side.

Proposal 6: For NR Rel-17 UEs, PDCCH monitoring capability is defined per BWP and configuration of 480K/960K SCS for a BWP implies multi-slot-based capability for that BWP.

Proposal 7: PDCCH monitoring capability for a serving cell is the capability for its active BWP or configured first active BWP when it is deactivated.

Proposal 8: For NR Rel-17 UEs, supported (X, Y) value should be determined first and then decide corresponding BD/CCE budget value case by case.

Proposal 9: For multi-cell operation, the following alternatives could be considered:

Alt. 1: Serving cells with the same PDCCH monitoring type including multi-slot-based capability are grouped together for further BD/CCE budget calculation;

Alt. 2: Transfer the serving cell with multi-slot-based capability to equivalent serving cell with slot-based capability for further BD/CCE budget calculation

Proposal 10: In multi-slot-based PDCCH monitoring capability case, PDCCH candidates could be allocated to multiple slots in granularity of SS and slot.

Proposal 11: Search space configuration should be improved for 480K/960K SCS.

- References

- RP-202925, RAN #90-e, Electronic Meeting, 7-11 December 2020.

- TR 38.822 v15.0.1, User Equipment (UE) feature list, July 2019.

- TS 38.306 v15.5.0, User Equipment (UE) radio access capabilities, March 2019.

- TS 38.306 v16.3.0, User Equipment (UE) radio access capabilities, Dec. 2021.

- Chairman Notes, RAN1 104 e-meeting, February 2021.

- Chairman Notes, RAN1 104bis e-meeting, April 2021.

- TS 38.213 v16.4.0, Physical layer procedures for control, Sept. 2020.

- TS 38.331 v16.3.1, Radio Resource Control (RRC) protocol specification, Jan. 2021.