3GPP TSG RAN WG1#106-e                                R1-2106844

e-Meeting, August 16th – 27th, 2021

Source: 	ZTE, Sanechips

Title:	Discussion on DL MIMO layers for RedCap UEs

Agenda Item:	8.6.1.4

Document for:	Discussion

- Introduction

In RAN1#105 e-meeting, specification impacts were discussed for maximum number of DL MIMO layers and relaxed maximum modulation order for RedCap. And the related agreements were reached as below [1]:

- For a RedCap UE, when motivated by reduced max number of DL MIMO layers modifications to CSI measurement and/or reporting mechanisms are not pursued in Rel-17.

- For a RedCap UE, 64QAM MCS tables (Table 5.1.3.1-1 in TS 38.214 for DL and UL OFDM and Table 6.1.4.1-1 in TS 38.214 for UL w/ transform precoding respectively) are the “default” ones and are mandatory.

- The following is optionally supported by RedCap UEs:

- 256QAM MCS tables (Table 5.1.3.1-2 in TS 38.214 for DL and UL OFDM)

- 64QAM low SE MCS tables (Table 5.1.3.1-3 and Table 6.1.4.1-2 in TS 38.214)

- For a RedCap UE, “CQI table 1” (Table 5.2.2.1-2 in TS 38.214), that corresponds to MCS Table 5.1.3.1-1 in TS 38.214, is mandatory.

- The following is optionally supported by a RedCap UE:

- “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) that corresponds to 256QAM MCS table

- “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) that corresponds to 64QAM low SE MCS table

- Both 256QAM MCS table for PDSCH and “CQI table 2” (Table 5.2.2.1-3 in TS 38.214) are supported by a RedCap UE indicating support of 256QAM for PDSCH.

- For a RedCap UE, support of 64QAM low SE MCS table for PDSCH and support of “CQI table 3” (Table 5.2.2.1-4 in TS 38.214) are not coupled and capability of each can be reported independent of the other.

- For a RedCap UE, support of 64QAM low SE MCS table for PDSCH and support of 64QAM low SE MCS tables for PUSCH are not coupled and capability of each can be reported independent of the other.

In this contribution, we discuss some remaining issues on maximum number of DL MIMO layers.

- Discussion

Higher layer parameter

In NR, the higher layer parameter maxMIMO-Layers indicates the maximum number of MIMO layers to be used for PDSCH in all BWPs of this serving cell. Wherein, the bitwidth of 3 bits indicates 1 to 8 layers. It can be observed that the value of this parameter is not appropriate for RedCap UEs. Since the maximum number of MIMO layers is same as number of UE Rx branches, this parameter is not needed for 1Rx UEs and it can be modified to 1-bit signaling which indicates the maximum number of layers for PDSCH for 2Rx UEs.

Additionally, the existing higher layer parameter maxNrofCodeWordsScheduledByDCI is used to indicate the maximum number of code words that a single DCI may schedule. Since only single codeword transmission is supported for RedCap UEs, this parameter is not required.

Therefore, the modification or removal of higher layer parameters should be considered to simplifying signaling and save overhead for RedCap UEs. Then, which higher layer parameters are affected on MIMO layers should be discussed by RAN1. And whether/ how to optimize these parameters is determined by RAN2.

Proposal 1: RAN1 should discuss which higher layer parameters are affected on MIMO layers for RedCap UEs.

- Whether/ how to optimize these parameters is determined by RAN2.

DCI fields

Due to smaller number of MIMO layers, the size of one downlink transport block will be reduced for RedCap UEs. Consequently, the number of code block groups in a TB is impacted. In the existing DCI format 0_1 and 1_1, CBG transmission information field is used to indicate each code block group in a transport block. This field can be configured as 0, 2, 4, 6 or 8 bits by higher layer. The configurable field size can be compatible with RedCap UE requirements.

Moreover, modulation and coding scheme, new data indicator and redundancy version fields for transport block 2 are not required in DCI format 1_1 for RedCap UEs. In the existing specifications, these three fields are only present if higher layer parameter maxNrofCodeWordsScheduledByDCI equals 2. When single code word is scheduled for PDSCH, the size of these fields is 0 bits. So they are not present in DCI format 1_1 for RedCap UEs.

In DCI format 1_1 and 1_2, antenna ports field indicates DL DMRS configurations, as defined by Tables 7.3.1.2.2-1/2/3/4 and Tables 7.3.1.2.2-1A/2A/3A/4A in TS 38.212. The bitwidth of this field is 4, 5 or 6 bits depending on different DMRS configurations if it is present. Considering reduced number of MIMO layers, less DMRS ports are required. For 2Rx RedCap UEs, two DMRS ports can be configured to support up to 2-layer transmission. In this case, antenna ports field still need the bitwidth of 4, 5 or 6 bits which are not reduced compared to the existing field size. But for 1Rx RedCap UEs, since only single DMRS port is used for 1-layer transmission, antenna ports field can be reduced by 1 bit after the capability on maximum number of MIMO layers is reported.

Proposal 2: For 1Rx RedCap UEs, antenna ports field can be reduced by 1 bit in DCI format 1_1/1_2 after the capability on maximum number of MIMO layers is reported.

- Conclusion

In this contribution, we have discussed the remaining issues on maximum number of DL MIMO layer for RedCap UEs. And the following proposals are given:

Proposal 1: RAN1 should discuss which higher layer parameters are affected on MIMO layers for RedCap UEs.

- Whether/ how to optimize these parameters is determined by RAN2.

Proposal 2: For 1Rx RedCap UEs, antenna ports field can be reduced by 1 bit in DCI format 1_1/1_2 after the capability on maximum number of MIMO layers is reported.

- Reference

- 3GPP, Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0, RAN1#105-e