3GPP TSG-RAN WG1 #106-e	R1-2106924

e-Meeting, August 16th – 27th, 2021

Title:	Draft reply LS on physical layer aspects of small data transmission

Response to:	R1-2106405 (R2-2106561)

Release:	Rel-17

Work Item:	NR_SmallData_INACTIVE-Core

Source:	RAN WG1

To:	RAN WG2

Contact Person:

#### Name:	Lei Zhou

E-mail Address:	zhoulei@catt.cn

- Overall Description:

RAN1 thanks RAN2 for the reply LS R1-2106405 (R2-2106561) and would like to provide the following responses.

For RA-SDT (for both 2-step RA and 4-step RA), RAN1’s understanding is configuration parameters include ssb-perRACH-OccasionAndCB-PreamblesPerSSB, prach-ConfigurationIndex, msg1-FrequencyStart and msg1-FDM for 4-step RA  and msgA-CB-PreamblesPerSSB-PerSharedRO, msgA-SSB-SharedRO-MaskIndex, msgA-PRACH-ConfigurationIndex, msgA-RO-FrequencyStart and msgA-RO-FDM for 2-step RA for the PRACH resource configuration when PRACH occasions are shared between SDT and non-SDT and configuration parameters include totalNumberOfRA-Preambles, prach-ConfigurationIndex, msg1-FrequencyStart and msg1-FDM for 4-step RA for 4-step RA and msgA-TotalNumberOfRA-Preambles, msgA-PRACH-ConfigurationIndex, msgA-RO-FrequencyStart and msgA-RO-FDM for 2-step RA for the PRACH resource configuration when PRACH occasions are separately configured for SDT and non-SDT.

For CG-SDT, RAN1 confirms the agreement #3 on WA: UE-specific search space is configured for UEs performing CG-SDT and the agreement #4 on CG-SDT resource can be configured on either initial BWP or separate SDT BWP in RAN2 reply LS (R2-2106561). Regarding the agreement #5 on some feedback being beneficial for subsequent transmission, RAN1 thinks explicit ACK is not necessary and current mechanism including calculation of HARQ process ID and retransmission based on dynamic grant on CG transmission in licensed band operation can be reused for subsequent transmission in CG-SDT.

2. Actions:

To RAN2

ACTION: RAN1 asks RAN2 to kindly take the above into account on small data transmission for RA-SDT and CG-SDT.

3. Date of Next TSG WG RAN1 Meetings:

TSG-RAN WG1 Meeting #106bis-e	11th Oct. – 19th Oct. 2021		e-Meeting.

TSG-RAN WG1 Meeting #107-e                                   11th Nov. – 19th Nov. 2021              e-Meeting.