3GPP TSG RAN WG1 #106-e	R1- 2107062

e-Meeting, August 16th – 27th, 2021

Agenda item:		8.5.6

Source: 	Nokia, Nokia Shanghai Bell

Title: 		Additional views on Inactive Mode Positioning and on-demand PRS

Document for: 	Discussion and Decision

# Introduction

In Rel-16 native NR positioning support was standardized. At RAN#90 a new WI was approved on enhancements in Rel-17 to positioning [1]. This contribution discusses our views on the enhancements related to on-demand PRS and inactive mode positioning. Our companion contributions discuss our others views [2-6]. The objectives from the WID are

- Specify methods, measurements, signalling and procedures to support positioning for UEs in RRC_ INACTIVE state, for UE-based and UE-assisted positioning solutions, including [RAN2, RAN1, RAN3,RAN4]:

  - DL NR positioning methods and RAT-independent positioning methods

    - Support of UE positioning measurements for UEs in RRC_INACTIVE state

    - Reporting of positioning measurement or location estimate performed in RRC_INACTIVE when the UE is in RRC_INACTIVE state

Note: this work will be coordinated with the SDT WI.

  - As 2nd priority:

    - UL and DL+UL NR positioning methods

    - Support of gNB positioning measurements for UEs in RRC_INACTIVE state

- Specify on-demand transmission and reception of DL PRS for DL and DL+UL positioning for UE-based and UE-assisted positioning solutions, including: [RAN2, RAN1, RAN3]

  - UE-initiated request of on-demand DL PRS transmission;

  - LMF (network)-initiated request of on-demand DL PRS transmission;

# Discussion

## Inactive Mode Positioning

At RAN1 #105e, the following was made:

Agreement:

NR positioning supports DL PRS-RSRP (section 5.1.28 in the TS 38.215) and DL RSTD (section 5.1.29 in the TS 38.215) measurements by UEs in RRC_INACTIVE state

- FFS additional potential impact on RAN1

Thus, at least DL-based positioning solution(s) supports location estimation of the UEs in the RRC_INACTIVE state. Based on the agreement, RAN1 may need further discussion on the potential impact such as if we can reuse the DL PRS processing capability to the UEs in the RRC_INACTIVE state and if the QCL configuration of each PRS resource is also resusable in RRC_INACTIVE. First of all, RAN1 needs discussion on the target positioning accuracy of the UEs in the RRC_Inactive state, especially for the UE-assisted positioning. It is necessary to identify whether NR positioning is able to support the same positioning accuracy as the RRC_Connected or not. Also, an investigation or analysis is necessary to find the reporting overhead to report the minimum measurement information to achieve the target positioning accuracy. In addition, the discussion may include identifying available SDT overhead to report positioning measurements.

Proposal 1: RAN1 needs discussion on the target positioning accuracy for both UE-based and UE-assisted positioning of the UEs in the RRC_Inactive state but the final answer may also involve RAN4. For UE-assisted based positioning, clarify the available reporting overhead by SDT and the required reporting overhead to achieve the target performance.

For the 2nd priority objectives for UL/UL+DL positioning in RRC_Inactive, the necessity needs to be first discussed considering the LS (R2-2106551) sent from RAN2. If it is necessary, the support of SRS transmission of RRC_Inactive needs detailed discussions such as configuration method ​and triggering mechanism of SRS transmission. Furthermore, it may involve configuration and update timing advance to transmit SRS resource(s) and also needs to consider power control issue.

The idea of the on-demand PRS concept is to allow UEs to express their PRS configuration preference or even request specific PRS resources from the LMF. The LMF may consider UE feedback and grant such requests as well as change PRS configuration more dynamically based on its own criteria, e.g., to reflect the presence of a new UE whose positioning session requires an update of the PRS configuration.

However, UEs in RRC_INACTIVE are currently not aware of changes in PRS configuration, i.e., their assistance data (AD) ages. More specifically, UEs in RRC_INACTIVE use the AD obtained before going into the INACTIVE state and continue using them for positioning procedures and measurements as long as they remain in the INACTIVE state.

Similarly, UE mobility induces AD aging due to new conditions arising in the UE’s environment, e.g., surroundings blocking line-of-sight to the configured TRPs and yielding lower positioning accuracy, thus requiring updates on the TRP list and/or their PRS configurations, hence AD.

Observation 1: UEs in RRC_INACTIVE state relying on assistance data obtained while in RRC_CONNECTED state without updates to the assistance data and/or mobility conditions can result in the aging of assistance data provided to UEs.

A gNB has the following options to update AD in RRC_INACTIVE UEs:

- Via broadcast of posSIB in any RRC state: Broadcast was designed for the delivery of static AD whereby each gNB advertises not only it’s own PRS configuration but also those of neighboring cells. Each of the cells broadcasting the AD must therefore update their SI if there is a PRS configuration change in any neighboring cell. From the UE perspective, relatively long periods of posSIB broadcast may imply a slow AD refresh rate. Any consequent performance degradation such as temporarily lower accuracy is problematic in case of safety-critical and/or high-speed scenarios.

- Via LPP unicast in RRC_CONNECTED: The network can bring the UEs into RRC_CONNECTED state to deliver AD via standard LPP unicast procedure. However, this option negates the primary objective of the RRC_INACTIVE state, that is, the conservation of power. The cumulative energy expenditure due to frequent RRC state transitions over longer time periods may be non-negligible for battery-powered devices.

- Via SDT in RRC_INACTIVE: In accordance with [2], SDT could be exploited to deliver LPP messages even to RRC_INACTIVE UEs. However, the SDT mechanism is being studied for UL usage only in Release 17, which implies that updates on AD can be delivered via SDT only for UE-triggered requests. In other words, the network itself cannot reach UEs (LMF-triggered updates on AD) in RRC_INACTIVE as DL SDT will not be specified in Release 17. In this context, it is also noteworthy that the LMF is not aware of the UE RRC state, as well as that the gNBs cannot monitor whether RRC_INACTIVE UEs have an active positioning session and/or are in need of an AD update.

In summary, to update AD of RRC_INACTIVE UEs, network can either: i) broadcast posSIB; ii) force UEs transition to RRC_CONNECTED to re-enable LPP unicast; or iii) rely on LPP delivery opportunities created by UE-originated UL SDT. However, all these approaches have drawbacks, where the process of updating AD in RRC_INACTIVE UEs is characterized by a trade-off between timeliness, overhead, and power consumption.

In general, it is also observed in this context that the LMF does not necessarily need to provide the same AD to all UEs in a given area (e.g., via posSIB broadcasts). It has the possibility to also consider the delivery of UE-specific AD to enable localization with an individual context or optimization, which would be enabled via the on-demand PRS mechanism. This is especially true in FR2 scenarios where selective configuration and optimization of PRS can be carried out in the beam domain, specifically for each UE. Such configuration updates would be especially problematic for UEs in RRC_INACTIVE.

Observation 2: Current mechanisms to update AD for positioning are mainly designed for static scenarios and RRC_CONNECTED UEs in mind, thus can fail in scenarios involving RRC_INACTIVE UEs and dynamic condition changes due to on-demand PRS and mobility.

In addition, the overhead of updating the AD should be considered as the UE power consumption is targeted to be mitigated. One potential way to reduce the overhead of updating the AD is to consider a partial update of the assistance data. For example, the UE may received AD for a new TRP only and the rest of the AD is left unchanged.

Proposal 2: RAN1 to study partial updates of PRS AD for UEs in RRC_INACTIVE mode to reduce overhead and power consumption.

In conclusion, update of AD for UEs in RRC_INACTIVE state is one of major challenging issues to effectively support positioning for RRC_INACTIVE. This issue is discussed in more details in our companion contribution [7].

## On-demand PRS

The concept of on-demand PRS refers to the ability of the network to dynamically configure PRS resource when and where needed, which is in contrast to LTE PRS configuration where the resource allocation to PRS is static. On-demand PRS can either be initiated by the LMF or the UE, cases which are referred to as LMF-initiated and UE-initiated on-demand PRS, respectively.

The benefits of on-demand PRS are primarily related to enhanced network efficiency owing to the targeted PRS transmissions and thus the minimization of unused PRS resources. In addition, on-demand PRS promises latency reduction compared to legacy PRS schemes, which stems from a) the transmission of PRS right upon request instead of fixed periodic fashion; b) the minimization of latency caused by beam-sweeping across all beam directions, since with on-demand PRS only a limited set of beams is sweeped and thus the waiting time between two consequetive PRS transmissions from the same beam is minimized. Moreover, on-demand PRS offers enhanced accuracy since it allows the network to dynamically increase certain PRS configuration parameters which lead to an accuracy enhancement upon request.

RAN1 started discussion at RAN1 #105e for both LMF-initiated and UE-initiated on-demand PRS request and reached the following agreement.

Agreement:

- For potential signaling of one or more parameters (such signaling is not yet agreed) for both UE- and LMF- initiated on-demand DL PRS request, consider at least the following (all parameters will not necessarily be supported and all parameters may not be applicable to both UE-initiated and LMF-initiated on-demand DL PRS request)

  - Start/end time of DL PRS transmission

  - DL PRS resource bandwidth

  - DL-PRS resource set IDs

  - DL PRS resource IDs

  - DL PRS transmission periodicity and offset

  - DL PRS resource repetition factor

  - Number of DL PRS symbols per DL PRS resource

  - DL PRS muting patterns

  - DL PRS QCL information

  - Number of TRPs

  - Number of PRS resources per PRS resource set

  - Number frequency layers or frequency layer indicator

  - Beam directions

  - Combsize, start PRB, Point A of DL PRS

  - ON/OFF indicator of the DL PRS

- FFS additional parameters indicated for UE and/or LMF initiated on-demand DL PRS request

In the LS (R2-2106607), RAN2 requested RAN1 to provide the list of parameters for on-demand PRS considering the following RAN2 agreement.

- The network can signal predefined PRS configurations to the UE and the UE can select one to request. FFS if the UE can request a configuration with different parameters and exactly which parameters are flexible. - Define a new LPP assistance data IE which can contain a set of possible on-demand DL-PRS configurations, where each on-demand DL-PRS configuration has an associated identifier. - This new LPP assistance data IE can be included in an LPP Provide Assistance Data message and/or a new posSIB. - The procedure(s) for on-demand DL-PRS should support at least the following functionality (up to RAN3 what is in NRPPa vs. OAM, etc.): - Providing the requested on-demand DL-PRS configuration information from an LMF to the gNB (e.g., explicit parameter or identifier of a predefined DL-PRS configuration), and confirmation of the request by the gNB - Provision of (possible/allowed) on-demand DL-PRS configurations that the gNB can support from a gNB to an LMF - TRP capability transfer (e.g., whether the RAN node supports the reconfiguration of DL-PRS, etc.)

The listed parameters in the previous RAN1 agreement above are not concretely agreed parameters but potential candidates to be discussed further. Thus, we need to start discussion to make a set of lists without excluding parameters that were not on the current list, but it is necessary to narrow down.

The ensuing two subsections elaborate on such PRS configuration parameters that we are considering. The cases of LMF-initiated on-demand PRS (where PRS configuration parameters are reported by UE to LMF) and UE-initiated on-demand PRS (where PRS configuration parameters are requested by the UE to LMF) are treated separately.

## Reported Parameters UE to LMF

In the LMF-initiated on-demand PRS case, the LMF requests feedback from the UE regarding the quality of reference signals that the UE can detect. This feedback is utilized by the LMF to request concrete PRS configurations from the respective gNBs. Details on the procedure associated with such UE feedback and subsequent request for PRS configuration are given in [7].

The parameters contained in such UE feedback report should relate to required information that the LMF can process to determine a) which direction the PRS transmissions should be transmitted per gNB; b) the resources allocated to such PRS transmissions. As such, the reported parameters from UE to LMF may include at least the following: L1-RSRP and/or L1-SINR reports; resource and/or resource set indices associated with DL Reference Signals (RS), e.g., Synchronization Signal Blocks (SSB) reports; PRS quality measurement reports. The reported parameters should be beam specific, that is including the DL Tx beam index information. Additionally, the reported parameters may include beam failure indication that assists LMF on which beam is currently detected at UE and thus should convey PRS.

Proposal 3: UE to LMF reported parameters include beam-specific measurement reports that assist the LMF determine and request certain PRS resources to the gNB.

For on-demand PRS, UE does not necessarily have to support new measurements and we do not envision RAN1 to define new measurements. Existing SSB based RSRP and PRS based measurements should suffice. However, there may be new configurations of existing measurements required (for example, different measurement thresholds configured at the UE) such that the UE feedback covers a sufficiently large number of neighbouring TRPs and PRS resources – beams – per TRP. It is noted that such new configurations should not be confused with L3 reporting measurement configurations, since for positioning purposes the number of reported reference signals by neighboring gNbs is considerably larger than in the case of mobility.

## Requested PRS Parameters

In the UE-initiated on-demand PRS case, the UE requests to the network PRS parameters associated to a target positioning performance. The request is initiated by the UE to the LMF. A typical example is the case of UE-based positioning where the UE identifies during an ongoing positioning session that the current allocated PRS bandwidth (BW) is not sufficient for the UE to meet certain accuracy requirements, hence the UE requests for additional bandwith to be allocated to PRS resources. Similarly, in a UE-based positioning session the UE may request to LMF for higher number of consecutive positioning subframes NPRS, so as to ensure enhanced measurement precision. In the scenario of periodic location updates, the UE may request for higher PRS periodicity (e.g., more frequent positioning occasions), to account for a given location update rate that meets its requirements. In addition, request of beam direction and/or QCL source information should be supported by on-demand request. In the current system, the gNB may not be able to know the beam direction corresponding to the LoS direction since the UE reports signal power measurement such as RSRP for the DL RS (e.g., SSB, CSI-RS) to the gNB. Thus, QCL configuration based on SSB is difficult to guarantee so that the UE can use the reception beam toward LoS direction. However, since the UE can continuously estimate propagation time of PRS resources and/or other DL RS resource transmitted from each TRP, the UE probably knows information on the beam direction close to the LoS better than the network. Thus, the UE's request on the beam direction can helpful in order to use the beam close to the LoS direction. In addition, the UE is able to find the TRPs providing LoS path by LoS/NLoS detection algorithm so UE’s on-demand request on the preferable TRP is also necessary.

Proposal 4: Requested PRS parameters include preferable TRP information, beam directions, DL PRS QCL information, PRS resource ID, PRS resource set ID, PRS bandwidth, number of consecutive subframes within a positioning occasion and PRS periodicity (periodicity of positioning occasions) for both UE-initiated and LMF-initiated.

Such requested PRS parameters are also related to the LMF request to the gNB for configuring certain PRS resources, and are associated to both UE-initiated and LMF-initiated on-demand PRS. That is, the request for certain PRS BW, NPRS or PRS periodicity can be both UE to LMF (case of UE-initiated on-demand PRS) and LMF to gNB (case of LMF-initiated and UE-initated on-demand PRS). For some parameters such as start/end time of DL PRS transmission and DL PRS muting patterns, study and discussion needs to clarify if UE-initiated on-demand is feasible and effective.

## Expected AoD

RAN1 #104bis has discussed signaling an indication of expected AoD/ZoD value and uncertainty (of the expected AoD/ZoD value) range(s) the LMF to the gNBs/TRPs. This information can be useful for gNBs/TRPs to determine more accurate Tx beamforming and also potentially reduce beam sweep overhead in on-demand PRS framework. As shown in Figure 1, a TRP originally needs to perform beam sweeping towards 360º in 6 PRS occasions. Hence, the TRP has to apply relative wide beam to guarantee the coverage. After receiving expected AoD from LMF, the TRP is able to finetune the Tx beamforming from wide beam to narrow beam, transmitted still in 6 PRS occasions. Such operation can improve the positioning accuracy especially for DL-AoD method and it can help to reduce the network overhead further when applying on-demand PRS.

Figure 1: Tx beamforming update according to expected AoD

Proposal 5: Support of indication of expected AoD/ZoD value and uncertainty (of the expected AoD/ZoD value) range(s) is signaled by the LMF to gNBs/TRPs at least for LMF-initiated on-demand PRS.

# Conclusion

We made the following proposals in this paper:

Proposal 1: RAN1 needs discussion on the target positioning accuracy for both UE-based and UE-assisted positioning of the UEs in the RRC_Inactive state but the final answer may also involve RAN4. For UE-assisted based positioning, clarify the available reporting overhead by SDT and the required reporting overhead to achieve the target performance.

Proposal 2: RAN1 to study partial updates of PRS AD for UEs in RRC_ INACTIVE mode to reduce overhead and power consumption.

Proposal 3: UE to LMF reported parameters include beam-specific measurement reports that assist the LMF determine and request certain PRS resources to the gNB.

Proposal 4: Requested PRS parameters include preferable TRP information, beam directions, DL PRS QCL information, PRS resource ID, PRS resource set ID, PRS bandwidth, number of consecutive subframes within a positioning occasion and PRS periodicity (periodicity of positioning occasions) for both UE-initiated and LMF-initiated.

Proposal 5: Support of indication of expected AoD/ZoD value and uncertainty (of the expected AoD/ZoD value) range(s) is signaled by the LMF to gNBs/TRPs at least for LMF-initiated on-demand PRS.

# References

[1] RP-210903, Revised WID on NR Positioning Enhancements, CATT, Intel Corporation, Ericsson.

[2] R1-2107057, Views on on mitigating UE and gNB Rx/Tx timing errors, Nokia, Nokia Shanghai Bell.

[3] R1-2107058, Views on enhancing UL AoA, Nokia, Nokia Shanghai Bell.

[4] R1-2107059, Views on enhancing DL AoD, Nokia, Nokia Shanghai Bell.

[5] R1-2107060, Views on PHY Latency Reductions, Nokia, Nokia Shanghai Bell.

[6] R1-2107061, Views on LoS/NLoS Identification and Mitigation, Nokia, Nokia Shanghai Bell.

[7] R2-2108703 Considerations on positioning RRC_INACTIVE, Nokia, RAN2 #115, August 2021