3GPP TSG RAN WG1 #106-e	R1-2107268

e-Meeting, August 16th – 27th, 2021

Source:	OPPO

Title:	Discussion on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH

Agenda Item:	7.2.5

Document for:	Discussion and Decision

Introduction

In this contribution, we further discuss the remaining issues on SPS enhancements, focusing on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH reception.

Discussion

Collision handling and HARQ-ACK bit generation for overlapping SPS PDSCHs and SPS release were widely discussed in previous RAN1 meetings and according to the achieved agreements, there was consensus for the following cases:

- In a slot with more than one SPS PDSCHs each without a corresponding PDCCH, UE receives and decodes one or more SPS PDSCHs according to the procedure cited below (the second agreement), and UE only generate HARQ-ACK bit for the SPS PDSCH(s) which UE receives.

- Support the case that in a slot SPS release PDCCH is received before the end of the SPS PDSCH reception for the same SPS configuration corresponding to the SPS release PDCCH and the HARQ-ACK information for the SPS PDSCH release and the SPS PDSCH receptions need be multiplexed in a same PUCCH. In such a case, UE does not expect to receive the SPS PDSCH and 1 bit HARQ-ACK is generated for SPS release PDCCH.

Agreements: In a slot with more than one SPS PDSCHs each without a corresponding PDCCH, for Type-1 HARQ-ACK codebook and without HARQ-ACK feedback for dynamic scheduled PDSCH and/or for SPS PDSCH release in the slot, or for Type-2 HARQ-ACK codebook, HARQ-ACK feedback for a SPS PDSCH should not be included in the HARQ-ACK codebook if the SPS PDSCH would not be received among overlapping SPS PDSCHs without associated PDCCH. Agreements: In case of collision in time domain among SPS PDSCHs each without a corresponding PDCCH after excluding SPS PDSCHs overlapping semi-static UL symbols, - A UE receives and decodes one or more of SPS PDSCHs within a group of overlapping SPS PDSCHs on the same serving cell according to the following procedure. - Step 0: set j=0-number of selected PDSCH for decoding. Set Q to set of activated SPS PDSCHs within a slot - Step 1: A UE receives and decodes one of SPS PDSCHs with the lowest SPS configuration index within Q, set j=j+1. Designate the received SPS PDSCH as survivor SPS PDSCH. - Step 2: The survivor SPS PDSCH in step 1 and any other SPS PDSCH(s) overlapping (even partially) with the survivor SPS PDSCH in step 1 are excluded from Q. - Step 3: Repeat step 1 and 2 until the group is empty or j≥N, where N is the number of unicast PDSCHs in a slot supported by the UE Agreement - At least, support the case that in a slot SPS release PDCCH is received before the end of the SPS PDSCH reception for the same SPS configuration corresponding to the SPS release PDCCH - 1 bit HARQ-ACK is generated for SPS release and a UE does not expect to receive the SPS PDSCH if HARQ-ACKs for the SPS release and the SPS reception would map to the same PUCCH. - FFS whether and how to support the HARQ-ACK for the SPS release and the SPS reception mapping to different PUCCHs - FFS whether and how to support the case that SPS release PDCCH is received after the end of the SPS PDSCH for the same SPS configuration

However, there is a case missing, as illustrated in Figure 1, according to the pseudo-code for generation of Type-1 HARQ-ACK codebook, the HARQ-ACK information for {SPS release of configuration 2, SPS PDSCH of configuration 1, SPS PDSCH of configuration 2} would share one HARQ-ACK bit. Based on the agreements above, the HARQ-ACK information for SPS PDSCH of configuration 2 would not be generated. Then, the remaining issue is, in terms of HARQ-ACK for SPS release of configuration 2 and SPS PDSCH of configuration 1, which HARQ-ACK information should UE report for the corresponding HARQ-ACK bit in a HARQ-ACK codebook. The following alternatives can be considered:

- Alt 1: UE reports the HARQ-ACK information corresponding to the SPS release of configuration 2

  - In such a case, gNB has no idea whether SPS PDSCH of configuration 1 is correctly received, hence retransmission of SPS PDSCH of configuration 1 is needed.

- Alt 2: UE reports the HARQ-ACK information corresponding to the SPS PDSCH of configuration 1

  - In such a case, gNB has no idea whether SPS release of configuration 2 is correctly received and gNB would retransmit SPS release of configuration 2. Then, for the SPS PDSCH occasion of configuration 2 during the time gap between the two SPS release, gNB would have no idea whether UE reports the corresponding HARQ-ACK information, hence other PDSCH transmissions whose HARQ-ACK information is in the same codebook with the HARQ-ACK information corresponding to the SPS PDSCH occasion of configuration 2 are also affected.

- Alt 3: UE reports the binary AND/OR of the HARQ-ACK information corresponding to both SPS release of configuration 2 and SPS PDSCH of configuration 1

  - Take binary AND as an example, UE reports ACK only if it correctly receives both SPS release of configuration 2 and SPS PDSCH of configuration 1. In such a case, NACK report by UE would lead to some confusion to gNB, that is, gNB would not know which information is not successfully received by UE. Then, gNB would retransmit both SPS PDSCH of configuration 1 and SPS release of configuration 2.

Based on the analysis above, it can be seen that drop HARQ-ACK information of SPS release leads to serious confusion and problem not only for SPS PDSCH of configuration 2, but also for the PDSCH transmissions whose HARQ-ACK information is in the same HARQ-ACK codebook with the HARQ-ACK information corresponding to SPS PDSCH of configuration 2. So to our understanding, the HARQ-ACK information for SPS release is more important than that of SPS PDSCH. Therefore, from perspective of performance and implementation simplicity, Alt 1 is more preferred.

Figure 1: Type-1 HARQ-ACK codebook for overlapping SPS PDSCHs and SPS release

Proposal 1: If a UE is configured or indicated to receive more than one SPS PDSCHs and SPS release PDCCH whose HARQ-ACK information would be multiplexed in a same PUCCH and their HARQ-ACK information share same location in the Type-1 HARQ-ACK codebook, UE reports HARQ-ACK information for SPS release PDCCH and does not generate HARQ-ACK information for the more than one SPS PDSCHs.

Proposal 2:  Adopt the following text proposal on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH:

9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** For the set of slot timing values , the UE determines a set of occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. If a UE is configured to receive more than one SPS PDSCHs and indicated to receive SPS PDSCH release in a slot whose HARQ-ACK information would be multiplexed in a same PUCCH and their HARQ-ACK information share same location in the Type-1 HARQ-ACK codebook, UE reports HARQ-ACK information for SPS PDSCH release and does not generate HARQ-ACK information for the more than one SPS PDSCHs. *** Unchanged text is omitted ***

Conclusion

In this contribution, we further discuss the remaining issues on SPS enhancements with focus on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH reception and the following proposals are made:

Proposal 1: If a UE is configured or indicated to receive more than one SPS PDSCHs and SPS release PDCCH whose HARQ-ACK information would be multiplexed in a same PUCCH and their HARQ-ACK information share same location in the Type-1 HARQ-ACK codebook, UE reports HARQ-ACK information for SPS release PDCCH and does not generate HARQ-ACK information for the more than one SPS PDSCHs.

Proposal 2:  Adopt the following text proposal on Type-1 HARQ-ACK codebook generation for SPS release and SPS PDSCH:

9.1.2 Type-1 HARQ-ACK codebook determination *** Unchanged text is omitted *** For the set of slot timing values , the UE determines a set of occasions for candidate PDSCH receptions or SPS PDSCH releases according to the following pseudo-code. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to a single SPS PDSCH release is same as for a corresponding SPS PDSCH reception. A location in the Type-1 HARQ-ACK codebook for HARQ-ACK information corresponding to multiple SPS PDSCH releases by a single DCI format is same as for a corresponding SPS PDSCH reception with the lowest SPS configuration index among the multiple SPS PDSCH releases. If a UE is configured to receive more than one SPS PDSCHs and indicated to receive SPS PDSCH release in a slot whose HARQ-ACK information would be multiplexed in a same PUCCH and their HARQ-ACK information share same location in the Type-1 HARQ-ACK codebook, UE reports HARQ-ACK information for SPS PDSCH release and does not generate HARQ-ACK information for the more than one SPS PDSCHs. *** Unchanged text is omitted ***

Reference

- RAN1 #100-e Chairman notes

- RAN1 #101-e Chairman notes