3GPP TSG RAN WG1 #106-e                                        R1-2107473

e-Meeting, August 16th – 27th, 2021

Source: 	ETRI

Title:	Enhancements for unlicensed band URLLC/IIoT

Agenda Item:	8.3.2   Enhancements for unlicensed band URLLC/IIoT

Document for:	Discussion/Decision

Introduction

In RAN1 #105-e, the following decisions were made regarding enhancements for URLLC/IIoT operation in unlicensed bands [1]:

Agreement:

- Both “CG-UCI based procedures” and “CG-DFI based procedures” are enabled or disabled for unlicensed using one RRC parameter i.e. cg-RetransmissionTimer-r16.

- Note: Procedures based on CG-UCI rely on UE including CG-UCI in CG PUSCH at least as in Rel-16 where the values of the respective fields of CG-UCI are decided by UE.

- Note: Procedures based on CG-DFI rely on automatic re-transmission on CG configuration and reception of CG downlink feedback information (DFI) in DCI for re-transmissions

Agreement:

In semi-static channel access mode when a UE can operate as UE-initiated COT,

- To determine whether a configured UL transmission that is aligned with a UE FFP boundary and ends before the idle period of that UE FFP, is based on UE-initiated COT or sharing a gNB-initiated COT:

  - If the transmission is confined within a gNB FFP before the idle period of that gNB FFP, and the UE has already determined that gNB is initiated that gNB FFP, UE assumes that the configured UL transmission corresponds to gNB-initiated COT. Otherwise, UE assumes that the configured UL transmission corresponds to UE-initiated COT

Agreement:

In semi-static channel access mode when a UE can operate as initiating device,

- To determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT:

- Determination based on the content in the scheduling DCI

  - FFS on whether the corresponding field(s) can be absent in DCI

    - If absent, determination based on the rules applied for configured UL transmissions is applied

  - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

In this contribution, remaining issues on enhancements for unlicensed band URLLC/IIoT are discussed.

Discussion

UE FFP configuration

It was agreed that the offset value for configuration of a UE-FFP for a serving cell has a symbol level granularity. A remaining issue is the reference numerology used to determine the symbol duration. The reference SCS can be provided by gNB or can be determined based on a predetermined rule. In either case, the smallest SCS among the configured SCSs in the cell may serve as the reference SCS considering the BWP switching operation across different numerologies. If a BWP is configured with u=2 and ECP, a simple rule may be defined to align the FFP boundary to a ECP symbol boundary.

Proposal 1: The symbol offset for UE FFP configuration can be determined based on the smallest SCS among the configured SCSs in a cell.

COT-initiator determination for scheduled UL transmissions

Agreement: (@RAN1#105-e) In semi-static channel access mode when a UE can operate as initiating device, - To determine whether a scheduled UL transmission is based on UE-initiated COT or sharing a gNB-initiated COT: - Determination based on the content in the scheduling DCI - FFS on whether the corresponding field(s) can be absent in DCI - If absent, determination based on the rules applied for configured UL transmissions is applied - FFS whether/how to handle the case when the gNB schedules an UL transmission in the next gNB’s FFP period

There are two remaining issues with COT-initiator determination for scheduled UL. One is for COT initiator indication whether a new field is introduced or the existing channel access field is reused, and the other is whether the corresponding field(s) can be absent in DCI. Another related question is necessity of adding channel access fields in Rel-17 DCI 0_2 and 1_2. In our view, it is natural for DCI 0_2/1_2 to have the channel access fields to fully install Rel-16 URLLC functionalities to Rel-17 unlicensed operation for FBE and LBE (although LBE may be less motivated for URLLC/IIoT).

Proposal 2: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

Regarding the above two issues, our views are provided in the following with yellow highlights based on a question list provided by the FL at the end of the last meeting [2]:

- COT-initiator indicated in scheduling DCI (0_0/1_0, 0_1/1_1, 0_2/1_2) for UL transmission. - How to indicate COT-initiator in scheduling DCI (0_0/1_0, 0_1/1_1, 0_2/1_2)? - Using LBT fields in DCI - Indicaiton always present - Indication can be configured to be absent - Using other methods, including others existing fields than LBT fields or new fields - Indicaiton always present - Indication can be configured to be absent - Can the LBT fields in Rel-16 DCI 0_1/1_1 to be included in Rel-17 DCI 0_2/1_2 for FBE? - Yes - Same field size as Rel-16 (always 2 bits) - Configurable field size as compared to Rel-16, e.g. 0 or 2 bits - No - Can the LBT fields in Rel-16 DCI 0_1/1_1 to be included in Rel-17 DCI 0_2/1_2 for LBE? - Yes - Same configurable field size as Rel-16 - Reduce configurable field size as compared to Rel-16, e.g. max 2-bits. - No

Proposal 3: COT initiator indication for scheduled UL is based on existing channel access fields in scheduling DCI, which can be configured to be absent (i.e., 0 bit) in Rel-17 DCI 0_2/1_2 for FBE.

Cross-FFP scheduling

Agreement: (@RAN1#104b-e) - In semi-static channel access mode, the gNB can schedule by a DCI UL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The UL transmission can occur only if the corresponding channel access requirements are met. - FFS on details. Agreement: (@RAN1#104b-e) - In semi-static channel access mode, the gNB can schedule by a DCI DL transmission(s) in a later g-FFP that is different from the g-FFP that carries the scheduling DCI. - The DL transmission can occur only if the corresponding channel access requirements are met. - FFS on details.

In RAN1#104b-e, it was clarified that cross-FFP scheduling is allowed for DL and UL transmissions in FBE. A remaining issue is how to define channel access requirements for the UL cross-FFP scheduling case. In our view, the requirement is already stated clearly in TS 37.213 as:

- A UE may transmit UL transmission burst(s) after detection of a DL transmission burst(s) within the channel occupancy time.

Basically, no other channel access requirement is needed. However, there is one issue to be resolved, which is illustrated in Fig. 1. For self-FFP scheduling, the UL grant ‘grants an authorization’ for the PUSCH transmission. However, in case of cross-FFP scheduling, the UL grant cannot grant the PUSCH from channel access point of view because the UL grant is not within the same FFP. Therefore, as shown in Fig. 1, UE should receive another DL signal (i.e., DL transmission burst) within the same FFP to be granted for PUSCH transmission. The same argument may hold for CG-PUSCH and all other configured UL transmissions which do not have a UL grant.

Observation 1: For scheduled UL based on cross-FFP scheduling or for configured UL, UE should receive a DL signal other than a UL grant to be granted for PUSCH transmission within the same FFP.

Fig. 1. Self-FFP scheduling vs. cross-FFP scheduling

On the other hand, the DL signal detection for UL authorization necessitates some processing time to UE. If not defined in the spec, the processing time for the DL detection would be different among UEs depending on their implementation or capability. Then, gNB may not know the exact timing from which UE is ready to transmit UL in a shared gNB COT. This issue is illustrated in Fig. 2.

Fig. 2. Processing time for UE’s DL signal detection for UL authorization

In Fig. 2, UE A may be capable to decode the DL signal in a shared COT until before the starting position of the CG-PUSCHs. Thus, UE A can transmit the CG-PUSCHs. However, UE B may not able to finish the DL detection at the starting position of the 1st CG-PUSCH. Then, UE B may not transmit the 1st CG-PUSCH. Depending on cases, UE B may drop the whole CG-PUSCH transmission including possible repetitions, which may severely degrade the UL delay performance for a TB (and also for piggybacked UCI). In Rel-16 NR-U, the delay due to UL dropping may not be critical because eMBB is the main use case. However, such a relaxed delay requirement is not applicable for high end URLLC applications. Furthermore, the gNB may need to perform blind detection for UL reception because it does not know whether the UL is actually transmitted or not, especially if the UL is allocated in the earlier part of the gNB COT.

Therefore, in order to avoid the UL scheduling/transmission uncertainty, it is necessary to define a UE processing time for detection of the DL signal granting UL authorization (and UL preparation) in a shared COT. Such enhancement is considered essential because it may highly impact the UL reliability performance for unlicensed band URLLC/IIoT.

Observation 2: The UL reliability performance of unlicensed URLLC can be severely degraded if UE’s processing time for DL detection to share a COT is unknown to gNB.

Proposal 4: For gNB-to-UE COT sharing, define a UE processing time for detection of the DL signal granting UL authorization (and UL preparation).

A similar issue is identified in the UE-to-gNB COT sharing. Since gNB should spend some time for UL burst detection, gNB can realize whether sharing a UE COT is possible or not after some delay from the UL reception. If UE can know the gNB’s processing time for UL detection, UE may skip the DL reception operation (e.g., micro-sleep) during that time for power saving purpose.

Proposal 5: For UE-to-gNB COT sharing, consider defining processing time for gNB’s UL burst detection for UE power saving purpose.

Fig. 3. Processing time for gNB’s UL detection

PUSCH repetition type B

Agreement: (@RAN1#104b-e) - Select one of the following options (aiming for RAN1#105-e): - Option 1: Do not support PUSCH repetition Type Bwhen using based on NR-U Rel-16 based CG for unlicensed band operation. - Option 2: Support enhancements of PUSCH repetition Type B when using based on NR-U Rel-16based CG for unlicensed band operation. FFS whether/how to enhance Agreements: (@RAN1#104b-e) - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether PUSCH segmentation should take into account the idle period of an FFP. - FFS on details Agreements: (@RAN1#104b-e) - For PUSCH repetition Type B enhancements on unlicensed spectrum, further study whether orphan symbol(s) are transmitted if they are between two actual repetitions that are transmitted. FFS on details

PUSCH repetition type B was introduced to meet tight UL latency and reliability requirements of URLLC applications. It is obvious that type B repetition is also essential in unlicensed URLLC, and there seems no technical problem to support it at least for FBE.

For PUSCH repetition type B in FBE, a nominal PUSCH repetition may include symbol(s) overlapping with the idle period, i.e., idle symbol(s), that belongs to an associated FFP. In that case, rather than being dropped, the nominal PUSCH repetition can be segmented into one or more actual PUSCH(s) by the idle symbol(s). That is, the idle symbol(s) can be regarded as invalid symbol for PUSCH repetition type B.

There was a proposal to transmit orphan symbol(s) if they are between two actual repetitions that are transmitted. A main purpose seems to guarantee contiguous PUSCH transmissions. However, from our understanding, UL transmissions need not necessarily be contiguous in FBE. If not contiguous, UE can just do short LBT before every bunch of UL transmission. Thus special handling of orphan symbol(s) for FBE seems not necessary.

Proposal 6: For FBE, a symbol overlapping with idle period of a FFP associated to PUSCH transmission is regarded as invalid symbol for PUSCH mapping type B.

Observation 3: It seems that no special handling of orphan symbol(s) for PUSCH repetition type B in FBE is needed.

Conclusion

In this contribution, remaining issues on enhancements for unlicensed band URLLC/IIoT are discussed. The following observations and proposal are made:

Proposal 1: The symbol offset for UE FFP configuration can be determined based on the smallest SCS among the configured SCSs in a cell.

Proposal 2: Support the channel access fields in Rel-16 DCI 0_1 and 1_1 to be included in Rel-17 DCI 0_2 and 1_2, respectively.

Proposal 3: COT initiator indication for scheduled UL is based on existing channel access fields in scheduling DCI, which can be configured to be absent (i.e., 0 bit) in Rel-17 DCI 0_2/1_2 for FBE.

Observation 1: For scheduled UL based on cross-FFP scheduling or for configured UL, UE should receive a DL signal other than a UL grant to be granted for PUSCH transmission within the same FFP.

Observation 2: The UL reliability performance of unlicensed URLLC can be severely degraded if UE’s processing time for DL detection to share a COT is unknown to gNB.

Proposal 4: For gNB-to-UE COT sharing, define a UE processing time for detection of the DL signal granting UL authorization (and UL preparation).

Proposal 5: For UE-to-gNB COT sharing, consider defining processing time for gNB’s UL burst detection for UE power saving purpose.

Proposal 6: For FBE, a symbol overlapping with idle period of a FFP associated to PUSCH transmission is regarded as invalid symbol for PUSCH mapping type B.

Observation 3: It seems that no special handling of orphan symbol(s) for PUSCH repetition type B in FBE is needed.

Reference

[1] Chair's Notes RAN1#105-e.

[2] R1-2106048, “Summary#5 - Enhancements for IIOT/URLLC on Unlicensed Band,” Moderator (Ericsson), RAN1#105-e.