3GPP TSG RAN WG1 #106-e			                                                                    R1-2107581

e-Meeting, August 16th – 27th, 2021

Source:	Intel Corporation

Title:	Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz

Agenda item:	8.2.5

Document for:	Discussion/Decision

# Introduction

At the RAN1#105-e meeting, the following agreements and conclusions were made for PDSCH and PUSCH enhancement (related to multi-TTI scheduling) for extending NR to up to 71GHz [1].

Agreement: - Do not use fallback DCI (i.e., DCI formats 0_0 and 1_0) for multi-PDSCH/PUSCH scheduling. - Use DCI format 0_1 to schedule multiple PUSCHs with a single DCI. - Use DCI format 1_1 to schedule multiple PDSCHs with a single DCI. Conclusion: For a DCI that can schedule multiple PUSCHs, - CSI-request: When the DCI schedules M PUSCHs, the PUSCH that carries the aperiodic CSI feedback is M-th scheduled PUSCH for M <= 2, or (M-1)-th scheduled PUSCH for M > 2. Agreement: - If a PDSCH among multiple PDSCHs that are scheduled by a single DCI is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not receive the PDSCH. - FFS on how to handle HARQ-related issue for the PDSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PDSCHs by a single DCI, where every PDSCH is collided with uplink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. - If a PUSCH among multiple PUSCHs that are scheduled by a single DCI is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated, the UE does not transmit the PUSCH. - FFS on how to handle HARQ-related issue for the PUSCH (e.g., HARQ process numbering) - The UE does not expect to be scheduled with multiple PUSCHs by a single DCI, where every PUSCH is collided with downlink symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated. Agreement: For TDRA in a DCI that can schedule multiple PDSCHs (or PUSCHs), - A row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots. - FFS: The maximum value of the gap between two consecutively scheduled PDSCHs or between two consecutively scheduled PUSCHs - FFS: The maximum value of the gap between the first scheduled PDSCH and the last scheduled PDSCH or between the first scheduled PUSCH and the last scheduled PUSCH - FFS: Details to introduce the gap between PDSCHs or between PUSCHs Agreement: For enhancements of generating type-1 HARQ-ACK codebook corresponding to DCI that can schedule multiple PDSCHs, the set of candidate PDSCH reception occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. - The set of DL slots includes all the unique DL slots that can be scheduled by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - The set of SLIVs corresponding to a DL slot (belonging to the set of DL slots) at least include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing. - FFS: details of further pruning of the set of SLIVs - FFS: impact if receiving more than one PDSCH in a slot is allowed, e.g., handling of overlapped SLIVs from different rows in the same and different DL slot - FFS impact of time domain bundling, if supported Agreement: - At least for 120 kHz SCS, for a DCI that can schedule multiple PUSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, - If CBG-based (re)transmission is configured, CBGTI field is not present when more than one PUSCHs are scheduled, but is present when a single PUSCH is scheduled, as in Rel-16. - FFS: - For 480/960 kHz SCS, whether to apply the same behavior with 120 kHz SCS or not to support CBGTI field configuration in the DCI that can schedule multiple PUSCHs - For a DCI that can schedule multiple PDSCHs and is configured with the TDRA table containing at least one row with multiple SLIVs, whether/how to configure CBGTI/CBGFI fields Agreement: If Alt 1 (C-DAI/T-DAI is counted per DCI) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - At least two sub-codebooks are generated for a PUCCH cell group where - The first sub-codebook is for the following cases: - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing rows each with a single SLIV - Any DCI that is not configured with CBG-based scheduling and is configured with TDRA table containing at least one row with multiple SLIVs and schedules only a single PDSCH - The second sub-codebook is for the following case: - Any DCI that is configured with TDRA table containing at least one row with multiple SLIVs and schedules multiple PDSCHs - FFS: Methods (if needed) to align the size of HARQ-ACK feedback corresponding to different DCIs - FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases - FFS: SPS PDSCH release, SCell dormancy indication without scheduled PDSCH - FFS: 2 or 3 sub-codebooks if CBG is configured for a serving cell in the PUCCH cell group - FFS: impact of time domain bundling, if supported, e.g., the number of sub-codebooks including single codebook if all A/N bits are bundled into a single bit per DCI Agreement: If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks - FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

In this contribution, we present our view on PDSCH and PUSCH enhancements, with primary focus on multi-PUSCH and multi-PDSCH scheduling, HARQ-ACK feedback and potential enhancements to PTRS and DMRS.

- Multi-PDSCH/PUSCH scheduling

## Number of PDSCHs/PUSCHs

At the RAN1#104b-e meeting, it was agreed that maximum number of PDSCHs that can be scheduled with a single DCI in Rel-17 is 8 for SCS of 480 and 960 kHz. In addition, maximum number of PUSCHs that can be scheduled with a single DCI in Rel-17 is 8 [2].

Given that a single DCI can schedule up to 8 PUSCHs for different SCSs, including 120/480/960 kHz SCS, it is straightforward to apply similar concept for multi-PDSCH scheduling for system operating between 52.6GHz and 71GHz carrier frequency. In our view, it is not desirable to differentiate the feature on the support of multi-PDSCH/PUSCH based on the subcarrier spacing. In this regard, maximum number of PDSCHs that can be scheduled with a single DCI is 8 for 120 kHz SCS, similar to 480/960 kHz.

Proposal 1

- Maximum number of PDSCHs that can be scheduled with a single DCI is 8 for 120 kHz SCS.

Further, as defined in NR, multiple PDSCHs or PUSCHs can be multiplexed in a TDM manner within a slot for SCS of 120kHz. As mentioned above, for system operating between 52.6GHz and 71GHz carrier frequency, it is not appropriate to differentiate the functionality of multi-PDSCH/PUSCH based on the subcarrier spacing. Hence, in our view, for all the supported SCSs including 120/480/960 kHz for NR 52.6-71 GHz, UE can be scheduled with more than one PDSCHs/PUSCHs in a slot for multi-PDSCH/PUSCH scheduling. In this case, more than one SLIVs per slot in a row in TDRA table for multi-PDSCH/PUSCH scheduling are supported.

Proposal 2

- For NR 52.6-71 GHz, UE can be scheduled with more than one PDSCHs/PUSCHs in a slot for multi-PDSCH/PUSCH scheduling for 120/480/960 kHz SCS.

  - More than one SLIVs per slot in a row in TDRA table for multi-PDSCH/PUSCH scheduling are supported.

## CBG based transmission

For system operating between 52.6GHz and 71GHz carrier frequency, when a large subcarrier spacing, e.g., 480kHz or 960kHz is used, symbol and slot duration is very short, which may pose certain constraint for scheduler implementation. To alleviate scheduler constraint and relax higher layer processing burden, it is more desirable to consider a prolonged scheduling unit. More specifically, multi-TTI based scheduling can be employed, where one PDCCH can be used to schedule multiple PDSCHs or PUSCH carrying independent TBs. Based on this mechanism, scheduler implementation and higher layer processing burdened can be relaxed, while maintaining same peak data rate.

For this range of carrier frequency, it is envisioned relatively large transport block size can be supported even in case when more than one PDSCHs or PUSCHs are scheduled, which can help achieve much higher data rate. In this regard, CBG based operation for multi-PDSCH/PUSCH scheduling is beneficial in term of improving overall system level spectrum efficiency, given the fact that unused resources which are released from the correctly received data can be allocated for other DL or UL transmission.

At the RAN1#105-e meeting, it was agreed that for 120kHz subcarrier spacing, for multi-PUSCH scheduling, if CBG based transmission is configured, CBGTI field is present when single PUSCH is scheduled and is not present when more than one PUSCHs are scheduled [1]. In our view, same mechanism can be reused for multi-PDSCH scheduling, e.g., CBG based transmission is supported for multi-PDSCH scheduling when a single PDSCH is scheduled. Further, this can also be extended to the case when 480/960kHz subcarrier spacing is used for data transmission.

Based on the discussions above, for multi-PDSCH/PUSCH scheduling, CBG based transmission is supported for 120/480/960kHz subcarrier spacing when a single PDSCH/PUSCH is scheduled.

Proposal 3

- For multi-PDSCH/PUSCH scheduling, CBG based transmission is supported for 120/480/960kHz subcarrier spacing when a single PDSCH/PUSCH is scheduled.

## TDRA for multi-PDSCH/PUSCH scheduling

At the RAN1#105-e meeting, it was agreed that for multi-PDSCH/PUSCH scheduling, a row of the TDRA table can indicate PDSCHs (or PUSCHs) that are in consecutive or non-consecutive slots [1]. This mainly aims to allow multiple transmissions for different UEs and different beam directions to be multiplexed in a same COT, and hence improve the system level spectrum efficiency. Figure 1 illustrates one example of non-consecutive slots for multi-PDSCH scheduling.

Figure 1. Non-consecutive slots for multi-PDSCH scheduling

To enable non-consecutive slots for multi-PDSCH/PUSCH scheduling, one straightforward approach is to configure separate k0/k2, mapping type, and SLIV for each scheduled PDSCH/PUSCH in each row of TDRA table, which can provide good flexibility on the resource allocation in time domain.

Proposal 4

- For multi-PDSCH/PUSCH scheduling, separate k0/k2, mapping type and SLIV for each scheduled PDSCH/PUSCHs can be configured in each row of TDRA table.

## 2nd TB for multi-PDSCH scheduling

At the RAN1#104b-e meeting, it was agreed that non-consecutive resources in time can be allocated for different PDSCHs for multi-PDSCH scheduling. In addition, signalling of MCS, NDI, RV for the 1st TB and HARQ process number for the scheduled PDSCHs was agreed [2]. Note that for system operating between 52.6GHz and 71GHz carrier frequency, it is more desirable to support the scheduling of 2nd TB for scheduled PDSCHs in order to achieve higher data rate, which would be a critical factor to differentiate this band compared to NR operating in FR1 or FR2. This is also preferrable when comparing 5G technology with other competing solutions for this frequency band.

Hence, in our view, scheduling of 2nd TB for scheduled PDSCHs is supported for multi-PDSCH scheduling. Further, as defined in NR, separate MCS, NDI and RV can be indicated for the 2nd TB for the scheduled PDSCHs. Similar to the indication for 1st TB, when 2nd TB for the PDSCHs is scheduled, MCS for 2nd TB in the DCI is commonly applied for the 2nd TB of each PDSCH. For NDI of the 2nd TB, an NDI bitmap is used, which is signalled for each scheduled PDSCH. In addition, for RV of the 2nd TB, a RV bitmap is used when more than one PDSCHs are scheduled, i.e., 1 bit for each PDSCH and 2 bits when a single PDSCH is scheduled.

Proposal 5

- For multi-PDSCH scheduling

  - Scheduling of 2nd TB is supported.

  - For 2nd TB, separate MCS, NDI and RV are signaled from 1st TB.

  - For 2nd TB, similar mechanisms for signaling of MCS, NDI and RV for 1st TB are reused.

## DCI content

For system operating between 52.6GHz and 71GHz carrier frequency, given the fact that contiguous frequency domain resource allocation for uplink transmission is considered, intra-slot frequency hopping can be supported in order to exploit the benefit of frequency diversity in case of multi-PUSCH scheduling.

In addition, enhancement on FDRA, e.g., by increasing RBG size or changing allocation granularity may not be needed as relatively small number of PRBs for resource allocation can be possible for higher carrier frequency.

Proposal 6

- For multi-PUSCH scheduling,

  - Support intra-slot frequency hopping for scheduled PUSCHs.

  - Do not support enhancement on FDRA.

To reduce DCI size for multi-PDSCH scheduling, some fields in the DCI can be commonly applied for the transmission of scheduled PDSCHs. For instance, same carrier indicator, BWP indicator, frequency domain resource allocation, VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, ZP CSI-RS trigger and DMRS configuration including antenna port, DMRS sequence initialization, etc., can be applied for all the scheduled PDSCHs.

Proposal 7

- For multi-PDSCH scheduling

  - Carrier indicator, BWP indicator, frequency domain resource allocation, VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, ZP CSI-RS trigger and DMRS configuration including antenna port, DMRS sequence initialization, etc., can be applied for all the scheduled PDSCHs.

- HARQ-ACK feedback

  - Type-1 HARQ-ACK codebook

In the agreement on Type-1 HARQ-ACK codebook generation in last meeting, the set of occasions corresponding to a UL slot with HARQ-ACK transmission is determined based on a set of DL slots and a set of SLIVs corresponding to each DL slot belonging to the set of DL slots. There are two options to determine the set of SLIVs corresponding to a DL slot (belonging to the set of DL slots).

- Option 1: the set of SLIVs just include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing.

- Option 2: the set of SLIVs include all the SLIVs that are included in any row in the TDRA table.

Option 2 results in a larger size of the set of SLIVs than Option 1 and is not good in HARQ-ACK codebook size reduction. Therefore, Option 1 is preferred. Further, the impact of the TDD UL-DL configuration should be considered in the determination of the set of SLIVs for a DL slot. If the SLIV(s) of a row in TDRA table overlaps with a UL symbol in a slot that is indicated by the TDD UL-DL configuration, the SLIV(s) is considered invalid and not included in the set of SLIV of the slot.

In existing NR Type-1 HARQ-ACK codebook generation, the overlap handling and occasion allocation is performed independently in each slot, which may not explore all the means for overhead reduction in multi-PDSCH scheduling. For two possible TDRAs of multi-PDSCH scheduling, even though the SLIVs of the two TDRAs do not overlap in a slot, the two TDRAs may overlap in another slot. Consequently, the same occasion(s) can be shared by the two TDRAs. As shown in Figure 1, if the SLIVs in slot n-2 are processed independently, 2 occasions are necessary since SLIV 1-4 of the first row and SLIV 2-2 of the second row do not overlapped. However, the first row and the second row overlap in slot n-3 in multi-PDSCH scheduling. That is, gNB cannot simultaneously schedule both the first row and the second row. Therefore, it is sufficient to allocate a single occasion for slot n-2 (shared by SLIV 1-4, SLIV 2-2 and SLIV 3-1).

Figure 1: Configured SLIVs of multiple rows in TDRA table

In an extreme case, it is possible that the set of SLIVs corresponding to a DL slot is empty after considering the TDD UL-DL configuration. consequently, no occasion needs to be allocated for the slot.

Proposal 8

- For Type-1 HARQ-ACK codebook generation,

- the set of SLIVs just include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing, considering the TDD UL-DL configuration.

- to allocate the occasion(s) for a DL slot, the overlap checking is performed across the SLIVs in the multiple slots of the rows in TDRA table

  - Type-2 HARQ-ACK codebook

The discussions on Type-2 HARQ-ACK codebook focus the down-selection between Alt 1 and Alt 2. C-DAI/T-DAI is counted per DCI in Alt 1. The 2 bits for a DAI enable UE to identify the missing of up to 3 consecutive PDCCHs. On the other hand, C-DAI/T-DAI is counted per PDSCH in Alt 2. If the number of PDSCHs scheduled by a DCI can be up to 8, the size of C-DAI/T-DAI needs to be 5 bits to ensure that at most 3 consecutive missed DCIs can be resolved.

One main concern for Alt. 2 is the largest DAI overhead. Further, based on the agreement in last meeting, C-DAI size in fallback DCI is still 2 bits, which causes complexity. If a single codebook is used, the C-DAI in fallback DCI needs additional interpretation. On the other hand, if a separate sub-codebook is used to carry HARQ-ACK feedback for all fallback DCIs, it is lack of protection of the size of the sub-codebook since T-DAI field is not present in fallback DCI. Therefore, we prefer to design Type-2 HARQ-ACK codebook based on Alt 1. In fact, Alt. 1 maximizes reusing the existing design of dual HARQ-ACK sub-codebooks. If time bundling is adopted to generate 1 or 2 HARQ-ACK bits per multi-PDSCH DCI, a single HARQ-ACK codebook can be used for the HARQ-ACK transmission.

For Alt 1, there was a proposal to increase the number of sub-codebooks. For example, one sub-codebook for the use if single PDSCH is scheduled and for the case that CBG-based transmission is not configured; a second sub-codebook is for multi-PDSCH scheduling when multiple PDSCHs are scheduled; a third sub-codebook is to carry CBG-based HARQ-ACK feedback. Though such proposal may have better control on the number of HARQ-ACK bits for each DCI, it results in the difficulty to protect the size of a sub-codebook. Assuming T-DAI is only indicated for the scheduled sub-codebook, the possible wrong sizes of the other two sub-codebooks result in incorrect overall HARQ-ACK codebook size, which would introduce increased failure probability of HARQ-ACK transmission.

Since the C-DAI counts the number of PDCCH, the same number of HARQ-ACK bits is associated with each PDCCH in a sub-codebook. For the first sub-codebook, the number of HARQ-ACK bits per DCI is 2 if at least one serving cell is configured with PDSCH transmission with 2 TBs. Otherwise, the number of HARQ-ACK bits per DCI is 1. For the second sub-codebook, the number of HARQ-ACK bits per DCI has to be determined by the maximum number of configured HARQ-ACK bits per DCI. As proposed in above Proposal 3, we assume it is not supported that more than one PDSCHs with CBG based transmission are scheduled by a multi-PDSCH DCI. Therefore, denote the maximum number of TBs that can be scheduled by a multi-PDSCH DCI as M and the number of configured CBGs for a PDSCH as N, the number of HARQ-ACK bits per DCI in the second sub-codebook equals to the maximum of all configured values M and N among all the configured cells.

Regarding the following FFS point,

•	FFS: Whether HARQ-ACK bits for 2 PDSCHs scheduled by this DCI can be included in the first sub-codebook in some cases

We think it can be supported if 2 HARQ-ACK bits per DCI are reported in the first sub-codebooks. Otherwise, the number of reported HARQ-ACK bits for the DCI that schedules 2 PDSCHs or one PDSCH with 2 TBs has to be max(all values M, all values N). For example, the number of HARQ-ACK bits may be up to 16 bits (maximum 8 PDSCHs with 2 TBs per PDSCH), which leads to increased HARQ-ACK overhead of 14 bits per DCI.

A further FFS point is regarding the handling of HARQ-ACK feedback for SPS PDSCH release, SCell dormancy indication without scheduled PDSCH. Since there is no scheduled PDSCH, the function of the HARQ-ACK feedback is to confirm the detection of the DCI. Therefore, single HARQ-ACK bit is sufficient. In our view, it is straightforward to include the 1-bit HARQ-ACK in the first sub-codebook for the DCI indicating SPS PDSCH release, SCell dormancy indication without scheduled PDSCH.

Proposal 9

- Type-2 HARQ-ACK codebook is generated with Alt 1 ‘C-DAI/T-DAI counted per DCI’

- Two sub-codebooks are generated for a PUCCH cell group

    - If time bundling is configured, a single HARQ-ACK codebook may be adopted.

- If 2 HARQ-ACK bits are generated for a multi-PDSCH DCI, it is included in the first sub-codebook if 2 HARQ-ACK bits per DCI is reported in the first sub-codebooks

- Same number of HARQ-ACK bits is associated with each DCI in a sub-codebook

    - Denote the maximum number of TBs that can be scheduled by a multi-PDSCH DCI as M and the number of configured CBGs for a PDSCH as N, the number of HARQ-ACK bits per DCI in the second sub-codebook equals to the maximum of all configured values M and N among all the configured cells

- 1 HARQ-ACK bit is included in the first sub-codebook for the DCI indicating SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

  - Time domain bundling of HARQ-ACK feedback

For system operating between 52.6GHz and 71GHz carrier frequency, the slot or one PDSCH transmission is very short. For instance, slot duration for 960kHz subcarrier spacing is approximately 15.60µs. This indicates that consecutive PDSCHs may very likely experience similar channel conditions. In this case, for HARQ-ACK feedback of multi-PDSCH scheduling, it may be more desirable to consider time domain bundling of HARQ-ACK feedback, which can also help in reducing the HARQ-ACK codebook size. Since the multiple PDSCHs are scheduled by the same DCI, they are always being received or missed in same manner. Therefore, there is no error case for time domain bundling over the multiple PDSCHs that are scheduled by the same DCI. Figure 3 illustrates one example of time-domain bundling of HARQ-ACK feedback. In the figure, it is assumed bundling size of 2 slots for HARQ-ACK feedback.

Figure 3. Time-domain bundling of HARQ-ACK feedback

Time domain bundling can be easily incorporated into Type-2 HARQ-ACK codebook. Whether a single HARQ-ACK codebook or two sub-codebooks are used can be determined by the maximum number of bundled HARQ-ACK bits per multi-PDSCH DCI. For example, if the maximum number of bundled HARQ-ACK bits can be larger than 2, two sub-codebooks are used. On the other hand, if the maximum number of bundled HARQ-ACK bits per multi-PDSCH DCI is one or two, it needs further discussions whether a single HARQ-ACK codebook or two sub-codebooks are used. The same number of HARQ-ACK bits is still reported for each DCI in the single codebook or each of the two sub-codebooks. One discussion point is whether it should be included in the first or the second sub-codebook if only two bundled HARQ-ACK bits are generated for a multi-PDSCH DCI. We prefer to define the same behavior as the case when time bundling is not configured, that is, the two bundled HARQ-ACK bits can be included in the first sub-codebook if 2 HARQ-ACK bits per DCI is reported in the first sub-codebooks.

Adding time bundling to Type-1 HARQ-ACK codebook as proposed in Proposal 8 needs more discussions. Assuming X occasions are generated by Proposal 8, the adjacent occasions may be linked to non-overlap rows of TDRA with same or different K1 value. It is possible that gNB may schedule the multiple non-overlap rows simultaneously by multiple DCIs. However, if UE misses one of the multiple DCIs, UE generates the wrong bundled HARQ-ACK information. Therefore, it should be guaranteed that a bundled occasion corresponds to multiple HARQ-ACK bits that are associated with same multi-PDSCH DCI.

Proposal 10

- Time domain bundling can be supported in Type-2 HARQ-ACK codebook.

- FFS how to determine the number of sub-codebooks

- The same grouping of the two sub-codebooks by the number of bundled HARQ-ACK bits as the case that time bundling is not configured.

- Time domain bundling can be supported in Type-1 HARQ-ACK codebook.

- A bundled occasion corresponds to multiple HARQ-ACK bits that are associated with same multi-PDSCH DCI.

- PTRS Enhancements

## Potential PT-RS enhancements for CP-OFDM

Phase noise is one of the main performance limiting factors in FR2-2 comparing to the lower NR bands. Given the agreement made at RAN1#103-e [3], the complete support of system operation at SCS 120kHz is mandatory for FR2-2 devices. However, as we showed in our previous contributions [4][5], ensuring the support of all 64QAM MCSs and transmission ranks is challenging even with the advanced de-ICI filtering method proposed in [6]. High MCSs may require a combination of several receiver implementation techniques to be supported with rank 2 Tx, some MCSs may not be supported in a practical implementation at all. Therefore, introduction of a UE capability of supporting certain MCS/rank combinations is needed in FR2-2. In case appropriate UE capability to indicate lack of support for specific MCS/rank combination is not possible, RAN1 may need to investigate the possibility remove to support for several highest MCSs with rank 2. The particular MCS range subject to UE capability introduction or removal of support requires further study by RAN1.

Existing UE capability signaling, supportedModulationOrderDL and supportedModulationOrderUL, are not adequate to allow UE to indicate that it cannot support specific modulation. This is because the existing modulation order indication is only use to determine the peak data rate and not used to limit the modulation order that the gNB can use for scheduling. Therefore, RAN1 should further consider introduction of new UE capability to address the processing challenges for each subcarrier spacing. The following is the description for supportedModulationOrderDL and supportedModulationOrderUL.

| supportedModulationOrderDL | Indicates the maximum supported modulation order to be applied for downlink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for downlink. If not included: - for FR1, the network uses the modulation order signalled in pdsch-256QAM-FR1. - for FR2, the network uses the modulation order signalled per band i.e. pdsch-256QAM-FR2 if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12]. |
| --- | --- |
| supportedModulationOrderUL | Indicates the maximum supported modulation order to be applied for uplink in the carrier in the max data rate calculation as defined in 4.1.2. If included, the network may use a modulation order on this serving cell which is higher than the value indicated in this field as long as UE supports the modulation of higher value for uplink. If not included, - for FR1 and FR2, the network uses the modulation order signalled per band i.e. pusch-256QAM if signalled. If not signalled in a given band, the network shall use the modulation order 64QAM. In all the cases, it shall be ensured that the data rate does not exceed the max data rate (DataRate) and max data rate per CC (DataRateCC) according to TS 38.214 [12]. |

Proposal 11:	RAN1 to introduce UE capability of supporting high MCS/rank combinations in FR2-2. Details of the capability signal is FFS.

The current section provides the results of our internal verification of PN compensation performance potentially achievable in a realistic receiver implementation. We show the limits of performance improvement solely by means of advanced Rx processing as well as evaluate the gains achievable with some PT-RS enhancements in the spec (block PT-RS pattern, PT-RS power boosting).

### Receiver implementation based PN compensation

Several methods can be used to improve PN compensation at the receiver. In this section we focus on the ones that don’t require any specification changes and, at the same time, can be implemented at UE with a increased complexity.

#### De-ICI filtering

The frequency domain de-ICI filtering proposed in [6] significantly improves PN compensation performance comparing to CPE compensation, however at the cost of adding substantial complexity. In order to keep the computational overhead reasonable, we limit our evaluation by 3, 5 and 7-tap de-ICI filters.

Two de-ICI filter coefficients estimation strategies are considered in this paper. The first is based on direct coefficients A(I) estimation using filtering MSE minimization criteria (described in detail in Sec. 2.1 of [6]):

A(I)=argminARk0+uRk0+u-1⋯Rk0-uRk1+uRk1+u-1⋯Rk1-u⋮⋮⋱⋮RkN-1+uRkN-1+u-1⋯RkN-1-uA-uA-u+1⋮Au-Hk0Sk0Hk1Sk1⋮HkN-1SkN-12=argminARA-x2=RHR-1RHx

where R matrix’s rows are the received QAM symbols at each PT-RS tone and 2u tones around it. Hki and Ski are the corresponding PT-RS tone channel coefficient and the transmitted PT-RS symbol, 2u+1 defines the size of the filter. RHR represents the autocorrelation matrix of the received symbols (distorted by PN). Thanks to the fact that R can contain not only received PT-RS, but also data symbols received at the neighboring tones, the number of terms for averaging the autocorrelation matrix elements is always equal to the number of PT-RS N.

The second approach derives de-ICI filter coefficients from the central taps of PN spectrum estimate, relying on the Fourier transform property: Fejψt=Ψω ⇒ Fe-jψt=Ψ*-ω where ejψt is a PN realization and Ψω is its spectrum. The property says that the PN compensating filter taps should be a complex conjugate of PN spectrum taps taken in the reversed order. So, the filter coefficients are derived as:

AII=PΨ*=PargminΨHk0+uSk0+uHk0+u-1Sk0+u-1⋯Hk0-uSk0-uHk1+uSk1+uHk1+u-1Sk1+u-1⋯Hk1-uSk1-u⋮⋮⋱⋮HkN-1+uSkN+uHkN-1+u-1SkN+u-1⋯HkN-1-uSkN-1-uΨ-uΨ-u+1⋮Ψu-Rk0Rk1⋮RkN-12*=

=PXHX-1XHr*

where P is the antidiagonal permutation matrix. Please note, that unlike in Sec. 2.2 of [x4] the matrix X here contains all the PT-RS tones in an OFDM symbol, not just the tones of a single PT-RS block. XHX represents the autocorrelation matrix of the PT-RS sequence passed through the fading channel (not distorted by PN). The number of terms for its averaging is N(1 – 2u/NB), where NB is the number of tones in PT-RS block. It’s worth to mention, that the number of terms is always less than the number of PT-RS, however the difference becomes smaller as NB increases.

Please note, that both approaches don’t enforce any specific property on the de-ICI filter coefficients. However, the ties between the compensating filter taps and the PN spectrum taps allow to exploit the PN spectrum conjugate anti-symmetry property to improve the filter taps estimation [7]:

Ai=ReAi-A-i/2+j ImAi+A-i/2,  i≠0

This operation is very low-complex, while provides some performance gain. We recommend to consider it as a part of reasonably advanced receiver implementation for performance evaluations.

Observation 1:	De-ICI filtering performance can be improved by using conjugate anti-symmetric filter.

In our view, the PT-RS sequence type doesn’t have a fundamental impact on de-ICI performance, although it’s cyclic properties can facilitate low complexity filter coefficients estimation. However, as shown in [8] the coefficients estimation complexity is a smaller portion of the one of de-ICI processing, while the larger one is the frequency domain de-ICI filtering itself. In our evaluation the original Rel-15 PT-RS Gold sequence was used.

Lastly, we should note that de-ICI filtering has a fundamental performance limitation due to the finite filter bandwidth BA = SCS ‧ (2u+1). The performance upper bound of the frequency domain filtering can be assessed by passing PN realizations through an ideal high pass filter with the stopband width equal to BA. The residual high-frequency PN components can be partly compensated only with time domain processing.

#### CP-based time domain phase ramp pre-compensation

The time domain PN compensation method that doesn’t require additional IFFT/FFT is the CP-based linear phase error ramp pre-compensation [7]. It leverages the fact that the main source of difference between a CP and the end of an OFDM symbol is PN (CIR is assumed to be constant within the OFDM symbol).

Figure 4.1.1-1 CP-based linear phase error ramp estimation

As the first step, the phase difference Δφ should be estimated as the argument of cross-correlation between CP and the corresponding ending samples:

Δφ=argn=LCIRNCP-1rn+NFFTrn*

Please note, that the first LCIR samples of CP are excluded from the processing to avoid the disturbance caused by intersymbol interference. LCIR should be chosen in accordance with the sampling rate and channel delay spread. Since this study primarily focuses on 120kHz SCS and delay spread values <10ns, LCIR = 50 was used.

The second step is compensating the linear phase error ramp within an FFT window:

rn=rne-jΔφNFFTn-NCP,  NCP≤n<NCP+NFFT

This compensation technique doesn’t have bandwidth limitations, so it can be used as a pre-processing step before FFT and frequency domain de-ICI filtering. It also has moderate computational complexity. In our view, CP-based linear phase ramp pre-compensation can be considered as a part of reasonably advanced receiver implementation for performance evaluations.

Observation 2:	De-ICI filtering performance can be improved by using CP-based linear phase ramp pre-compensation.

#### Time domain compensation

The other way to bypass the bandwidth limitation of the frequency domain filtering is applying de-ICI compensation in time domain. This method involves additional IFFT of the zero-padded filter coefficients A. The elements of the resulting time domain vector a may not have unit magnitude since neither of the two de-ICI filter estimation approaches ensures this property. PN compensating vector should be derived by forcing the magnitude to 1 for each element of a. This operation effectively leads to the extension of the de-ICI compensation bandwidth. The compensation is done by elementwise multiplication of time domain OFDM symbol samples with the PN compensating vector:

rn=rn∙expj argk=-uuAkej2πNFFTkn

This method helps to overcome the bandwidth limitation of the frequency domain de-ICI but requires additional IFFT for the filter and second FFT for OFDM samples after the compensation. Although there is a clear performance benefit, the computational price is pretty high.

Observation 3:	De-ICI performance can be improved by applying the unit-magnitude de-ICI compensation in time domain.

### PT-RS enhancements

In this section we discuss PN compensation improving methods, which require specification changes. The evaluation methodology we used is also described here.

#### Block PT-RS

Grouping PT-RS tones into blocks is required for PN spectrum-based de-ICI filter estimation work. It may also be useful to form a guard band around the PT-RS tones used in x (direct filter coefficients estimation) or r vectors (PN spectrum-based estimation). In this paper we provide the simulation results for the full range of PT-RS block sizes NB from 1 to 128 with both de-ICI filter estimation approaches.

The uniform grid of PT-RS blocks was assumed in the evaluation. The total number of PT-RS tones N=128 (corresponding to PT-RS frequency density K=2 with NPRB=256) was kept the same during all the evaluations. With increasing NB, the tones were only redistributed to form fewer blocks of a larger size (see Figure 4.1-2).

Figure 4.1-2 The uniform PT-RS block grid

Only the central N(1 – 2NGB/NB) tones of each PT-RS block were used in x vector of the direct filter coefficients estimation algorithm, where NGB is the number of guard band tones at each PT-RS group edge. Similarly, only the central N(1 – 2(u+NGB)/NB) tones of a block was used in r vector of the PN spectrum-based estimation algorithm.

#### PT-RS power boosting

Another promising method of improving PN compensation performance is more flexible PT-RS power boosting, which however would require power redistribution from data to PT-RS REs. Some new signalling would also be needed to indicate the relative DM-RS to data RE power. However, we don’t investigate the trade-off between PN compensation improvement and the data SNR reduction in this paper. As an initial step, we provide the evaluation of boosting the PT-RS power while keeping the same power for data REs. The more detailed study is expected later.

### Simulation results

In order to evaluate the usefulness of block PT-RS pattern and PT-RS power boosting, it’s good to derive the set of MCS/rank combinations subject to UE capability in cases the above enhancements are present or not. The next section provides insight on the achievable performance improvement without specification changes.

#### Receiver implementation based PN compensation

Firstly, MCS27 – the highest achievable MCS under the simulation assumptions [9] – was simulated with rank 1 (see Figure 4.1-3). It can be seen that 10% BLER can be achieved below 30dB SNR with 7-tap de-ICI filter. So, in this scenario UE doesn’t even need to apply any advanced receiving algorithms to support the full range of MCSs with rank 1.

Observation 4:	all the MCSs can be supported with rank 1 without any enhancements in the specification with advanced PN compensation technique.

Figure 4.1-3 High MCS performance with rank 1 120kHz

Figure 4.1-4 High MCS performance with rank 2 120kHz

Observation 5:	MCSs up to MCS22 for CP-OFDM can be supported with rank 2 and SCS120kHz using frequency domain de-ICI filtering.

Observation 6:	MCSs up to MCS25 for CP-OFDM can be supported with rank 2 and SCS120kHz using the combination of CP-based pre-compensation, conjugate antisymmetric de-ICI filter property and the unit-magnitude de-ICI compensation in time domain.

Observation 7:	MCS26 for CP-OFDM and above cannot be supported with rank 2 and SCS120kHz by any means of a realistic UE implementation.

If no PT-RS enhancement will be adopted in the specification, supporting MCSs 23-25 with rank 2 and SCS120kHz should be a UE capability. The specification should not expect a UE to support MCSs 26-28 with rank 2 and SCS120kHz in that case.

Figure 4.1-5 High MCS performance with rank 2 960kHz

Observation 8:	MCSs up to MCS24 for CP-OFDM can be supported with rank 2 and SCS960kHz using frequency domain de-ICI filtering.

Observation 9:	MCSs up to MCS26 for CP-OFDM can be supported with rank 2 and SCS960kHz using the combination of CP-based pre-compensation, conjugate antisymmetric de-ICI filter property and the unit-magnitude de-ICI compensation in time domain.

Observation 10:	MCS27 and above cannot be supported with rank 2 and SCS960kHz by any means of a realistic UE implementation for CP-OFDM.

If no PT-RS enhancement will be adopted in the specification, supporting MCSs 24-26 with rank 2 and SCS960kHz should be a UE capability. The specification should not expect a UE to support MCSs 27-28 with rank 2 and SCS960kHz in that case.

#### PT-RS enhancements

The full set of different PT-RS block sizes NB from 1 to 128 was simulated with the same total PT-RS overhead using PN spectrum-based (solid curves) and direct de-ICI filter coefficients estimation (dashed curves) to find the best performing conditions for both of them. Also, the performance of the high pass filter suppressing PN spectrum components within de-ICI filter bandwidth is provided for reference (dash-dotted lines).

It can be seen that the performance of PN-spectrum based estimation algorithm drastically varies among the PT-RS block sizes. The reason is the ICI suppression level depends on the quality of estimating the autocorrelation matrix XHX. The number of averaging terms used to estimate the matrix elements shrinks significantly when NB is small, while it becomes almost the same as in direct coefficients estimation case when NB is larger. That’s why PN-spectrum based approach is inferior to the direct one at the small PT-RS block sizes, while it outperforms the competing algorithm at large PT-RS blocks.

One can also notice the degradation trend of both algorithms performance as NB grows above ~40. This is due to the increased probability of grouped PT-RS fading in a frequency-selective channel. That effect doesn’t let PN-spectrum based approach to become the overall winner in a fading channel (see Figure 4.1-6, TDL-A), because frequency selectivity eliminates its advantage at large PT-RS block size.

However, in almost flat LoS channel (see Figure 4.1-6, TDL-E) the PN-spectrum based algorithm can significantly (~1.5dB at MCS22) outperform the direct coefficients estimation at its best performing NB ~ 40. Since LoS condition is common in mmWave communication, that fact gives a strong motivation for supporting block PT-RS pattern in FR2-2.

Proposal 12:	Adopt block PT-RS pattern for use in FR2-2.

|  | Rank 1 | Rank 2 |
| --- | --- | --- |
| TDL-E 1 ns |  |  |
| TDL-A 5 ns |  |  |

Figure 4.1-6 De-ICI filtering performance as a function of PT-RS block size

Figure 4.1-7 shows how advanced Rx implementation algorithms, PT-RS power boosting and the usage of 1 edge subcarrier of a PT-RS block as a guard band can improve PN compensation performance across different NB. It can be seen that PT-RS power boosting gain increases with the block size NB from 0 to ~0.7dB. So, it’s worth to define flexible PT-RS power boosting only in together with block PT-RS. However, the gain is quite modest in the evaluated scenario, so it would be premature to recommend adoption of the power boosting based on these results. RAN1 should continue studying this aspect since it can reduce the number of non-supported MCSs/MCSs subject to UE capability.

Observation 11:	6dB PT-RS power boosting improves the MCS22 PN compensation performance from 0 to 0.7 dB depending on PT-RS block size.

Proposal 13:	RAN1 to continue studying PT-RS power boosting aspects.

Figure 4.1-7 De-ICI filtering performance improvement by implementation and specification-based enhancements

## Potential PT-RS enhancements for DFT-s-OFDM

### PN robust CBs mapping

Grouped PTRS samples are added before transform precoding (DFT) to compensate phase noise impact in NR UL. NgroupPT-RS sample groups are distributed over pre-DFT domain to capture time variation of PN, while Nsampgroup samples within a group are intended to improve CPE estimation quality for that group. Five PTRS patterns are currently supported in NR (see Figure 9).

samples / t

NgroupPT-RS

2

2

4

4

8

Nsampgroup

2

4

2

4

4

Figure 9. NR UL PTRS patterns (per DFT-s-OFDM symbol)

PN estimation error is unequally distributed within a DFT-s-OFDM symbol. If the gap between PTRS groups is comparable to the typical time variation of PN, the quality of CPE compensation at a sample significantly depends on the distance to the closest PTRS group. So, the samples in a middle between the groups get worse PN compensation than the other ones. A DFT-s-OFDM symbol is often shared between several code blocks (CBs) of a transport block (TB). So, the PN compensation performance may be different among the CBs depending on the location of their corresponding data samples.

Figure 6 provides the examples of the possible data and PTRS samples mapping together. The current NR specifies CBs concatenation on a bit level to form a TB together with TB CRC. Given the consecutive mapping of modulated QAM symbols to DFT-s-OFDM samples, the data samples corresponding to different CBs are also concatenated consequently. With certain PTRS patterns it may cause some CBs to suffer from PN more than the others. In the left example below CB0 and CB2 usually get better CPE compensation than CB1. This systematic unbalance between CBs BLERs increases TB BLER.

Observation 12:	Unequal distribution of PN estimation error among DFT-s-ODFM samples may lead to systematic unbalance between code blocks’ BLERs.

In the right example the data samples corresponding to different CBs are interlaced. The interlacing may be done on QAM symbol level within each DFT-s-OFDM symbol or a group of DFT-s-OFDM symbols. An illustration of the interlacing is shown in Figure 10. In this case the PN estimation error is equally distributed among the CBs, so there is no systematic unbalance between CBs BLERs.

CB2

CB1

CB0

PN1

PN2

PN1

PN2

Figure 10. Examples of concatenated (left) and interlaced (right) CBs mapping

The simulation has been done to evaluate the performance of all five NR PTRS patterns with regular CB mapping and interleaved mappings. The interleaving used in the evaluation was a simple modulation order level interleaving of all CBs. Firstly, the results (in Figure 11) show that only the patterns with 4 and 8 PTRS groups are applicable at 60GHz with 120kHz SCS. Secondly, given the same number of groups the patterns with larger number of samples in a group perform worse due to higher overhead. Finally, it can be seen that CBs interlacing within each DFT-s-OFDM symbol provides performance gain over CBs concatenation in all the scenarios. Larger gains are expected in peak data rate scenario, when the higher MCSs are used.

Figure 11. PUSCH BLER performance with different PTRS patterns and CBs mapping

Observation 13:	PUSCH PTRS patterns with only 4 and 8 PTRS groups provide acceptable performance with 120kHz SCS.

Observation 14:	Code blocks interlacing within a DFT-s-OFDM symbol provides performance gain from 0.5dB to 1.7dB at MCS22.

However, the details of the interlacing mapping require further study. This may include interleaving depth, handling of PTRS resource mapping with time density L>1, handling of different number of bits/modulated symbols for each CB in a DFT-s-OFDM symbol.

Proposal 14:	RAN1 to consider code blocks interlacing for PUSCH with transform precoding.

- DMRS Enhancements

Enhancements to DMRS for NR extension up to 71 GHz has been considered since RAN1 #104-e. The main motivation for them is to improve channel estimation performance especially for high subcarrier spacing (480 kHz/960 kHz) in frequency selective channels. This may be important for DMRS structures with comb-like mapping in the frequency domain, such as NR DMRS Type-1, as discussed further below.

In NR, DMRS utilizes frequency domain OCC to multiplex DMRS ports within the same resource elements. Even if the UE is receiving the transmission using rank-1, the UE needs to de-spread the OCC before channel estimation since the UE is not aware of whether other UEs are multiplexed in the same resources using an orthogonal DMRS port or not. Therefore, the UE will need to always assume that there may be multiple DMRS ports multiplexed when processing DMRS for channel estimation.

The orthogonality of the OCC only works when there is no channel variation across the two REs with OCC. For smaller subcarrier spacing, this is typically not an issue as even with severe channel delay spreads, the channel is relatively flat and channel estimation can be performed with minimal performance loss. For larger subcarrier spacing, the channel becomes wide enough that there is some channel selectivity across the OCC REs, and this starts to impact high order modulation reception, where good channel estimation is required.

There are basically two solutions proposed to cope with this issue. In the first one, there is dynamic indication to UE, i.e., via scheduling DCI, that no other ports are multiplexed onto the same DMRS REs. In other words, a CDM group indicated in the scheduling DCI contains only a single DMRS port. In this case, the UE does not need to perform OCC de-spreading operation which would degrade the performance previously. In the second solution, it’s proposed to introduce a new DMRS RE mapping pattern where all OFDM subcarriers within a symbol are utilized for DMRS transmission. It avoids comb-like structures with frequency gaps between consecutive OFDM subcarriers and, therefore, is referred to as the full-density DMRS.

Below, the evaluation results are provided for both NR DMRS Type-1 without OCC de-spreading and the full-density DMRS. The performance of the conventional NR DMRS Type-1 with OCC de-spreading operation in the frequency domain is also provided as the reference. The results for PDSCH transmission with rank-1 are given in Figure 4, whereas the results for PDSCH transmission with rank-2 are given in Figure 5. There is no phase noise modelling used in the evaluations.

Figure 4. Evaluation results for PDSCH performance with realistic channel estimation based on DMRS. Rank-1

Figure 5. Evaluation results for PDSCH performance with realistic channel estimation based on DMRS. Rank-2

Please note, that for rank-2 results in Figure 5 ports 0, 1 for NR DMRS Type-1 assumes CDM (and, thus, OCC de-spreading operation at the Rx) whereas ports 0, 2 assumes FDM (and, thus, no OCC de-spreading at the Rx).

From the presented evaluation results, it can be clearly seen the issue of PDSCH performance degradation impacted by channel estimation for the case of conventional NR DMRS Type-1 which is caused by OCC de-spreading. At the same time, it can be observed that both NR DMRS Type-1 without OCC de-spreading and the full-density DMRS improve the PDSCH performance and demonstrate almost the same results. Support of full density DMRS allows to maximize number of orthogonal DMRS layers which could be beneficial in MU-MIMO scenarios. The 60 GHz deployments are expected to use of narrow beams coupled with short time durations (from higher subcarrier spacing). Because of this, we expect more prevalent usage of MU-MIMO to multiplex users instead of relying on frequency domain multiplexing. Therefore, having many orthogonal DMRS ports is expected to bring meaningful benefits for 60 GHz. Supporting DMRS without OCC de-spreading is something simple that could be standardized with very minimal specification effort. So from this perspective, we believe there is value in supporting DMRS without OCC de-spreading as well. Based on that, it’s proposed to support either NR DMRS Type-1 without OCC de-spreading or the full-density DMRS.

Proposal 15:

- For DMRS enhancement in NR extension up to 71 GHz support either indication to UE that CDM groups, signaled in scheduling DCI, do not contain potential co-scheduled DMRS ports (aka DMRS without OCC de-spreading) or full-density DMRS (without any OCC de-spreading restrictions).

- Conclusions

In this contribution, we discussed enhancements for PDSCH and PUSCH for NR extensions up to 71 GHz. The following is a summary of the proposals:

Proposal 1

- Maximum number of PDSCHs that can be scheduled with a single DCI is 8 for 120 kHz SCS.

Proposal 2

- For NR 52.6-71 GHz, UE can be scheduled with more than one PDSCHs/PUSCHs in a slot for multi-PDSCH/PUSCH scheduling for 120/480/960 kHz SCS.

  - More than one SLIVs per slot in a row in TDRA table for multi-PDSCH/PUSCH scheduling are supported.

Proposal 3

- For multi-PDSCH/PUSCH scheduling, CBG based transmission is supported for 120/480/960kHz subcarrier spacing when a single PDSCH/PUSCH is scheduled.

Proposal 4

- For multi-PDSCH/PUSCH scheduling, separate k0/k2, mapping type and SLIV for each scheduled PDSCH/PUSCHs can be configured in each row of TDRA table.

Proposal 5

- For multi-PDSCH scheduling

  - Scheduling of 2nd TB is supported.

  - For 2nd TB, separate MCS, NDI and RV are signaled from 1st TB.

  - For 2nd TB, similar mechanisms for signaling of MCS, NDI and RV for 1st TB are reused.

Proposal 6

- For multi-PUSCH scheduling,

  - Support intra-slot frequency hopping for scheduled PUSCHs.

  - Do not support enhancement on FDRA.

Proposal 7

- For multi-PDSCH scheduling

  - Carrier indicator, BWP indicator, frequency domain resource allocation, VRB-to-PRB mapping, PRB bundling size indicator, rate matching indicator, ZP CSI-RS trigger and DMRS configuration including antenna port, DMRS sequence initialization, etc., can be applied for all the scheduled PDSCHs.

Proposal 8

- For Type-1 HARQ-ACK codebook generation,

- the set of SLIVs just include all the SLIVs that can be scheduled within the DL slot by any row index r of TDRA table in DCI indicating the UL slot as HARQ-ACK feedback timing, considering the TDD UL-DL configuration.

- to allocate the occasion(s) for a DL slot, the overlap checking is performed across the SLIVs in the multiple slots of the rows in TDRA table

Proposal 9

- Type-2 HARQ-ACK codebook is generated with Alt 1 ‘C-DAI/T-DAI counted per DCI’

- Two sub-codebooks are generated for a PUCCH cell group

    - If time bundling is configured, a single HARQ-ACK codebook may be adopted.

- If 2 HARQ-ACK bits are generated for a multi-PDSCH DCI, it is included in the first sub-codebook if 2 HARQ-ACK bits per DCI is reported in the first sub-codebooks

- The same number of HARQ-ACK bits are associated with each DCI in a sub-codebook

    - denote the maximum number of TBs that can be scheduled by a multi-PDSCH DCI as M and the number of configured CBGs for a PDSCH as N, the number of HARQ-ACK bits per DCI in the second sub-codebook equals to the maximum of all configured values M and N among all the configured cells

- 1 HARQ-ACK bit is included in the first sub-codebook for the DCI indicating SPS PDSCH release, SCell dormancy indication without scheduled PDSCH

Proposal 10

- Time domain bundling can be supported in Type-2 HARQ-ACK codebook.

- FFS how to determine the number of sub-codebooks

- The same grouping of the two sub-codebooks by the number of bundled HARQ-ACK bits as the case that time bundling is not configured.

- Time domain bundling can be supported in Type-1 HARQ-ACK codebook.

- A bundled occasion corresponds to multiple HARQ-ACK bits that are associated with same multi-PDSCH DCI.

Proposal 11:	RAN1 to introduce UE capability of supporting high MCS/rank combinations in FR2-2. Details of the capability signal is FFS.

Observation 1:	De-ICI filtering performance can be improved by using conjugate anti-symmetric filter.

Observation 2:	De-ICI filtering performance can be improved by using CP-based linear phase ramp pre-compensation.

Observation 3:	De-ICI performance can be improved by applying the unit-magnitude de-ICI compensation in time domain.

Observation 4:	all the MCSs can be supported with rank 1 without any enhancements in the specification with advanced PN compensation technique.

Observation 5:	MCSs up to MCS22 for CP-OFDM can be supported with rank 2 and SCS120kHz using frequency domain de-ICI filtering.

Observation 6:	MCSs up to MCS25 for CP-OFDM can be supported with rank 2 and SCS120kHz using the combination of CP-based pre-compensation, conjugate antisymmetric de-ICI filter property and the unit-magnitude de-ICI compensation in time domain.

Observation 7:	MCS26 for CP-OFDM and above cannot be supported with rank 2 and SCS120kHz by any means of a realistic UE implementation.

Observation 8:	MCSs up to MCS24 for CP-OFDM can be supported with rank 2 and SCS960kHz using frequency domain de-ICI filtering.

Observation 9:	MCSs up to MCS26 for CP-OFDM can be supported with rank 2 and SCS960kHz using the combination of CP-based pre-compensation, conjugate antisymmetric de-ICI filter property and the unit-magnitude de-ICI compensation in time domain.

Observation 10:	MCS27 and above cannot be supported with rank 2 and SCS960kHz by any means of a realistic UE implementation for CP-OFDM.

Proposal 12:	Adopt block PT-RS pattern for use in FR2-2.

Observation 11:	6dB PT-RS power boosting improves the MCS22 PN compensation performance from 0 to 0.7 dB depending on PT-RS block size.

Proposal 13:	RAN1 to continue studying PT-RS power boosting aspects.

Observation 12:	Unequal distribution of PN estimation error among DFT-s-ODFM samples may lead to systematic unbalance between code blocks’ BLERs.

Observation 13:	PUSCH PTRS patterns with only 4 and 8 PTRS groups provide acceptable performance with 120kHz SCS.

Observation 14:	Code blocks interlacing within a DFT-s-OFDM symbol provides performance gain from 0.5dB to 1.7dB at MCS22.

Proposal 14: RAN1 to consider code blocks interlacing for PUSCH with transform precoding.

Proposal 15:

- For DMRS enhancement in NR extension up to 71 GHz support either indication to UE that CDM groups, signaled in scheduling DCI, do not contain potential co-scheduled DMRS ports (aka DMRS without OCC de-spreading) or full-density DMRS (without any OCC de-spreading restrictions).

# References

- Chairman’s notes, RAN1 #105 e-Meeting, May 2021.

- Chairman’s notes, RAN1 #104b e-Meeting, April 2021.

- Chairman’s notes, RAN1 #103 e-Meeting, October – November 2020.

- R1-2100647 “Discussion on PDSCH/PUSCH enhancements for extending NR up to 71 GHz”, Intel Corporation, RAN1 #104b e-Meeting, April 2021

- R1-2008805 “Discussion on Required Changes to NR in 52.6 – 71 GHz”, Intel Corporation, RAN1#103 e-Meeting, October – November 2020.

- R1-2005922 “On phase noise compensation for OFDM”, Ericsson, RAN1#102 e-Meeting, August 2020.

- A. Lomayev, V. Kravtsov, M. Genossar, A. Maltsev, A. Khoryaev, “Method for Phase Noise Impact Compensation in 60 GHz OFDM Receivers”, Radioengineering, vol. 29, no. 1, pp. 159–173, April 2020.

- R1-2102792 “PDSCH/PUSCH enhancements”, Ericsson, RAN1#104b e-Meeting, April 2021.

- 3GPP TR 38.808

# Appendix: Main simulation parameters for DMRS evaluations

| Parameter | Value |
| --- | --- |
| Channel Model | TDL-A, DS = 20 ns |
| SCS | 960 kHz |
| Bandwidth | 400 MHz (32 PRBs) |
| Channel Estimation | LMMSE with block size = 2PRBs |
| MCS | Rank-1: MCS22, MCS25 Rank-2: MCS20, MCS23 |
| Power Boosting | Full Density DMRS: 0 dB NR DMRS Type-1: 3 dB |
| OCC De-Spreading | ON: Full Density DMRS, NR DMRS Type-1 OFF: NR DMRS Type-1 |
| PMI Granularity | Sub-band (2 PRB) |
| PN Modelling, Estimation and Compensation | OFF |