3GPP TSG RAN WG1 #106-e                                      R1-2107781

e-Meeting, August 16th – 27th, 2021

Source:	ZTE

Title:	Discussion on additional enhancements for IoT-NTN

Agenda Item:	8.15.3

Document for:   Discussion

- Introduction

In RAN#92e meeting [1], the IoT-NTN WI has been endorsed. In this contribution, following the discussion in SI, continuous analysis on the issues for power consumption and discontinuous service are elaborated.

- Discussion on discontinuous service

In NTN systems, the service may be discontinuous due to the satellite constellation configuration or limited satellite capability (e.g., cube satellite). For a satellite with earth moving beam, a beam sweeps the serving area with the movement of the satellite. The UEs in this serving area experience time-variant DL RSRP. The service at the serving beam edge may be discontinuous. For a satellite with earth fixed beam, a steerable serving beam will provide a relatively long serving time. However, once the elevation of service link (or feeder link) is too small, beam switching will occur and the service may be discontinuous.

In current NR system, a UE attempts to initialize handover or radio link failure procedure when channel quality is poor. In NTN case, this attempt may be invalid or inefficient with unnecessary power consumption. To solve this problem, current DRX/eDRX or PSM mechanism can be reused with some enhancement specific to IoT over NTN.

2.1 Assistance information based enhancement on discontinuous coverage

In general, the discontinuous service due to the satellite constellation configuration or limited satellite capability is highly predictable at the network side. So, it is possible for the eNB to provide assistance information to UEs as following examples to reduce unnecessary power consumption at the UE side:

- Assist time information of a satellite for a given area: The assist time information can refers to the time interval, in which, the service from current will be available or unavailable. The indication of such information is more general comparing to the ephemeris only since the discontinuous service may be caused by service link switching, feeder link switching, traffic load balance, satellite malfunction, inter-satellite routine change due to congestion and so on. In addition, indication of such assist time information is beneficial to IoT devices with limited power and capability. The corresponding time duration of current and/or next satellites for a given area can be pre-calculated at eNB side with indication to UEs.  For example, if the assist information refers to the time that the service is unreachable, within the indicated period, the reception of DL signaling including measurement for RLF or handover and scheduling/data for both DL and DL is assumed to be disabled. Since this serving time is common information for all UEs or a group of UEs, it can be indicated via common signaling or UE-specific signaling.

- Grouping information based UE’s location: Accordingly to the GNSS assumption for NTN UEs, it is possible for UEs to report their location to the eNB. The location information can be used for UE grouping. The eNB can predict the service discontinuity according to the location of satellite and UEs. Then corresponding transmission control can be carried out at the UE side or the eNB side.

Proposal 1: Indication of assist time information of each satellite to UE should be supported to handle the discontinuous service.

2.3 Discussion on the enhancement on legacy procedure

In current LTE system, DRX, eDRX or PSM mechanism has been used to deal with the discontinuous service, which is main targeted for the power saving based on the characteristic of IoT traffic. For example, in NB-IoT, discontinuous transmission is supported at the UE side using the eDRX and PSM mechanism. As shown in Figure 1, an NB-IoT UE will periodically enter the idle mode or deep sleep mode to save power, which is determined by the corresponding timers.

Figure 1. The state transition of an NB-IoT UE

For another example, as shown in Figure 2 for LTE systems, short DRX cycle and long DRX cycle are used for RRC_CONNECTED UE to save power consumption via discontinuous transmission. For RRC_IDLE UE, the discontinuous transmission is controlled by paging cycle.

Figure 2. The state transition of an LTE UE

It is observed that current mechanism to support discontinuous transmission is generally period based (i.e., using various cycles). In NTN, however, the discontinuous transmission caused by the satellite movement may not occur periodically. Therefore, based on the assistance information in above, it is necessary to study enhanced DRX, eDRX or PSM mechanism for NTN scenarios with following examples:

- Based on the information of deployment, since the eNB may have the information on the beam deployment and satellite information, prediction on the discontinuous service can be expected at eNB side, e.g., within a period of time. Then, the trigger indication can be sent to UE to enforce the UE into PSM mode for power saving with an instant PSM timer on demand.

- If a UE has the information w.r.t its own location and the satellite’s position and beam deployment or based on the tracking of channel quality, the prediction on the discontinuous service can be conducted at UE side. In this case, once the UE is scheduled or wake-up based on DL indication, the UE can report a service delay request to the eNB with a timer value to indicate the eNB to postpone the following transmission including DL/UL grant.

Proposal 2: Enabling of on-demand PSM can be considered to achieve the UE power saving due to the discontinuous service.

Proposal 3: The postponed DL/UL transmission based on the UE reported time information can be considered for IoT over NTN scenarios.

- Conclusions

In this contribution, power consumption and discontinuous service issues and possible solutions to support IoT over NTN is conducted with following proposals:

Proposal 1: Indication of assist time information of each satellite to UE should be supported to handle the discontinuous service.

Proposal 2: Enabling of on-demand PSM can be considered to achieve the UE power saving due to the discontinuous service.

Proposal 3: The postponed DL/UL transmission based on the UE reported time information can be considered for IoT over NTN scenarios.

- References

- RP-211601, WID on NB-IoT/eMTC support for NTN