3GPP TSG RAN WG1 #106-e			R1-2108154

e-Meeting, August 16th – 27th, 2021

Source:	WILUS Inc.

Title:	Discussion on intra-UE multiplexing/prioritization for URLLC/IIoT

Agenda item:	8.3.3

Document for:	Discussion/Decision

- Introduction

From RAN1#104-e to RAN1#105-e meeting [1]-[3], RAN1 made a progress on intra-UE multiplexing/ prioritization for URLLC/IIoT. In this contribution, we provide our views on intra-UE multiplexing of different channels.

Multiplexing UCIs of different priorities in a PUCCH

The total # bits of {LP HARQ-ACK + HP HARQ-ACK} > 2

Code rate determination for separate coding

Agreements at RAN1#104-e: [1] For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, - Use a PUCCH resource in the second PUCCH-Config (the PUCCH-config containing the PUCCH resource of the HP HARQ-ACK) at least in case the total number of LP and HP HARQ-ACK bits is more than 2. - FFS: The PUCCH resource is configured dedicated for multiplexing of HP HARQ-ACK and LP HARQ-ACK. - FFS in case the total number of LP and HP HARQ-ACK bits is 2. - FFS details Agreements at RAN1#104b-e:[2] For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, support separate coding for the two HARQ-ACKs. - FFS for HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s). - (working assumption) Drop CSI (including part 1 and part2, if exist) if CSI would multiplex on a PUCCH which has HP A/N. - FFS Strive to let HP A/N reuse the encoder, rate matching equation, and RE mapping rules in Rel-15 for A/N+CSI-1. - FFS Strive to let LP A/N reuse the encoder, rate matching equation, and mapping rules in Rel-15 for CSI-2. Agreements at RAN1#105-e:[3] For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is more than 2, - For HP HARQ-ACK or LP HARQ-ACK of 1-2 bit(s), support separate coding. Down-select from the two options: - Option 1: Reuse R15 TS 38.212 Clause 5.3.3.1 for 1-bit. Reuse R15 TS 38.212 Clause 5.3.3.2 for 2-bit. - Option 2: Reuse R15 TS 38.212 Clause 5.3.3.3, i.e., padding to 3 bits and using RM coding. - For HP HARQ-ACK or LP HARQ-ACK >2 bit(s), HP HARQ-ACK and LP HARQ-ACK are separately encoded according to R15 TS 38.212 Clause 5.3.3.3 or Clause 5.3.1. - FFS rate matching equation and RE mapping rules for PF2/3/4. Rel-15 is baseline if available.

In the RAN1#104b-e meeting, RAN1 made a consensus to support separate coding for LP HARQ-ACK and HP HARQ-ACK. For separate coding, it needs to determine two code rates, one for LP HARQ-ACK and another for HP HARQ-ACK.

If a LP PUCCH overlaps with a HP PUCCH, the HP PUCCH resource is used to multiplex HP HARQ-ACK and LP HARQ-ACK. Since the HP PUCCH resource has a code rate for HP HARQ-ACK only, a code rate for LP HARQ-ACK can be additionally configured or derived. Based on enhancements of RRC configuration, there are two options.

- Option 1. Additional code rate for LP HARQ-ACK in a HP PUCCH format is configured.

- Option 2. Code rate configured in LP PUCCH format is reused.

In option 1, a gNB may configure two code rates in a HP PUCCH format 2/3/4, where one is used for HP HARQ-ACK transmission and another one is used for LP HARQ-ACK (only when multiplexing LP HARQ-ACK and HP HARQ-ACK). This option is straightforward and has more flexibility to configure code rate of LP HARQ-ACK considering the code rate of HP HARQ-ACK, but it requires additional RRC signalling overhead.

In option 2, a UE can reuse the code rate already configured in LP PUCCH format. This option can be used without any changes of RRC configurations so that it is preferred in terms of RRC signalling overhead. One remaining issue in option 2 is which code rate for LP PUCCH format should be used if the original LP PUCCH format is different from the PUCCH format after multiplexing LP HARQ-ACK and HP HARQ-ACK. For example, the original LP PUCCH format is PF3, but the PUCCH format after multiplexing LP HARQ-ACK and HP HARQ-ACK can be changed to PF2. It is because HP PUCCH resource is used for multiplexing LP HARQ-ACK and HP HARQ-ACK. In this case, the UE can use either the code rate in LP PUCCH format 3 (original LP PUCCH format) or the code rate in LP PUCCH format 2 (new PUCCH format after multiplexing). The reason why the code rate is configured in each of PUCCH format is that the PUCCH format can have different usages e.g., PF2 for low-latency or PF3 for high-reliability, etc. Therefore, it seems reasonable to use the code rate in the new PUCCH format after multiplexing LP HARQ-ACK and HP HARQ-ACK.

- Proposal 1. To determine a code rate for LP HARQ-ACK for multiplexing, consider the following two options.

  - Option 1. Additional code rate for LP HARQ-ACK in a HP PUCCH format is configured by RRC.

  - Option 2. Code rate configured in LP PUCCH format is reused.

Another issue is the case where the second PUCCH-Config only includes the first PUCCH resource set (for 1- or 2-bit HARQ-ACK information). In this case, the UE cannot multiplex LP HARQ-ACK and HP HARQ-ACK in a PUCCH resource of the second PUCCH-Config since there are no PUCCH resources for more than 2 bits HARQ-ACK information. There are two approaches to address this issue. The first approach is to allow a configuration of dedicated PUCCH resource(s) for multiplexing HP HARQ-ACK and LP HARQ-ACK only if the second PUCCH-Config contains the first PUCCH resource set (for 1- or 2-bit HARQ-ACK information). So, the UE can multiplex HP HARQ-ACK and LP HARQ-ACK in the dedicated PUCCH resource. The second approach is that this case is regarded as an error case, i.e., the UE is not expected to multiplex HP HARQ-ACK and LP HARQ-ACK if the total number of LP HARQ-ACK and HP HARQ-ACK is more than 2 bits for the case where the second PUCCH-Config only includes the first PUCCH resource set (for 1- or 2-bit HARQ-ACK information.

- Proposal 2: Further discuss whether/how to multiplex HP HARQ-ACK and LP HARQ-ACK in a PUCCH resource if the second PUCCH-Config contains only the first PUCCH resource (for 1- or 2-bit HARQ-ACK information).

HP-UCI and LP-UCI mapping in a PUCCH format 2/3/4

When the separately encoded UCIs are mapped to the selected PUCCH resource, it should be determined in RAN1 how to map the different UCIs with priorities.

In Rel-15, these PUCCH format 3/4 support two separate coded UCIs, where the first coded UCI includes HARQ-ACK, SR and CSI part 1 and the second UCI includes CSI part 2. The first coded UCI has higher priority than the second coded UCI so that the first coded UCI is mapped to symbols adjacent to DMRS symbols. For example, Figure 2 shows PUCCH format 3 where the first UCI occupies 100 REs and the second UCI occupies 44 REs.

Figure 1. RE mapping of PUCCH format 3 (1 RB and 14 symbols)

To multiplex LP HARQ-ACK and HP HARQ-ACK, the mapping rule of PUCCH format 3/4 in Rel-15 can be reused. Here, the first coded UCI includes HP HARQ-ACK and the second coded UCI includes LP HARQ-ACK. Based on Figure 1, the mapping rule of PUCCH format 3/4 in Rel-15 is enough to provide higher reliability, but it may result in longer latency to HP HARQ-ACK due to the HP HARQ-ACK is mapped to later symbols. For example, 100 REs for HP HARQ-ACK can be mapped to all REs in OFDM symbol 0/1/2/4/5/6/7/8 and 4 REs in OFDM symbol 9, which can reduce latency to HP HARQ-ACK of 4 symbols (ending OFDM symbol is changed from 13 to 9).

- Proposal 3: For PUCCH format 3/4,

  - To maximize reliability of HP HARQ-ACK, the mapping rule of PUCCH format 3/4 in Rel-15 can be reused., i.e., the HP HARQ-ACK is mapped to adjacent symbols to DMRS symbols.

  - To minimize latency of HP HARQ-ACK, the HP HARQ-ACK can be mapped to earlier symbols.

In Rel-15, PUCCH format 2 cannot support to multiplex two separate coded UCIs. It is important to multiplex LP HARQ-ACK and HP HARQ-ACK in a PUCCH format 2 because the PUCCH format 2 is suitable not only for small latency services but also for 2-symbol sub-slot PUCCH configuration. Therefore, it is needed to introduce a new mapping rules of PUCCH format 2 for multiplexing LP HARQ-ACK and HP HARQ-ACK. Since PUCCH format 2 has one or two OFDM symbols, there are no rooms to reduce latency of HP HARQ-ACK. To maximize reliability of HP HARQ-ACK, it would be better to map the HP HARQ-ACK across RBs as a distributed manner, i.e., HP HARQ-ACK is distributed to REs across RBs as much as possible.

- Proposal 4: For PUCCH format 2,

  - Support PUCCH format 2 to multiplex LP HARQ-ACK and HP HARQ-ACK

  - To maximize reliability of HP HARQ-ACK, the HP HARQ-ACK is distributed to REs across RBs as much as possible.

UCI compaction

When encoding the LP-UCI, the required # of RBs may exceed the limit of PUCCH formats. In this case, a size of the LP-UCI may be reduced. In fact, if the LP-UCI is HARQ-ACK information, rather than dropping some part of HARQ-ACK information, it would be better to bundle some part of HARQ-ACK information. For HARQ-ACK bundling, we need to decide which HARQ-ACK bits would be bundled. For example, consider the case where 10 bits for HARQ-ACK information are needed to be bundled into 5 bits. One scheme is to bundle adjacent 2 bits into a 1 bit. The other scheme is to bundle the last 6 bits into a 1 bit. Both schemes can bundle into 5 bits, but the performance can be different depending on the states of the HARQ-ACK information. For example, in the first scheme of bundling adjacent 2 bits into 1 bit, the poor performance would be expected for the case with the lower correlation of adjacent 2 bits. So, it should be further studied how to bundle HARQ-ACK information in Rel-17 URLLC/IIoT WI.

- Proposal 5: If the required # of RBs for low-priority HARQ-ACK information exceeds the limit of PUCCH formats, then bundle the low-priority HARQ-ACK information. Detail bundling rules should be further discussed in Rel-17 URLLC/IIoT WI.

The total # bit of {LP HARQ-ACK + HP HARQ-ACK} = 2

Encoding of HP HARQ-ACK and LP HARQ-ACK in PF0/PF1

If the total bit-length of LP HARQ-ACK and HP HARQ-ACK is no more than 2 bits, then one of PF0 or PF1 is used to transmit LP HARQ-ACK and HP HARQ-ACK. In this case, the issue is how to multiplex 2 HARQ-ACK bits with or without SR in PF0 or PF1. At the previous RAN1#105-e meeting, the following was agreed for the case of multiplexing LP HARQ-ACK and HP-HARQ-ACK in a PF0 or PF1.

Agreements at RAN1#105-e meeting: For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUCCH in R17, when the total number of LP and HP HARQ-ACK bits is 2, treat the two bits as HARQ-ACK bits with High priority. - Rel-15 design (for PF0 and PF1) is baseline.

In this section, we describe the remaining cases except for the case of multiplexing LP HARQ-ACK and HP-HARQ-ACK in a PF0 or PF1, and we provide our views on each case of remaining cases.

Case 2) Multiplexing LP HARQ-ACK and HP SR in a PUCCH format 0 or 1

Case 2-1) HP SR with PF0 and LP HARQ-ACK with PF0

Agreements at RAN1#104-e: [1] When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.3: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In Rel-15, when a PUCCH carrying SR with PF0 overlaps with a PUCCH carrying HARQ-ACK with PF0, then SR and HARQ-ACK are multiplexed into the PUCCH carrying HARQ-ACK. This method can be directly reused for multiplexing HP-SR with PF0 and LP HARQ-ACK with PF0.

To multiplex 1 HP-SR with PF0 and 1-bit LP HARQ-ACK with PF0, it is enough to reuse the CS mapping rule defined in Rel-15, where CS=0, 3, 6, 9 is used to indicate 4 states (LP HARQ-ACK, HP-SR) = (NACK, negative SR), (NACK, positive SR), (ACK, negative SR), and (ACK, positive SR), respectively. Note that the minimum CS distance between negative SR and positive SR of HP-SR is 3, which is quite large to ensure the reliability of HP-SR.

To multiplex with 1 HP-SR with PF0 and 2-bit LP HARQ-ACK in PF0, the Rel-15 CS mapping rule can be also used, where CS=0, 3, 6, 9 is used to indicate 2-bit LP HARQ-ACK with negative SR, and CS=1, 4, 7, 10 is used to indicate 2-bit LP HARQ-ACK with positive SR as shown in Figure 2-(a). In this case, the minimum CS distance between negative SR and positive SR of HP-SR is 1 (for example CS=0 and CS=1) and the minimum CS distance between ACK and NACK of LP HARQ-ACK is 2 (for example CS=1 and CS=3). Thus, the LP HARQ-ACK is more robust than the HP-SR, which cannot be suitable to protect HP-SR reliability. A simple way is to swap the CS positions of LP HARQ-ACK and HP-SR. For example, CS=0, 3, 6, 9 is used to indicate 1-bit (MSB) LP HARQ-ACK and HP-SR if 1-bit (LSB) LP HARQ-ACK is NACK, and CS=1, 4, 7, 10 is used to indicate 1-bit (MSB) LP HARQ-ACK and HP-SR if 1-bit (LSB) LP HARQ-ACK is ACK, which is shown in Figure 2-(b). Based on this CS mapping, the HP-SR reliability can be higher than LP HARQ-ACK. In addition, to achieve higher HP-SR reliability and reduce CS overheads, we can also consider bundling of 2-bit LP HARQ-ACK, where 1 HP-SR and 1-bit bundled LP HARQ-ACK can be multiplexed by using the same way of 1 HP-SR and 1-bit LP HARQ-ACK.

Figure 2. CS mapping for 2-bit LP HARQ-ACK and 1 HP-SR

- Proposal 6: To multiplex HP-SR with PF0 and LP HARQ-ACK with PF0, we propose

  - If HP-SR is negative, then transmit LP HARQ-ACK on HARQ-ACK resource.

    - In case of 1-bit LP HARQ-ACK, use 2 CSs, i.e., {0, 6} CS index

    - In case of 2-bit LP HARQ-ACK, use 4 CSs, i.e., {0, 3, 6, 9} CS index

  - If HP-SR is positive, then transmit LP HARQ-ACK and HP-SR on HARQ-ACK resource

    - In case of 1-bit LP HARQ-ACK, use 2 CSs, i.e., {3, 9} CS index

    - In case of 2-bit LP HARQ-ACK, use 4 CSs, i.e., {1, 4, 7, 11} CS index

      - To enhance HP-SR reliability, 2-bit LP HARQ-ACK can be bundled to 1-bit and then the 1-bit bundled LP HARQ-ACK is treated as 1-bit LP HARQ-ACK, i.e., use 2 CSs, {3, 9} CS index .

Case 2-2) HP SR with PF0 and LP HARQ-ACK with PF1

In Rel-15, it was not allowed to multiplex SR with PF0 and HARQ-ACK with PF1 in a PUCCH resource. So, it needs to define a new multiplexing rule to support multiplexing of HP-SR with PF0 and LP HARQ-ACK with PF1. In the RAN1#104-e meeting, several options were listed [2].

Agreements at RAN1#104-e: [1] When a PUCCH carrying HP SR with PF0 overlaps with a PUCCH carrying LP HARQ-ACK with PF1, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The positive SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: The UE does not transmit negative SR. - Opt.1b: For negative SR, the UE transmit only HARQ-ACK on the HARQ-ACK resource. - Opt.1c: For negative SR, the UE transmits SR and HARQ-ACK on the SR resource - FFS: whether with power boost to transmit multiplexed payload or not. - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - FFS on conditions of multiplexing. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: For positive SR, transmit SR on the SR resource and drop HARQ-ACK. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.5: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

Option 1 is to use the HP-SR resource for multiplexing positive HP-SR and LP HARQ-ACK. To support Option 1, a gNB should reserve some of CSs in PF0 for the HP-SR, because the gNB does not know whether the UE transmits positive HP-SR or not. If the UE transmits negative HP-SR, then the reserved CSs in PF0 would be wasted so that this option is not preferred in terms on UL resource efficiency.

Option 2 is to use the LP HARQ-ACK resource for multiplexing HP-SR and LP HARQ-ACK. Again, in Option 2a, 1 PRB (based on the offset) should be reserved as in Option 1. So, Option 2a is also not preferred in terms of UL resource efficiency. In Option 2b, the HP-SR and the 1-bit LP HARQ-ACK are multiplexed by modulating to QPSK symbol and transmitted via PF1 for the LP HARQ-ACK and for the case of 2-bit LP HARQ-ACK, the LP HARQ-ACK is reduced/compressed to 1-bit. Since this option 2b does not require any reserved resources, it is preferred in terms of UL resource efficiency.

Option 3 requires blind detection at the gNB side to detect positive or negative SR on separate PUCCH resource. In fact, such a blind detection was used in Rel-15 multiplexing SR with PF1 and HARQ-ACK with PF1. However, contrary that Rel-15 where two PUCCH resources are PF1, one PUCCH resource is PF0 and another PUCCH resource is PF1. Therefore, it is unclear how to blindly detect two separately different PUCCH formats and its performance.

Option 4 does not multiplex HP-SR and LP HARQ-ACK so that it should be avoided.

Based on this discussion, we propose to support Option 2b for multiplexing HP-SR with PF0 and LP HARQ-ACK with PF1.

- Proposal 7: We propose to support Option 2b for multiplexing HP-SR with PF0 and LP HARQ-ACK with PF1.

  - To multiplex HP-SR with PF0 and LP HARQ-ACK with PF1, use the HARQ-ACK resource.

    - Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

Case 2-3) HP SR with PF1 and LP HARQ-ACK with PF0

Agreements at RAN1#104-e: [1] When a PUCCH carrying HP SR with PF1 overlaps with a PUCCH carrying LP HARQ-ACK with PF0, further study the following options (proponents are encouraged to provide more details and analysis): - Opt.1: The SR and HARQ-ACK are multiplexed and transmitted on the SR resource. - Opt.1a: For positive SR, the UE transmits the PUCCH in the resource using PUCCH format 1 for SR. The value of cyclic shift of sequence, i.e., , of this PUCCH format 1 is determined by HARQ-ACK, and the bit, i.e., b(0), of this PUCCH format 1 is determined by SR. For negative SR, the UE transmits only a PUCCH with HARQ-ACK information and drops the PUCCH with negative SR. - Opt.1b: SR and HARQ-ACK are multiplexed and modulated to be transmitted on the SR resource - Opt.2: The SR and HARQ-ACK are multiplexed and transmitted on the HARQ-ACK resource. - Opt.2a: If SR is positive, an offset (e.g. 1 PRB) is added to the starting PRB of the HARQ-ACK PUCCH resource. - Opt.2b: Using 4 CS values as for SR+1-bit HARQ-ACK in Rel-15/16. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit. - Opt.2c: If SR is positive, SR is multiplexed on HARQ-ACK resource in the same way as Rel-15. If SR is negative, transmit only HARQ-ACK on HARQ-ACK resource. - Opt.2d: HP SR and LP HARQ-ACK are multiplexed by the Rel-15 cyclic shift only if latency requirement for HP SR is met. Otherwise, drop the LP HARQ-ACK and only transmit the HP SR on its resource. - Opt.3: For positive SR, transmit HARQ-ACK on the SR resource. For negative SR, transmit HARQ-ACK on the HARQ-ACK resource. - Opt.4: No enhancement over Rel-16. - Other options not excluded. - FFS: Whether/How to differentiate HP SR and LP SR when multiplexed with LP HARQ-ACK?

In Rel-15, when a PUCCH carrying SR with PF1 overlaps with a PUCCH carrying HARQ-ACK with PF0, then SR and HARQ-ACK are multiplexed into the PUCCH carrying HARQ-ACK. To strive for common design, the multiplexing method proposed in Case 2-1 can be directly used for multiplexing HP SR with PF1 and LP HARQ-ACK with PF0.

- Proposal 8: To multiplex HP-SR with PF1 and LP HARQ-ACK with PF0, reuse multiplexing rule for HP-SR with PF0 and LP HARQ-ACK with PF0.

- PUCCH vs PUSCH

Multiplexing with LP HARQ-ACK and HP HARQ-ACK into a PUSCH

Agreement at RAN1#104bis-e: [2] For multiplexing a high-priority (HP) HARQ-ACK and a low-priority (LP) HARQ-ACK into a PUSCH in R17, support separate coding for the two HARQ-ACKs. - It is understood that it is intended that the number of encoding chains for all UCI multiplexing combinations in Rel-17 should not exceed that in Rel-15/16.

To multiplex HP HARQ-ACK and LP HARQ-ACK in a PUSCH, RAN1 agreed to support separate coding. The remaining issue is how to determine the number of REs for HP HARQ-ACK and LP HARQ-ACK and how to map the REs into a PUSCH in case of UCI on PUSCH

To determine the number of REs (QHP-ACK') for HP HARQ-ACK, we can reuse the rule to determine the number of HARQ-ACK by using beta_offset, βoffset,HP-to-XHARQ-ACK, which is denoted by beta_offset value for HP HARQ-ACK and X=LP or X=HP PUSCH.

QHP-ACK'=minOHP-ACK+LHP-ACK∙βoffset,HP-to-XHARQ-ACK∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=l0Nsymb,allPUSCH-1MscUCIl.

Similarly, as in HP HARQ-ACK, the number of REs (QLP-ACK') for LP HARQ-ACK can be determined as follows:

QLP-ACK'=minOLP-ACK+LLP-ACK∙βoffset,LP-to-XHARQ-ACK∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=l0Nsymb,allPUSCH-1MscUCIl-QHP-ACK',

where beta_offset, βoffset,LP-to-XHARQ-ACK, which is denoted by beta_offset value for LP HARQ-ACK and X=LP or X=HP PUSCH. The difference between two equations above is the maximum available REs α∙l=l0Nsymb,allPUSCH-1MscUCIl-QHP-ACK' by considering the REs for HP HARQ-ACK.

After determination of QHP-ACK' and QLP-ACK', we should decide which REs in PUSCH are used for HP HARQ-ACK and LP HARQ-ACK. Rather than introducing a new RE mapping rules in PUSCH, we can reuse Rel-15 RE mapping rules as much as possible.

- 1st step) the UE select QACK' RE positions for HP HARQ-ACK and LP HARQ-ACK by using Rel-15 RE mapping rules with QACK'=QHP-ACK'+QLP-ACK'.

- 2nd step) the UE select QHP-ACK' RE positions for HP HARQ-ACK among the RE positions determined in the 1st step. The remaining QLP-ACK' RE positions are used for LP HARQ-ACK.

In the 2nd step, there are several ways to select QHP-ACK' RE positions for HP HARQ-ACK among the RE positions determined in the 1st step. To minimize latency of HP HARQ-ACK, it would be better to select earlier symbols. To increase reliability of HP HARQ-ACK, it would be better to select earlier symbols also because the earlier symbol is the nearest to front-loaded DMRS symbol. In addition, to obtain frequency diversity sufficiently, the REs for HP HARQ-ACK can be selected from the RE positions determined in the 1st step by considering subcarrier index of REs.

- Proposal 9: To multiplex LP HARQ-ACK and HP HARQ-ACK into a PUSCH in case of UCI on PUSCH, reuse Rel-15 rules as much as possible.

  - The number of REs for LP HARQ-ACK and HP HARQ-ACK is determined by Rel-15 RE calculation rules for HARQ-ACK.

  - RE positions for LP HARQ-ACK and HP HARQ-ACK in a PUSCH are determined by Rel-15 RE mapping rules for HARQ-ACK.

Configuration of beta offset and UL-DAI

Figure 3. Multiplexing UCIs with different priorities in a PUSCH

At the RAN1#102-e meeting, it was agreed to support multiplexing of HP HARQ-ACK on PUCCH with LP-PUSCH (with UL-SCH + A/N and/or CSI), and multiplexing of LP HARQ-ACK on PUCCH with HP-PUSCH (with UL-SCH + A/N and/or CSI). Consequently, HP/LP-PUSCH may contain LP HARQ-ACK and HP HARQ-ACK.

In this case, we need to answer the following questions.

- Q1) How to indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed?

- Q2) How to indicate “beta offset” for LP HARQ-ACK and/or HP HARQ-ACK?

Regarding Q1, the presence of HARQ-ACK information is determined by UL-DAI value in a DCI format scheduling PUSCH transmission. In case of Type-1 HARQ-ACK codebook, 1-bit UL DAI value indicates the presence of Type-1 HARQ-ACK codebook. In case of Type-2 HARQ-ACK codebook, 2-bit UL DAI value indicates the presence and size of Type-2 HARQ-ACK codebook. The case where LP HARQ-ACK and HP HARQ-ACK are multiplexed with a PUSCH, at least two UL-DAI values should be contained in a DCI format scheduling the PUSCH. For example, when LP-UCI is configured with Type-1 HARQ-ACK codebook and HP-UCI is configured with Type-2 HARQ-ACK codebook, the DCI format scheduling PUSCH transmission should contain two UL-DAI values, one is 1-bit UL DAI for LP-UCI and the other is 2-bit UL DAI for HP-UCI.

Regarding Q2, two beta offset indicators are necessary in a DCI format scheduling PUSCH transmission similar to Q1 case. Here, one beta offset indicator indicates the beta offset value for LP-UCI and the other beta offset indicator indicates the beta offset value for HP-UCI. Without two beta offset indicators, the UE cannot dynamically determine the number of modulation symbols for LP HARQ-ACK and HP HARQ-ACK. For example, if a single beta offset indicator is indicated by the DCI format scheduling PUSCH transmission, then the UE can dynamically determine the number of modulation symbols for one priority and semi-statically determine the number of modulation symbols for another priority.

- Proposal 10: In case of HP-PUSCH or LP-PUSCH contains LP HARQ-ACK and HP HARQ-ACK, it should be discussed how to indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed and “beta offset” for LP HARQ-ACK and/or HP HARQ-ACK.

- Conclusion

In this contribution, intra-UE multiplexing and prioritization were discussed, and the followings were proposed:

- Proposal 1. To determine a code rate for LP HARQ-ACK for multiplexing, consider the following two options.

  - Option 1. Additional code rate for LP HARQ-ACK in a HP PUCCH format is configured by RRC.

  - Option 2. Code rate configured in LP PUCCH format is reused.

- Proposal 2: Further discuss whether/how to multiplex HP HARQ-ACK and LP HARQ-ACK in a PUCCH resource if the second PUCCH-Config contains only the first PUCCH resource (for 1- or 2-bit HARQ-ACK information).

- Proposal 3: For PUCCH format 3/4,

  - To maximize reliability of HP HARQ-ACK, the mapping rule of PUCCH format 3/4 in Rel-15 can be reused., i.e., the HP HARQ-ACK is mapped to adjacent symbols to DMRS symbols.

  - To minimize latency of HP HARQ-ACK, the HP HARQ-ACK can be mapped to earlier symbols.

- Proposal 4: For PUCCH format 2,

  - Support PUCCH format 2 to multiplex LP HARQ-ACK and HP HARQ-ACK

  - To maximize a reliability of HP HARQ-ACK, the HP HARQ-ACK is distributed to REs across RBs as much as possible.

- Proposal 5: If the required # of RBs for low-priority HARQ-ACK information exceeds the limit of PUCCH formats, then bundle the low-priority HARQ-ACK information. Detail bundling rules should be further discussed in Rel-17 URLLC/IIoT WI.

- Proposal 6: To multiplex HP-SR with PF0 and LP HARQ-ACK with PF0, we propose

  - If HP-SR is negative, then transmit LP HARQ-ACK on HARQ-ACK resource.

    - In case of 1-bit LP HARQ-ACK, use 2 CSs, i.e., {0, 6} CS index

    - In case of 2-bit LP HARQ-ACK, use 4 CSs, i.e., {0, 3, 6, 9} CS index

  - If HP-SR is positive, then transmit LP HARQ-ACK and HP-SR on HARQ-ACK resource

    - In case of 1-bit LP HARQ-ACK, use 2 CSs, i.e., {3, 9} CS index

    - In case of 2-bit LP HARQ-ACK, use 4 CSs, i.e., {1, 4, 7, 11} CS index

      - To enhance HP-SR reliability, 2-bit LP HARQ-ACK can be bundled to 1-bit and then the 1-bit bundled LP HARQ-ACK is treated as 1-bit LP HARQ-ACK, i.e., use 2 CSs, {3, 9} CS index .

- Proposal 7: We propose to support Option 2b for multiplexing with HP-SR with PF0 and LP HARQ-ACK with PF1.

  - To multiplex HP-SR with PF0 and LP HARQ-ACK with PF1, use the HARQ-ACK resource.

    - Applying QPSK for SR+1-bit HARQ-ACK. For the case of 2-bit HARQ-ACK, the HARQ-ACK is reduced/compressed to 1-bit.

- Proposal 8: To multiplex HP-SR with PF1 and LP HARQ-ACK with PF0, reuse multiplexing rule for HP-SR with PF0 and LP HARQ-ACK with PF0.

- Proposal 9: To multiplex LP HARQ-ACK and HP HARQ-ACK into a PUSCH in case of UCI on PUSCH, reuse Rel-15 rules as much as possible.

  - The number of REs for LP HARQ-ACK and HP HARQ-ACK is determined by Rel-15 RE calculation rules for HARQ-ACK.

  - RE positions for LP HARQ-ACK and HP HARQ-ACK in a PUSCH are determined by Rel-15 RE mapping rules for HARQ-ACK.

- Proposal 10: In case of HP-PUSCH or LP-PUSCH contains LP HARQ-ACK and HP HARQ-ACK, it should be discussed how to indicate the presence of LP HARQ-ACK and/or HP HARQ-ACK to be multiplexed and “beta offset” for LP HARQ-ACK and/or HP HARQ-ACK.

References

- Final Report of 3GPP TSG RAN WG1 #104-e v1.0.0

- Final Report of 3GPP TSG RAN WG1 #104b-e v1.0.0

- Draft Report of 3GPP TSG RAN WG1 #105-e v0.2.0