3GPP TSG RAN WG1 #106-e			R1-2108349

e-Meeting, August 16th – 27th, 2021

Source:	Intel Corporation, Qualcomm, NTT DOCOMO, Ericsson, Bosch

Title:	Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case

Agenda item:	8.16

Document for:	Discussion and Decision

Introduction

Release 16 NR introduced support of sidelink V2X communication for unicast, groupcast, and broadcast V2X traffic services. One of the main differences of NR comparing to LTE sidelink is physical layer support of HARQ feedback and retransmissions. HARQ feedback is realized in two regimes: ACK-NACK information and NACK-only information, where the latter is assumed to be used for connection-less groupcast mode using communication range information.

During the normative and maintenance work on specification of NACK-only feedback regime it was identified that NACK-only groupcast regime is susceptible to half-duplex collision on sidelink control channel which could limit the achievable performance even at very short distances, i.e. on high SNR links [1]-[4].

In this TEI proposal, the technical issue of half-duplex collision in NACK-only HARQ feedback regime is first illustrated by evaluation and then proposed to be solved by a limited specification work.

# Motivation

In V2X sidelink resource allocation Mode-1 and Mode-2, UEs can use groupcast transmissions with NACK only feedback. In this mode of operation, receivers within target communication range from the transmitter provide NACK feedback in case of unsuccessful reception and do not provide ACK in case of successful reception. The susceptibility to half-duplex collisions can be illustrated by the following simple example:

- UE1, UE2, and UE3 operate with groupcast NACK only feedback and are group members (i.e., within target communication range from each other).

- UE1 and UE2 selected/were granted with resources in the same slot and transmitted in slot ‘n’.

- UE3 has successfully received UE1 and UE2 transmissions and thus has not provided HARQ feedback.

- UE1 and UE2 were not able to receive each other transmissions.

- Due to lack of NACK feedback UE1 and UE2 assume successful reception by UEs within target communication range.

  - In Mode-1, the UE reports ACK to gNB, and gNB considers successful transmission on SL, thus do not grant retransmissions

  - In Mode-2, the UE reports ACK to higher layer, and the higher layer does not grant retransmissions

- UE1 and UE2 stop transmissions of TBs without receiving each other transmissions.

The above problem was also confirmed by system level evaluations [1][3][4]. Figure 1 shows comparative analysis of the current Rel.16 design vs scenario when two blind retransmissions are used for groupcast communication with NACK only feedback according assumptions listed in Annex. As it can be seen, the Rel.16 solution does not achieve PRR equal to 1 even at short communication distances.

Figure 1: Illustration of the Rel.16 groupcast communication with NACK only feedback

Since the half-duplex collision is a fundamental issue in distributed communication systems, it may not be possible to completely avoid it. But it is possible to apply a simple enhancement which reduces the issue dramatically. Such an enhancement is to allow a UE to transmit at least two TB (re-)transmissions without considering the feedback, thus increasing the chances that at least one of the two control channels were successfully received, as illustrated in the analysis above.

Observation 1

- Rel-16 groupcast sidelink communication with NACK-only is susceptible to half-duplex issue which could limit the achievable reliability even at very high SNR links

# Possible Solutions

To mitigate the illustrated half-duplex problem for groupcast NACK only sidelink communication, there could be different solutions with difference spec impact, as was analyzed in the previous submission [5]. The following options were considered in descending order of spec impact / generalization:

- Option 1: Introduce a configurable minimum number of blind retransmissions N and support of mixing blind retransmissions and feedback-based retransmissions for a TB in MAC procedures

- Option 2: Introduce a configurable minimum number of retransmissions N performed w/o considering the feedback from receivers

- Option 3: Introduce a fixed number of minimum two retransmissions and support of mixing blind retransmissions and feedback-based retransmissions for a TB in MAC procedures

- Option 4: Introduce a fixed number of minimum two retransmissions performed w/o considering the feedback from receivers

All the above options are backward compatible with Release-16 receivers in the same resource pool. However, options 1 and 3 require potentially substantial MAC specification changes to introduce the behavior of mixed blind and feedback-based transmission for the same TB.

At the same time, options 2 and 4 completely avoid MAC spec impact, and only require new RRC fields to enable and disable the new behavior. Additionally, option 2 provides more control and flexibility by introducing the number of blind (re-)transmission a UE can start its TB transfer. A simple sketch of option 2/4 is illustrated in Figure 2. Note, we don’t consider this kind of RRC changes as a cross-WG impact violating TEI conditions, since any new TEI work should have the RRC signaling of enabling/disabling the feature.

Figure 2: Illustration of Option 2 and 4 for mitigation of half-duplex for NACK-only feedback regime

The following observation is highlighted regarding option 1 and 3:

Observation 2

- The most flexible way to mitigate the half-duplex issue for sidelink V2X communication in groupcast NACK-only feedback regime is to introduce an explicit mix of blind and feedback-based HARQ retransmissions, that however has a substantial cross-WG impact to MAC specification

Observation 3

- Options 2 and 4 do not have MAC specification impact and provide similar result as Option 1 and 3, thus may be recommended for further work in R17 TEI

Considering the above observations, it is suggested to focus on Option 2 or 4 for further work in this TEI. The example text proposals can be found in section 4:

Proposal

- Agree on Release 17 TEI work to introduce mitigation of half-duplex issue for sidelink V2X communication in groupcast NACK-only feedback regime by introducing a minimum number of retransmissions performed without considering the feedback from receivers

  - This functionality is enabled/disabled by (pre-)configuration

# Text Proposal to TS 38.213

Except RRC enabling / disabling of the feature, there are potentially two places that require changes, and both are in TS 38.213 section 16. The first change is to introduce the behaviour of ignoring the feedback contents when needed in Mode-2 operation, and the second change is to modify the reported feedback to gNB in Mode-1. The text proposal is based on Option 4, but it can be easily extended to Option 2 if needed.

### 16.3.1 UE procedure for receiving HARQ-ACK on sidelink A UE that transmitted a PSSCH scheduled by a SCI format 2-A or a SCI format 2-B that indicates HARQ feedback enabled, attempts to receive associated PSFCHs according to PSFCH resources determined as described in clause 16.3. The UE determines an ACK or a NACK value for HARQ-ACK information provided in each PSFCH resource as described in [10, TS 38.133]. The UE does not determine both an ACK value and a NACK value at a same time for a PSFCH resource. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report to higher layers. For generating the HARQ-ACK information, the UE can be indicated by a SCI format to perform one of the following - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - report to higher layers HARQ-ACK information with same value as a value of HARQ-ACK information that the UE determines from the PSFCH reception - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - report an ACK value to higher layers if the UE determines an ACK value from at least one PSFCH reception occasion from the number of PSFCH reception occasions in PSFCH resources corresponding to every identity MID of UEs that the UE expects to receive corresponding PSSCHs as described in Clause 16.3; otherwise, report a NACK value to higher layers - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is not the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one or the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is equal to one, report to higher layers an ACK value if the UE determines absence of PSFCH reception for the PSFCH reception occasion; otherwise, report a NACK value to higher layers - if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one, report to higher layers a NACK value

## 16.5 UE procedure for reporting HARQ-ACK on uplink A UE can be provided PUCCH resources or PUSCH resources [12, TS 38.331] to report HARQ-ACK information that the UE generates based on HARQ-ACK information that the UE obtains from PSFCH receptions, or from absence of PSFCH receptions. The UE reports HARQ-ACK information on the primary cell of the PUCCH group, as described in clause 9, of the cell where the UE monitors PDCCH for detection of DCI format 3_0. For SL configured grant Type 1 or Type 2 PSSCH transmissions by a UE within a time period provided by sl-PeriodCG, the UE generates one HARQ-ACK information bit in response to the PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource, in a set of time resources. For PSSCH transmissions scheduled by a DCI format 3_0, a UE generates HARQ-ACK information in response to PSFCH receptions to multiplex in a PUCCH transmission occasion that is after a last time resource in a set of time resources provided by the DCI format 3_0. For each PSFCH reception occasion, from a number of PSFCH reception occasions, the UE generates HARQ-ACK information to report in a PUCCH or PUSCH transmission. The UE can be indicated by a SCI format to perform one of the following and the UE constructs a HARQ-ACK codeword with HARQ-ACK information, when applicable - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "10" - generate HARQ-ACK information with same value as a value of HARQ-ACK information the UE determines from a PSFCH reception in the PSFCH reception occasion and, if the UE determines that a PSFCH is not received at the PSFCH reception occasion, generate NACK - if the UE receives a PSFCH associated with a SCI format 2-A with Cast type indicator field value of "01" - generate ACK if the UE determines ACK from at least one PSFCH reception occasion, from the number of PSFCH reception occasions, in PSFCH resources corresponding to every identity MID of the UEs that the UE expects to receive the PSSCH, as described in Clause 16.3; otherwise, generate NACK - if the UE receives a PSFCH associated with a SCI format 2-B or a SCI format 2-A with Cast type indicator field value of "11" - if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is not the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one or the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is equal to one, generate ACK when the UE determines absence of PSFCH reception for each PSFCH reception occasion from the number of PSFCH reception occasions; otherwise, generate NACK - if the transmitted PSSCH scheduled by the SCI format 2-A or SCI format 2-B is the initial transmission and the maximum number of retransmissions for the TB provided by higher layers is larger than one, generate a NACK value

# Conclusions

In this contribution, we have shown a technical problem of NR sidelink groupcast communication with NACK only feedback and have shown simple changes that can address it with minimal changes in the Rel.16. Based on discussion we have the following proposal:

Proposal

- Agree on Release 17 TEI work to introduce mitigation of half-duplex issue for sidelink V2X communication in groupcast NACK-only feedback regime by introducing a minimum number of retransmissions performed without considering the feedback from receivers

  - This functionality is enabled/disabled by (pre-)configuration

References

- R1-1910650, “Resource Allocation Mode-2 for NR V2X Sidelink Communication”, Intel Corporation, October 2019 - R1-2103766, “Correction to PSFCH reception procedure for NACK-only case to mitigate half-duplex issue”, Intel Corporation, April 2021 - R1-2100673, “On Feasibility and Benefits of Inter-UE Coordination for Sidelink Mode-2 Design”, Intel Corporation, January 2021

- R1-2101910, “Reliability and Latency Enhancements for Mode 2”, Qualcomm Incorporated, January 2021 - R1-2105778, “Rel-17 TEI proposal for mitigating half-duplex issue in NR V2X groupcast NACK-only case regime”, Intel Corporation, Qualcomm Inc.

Annex: Evaluation Assumptions

In this section, we provide the list of evaluation assumptions used for analysis in this contribution. The set of main parameters used for NACK only evaluation assumptions is provided in Table 1.

Table 1: System level evaluation assumptions for NACK only groupcast communication studies

| Parameter | Value |
| --- | --- |
| Deployment scenario | - Highway Option A scenario from NR V2X methodology - Vehicle speed = 70 km/h |
| Channel model | TR 37.885, NR V2X Channel Model |
| Spectrum allocation | Carrier frequency: 6GHz Simulated Bandwidth: 40 MHz |
| Subcarrier spacing | 30 kHz |
| Tx/Rx Antenna Ports | 1Tx/2Rx |
| Communication Mode(s) | Groupcast, connection-less |
| UE-to-UE Association | - UE is considered as member of the group based on location principle - Each UE may be a member of a multiple groups |
| Traffic model | Aperiodic variable packet size traffic (TR 37.885 Aperiodic Model 1 traffic): - Packet size: uniform in the range [200..2000] B with quantization step of 200 B - Inter-packet arrival time: 50 ms + an exponential random variable with the mean of 50 ms - Latency requirement: 50 ms |
| TTI structure | NR Slot TTI: - Slot w/o PSFCH zone: 1 AGC symbol, 1 GAP symbol, 2 PSSCH symbols w/ pilots - Slot w/ PSFCH zone: 1 AGC symbol, 2 GAP symbol, 2 PSSCH symbols w/ pilots, 2 symbols for PSFCH |
| PSFCH Zone Allocation | Enabled, in every 2nd slot |
| Sidelink control TX parameters | 64 Bits QPSK Modulation |
| SCI/Data frequency resource allocation | - PSCCH: 10 PRB - PSSCH: 35 PRB |
| SCI/Data time resource allocation | - PSCCH: 3 Symbols - PSFCH: 2 Symbols - PSSCH: Remaining symbols in slot |
| Data Packet Tx parameters | Aperiodic variable packet size evaluations: - 200 B packet: QPSK, 2 TTI - 400 B packet: 16-QAM, 2 TTI - 600 B packet: 16-QAM, 2 TTI - 800 B packet: 16-QAM, 2 TTI - 1000 B packet: 16-QAM, 3 TTI - 1200 B packet: 16-QAM, 3 TTI - 1400 B packet: 64-QAM, 3 TTI - 1600 B packet: 64-QAM, 3 TTI - 1800 B packet: 64-QAM, 4 TTI - 2000 B packet: 64-QAM, 4 TTI |
| HARQ Feedback | Enabled, NACK-only |
| Feedback reporting distance | UEs within 200m report HARQ feedback |