3GPP TSG RAN WG1 Meeting #106-e	R1-21xxxxx

E-Meeting, August 16th – August 27th, 2021

Agenda Item:	6

Source:	Moderator (Huawei)

Title:	Feature lead summary on 106-e-LTE-6CRs-02

Document for:	Discussion and Decision

Introduction

This documents provides the summary of discussions on the corresponding email discussion, regarding the proposed CR in [1].

[106-e-LTE-6CRs-02] Email discussion/approval on Correction on cyclic shift for eMTC PUR (R1-2107688) – Yubo (Huawei)

- Issue 3: Correction on cyclic shift for eMTC PUR

- Discussion and decision by August 18, CR by August 20, final check by August 24

Discussion

In [1], a correction on cyclic shift for eMTC PUR is proposed due to the following reason:

| Reason for change: | The parameter pusch-CyclicShift in higher layer parameter PUR-PUSCH-Config is intended to be used in PUSCH corresponding to preconfigured uplink resource. However, in the current spec, it is not limited to PUR PUSCH. |
| --- | --- |
| Summary of change: | It is clarified that the parameter pusch-CyclicShift in higher layer parameter PUR-PUSCH-Config is for PUSCH corresponding to preconfigured uplink resource. |
| Consequences if not approved: | The parameter pusch-CyclicShift in higher layer parameter PUR-PUSCH-Config will also be used in non-PUR PUSCH mistakenly, such as msg3 etc. |

The proposed change is as following:

=======================Change to TS 36.211=====================================

<Unchanged parts are omitted>

##### 5.5.2.1.1	Reference signal sequence

<Unchanged parts are omitted>

The cyclic shift  in a slot  is given as if the ul-V-SPS-RNTI-r14 was used to transmit the most recent uplink-related DCI for the transport block associated with the corresponding PUSCH transmission. For PUSCH transmissions not using sub-PRB allocations, if pusch-CyclicShift in higher layer parameter PUR-PUSCH-Config is configured it provides the value of ncs,λ and the cyclic shift  in a slot  is given as for PUSCH (re)transmission corresponding to preconfigured uplink resource.

<Unchanged parts are omitted>

======================End of change to TS 36.211=================================

Please input your comment on the motivation and CR above:

| Companies | Comments |
| --- | --- |

Summary

- References

- R1-2107688	Correction on cyclic shift for eMTC PUR	Huawei, HiSilicon