3GPP TSG RAN WG1 #106-bis-e		      R1-2109117

e-Meeting, October 11th – 19th, 2021

Source:	NEC

Title:	Discussion on PDCCH monitoring enhancements supporting NR from 52.6GHz to 71 GHz

Agenda Item:	8.2.2

Document for:	Discussion and Decision

Introduction

At the previous RAN1#106-e meeting, the following agreement has been made [1],

Agreement:

For reporting the multi-slot PDCCH monitoring capability, at least the following values are supported:

- X=4 slots for SCS 480 kHz

- X=8 slots for SCS 960 kHz

Agreement:

- Revise Alt 1 in previous agreement to the following (this agreement does not select Alt. 1):

- Alt 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive slots in each slot group separately

  - Further discuss down-selection of Y within 1<=Y<=X/2 (both in units of slot) when X>1

  - FFS: Supported values/constraints of X and Y, e.g. Y<=X, Y=X

  - FFS: Restrictions on location of the Y slots within a slot group, e.g. the Y slots always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- FFS: What the UE capability defines for monitoring within the Y slots

Agreement:

Revise prior agreement including modifications to Alt. 1 as follows:

- Alt. 1: Use a fixed pattern of slot groups as the baseline to define the new capability.

  - Each slot group consists of X slots

  - Slot groups are consecutive and non-overlapping

  - The capability indicates the BD/CCE budget within Y consecutive slots in each slot group separately

    - The location of the Y slots within the X slots is maintained across different slot groups

  - Further discuss down-selection of Y within 1<=Y<=X/2 (both in units of slot) when X>1

  - FFS: Restrictions on location of the Y slots within a slot group, e.g. the Y slots always start at the first slot within a slot group

  - FFS: Further definition of capabilities

- FFS: The following issues for the search space configuration discussion

  - Whether a slot group is aligned with a slot boundary

  - Restrictions on location of the Y slots within a slot group, e.g. whether to restrict the location of a SS to be within the first Y slots within a slot group

- FFS: What the UE capability defines for monitoring within the Y slots

Agreement:

- A UE supporting 480 kHz SCS supports multi-slot PDCCH monitoring for 480 kHz SCS.

- A UE supporting 960 kHz SCS supports multi-slot PDCCH monitoring for 960 kHz SCS.

- FFS: whether to apply multi-slot PDCCH monitoring at all times and for all search spaces.

In this contribution, we would like to share our views on PDCCH monitoring enhancements on supporting NR from 52.6GHz to 71 GHz.

Discussion

  - Monitoring capability

For 480 kHz and 960 kHz SCS adopted beyond 52.6GHz, In the WID [3], PDCCH monitoring enhancement with multi-slot span is supported, it can maintain scheduling framework same as for smaller SCS (e.g. 120 kHz) when UE is configured to monitor the PDCCH every multiple slots. There are 3 alternatives for defining the multi-slot PDCCH monitoring capability. In previous meeting [1], there were many discussions on Alt1 (fixed pattern) due to its simplicity, but with fixed X and Y location, there may be some issues when monitoring Type 0/0A/1(without dedicated RRC configuration)/2 PDCCH CSS, how they could fit within the framework of Alt1? In our view, with fixed X and Y location, for example, Y slots always start at the first slot within a slot group, UE need to monitor the above CSS in extra slots other than Y if they don’t fall into Y slots, and in order to avoid the back-to-back overload issue, an extra window can be considered to calculate the PDCCH monitoring burden. In this extra window with size equivalent to X, the monitoring capability indicates the BD/CCE budget within the whole window.

Figure 1

Proposal 1: For Alt1 with fixed Y location, an extra window can be considered to calculate the PDCCH monitoring burden if Type 0/0A/1(without dedicated RRC configuration)/2 PDCCH CSS don’t fall into Y monitoring slots.

In previous meeting [1], it was agreed that at least the following values are supported: 4 slots for 480 kHz SCS and 8 slots for 960 kHz SCS, and other specific number of the multiple slots is in discussion. For some use cases such as low-latency services which require more frequent PDCCH monitoring, the flexibility will be reduced with the multi-slot based monitoring. To handle those use cases with low-latency, denser PDCCH monitoring occasion should be considered to support, such as per-slot, per 2-slots based monitoring for 480 kHz SCS and per 2-slots, per 4-slots based monitoring for 960 kHz SCS, and accordingly the associated BD/CCEs limit number needs to further study.

Proposal 2: Additional PDCCH monitoring group sizes should be supported: 1 or 2 for 480 kHz SCS, 2 or 4 for 960 kHz SCS, and further study the associated BD/CCEs limit number.

2.2     Search space set group switching

In TS 38.213 [2], search space set group switching is introduced for NR-U in Rel-16, and it’s used for UE to save power by switching from more frequent PDCCH monitoring to less frequent in a COT. There are 2 search space set groups for switching, one is dense monitoring periodicity and the other is sparse. Switching is in an active BWP and based on explicit or implicit switching indication, and the monitoring capability keeps the same before and after the switch.

For operation in unlicensed band beyond 52.6GHz, in our understanding, the current SSSG switching can be reused for 120 kHz SCS, since it has been concluded: for 120 kHz SCS, no multi-slot UE capability for PDCCH monitoring is needed. So the monitoring capability before and after the switch is the same, both are per-slot based. While for 480 kHz and 960 kHz SCS, PDCCH monitoring capability may be changed along with SSSG switching, e.g. there are 2 configured SSSG, the first search space set defines PDCCH is monitored per 2-slots, and the second set defines PDCCH is monitored per 4-slots, the monitoring time unit and capability is different before and after the switching. If this scenario is supported, how to adapt it to R16 SSSG switching needs to be discussed.

Proposal 3: For operation in unlicensed band with 480 kHz and 960 kHz SCS, consider whether/how to support SSSG switching along with changing different PDCCH monitoring capability.

As mentioned in [4], in R16, the switching boundary is the first slot that is at least Pswitch symbols after some switch indication and the timer decrement value is counted by slot. For 480 kHz and 960 kHz SCS, PDCCH monitoring capability is multi-slot based, e.g. 4 slots for 480 kHz SCS and 8 slots for 960 kHz SCS, the switching boundary and the timer counter should be modified to multi-slot based accordingly.

Proposal 4: For operation in unlicensed band with 480 kHz and 960 kHz SCS, the switching boundary and the timer counter should be modified to slot group based.

Currently, Pswitch, which means the SSSG switching time, is defined for SCS configuration 
μ = 0,1,2. For new SCSs adopted beyond 52.6GHz, to operate in unlicensed band, SSSG switching time  Pswitch should be defined and added in the table 10.4-1 of TS 38.213[2].

Proposal 5: Search space set group switching time Pswitch should be defined for new SCSs.

For 480 kHz and 960 kHz SCS, if PDCCH monitoring capability is based on per multi-slot, the UE will decode more DCI than per slot monitoring, the processing time will last longer, and it will have an influence in SSSG switching time estimation. So when we estimate minimum value of Pswitch, which monitoring capability is the reference, per slot or per multi-slot? It should be determined.

Proposal 6: For 480 kHz and 960 kHz SCS, which monitoring capability is the reference to estimate the search space set group switching time Pswitch should be determined.

Conclusion

In this contribution, we share our views on PDCCH monitoring enhancements on supporting NR from 52.6GHz to 71 GHz, and the following proposals are made:

Proposal 1: For Alt1 with fixed Y location, an extra window can be considered to calculate the PDCCH monitoring burden if Type 0/0A/1(without dedicated RRC configuration)/2 PDCCH CSS don’t fall into Y monitoring slots.

Proposal 2: Additional PDCCH monitoring group sizes should be supported: 1 or 2 for 480 kHz SCS, 2 or 4 for 960 kHz SCS, and further study the associated BD/CCEs limit number.

Proposal 3: For operation in unlicensed band with 480 kHz and 960 kHz SCS, consider whether/how to support SSSG switching along with changing different PDCCH monitoring capability

Proposal 4: For operation in unlicensed band with 480 kHz and 960 kHz SCS, the switching boundary and the timer counter should be modified to slot group based

Proposal 5: Search space set group switching time Pswitch should be defined for new SCSs.

Proposal 6: For 480 kHz and 960 kHz SCS, which monitoring capability is the reference to estimate the search space set group switching time Pswitch should be determined.

References

- 3GPP RAN1#106-e Chairman’s notes

- 3GPP TS 38.213 V16.4.0

- RP-202925, Extending current NR operation to 71GHz

[4]      R1-2103097, Discussion on PDCCH Enhancements for above 52.6 GHz, Apple Inc.